3GPP TSG-SA WG6 Meeting #69	S6-254247

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0328 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of Update API invoker's API list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Update API List service operation is missing: - Clause 8.26.3, triggering of revocation authorization procedure, when applicable. - In clause 10.1. - In clause 10.5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Completion of clause 8.26.3. Inclusion of the updae API list service operation in clauses 10.1 and 10.5.2a clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorization revocation procedure is not triggered for disenrolled service APIs. The Update API List service operation is not fully specified in TS 23.222. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.26.3, 10.1, 10.5.2a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.26.3	Procedure

Figure 8.26.3-1 illustrates the procedure for updating the API invoker API list on the CAPIF.

Pre-conditions:

1.	The API invoker has been onboarded as a recognized user of the CAPIF and associated API invoker profile is provisioned.

2.	The API invoker has visibility to new APIs information (e.g. updates on API catalogue or dashboard, API discovery).

Figure 8.26.3-1: Procedure for updating the API invoker profile on the CAPIF

1.	For updating of the API invoker API list on the CAPIF, the API invoker triggers update API invoker API list request towards the CAPIF core function, providing the information to be updated (e.g. enroll new APIs, disenroll APIs).

2.	The CAPIF core function updates the API invoker API list of the requesting API invoker, according to the grant from the CAPIF administrator or the API management. For the disenrolled API(s), the authorization corresponding to the API invoker is revoked from CAPIF, if available.

NOTE 1:	Completion of updating process can require explicit grant by the CAPIF administrator or the API management, which is left out-of-scope of this solution. CAPIF can handle the grant process internally without the need of explicit grant by the CAPIF administrator.

3.	The update API invoker API list response provides a success or failure indication. When not all the APIs included in the request are enrolled, the success result include information about the APIs that the API invoker can access. When the update status is failure, the reason for failure is included.

* * * Second Change * * * *

## 10.1	General

Table 10.1-1 illustrates the CAPIF core function APIs.

Table 10.1-1: List of CAPIF core function APIs

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| CAPIF_Open_Discover_Service_API | Open_Discover_Service_API | Open | Request/Response |
| CAPIF_Discover_Service_API | Discover_Service_API | API Invoker, CAPIF core function | Request/ Response |
|  | Subscribe_Event | API Invoker | Request/ Response |
|  | Update_Event_Subscription | API Invoker | Request/ Response |
|  | Notify_Event | API Invoker | Notify |
|  | Unsubscribe_Event | API Invoker | Request/ Response |
| CAPIF_Publish_Service_API | Publish_Service_API | API Publishing Function, CAPIF core function | Request/ Response |
|  | Unpublish_Service_API | API Publishing Function, CAPIF core function | Request/ Response |
|  | Update_Service_API | API Publishing Function, CAPIF core function | Request/ Response |
|  | Get_Service_API | API Publishing Function, CAPIF core function | Request/ Response |
|  | Subscribe_Event | API Publishing Function | Request/ Response |
|  | Update_Event_Subscription | API Publishing Functionr | Request/ Response |
|  | Notify_Event | API Publishing Function | Notify |
|  | Unsubscribe_Event | API Publishing Function | Request/ Response |
| CAPIF_Events API | Subscribe_Event | API Invoker, API Publishing Function, API Management Function, API Exposing Function | Request/ Response |
|  | Update_Event_Subscription | API Invoker, API Publishing Function, API Management Function, API Exposing Function | Request/ Response |
|  | Notify_Event | API Invoker, API Publishing Function, API Management Function, API Exposing Function | Notify |
|  | Unsubscribe_Event | API Invoker, API Publishing Function, API Management Function, API Exposing Function | Request/ Response |
| CAPIF_API_Invoker_management API | Onboard_API_Invoker | API Invoker | Request/ Response |
|  | Offboard_API_Invoker | API Invoker | Request/ Response |
|  | Update_API_List | API Invoker | Request/ Response |
|  | Subscribe_Event | API Management Function | Request/ Response |
|  | Update_Event_Subscription | API Management Function | Request/ Response |
|  | Notify_Event | API Management Function | Notify |
|  | Unsubscribe_Event | API Management Function | Request/ Response |
| CAPIF_API_Provider_Management_API | Register_API_Provider | API Management Function | Request/Response |
|  | Update_API_Provider | API Management Function | Request/Response |
|  | Deregister_API_Provider | API Management Function | Request/Response |
| CAPIF_Security API | Obtain_Security_Method | API Invoker | Request/ Response |
|  | Obtain_Authorization | API Invoker | Request/ Response |
|  | Obtain_API_Invoker_Info | API Exposing Function | Request/ Response |
|  | Revoke_Authorization | API Exposing Function | Request/ Response |
|  | Revoke_Authorization_Notify | API Invoker | Notify |
| CAPIF_Monitoring API | Subscribe_Event | API Management Function | Request/ Response |
|  | Update_Event_Subscription | API Management Function | Request/ Response |
|  | Notify_Monitoring_Service_Event | API Management Function | Notify |
|  | Unsubscribe_Event | API Management Function | Request/ Response |
| CAPIF_Logging_API_Invocation API | Log_API_Invocation | API exposing function | Request/ Response |
| CAPIF_Auditing API | Query_API_Invocation_Log | API management function | Request/ Response |
| CAPIF_Access_Control_Policy API | Obtain_Access_Control_Policy | API exposing function | Request/Response |
| CAPIF_Routing_Info API | Obtain_Routing_Info | API exposing function | Request/Response |

* * * Third Change * * * *

### 10.5.2a	Update_API_List operation

API operation name: Update_API_List

Description: Enrolls/disenrolls service APIs from the API invoker profile.

Known Consumers: API invoker.

Inputs: Refer subclause 8.26.2.1.

Outputs: Refer subclause 8.26.2.2.

See subclause 8.26.3 for the details of usage of this API operation.

* * * End of Change * * * *