3GPP TSG-SA WG6 Meeting #69	S6-254249

Wuhan, China 13th – 17th October 2025	(revision of S6-254xyz)

Source:	Ericsson

Title:	Pseudo-CR on solution for KI#1, CAPIF Administrator revocation of API Invoker enrollment information.

Spec:	3GPP TR 23.700-43 v0.1.0

Agenda item:	9.11

Document for:	Approval

Contact:	Fuencisla Garcia (fuencisla.garcia@ericsson.com)

1. Introduction

This pCR provides a solution for KI#1 for TR 23.700-43.

2. Reason for Change

KI#1 identifies several use cases where the API invoker may have changed its status in the API provider domain after onboarding in the CAPIF provider domain. E.g., the API invoker decides to terminate the service agreement with the API provider, or the API Provider decides to terminate the service agreement with the API Invoker because e.g., unpaid bills.

With the current level of specification, the termination of the service agreement between an API Invoker and an API Provider is communicated to the CSP via a BSS/OSS mechanism, and it is not further specified how this termination is communicated to the CAPIF Provider.

This CR proposes a solution where the CAPIF Administrator can update the API Invoker profile according to the API Provider domain provided information, e.g., to update the set of enrolled APIs and remove the API Invoker authorization information from the CCF, as necessary.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.1.0)

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | … |
| --- | --- | --- | --- |
| Sol #1 |  |  |  |
| Sol #2 |  |  |  |
| Sol #X. | X |  |  |

* * * Second Change * * * *

## 6.x	Solution #x: CAPIF Administrator revocation of API Invoker enrollment information

### 6.x.1	General

This solution relates to KI #1.

KI#1 identifies several use cases where the API invoker may have changed its status in the API provider domain after onboarding in the CAPIF provider domain. E.g., the API invoker decides to terminate the service agreement with the API provider, or the API Provider decides to terminate the service agreement with the API Invoker because e.g., unpaid bills.

With the current level of specification, the termination of the service agreement between an API Invoker and an API Provider is communicated to the CSP via a BSS/OSS mechanism, and it is not further specified how this termination is communicated to the CAPIF Provider.

This solution proposes that the CAPIF Administrator can update the API Invoker profile according to the API Provider domain provided information, e.g., to update the set of enrolled APIs and remove the API Invoker authorization information from the CCF, as necessary.

### 6.x.2	Architecture Impacts

There are no architecture impacts.

### 6.x.3	Solution description

#### 6.x.3.0	General

The following changes are proposed to TS 23.222:

- Update of Annex E to include the CAPIF administrator provisioning of per API provider domain and API Invoker service API enrollement information.

- Creation of a new Annex X to describe the handling of the API Invoker profile by the CAPIF Administrator.

##### 6.x.3.0.1	Updates to TS 23.222 Annex E

Annex E (normative):
Configuration data for CAPIF

The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator.

The configuration data for CAPIF is specified in table E-1.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.3.2 | List of service API authorization restrictions per API invoker |
|  | > API Invoker identity information |
|  | > Service API identification |
|  | > Per resource, operation authorization information. (see NOTE 2) |
|  | > Network slice identification |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE 1) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE 1) |
| Subclause 4.10 | List of access control policy (for an API provider domain) per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.1 | Onboarding information per API provider domain and API invoker. > Optionally, enrollment explicit grant > API Invoker Identification > API Provider identification > List of service API |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifier |
|  | >> Purpose |
|  | >> Scope |
| NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]. |  |

##### 6.x.3.0.2	Creation of new Annex to TS 23.222

Annex X (informative):
Onboarding handling by CAPIF Administrator

The CAPIF supports an onboarding procedure that enables an API Invoker to use a list of enrolled service APIs and registers the API Invoker as a recognized user of the CAPIF, an offboarding procedure that removes the API invoker as a recognized user of the CAPIF, and an update the list of enrolled service APIs procedure that updates the list of enrolled service APIs for an API Invoker.

The API Invoker onboarding, offboarding and update of the list of enrolled service APIs may be triggered by the API invoker via CAPIF-1 or CAPIF-1e or may be based on provisioning and triggered by the CAPIF administrator. When an API invoker onboarding procedure is triggered via CAPIF-1 or CAPIF-1e, the completion of the onboarding process may require explicit grant by the CAPIF administrator or the API management.

Whether the API Provider domain exposing the list of enrolled/disenrolled service APIs plays a role in the onboarding process (e.g. the API Invoker can only enrolled service APIs subscribed with the API Provider domain) is out of the scope of the specification. In case the API Provider domain plays a role, the CAPIF supports that the information provided by the API Provider domain can be used to validate an onboarding process or to provision/deprovision onboarding information via the interaction of the CAPIF adminstrator or the API management with CAPIF as shown in table E-1.

#### 6.x.3.1	Information flows

Not applicable.

### 6.x.4	Solution evaluation

This solution proposes that the API Provider domain provided information about the service APIs an API Invoker is allowed to enroll is used by the CAPIF Administrator or API Management to validate and/or update API Invoker onboarding information, e.g., to update the set of enrolled APIs and remove the API Invoker authorization information from the CCF, as necessary. There are no architecture impacts. There are no impacts on CAPIF APIs.

* * * End of Change * * * *

<Proposed change in revision marks>