3GPP TSG-SA WG6 Meeting #69	S6-254268

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0074 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding reference point for spatial map localization for objects and Spatial anchors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA6 agreed to have spatial anchors for mobile object witin spatial map. Further, the localization of the object within spatial map currently provides 3D posision. In order to display the spatial anchor or object wihtin spatial map, a reference point (or point of origin) is required and its refenfence can be used to display the spatial anchor or object. The issue is from spatial map development aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a reference point for the spatial map and spatial anchor (when added within spatial map). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The localization and tracting of the object will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.1, 7.3.3.1, 8.3.1.3.1, 9.3.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.3.2.1	Spatial anchor profile

Table 7.3.2.1-1 describes information elements for the spatial anchor profile stored in the SAn server.

Table 7.3.2.1-1: Spatial anchor profile

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor ID | M | Identifier of the discovered spatial anchor. |
| Position (NOTE 1) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| Spatial map position (NOTE 1) | O | Position information based on spatial map. It includes the identifier of the Spatial map and the corresponding localization information or object ID. |
| > Spatial map ID | O | Identifier of the spatial map |
| >> Spatial map layer ID | O | Identifier of the spatial map layer if Spatial map ID is present |
| >> Reference point | O | Reference point from where the spatial anchor is placed in the spatial map |
| >> Localization (NOTE 2) | O | 3D position within spatial map and pose of the VAL UE |
| >> Object ID (NOTE 2) | O | Identifier of the object on which the spatial anchor is produced. The object can be stationary or trackable. |
| Application specific Spatial anchor information | O | A container to contain application specific information. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > VAL Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| Service area | O | The service area where the spatial anchor is accessible. |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| >Spatial anchor group ID | O | Identifier of spatial anchor group that the spatial anchor associates with |
| Time period | O | Indicates time period or duration or time windows – comprising a start time and an expiry time. A spatial anchor is active during this time period. After the expiry time, the state of a spatial anchor becomes inactive. The time period may include one or more values (e.g. repeated over time/day). |
| Expiry handling indication | O | Indicates whether the spatial anchor has to be deleted or its state to be updated as inactive on expiry by the SAn server. Absence of this IE shall be considered that SAn server deletes the spatial anchor on expiry. |
| Spatial anchor state | O | Indicates the state of the spatial anchor (active or inactive). Spatial anchors in inactive state is not discoverable.Absence of this IE shall be considered as spatial anchor is in active state. |
| NOTE 1: One of the IEs shall be present. NOTE 2: If Spatial map ID is present, either one of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 7.3.3.1	Spatial map information

Table 7.3.3.1-1 describes information elements for the spatial map information stored in the SM server.

Table 7.3.3.1-1: Spatial map information

| Information element | Status | Description |
| --- | --- | --- |
| Spatial map ID | M | The identifier of the spatial map |
| Spatial map coverage | M | The three-dimensional space coordinates information of the spatial map |
| Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map |
| Spatial map media format | O | The media format of the spatial map. |
| Reference point | O | Reference point from where the object is localized within spatial map |
| List of spatial map layers | M | The list of spatial map layer information. At least one or more spatial map layers should be included. Each element includes the information described below. |
| > Spatial map layer ID | M | The identifier of the corresponding spatial map layer. |
| > Spatial map layer information | O | A container to carry the corresponding spatial map layer specific information |
| >> area | O | The area covered by this layer. This area can be the same or a subset of the Spatial map coverage space. |
| >> associated spatial map | O | The details about a spatial map associated with this layer. For example, if the layer is for a particular floor of the building, the detailed spatial map of that particular floor can be associated with this layer. |
| > List of objects | O | List of object information. Each element contains the information as specified in Table 7.3.3.1-2. |

Table 7.3.3.1-2 describes object information details for the spatial map information specified in the Table 7.3.3.1-1.

Table 7.3.3.1-2: Spatial map object information

| Information element | Status | Description |
| --- | --- | --- |
| Object ID | M | The identifier of the corresponding object |
| Object type | M | The type of object |
| Associated UE ID | O | Identifier of the UE associated with the object. |
| Object mobility information | M | Classifies whether the object is stationary or trackable. |
| > Object position | O | The 3D position of the object in the layer. For stationary object, this IE shall be present. For trackable object, 3D position of it shall be provided while it is being tracked, otherwise no value. |
| > Direction (NOTE) | O | Direction of the trackable object while it’s being tracked, otherwise no value. |
| > Velocity (NOTE) | O | Velocity of the trackable object while it’s being tracked, otherwise no value. |
| Object specific information | O | Object specific attribute information |
| NOTE: These IEs may be present when object is moving or changes it’s location frequently. |  |  |

NOTE 1:	Capability of SM server to distinguish whether object is stationary or trackable is outside the scope of this specification.

NOTE 2:	For the current release, the object specific information is implementation specific and out of scope.

Editor's Note: Spatial map management procedures to request object tracking need to be provided.

* * * Next Change * * * *

##### 8.3.1.3.1	Spatial anchor create request

Table 8.3.1.3.1-1 describes information elements for the spatial anchor create request from the SAn client or VAL server to the SAn server.

Table 8.3.1.3.1-1: Spatial anchor create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| VAL service information | M | Information about the application service associated with the spatial anchor. |
| > Service ID | M | The identifier of the application service. |
| > Service endpoint | M | Endpoint information (e.g., URI, FQDN, IP address) used to communicate with the application service. |
| > Service connection information | O | The connection information (e.g., DNN, DNAI(s)) for establishing connectivity to the application service. |
| List of spatial anchors | M | List of spatial anchors to be created. |
| > Position information | M | Position information of the spatial anchor |
| >> Position (NOTE 1) | O | The 3D geographic position of the spatial anchor in space (x, y and z coordinates). |
| >> Spatial map position (NOTE 1) | O | Position information of the spatial anchor within the spatial map. It includes the identifier of the Spatial map, identifier of spatial map layer(s) and the corresponding localization information (The localization information is specific to spatial map format) or object ID. |
| >>> Spatial map ID | O | Identifier of the spatial map |
| >>>> Spatial map layer ID | O | Identifier of the spatial map layer if Spatial map ID is present |
| >>>> Reference point | O | Reference point from where the spatial anchor is placed in the spatial map |
| >>>> Localization (NOTE 2) | O | 3D position within spatial map and pose of the VAL UE |
| >>>> Object ID (NOTE 2) | O | Identifier of the object on which the spatial anchor will be created |
| > Application specific Spatial anchor information | O | A container to contain application specific information. |
| > Service area | O | The service area where the spatial anchor is accessible. |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover the spatial anchor. |
| > Associated name | O | Name of the product/service associated with the spatial anchor |
| > Time period | O | Indicates time period or duration or time windows – comprising a start time and an expiry time. A spatial anchor is active during this time period. After the expiry time, the state of a spatial anchor becomes inactive. The time period may include one or more values (e.g. repeated over time/day). |
| > Expiry handling indication | O | Indicates whether the spatial anchor has to be deleted or its state to be updated as inactive on expiry by the SAn server. Absence of this IE shall be considered that SAn server deletes the spatial anchor on expiry. |
| > Spatial anchor resource identifier | M | The identifier to associate the resource information with the created spatial anchors. |
| > Spatial anchor group ID | O | Identifier of spatial anchor group that the spatial anchor associates with |
| NOTE 1: One of the IEs shall be present. NOTE 2: If Spatial map ID is present, either one of the IEs shall be present. |  |  |

* * * Next Change * * * *

##### 9.3.1.3.1	Create spatial map request

Table 9.3.1.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SM server for create spatial map request.

Table 9.3.1.3.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Notification Target Address | O | Notification target address (e.g. URL, URI, IP) where the notifications should be sent. |
| Spatial map media format | O | The media format requested for the spatial map. |
| Reference point | O | Reference point from where the object is localized within spatial map |
| Information to include | O | Information to be included in the spatial map |
| > Allowed entities list | O | Identity of entities (e.g. VAL client, VAL server or VAL service identity) which are permitted to discover and access the spatial map |
| > List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >> Spatial map layer information | O | Information to specify the corresponding spatial map layer specific information |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS. |

NOTE 1:	For the current release, the spatial map layer information is implementation specific and out of scope.

* * * End of Changes * * * *