3GPP TSG-SA WG6 Meeting #69	S6-254273

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Nokia

Title:	Pseudo-CR on solution for KI#1 – API invoker status change

Spec:	3GPP TR 23.700-43 (v0.1.0)

Agenda item:	9.11

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@nokia.com)

1. Introduction

This solution relates to KI #1.

2. Reason for Change

As per KI #1, there is no provision for API providing function to handle scenarios where e.g. the API invoker unsubsribes or became rogue. This pCR proposes to enable API provider domain to offboard particular API invoker when its status is changed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-43 (v0.1.0).

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | … |
| --- | --- | --- | --- |
| Sol #X | X |  |  |
| Sol #2 |  |  |  |
| Sol #... |  |  |  |

## 6.x	Solution #x: API provider domain offboarding API invoker

### 6.x.1	General

The API invoker receives onboarding enrolment information from the API provider domain, which is used by the API invoker to onboard to CCF. If the API invoker has unsubscribed to the services provided by the API provider domain, then the API invoker is required to be offboarded.

### 6.x.2	Architecture Impacts

This solution does not impact existing CAPIF architecture.

### 6.x.3	Solution description

Figure 6.x.3.1 illustrates the procedure for offboarding API invoker from the CCF initiated by the AEF.

Pre-conditions:

1.	The API invoker had received the onboarding enrolment information from the API provider domain and successfully onboarded to the CCF..

2.	The API provider domain function (i.e. AMF) decides to offboard the API invoker based on certain condition (e.g. the API invoker has unsubscribed to the services provided by the API provider domain functions).

Figure 6.x.3.1: Procedure for offboarding API invoker AMF

1.	The AMF triggers the offboarding of the API invoker.

2.	The AMF sends offboard API invoker request to the CAPIF core function with the details of the API invoker. The request message may include the reason for the offboarding.

3.	Upon receiving the information to offboard the API invoker, the CAPIF core function invalidates the API invoker authorization corresponding to all the service APIs and deletes the API invoker profile.

4.	The CAPIF core function sends an offboard API invoker notify to the API invoker. The notification message may include the reason for the offboarding.

4.	The CAPIF core function sends a offboard API invoker response to the AMF.

#### 6.x.3.1	Information Flows

The Offboard API invoker request and Offboard API invoker response messages are same as specified in clause 8.2.2.1 and clause 8.2.2.2 respectively. Table 6.x.3.1-1 describes the information flow offboard API invoker notify from the CAPIF core function to the API invoker.

Table 6.x.3.1-1: Offboard API invoker notify

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker whose has been Offboard. |
| Cause | M | The cause for revoking the API invoker. |

### 6.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

* * * End of Change * * * *