3GPP TSG-SA WG6 Meeting #69	S6-254282

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Lenovo

Title:	Solution #16 update

Spec:	3GPP TR 23.700-83 v1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This paper provides an update to Solution #16, to add the missing clauses and to remove the ENs.

2. Reason for Change

This paper provides an update to Solution #16, to add the missing clauses and to remove the ENs.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.0.0.

* * * First Change * * * *

## 7.16	Solution #16: Solution on AIMLE service migration support in UE roaming scenarios

### 7.16.1	Solution description

This solution addresses Key Issue #3. The solution provides a mechanism for supporting the continuity of the ML training / inference capability in scenarios where the ML model training / inference happens at the UE (e.g. AIMLE client) while the UE client is expected to move to an area served by another PLMN.

In this solution, the assumption is that the VAL UE undertakes an ML operation (such as training or inference) and is served by a source AIMLE server to which he is connected via H-PLMN, and the VAL UE is roaming while the operation is ongoing. In such scenario, to prevent AIML service disruption, this solution provides a mechanism to enable the migration of AIMLE service to the target AIMLE server who is bound to VPLMN.

Such solution requires that service / roaming agreements between AIMLE service providers in both PLMNs are in place to support service the roaming UEs.

Pre-conditions:

- The AIMLE-provided ML model training/inference support service is ongoing.

Figure 7.16.1-1 procedure for support for VAL UE roaming

1.	The AIMLE client via the VAL client or other SEAL clients (LMC, ADAEC) monitors UE mobility or analytics related to the expected or predicted mobility of the UE. The AIMLE client detects a mobility of UE to an area covered by different PLMN and sends a mobility report to AIMLE server indicating a cross-PLMN mobility event for the VAL UE.

2.	AIMLE server identifies (check with Core Network) the V-PLMN info and the time when the VAL UE is expected to reach V-PLMN.

3.	AIMLE server fetches from ML repository information on available AIMLE servers in target V-PLMN.

NOTE 1: If the ML repository is deployed by PLMN/MNO, then this step requires interaction among ML repositories of the same provider based on the service agreements or synchronization via the AIMLE servers. The mechanism for this coordination is up to implementation.

NOTE 2: It is assumed that the available AIMLE servers for the V-PLMN are pre-configured at the ML repository.

4.	AIMLE server sends a notification or request for selecting a new AIMLE server, where the message includes the list of available AIMLE servers in VPLMN and some context information (e.g. rating/ranking, vendor info, capability information, energy/load status).

5.	AIMLE client based on the received list, selects the T-AIMLE server from a list of AIMLE servers, based on the fetched information. The selection criteria can be:

- Capabilities (computational, processing) and coverage (geographical or topological area covered by T-AIMLE server)

- Load and energy status of the target AIMLE server, based on monitoring or querying possible overload or high energy consumption (can be provided by the target AIMLE service provider, e.g. edge platform provider).

- Expected delay / latency for accessing the T-AIMLE server (e.g., in case of relaying ML inference task). This can be estimating based on probes or based on relative location of platform hosting the T-AIMLE server or based on the interface / API statistics on delay or failures based on high delay.

- Vendor compatibility: this can be the case when the selection of a target AIMLE is based on which vendor provides the server. It can be either the same vendor only, or a list of selected vendors based on the service agreements among MNOs/SEAL providers.

- Cost for task migration: The computational effort and/or processing required, or time required, or energy required for offloading a task to the T-AIMLE server. Such cost may be based on the distance among servers (e.g. edge to regional cloud) or based on the task to be offloaded.

- A rate/ranking/weighting factor of target AIMLE server based on fulfilment of previous tasks and statistics. Such rate can be stored and updated at ML repository (or shared among repositories) after fulfilling each task based on consumer feedback or based on internal evaluation at the AIMLE (based on success or failure of tasks under similar conditions)

NOTE 3: Pre-defined policies from the S-AIMLE server or VAL server may be provided in previous steps to dictate the AIMLE server selection criteria (policies on minimum rating, allowed vendors, preferred AIMLE servers)

6.	The AIMLE client sends to the AIMLE server the selected T-AIMLE server and the expected time and area of roaming.

NOTE 4: The selection of the T-AIMLE server by the S-AIMLE server instead of AIMLE client is to be explored in normative phase.

7.	The AIMLE server requests or negotiates with target AIMLE server to undertake the task of serving the AIMLE client of interest.

8.	The AIMLE server requests AIMLE client to change serving AIMLE server and passes the target AIMLE server info, also possible limitations in target V-PLMN (lower priority, capacity limitations for ML operation). The AIMLE client confirms and connects to target AIMLE server via the source AIMLE server.

-----------UE of interest roams to target PLMN and AIMLE client connects to T-AIMLE server-------

9.	The T-AIMLE server sends a notification to S-AIMLE server about the roaming of the UE(s) of interest. Following the notification may also reach the VAL server (as expected consumer of the ML training/inference output).

10.	The T-AIMLE server if the task is about relaying the ML inference output over the VPLMN, it provides the output via the S-AIMLE server to the VAL server. If the T-AIMLE server is the new AIMLE server, it sends directly the output to VAL server.

### 7.16.2	Architecture Impacts

This solution requires that AIMLE functional model supports UE roaming scenarios, and that ML repository has informing on V-PLMN. Such architecture option is introduced in clause 6.2.2.

### 7.16.3	Corresponding APIs

This subclause provides a summary on the corresponding API for solution #16.

-	Fetch multi-MNO AIMLE server API (request / response model; API provider: ML repository; known consumer: AIMLE server; corresponding to step 3 of clause 7.16.1).

-	Select V-PLMN AIMLE server API (request / response model; API provider: AIMLE client; known consumer: AIMLE server; corresponding to steps 4 and 6 of clause 7.16.1).

Request AIMLE server change API (request / response model; API provider: AIMLE server; known consumer: AIMLE server; corresponding to step 7 of clause 7.16.1).

-	Notify AIMLE server change API (subscribe / notify model; API provider: AIMLE server; known consumer: AIMLE server or AIMLE client; corresponding to step 8, 9 of clause 7.16.1).

### 7.16.4	Solution evaluation

This solution addresses Key Issue #3 and introduces the capability to support the continuity of the ML training / inference capability in scenarios where the ML model training / inference happens at the UE (e.g. AIMLE client) while the UE client is expected to move to an area served by another PLMN.

This solution supports the selection of the T-AIMLE server by the AIMLE client, since this is a VAL UE-centric solution where the VAL UE is configured to select the best AIMLE server for continuing the AIML capability when roaming; however variants of this solution where the S-AIMLE server is selected instead of AIMLE client can be also possible and can be explored in normative phase.

This solution is feasible and doesn't introduce any dependency to 3GPP network systems.

* * * Next Change * * * *

## 7.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |  |  |
| Sol #3 |  |  |  |  |  |  |  |  |
| Sol #4 |  |  |  |  |  |  |  |  |
| Sol #5 |  |  |  |  |  |  |  |  |
| Sol #6 |  |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  |  |
| Sol #8 |  |  |  |  |  |  |  |  |
| Sol #9 |  |  |  |  |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |  |
| Sol #11 |  |  |  |  |  |  |  |  |
| Sol #12 |  |  |  |  |  |  |  |  |
| Sol #13 |  |  |  |  |  |  |  |  |
| Sol #14 |  |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  |  |  |
| Sol #16 |  |  | x |  |  |  |  |  |
| Sol #17 |  |  |  |  |  |  |  |  |
| Sol #18 |  |  |  |  |  |  |  |  |
| Sol #19 |  |  |  |  |  |  |  |  |
| Sol #20 |  |  |  |  |  |  |  |  |
| Sol #21 |  |  |  |  |  |  |  |  |
| Sol #22 |  |  |  |  |  |  |  |  |