3GPP TSG-SA WG6 Meeting #69	S6-254286

Wuhan, China 13th – 17th October 2025	(revision of S6-254xxx)

Source:	Lenovo

Title:	New Solution to Key Issue #3

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Manos Pateromichelakis (epateromiche@lenovo.com)

1. Introduction

This paper proposes a new solution to key issue 3.

2. Reason for Change

This paper proposes a new solution to key issue 3.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

## 6.x	Solution #x: Sensing Enablement for Spatial Map service

### 6.x.1	Solution description

This solution addresses key issue #3 and open points 1, 2 and 4.

In spatial mapping use case (as described in TS 22.856), spatial mapping enabler (SM server) will help classifying objects into modelling and tracking of stationary and moving objects. For stationary object, spatial mapping has to estimate the number of objects, type of object and position. Whereas for moving objects, spatial mapping has to determine the position, type of object, direction, speed. Once the spatial mapping service has sufficient information, it has to map all the stationary and moving objects related to UE’s environment.

The solution provides a method for determining the sensing requirement and the translation to sensing needs for an Localized Mobile Metaverse application. The solution introduces a Sensing Enablement service, to support sensing fusion (where sensing can be provided by multiple sources/technologies) from the UEs (which are assumed to deploy an enabler client) and the network (if network deploys a sensor function), to be utilized by the SM server.

### 6.x.2	Procedures

The procedure for sensing enablement solution is described in this sub-clause.

Pre-conditions:

- Sensing Enabler Server (SES) is authorized / in trusted MNO domain to perform sensing enablement.

- SES is connected to the Sensing Enabler Clients (SEC) in its service coverage area.

Figure 6.x.2-1: Sensing enablement for SM service

- The Spatial Map (SM) Server sends a subscription request to SES. This request includes the sensing enablement requirements, one or more VAL UE identifiers/addresses within the VAL service, a service area where this subscription applies (this can be geographical or topological area), application QoS requirements for the VAL service, optionally the list of IDs/types of objects to be sensed, permissions / restrictions / preferences for the sensors to be used, and event reporting criteria (to identify when to report the sensing data).

If the requestor is the SM server, the sensor data for the target area (e.g. objects) are needed for the spatial mapping creation (if not a map exists) or if spatial map exists, the SM server can request sensor data for specific users, or data used to identify in 3D space the position, viewing direction, angle, etc. of the specific user.

- The SES authorizes the request and provides a subscription response as a positive or negative acknowledgement.

- The SES may also request and receive from an Object Repository (at edge/cloud) information/ attributes for one or more object IDs (based on step 1). This information includes attributes / characteristics/ profile for the object to be sensed.

- The SES translates the sensing enablement service requirement in step 1 to sensing requirement. This includes the determination of the sensing area (topological) and the sensing technologies to be considered, as well as the objects to be sensed (target objects/UEs for the spatial mapping case). Also, this triggers the queries towards the network and UEs.

- The SES, acting as AF sends a query to the underlying 5GC (LMF or sensing function), to receive the sensing capabilities for the target area or for one or more target objects. 5GC sends the sensing capabilities / configurations back to the SES.

Editor’s Note: This step depends on SA2 progress on sensing data exposure.

6a. 	     The SES also broadcasts to the target service area (based on the area of interest in step 1) a query to identify possible sensing sources and capabilities at the VAL UEs. This query includes the VAL service ID, sensor capabilities requirement (non-3gpp, 3gpp), object(s) identifiers/names, sensing preferences request, area of interest and time of interest/horizon.

6b. 	       The recipient SECs send to the SES the requested information including their sensing capabilities (from both UE modem and VAL client side).

7. 		   The SES determines for the given SM service, the sensing entities to be used in the given area (this may include LiDAR sensing from VAL UE#1, RF sensing from VAL UE #2, etc).

- The SES collects the sensing outputs from the determined sources. Such sensing output may include identified objects and their location/mobility/orientation or can be just the identification of an obstacle at a given location with certain attributes (e.g. height, material, …).

- The SES performs sensing processing / abstraction / integration or fusion based on step 7 sensing output. This step also optionally includes the identification of the object based on the attributes and the mapping of the sensing data to the determined object.

- SES sends the sensing outputs to the consumer SM server.

- Optionally, SES sends also the mapping of the sensing data to the object to the Object Repository.

### 6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * Next Change * * * *