3GPP TSG-SA WG6 Meeting #69	S6-254320

Wuhan, China 13th – 17th October 2025	(revision of S6-253xxx)

Source:	Ericsson

Title:	Policy based configuration for Energy Efficient data delivery service

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Ashish S Sharma, ashish.sanjay.sharma@ericsson.com

1. Introduction

The pCR provides a solution for Key Issue #1 and #6.

2. Reason for Change

The SEALDD layer provides data transmission capability for the VAL servers. The VAL client can be a resource constraint device, like devices running on battery. And this requires the VAL servers to provide services to the VAL clients in an Energy-Efficient(EE) manner. The VAL server can request the SEALDD layer to leverage EE methods for the data delivery. In order to do so, the SEALDD layer enables the VAL server to install the EE policies to take into account while optimising the data connection establishment and transmission for EE. Based on the EE policy, the SEALDD server can instruct the SEALDD client to suspend the connections which are inactive to save energy. The connection suspension maintains the state of the connection, which helps to resume the connection instantly. Otherwise, if the SEALDD server deletes the connection, then the connection needs to be established again, which requires more energy and introduces a delay in the data delivery.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

### 9.10.3	Information flows

#### 9.10.3.1	SEALDD policy configuration request

Table 9.10.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the SEALDD policy configuration.

Table 9.10.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID). |
| Identities | O | Identifier of the VAL UEs or VAL users, or VAL UE group ID(as defined in clause 7.5 of 3GPP TS 23.434 [4]), or VAL UE identity list for which SEALDD policy applies |
| Slice identifier | O | The slice identifier (S-NSSAI) for which SEALDD policy applies. |
| SEALDD policy | O (See NOTE 4) | The SEALDD policy associated with application traffic identifiers, VAL UE identity. |
| > Quality guarantee policy | O (See NOTE 1) | Indicates the event (e.g. measurement threshold) to be measured for the quality guarantee |
| > Quality optimization policy | O (See NOTE 1) | Indicates the data transmission adjustment policy (e.g., adjustment need) to be performed in SEALDD layer. |
| > Bandwidth control policy | O (See NOTE 2) | Indicate the bandwidth control preference, e.g. re-allocating the bandwidth limit between different VAL users, including UL/DL |
| > Geofence policy | O (See NOTE 3) | Indicates the geofence policy per VAL service. |
| >> Geofence location | O (See NOTE 3) | Indicates the geofence location information for the mentioned VAL service |
| >>Geofence policy action | O (See NOTE 3) | Indicates the policy action like allowed or blocked. |
| > temporal policy | O (See NOTE 3) | Indicates the time period for which the sealdd traffic is allowed |
| > policy expiration time | O | Indicates the validity of the policy. |
| > Energy saving policy | O | Indicates the Energy Saving policy for different energy saving modes such as Performance, Balanced, Economy mode. |
| > Non-3GPP access measurement policy | O | Indicates the non-3GPP access (like WLAN) measurement policy (e.g. WLAN SSIDs/BSSID, signal strength thresholds(high/low), time-based threshold for offload to WLAN). |
| Multi-modal SEALDD policy | O (See NOTE 4) | Multi-modal SEALDD policy associated with set of individual SEALDD flows. |
| > Synchronization policy | O | Indicates the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2]. |
| > Multi-modal flows alignment policy | O | Indicates the information to do the multi-modal flows alignment, e.g., Multi-modal Service ID, maximum acceptable duration for traffic flow alignment. |
| > Policy expiration time | O | Indicates the validity period of the policy. |
| > UE-to-UE policy | O | Specifies UE-to-UE direct communication policy including proximity threshold, QoS threshold (e.g. bitrate, latency and jitter, PLR, PER) or QoE threshold (e.g. MOS as specified in ITU-T P.1203.3 [18]) for entering direct communication mode, and proximity threshold, QoS threshold or QoE threshold for leaving direct communication mode. |
| NOTE 1: This IE is used for the SEALDD enabled transmission quality guarantee, as specified in clause 9.9. NOTE 2: This IE is used for the SEALDD enabled bandwidth control, as specified in clause 9.8. NOTE 3: This IE is used for the SEALDD connection establishment and data delivery, as specified in clause 9.2 NOTE 4: At least one of these IEs shall be present. |  |  |

* * * Next Change * * * *

### 9.10.2	Procedures

#### 9.10.2.4	SEALDD client policy configuration request

Figure 9.10.2.4-1 illustrates the procedure for SEALDD client policy configuration request from the SEALDD server to the SEALDD client.

Pre-condition:

1.	The SEALDD connectivity has been established between the SEALDD client and SEALDD server.

Figure 9.10.2.4-1: SEALDD client policy configuration request

1.	Based on policy received from the VAL server(e.g., Multi-modal flows alignment policy, Energy saving policy), or SEALDD server determines that the policy needs to be configured to the SEALDD client, the SEALDD server sends the SEALDD client policy configuration request to the SEALDD client. The SEALDD server may combine different policies which are updated to fulfill the energy saving requirements in the request.

2.	The SEALDD client performs authorization check to verify whether the SEALDD server is authorized to configure the SEALDD client policy.

3.	Upon successful authorization, the SEALDD server stores the SEALDD client policy for later use (e.g. for multi-modal flows alignment) and replies to the SEALDD server with the SEALDD client policy configuration response.

* * * Next Change * * * *

#### 9.10.2.5	SEALDD client policy configuration update request

Figure 9.10.2.5-1 illustrates the procedure for SEALDD client policy configuration update request from the SEALDD server to the SEALDD client.

Pre-condition:

1.	The SEALDD connectivity has been established between the SEALDD client and SEALDD server.

Figure 9.10.2.5-1: SEALDD client policy configuration update request

1.	Based on policy received from the VAL server, or SEALDD server determines that the policy needs to be updated to the SEALDD client, the SEALDD server sends the SEALDD client policy configuration update request to the SEALDD client. The SEALDD server may combine different policies which are updated to fulfill the energy saving requirements in the request.

2.	The SEALDD client performs authorization check to verify whether the SEALDD server can be authorized to update the SEALDD client policy.

3.	Upon successful authorization, the SEALDD client updates the SEALDD client policy and replies to the SEALDD server with the SEALDD client policy configuration update response.

* * * Next Change * * * *

### 9.10.3	Information flows

#### 9.10.3.7	SEALDD client policy configuration request

Table 9.10.3.7-1 describes the information flow from the SEALDD server to the SEALDD client for requesting SEALDD client policy configuration.

Table 9.10.3.7-1: SEALDD client policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor |
| SEALDD-UU flow list | O (NOTE) | Indicates the list of SEALDD-UU flows. |
| > SEALDD-UU flow ID | O | Identity of the SEALDD-UU flow. |
| Multi-modal SEALDD flow ID | O (NOTE) | Identity of the multi-modal SEALDD flows associated with the SEALDD multi-modal connection |
| VAL service ID | M | Identity of the VAL service |
| UE ID | O | Identifier of specific UE or VAL user |
| >Configuration ID | M | Identifier of the SEALDD policy configuration. |
| Multi-modal flows alignment policy | O | Multi-modal flows alignment Policy |
| >Multi-modal Service ID | O | Identifier of Multi-modal Service |
| >Flows transmission requirement | O | Flows transmission requirement including the delay requirement, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time. for the associated flow |
| SEALDD UE-to-UE policy | O | Specifies UE-to-UE direct communication policy including, QoS threshold (e.g. bitrate, latency and jitter, PLR, PER) for entering direct communication mode, and QoS threshold for leaving direct communication mode. |
| SEALDD energy saving policy | O | Indicates the energy saving policy e.g., scheduled time period(s) of no active data transmission and the duration of no activity between the SEALDD client and SEALDD server (time period with corresponding sleeping interval) |
| NOTE: At least one of these IEs shall be present in the message. |  |  |

* * * End of Changes * * * *