3GPP TSG-SA WG6 Meeting #69	S6-254350

Wuhan, China 13th – 17th Oktober 2025	(revision of S6-254xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on Key issue DM of end-to-end encrypted calls

Spec:	3GPP TR 23700-37-030

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

This pCR adds a key issue about performing Discreet monitoring of end-to-end encrypted calls. The text originates from TR 23.784, but has been enhanced as required.

2. Reason for Change

This key issue should be addressed in the study.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37-020 v 0.3.0.

* * * First Change * * * *

## 5.X	Key issue X:	Discreet monitoring of end-to-end encrypted calls

An authorized MC user may perform discreet monitoring on end-to-end encrypted communications, where those communications can be private calls, individual communications with a server (e.g. pull from or push to a server), and group calls where the target of the discreet monitoring is engaged in group communications. To comprehend the communications, the authorized user's MC service client needs to be provided in near-realtime with the end-to-end encryption key used to decrypt the content of the call.

NOTE:	This key issue assumes that the decryption process is in the authorized user's MC service client, to avoid violating the stage 1 requirement [R-5.12-009] in 3GPP TS 22.179 [2] for end-to-end encryption of media traffic.

For private or individual communications, a session key is used to encrypt the content of the communications, which is protected in transfer between the call participants by an encryption process making use of the identity of the destination party. The mechanism is described in 3GPP TS 33.180 [6].

For group communications, a GMK is originated by the GMS and protected in transfer to group members one at a time by an encryption process making use of the identity of each group member. The mechanism is described in 3GPP TS 33.180 [6].

According to the requirement in 3GPP TS 22.280 [2] and reproduced in Annex A of the present document, the participants in the call shall not be aware of the discreet monitoring.

Issues:

-	To successfully discreet monitor private or individual communications without violating the requirement for end-to-end encryption, the authorized user's MC service client needs near-realtime access to the session key exchanged between the parties in the private or individual communication. To fulfil the requirement reproduced in Annex A.1, the parties in the call shall not have knowledge of the authorized user possessing the encryption key.

-	To successfully discreet monitor group communications without violating the requirement for end-to-end encryption, the authorized user's MC service client needs near-realtime access to the GMK for the group. To fulfil the requirement reproduced in Annex A.1, the group members in the call will not have knowledge of the authorized user possessing the encryption key.

* * * End of Changes * * * *