3GPP TSG-SA WG6 Meeting #69	S6-254619

Wuhan, China, 13th – 17th Oct 2025                                                (revision of S6-254174)

Source:	ZTE Corporation

Title:	Solution for KI#1 on support for sensing service registration

Spec:	3GPP TR 23.700-15 v0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Wei Luo <luo.wei11@zte.com.cn>

1. Introduction

This paper proposes a solution to KI#1 on support for sensing service registration procedure.

2. Reason for Change

This paper proposes a solution to KI# 1 on support for sensing service registration procedure. This solution addresses the key issue on how to perform Sensing Service registration for a Sensing Service Supplier.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.1.0.

* * * First Change * * * *

6.x	Solution #x: Support for sensing service registration

6.x.1	Solution description

This solution aims to address the issues identified in Key Issue 1, specifically on  Sensing Service Supplier registration.

Editor's note: This solution is built on functional architecture described in solution X for key issue 1. The sensing capabilities in the enabler layer may be provided by a separate SEAL sensing server or directly by the specific SEAL/VAL enabler server. The decision on whether a separate SEAL sensing server is needed will be based on the evaluation of the solutions and FFS.

In this solution, a functional entity (e.g., 5GC NF, vertical application server/client) which can provide sensing results may act as the Sensing Service Supplier.

Editor's note:	Whether the sensing service supplier can be 3rd party AF is FFS.

6.x.2	Procedures

Figure 6.x.2.1-1 illustrates the procedure of Sensing Service Supplier-triggered sensing service registration.

Figure 6.X.2.1-1: sensing service registration procedure

- The Sensing Service Supplier (e.g.,Sensing application server/client) sends sensing service registration request to the sensing enabler server, including the identifier of the Sensing Service Supplier and capabilities information. The capabilities information may include supported sensing types (e.g., object detection, tracking, environmental monitoring), target object type (e.g., humans, vehicles, infrastructure), sensing accuracy metrics (e.g.,range, positioning precision), supported sensing service areas and geographical limitations, supported sensing parameters (update frequency, latency) and so on.

- The sensing enabler server checks authorization for the Sensing Service Supplier 's registration request.

- After successful authorization, sensing enabler server sends sensing service registration response to the Sensing Service Supplier and stores the Sensing Service Supplier's information received in step 1 as sensing service supplier profile information.

6.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *