3GPP TSG-SA WG6 Meeting #69	S6-254663

Wuhan, China, 13th – 17th October 2025	(revision of S6-254152)

Source:	Huawei, Hisilicon

Title:	New solution of AIoT data processing and event notification

Spec:	3GPP TR 23.700-26 v0.1.0

Agenda item:	9.8

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This solution aims to address the issues identified in Key Issue 2 and key Issue 4.

2. Reason for Change

This AIoT application desires the applcation events rather than each AIoT device ID. So the enabler layer could make use of the reported AIoT device ID and other provisioned service information to create the application interested event notification.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS / TR <TS/TR number and version>.

* * * First Change * * * *

<Proposed change in revision marks>

7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #X |  | Y |  | Y |
| Sol #2 |  |  |  |  |

* * * Next Change * * * *

7.x	Solution #X: AIoT Data Processing management

7.x.1	Solution description

This solution address key issue#2, and 4 to introduce a solution for AIoT data processing of the network exposured AIoT inventory/command result.

The inventory results in the notification from 3GPP network is further processed and transformed into the application layer interested service events using the provisioned service data.

The figure 7.x.1-1 shows the high-level procedure of AIoT data processing management and event notification.

Pre-conditions:

- The AIoT task initiate service has been invoked by the AIoT application layer consumer, and the AIoT enabler has initiated the AIoT service request to the 3GPP core network.

Figure  7.X.1-1 AIoT Data Processing management

1.	 The AIoT enabler server receives the notification(s) with inventory/command result from the 3GPP core network as described in 3GPP TS 23.369 [23.369].

2.	The AIoT enabler processes the network expoured data from step 1 and transforms them into service event based on the provisioned service data and the AIoT taks initiate request previously received. The AIoT service event contains the information of “what AIoT device ID(s)”,  “in which area,” and “what happened”. For “what happened”, there are some examples (not limited to) as below:

Type1: AIoT device appearance, the AIoT device is being reported at the service region for the first time;

Type2: AIoT device disappearance, the AIoT device is not being reported during the limited time;

Type3: AIoT device stay, the AIoT device is continuously reported at the same service region;

Type4: AIoT device moving, the AIoT device is reported from different regions.

The reported AIoT device ID is transformed back to the application layer object ID.

Further the AIoT enabler server processes the network expoured data per AIoT service scenario type indicated in the AIoT task initiate service request.

-	Inbound or Outbound type: for all the AIoT devices appearance events during the task execution period, compare the objects ID(s) reported from 3GPP network and the objects ID lists from the AIoT tasks. If the ID(s) can match, then, the application objects can be successfully inbound or outbound; otherwise, it is considered an illegal operation and an alert information is triggered.

-	Inventory type: for all the AIoT devices appearance and stay events, the corresponding objects can be seen as the normal inventory information. However, for the AIoT devices disappearance event, an alert or other method such as specific AIoT device ID inventory should be initiated.

-	Monitor type: for all the AIoT devices appearance, disappearance, moving events, the AIoT device state changed, the AIoT enable layer should record and report.

4.	Report the interested application events to the application layer and the AIoT task is completed.

7.x.2	Architecture Impacts

This solution introduces AIoT service event notification capability to the AIoT enabler server.

7.x.3	Corresponding APIs

The AIoT service event notification service is introduced.

7.x.4	Solution evaluation

* * * Next Change * * * *