3GPP TSG-SA WG6 Meeting #69	S6-254745

Wuhan, China 13th – 17th October 2025	(revision of S6-254695)

Source:	Samsung

Title:	Pseudo-CR on Solution for AIMLE discovery

Spec:	3GPP TR 23.700-83 v1.0.0

Agenda item:	9.4

Document for:	Approval

Contact:	Narendranath (Naren) Durga Tangudu, n.tangudu@samsung.com

1. Introduction

This pCR proposes new solution for discovery of AIMLE server. Based on the agreed pCR in SA6#68 (S6-253648), this pCR proposes the solution related to new open issue on AIMLE discovery.

2. Reason for Change

S6-253648 agreed to new open issue on AIMLE discovery, under key issue #6. In Rel-19, the AIMLE capabilities assume that the VAL server and AIMLE servers are pre-confiugured with AIMLE server endpoint information (e.g., URI, IP address). Such pre-configured AIMLE server endpoint information poses the issues or architectural limitations.

This pCR proposes a solution for AIMLE discovery when there is need for availing services from another AIMLE server may arise at AIMLE client for various conditions like change in AIMLE client’s ML task requirement, AIMLE client’s mobility planning/requirements etc.

3. Conclusions

None.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.0.0.

* * * First Change * * * *

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #6 |
| --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |
| Sol #X |  |  |  |  | X |

* * * Next Change * * * *

## 7.X 	Solution #X: AIMLE server discovery

### 7.X.1 	Solution Description

#### 7.X.1.1 	General

This solution addresses key issue # 6, open issue #5 (“Whether and how the AIMLE services can support the discovery of AIMLE servers”). For instance, when the services from a AIMLE server cannot be consumed, due to not supported in a given location, ML models, etc, the AIMLE service consumer may require to discover an alternate AIMLE server. Each AIMLE client or VAL server is pre-configured with a Default AIMLE server URI. The Default AIMLE server exposes a discovery API that returns a list of AIMLE servers matching a given criteria. The need for availing services from another AIMLE server may arise at AIMLE client/VAL server for various conditions like change in AIMLE client's ML task requirement, AIMLE client's mobility planning/requirements etc. When such requirement arises, the AIMLE client invokes the AIMLE server discovery service to fetch the appropriate AIMLE server meeting the requirements.

This solution proposes to enable the AIMLE server to support the discovery of other AIMLE servers based on the discovery requirements from the AIMLE service consumers (i.e., AIMLE client, VAL server).

#### 7.X.1.2 	Procedure

Assumptions:

- AIMLE server is aware of the other AIMLE servers information, for example, AIMLE servers may have registered their capabilities with ML repository.

- There are various deployments that include multiple AIMLE servers, for example, hierarchichal, Edge deployments etc.

Pre-conditions:

- Each AIMLE client or VAL server is pre-configured with a default AIMLE server URI. The default AIMLE server exposes the discovery API that returns a list of AIMLE servers matching a given criteria.

Figure 7.X.1.2-1: AIMLE server discovery procedure

- An AIMLE service consumer (VAL server, AIMLE client) sends an AIMLE server discovery request message to discover the list of suitable AIMLE servers. The request message includes AIMLE server requirements to discover suitable servers based on discovery criteria like geographical proximity etc. The AIMLE server discovery request includes information as described in Table 7.X.1.3.1-1.

- the AIMLE server performs the authentication and authorization checks to determine if the requestor can discover AIMLE servers. If the requestor is authorized, the AIMLE server performs the discovery based on the discovery criteria in the request message. The server utilizes the provided criteria to filter and identify suitable AIMLE servers. If there are multiple AIMLE servers that fulfill the criteria, then the default AIMLE server selects the suitable servers, ensuring that the returned list is optimized for the requestor's needs, improving performance and reliability.

- default AIMLE servers send the list of the suitable AIMLE servers to the requestor. The information is included in Table 7.X.1.3.2-1. The available list may have an availability duration, and the requestor can communicate with any AIMLE server based on availability duration information. This duration informs the requestor about the time frame within which the servers are guaranteed to be available, allowing for better planning and resource allocation.

The discovery request needs to be initiated again by the AIMLE service consumer if the availability duration information is expired. This ensures that the information remains current and relevant, preventing issues related to outdated server availability.

Editor’s Note: The relation with CAPIF discovery is FFS.

#### 7.X.1.3	Information Flows

##### 7.X.1.3.1 	AIMLE server discovery request

Table 7.X.1.3.1-1 describes information elements for the AIMLE server discovery request from the AIMLE client/VAL server to the AIMLE server.

Table 7.X.1.3.1-1: AIMLE Server Discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server, AIMLE client). |
| AIMLE server discovery criteria | M | Discovery criteria for discovering suitable AIMLE servers for AI/ML operations. |
| > Location | O (NOTE) | Indicates the location information (e.g., cell Identity, Tracking Area Identity, GPS coordinates, VAL service area ID), serving area information of the AIMLE server to be discovered. |
| > VAL service ID | O (NOTE) | The identity of the VAL service for which the request applies. |
| > Task Type | O (NOTE) | The type of AI/ML operations that should be supported (e.g., model training, testing, inference, model split, etc). |
| > ML Models Supported | O (NOTE) | Types of ML model supported, identifiers of the ML models. |
| NOTE: At least one of the information elements shall be present. |  |  |

##### 7.X.1.3.2	 AIMLE server discovery response

Table 7.X.1.3.2-1 describes information elements for the AIMLE Server Discovery response from the AIMLE server to the AIMLE client / VAL server.

Table 7.X.1.3.2-1: AIMLE Server Discovery response

| Information element | Status | Description |
| --- | --- | --- |
| status | M | The status for the request: success or fail. |
| List of AIMLE servers | M | A list of AIMLE servers information and their end-point information to reach the AIMLE servers. |
| Availability Schedule | O | The availability duration or schedule of each AIMLE server in the list when they are available. |

### 7.X.2 	Architecture Impacts

Editor's Note: The architecture impacts of this solution are FFS.

### 7.X.3	 Corresponding APIs

Editor's Note: The corresponding APIs are FFS.

### 7.X.4	 Solution Evaluation

Editor's Note: The evaluation of the solution is FFS.

* * * End of Change * * * *