3GPP TSG-SA WG6 Meeting #69	S6-254791

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254728

Source:	Ericsson, InterDigital

Title:	New Solution for KI#3 on EAS Discovery and Selection for Support Energy Saving

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #3 (TR 23.700-44 clause 5.3), the study aspects on EDGE application enablement enhancement for energy efficiency include:

- Whether and how the EDGE application enablement layer needs to be enhanced to support energy saving and energy efficiency?

- How EDGE application enablement service could be enhanced to support energy saving and energy efficiency, including how to support the selection of an edge application server based on energy consumption information.

According to the use cases and comment functions identified in S6-254224, proper discovery and selection of edge application server can support energy saving. This pCR proposes procedure for enhance the EAS discovery services to support energy saving.

2. Reason for Change

This pCR proposes a new solution for Key Issue #3, to enhance the EAS discovery services to support energy saving.

3. Conclusions

This paper proposes solution on enhancements of the EAS discovery to support energy saving.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[5]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[aa]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

* * * Next Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #A |  |  | X |  |  |  |

* * * Next Change * * * *

## 6.A	Solution #A: Enhance Edge Services for Support Energy Saving

### 6.A.1	Solution Description

According to the key issue #3, there is a need for EDGE application enablement layer enhance to support energy saving. The services procedures and information flows introduced in 3GPP TS 23.558 [aa] for edge services, can be enhanced to support the energy saving requirements.

The procedure for EAS discovery as specified in clause 8.5.2.2 of 3GPP TS 23.558 [aa] is enhanced as follows (new text in bold italics):

#### 8.5.2.2 Request-response model Pre-conditions: 1. The EEC has received information (e.g. URI, IP address) related to the EES; 2. The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.11; and 3. The EES is configured with ECSP's policy for EAS discovery. NOTE 1: Details of ECSP's policy are out of scope. Figure 8.5.2.2-1: EAS Discovery procedure - The EEC sends an EAS discovery request to the EES. The EAS discovery request includes the requestor identifier [EECID] along with the security credentials and may include EAS discovery filters, EEC service continuity support, and may also include UE location to retrieve information about particular EAS(s) or a category of EASs, e.g. gaming applications, or Edge Applications Server(s) available in certain service areas, e.g. available on a UE's predicted or expected route. The request may include an EAS selection request indicator. If an e2e tunnel is used in the UE for applications (e.g. user configured tunnel service), and if the tunnel service provider and the ECSP are from the same organization and only if user grants the permission, then the EEC provides the tunnel information for the associated applications in the request to the EES considering user consent. The EAS discovery filter may include energy requirements to the EAS, e.g., Energy mode, Energy type supports. The Energy type is about requirement to the EAS on its supported energy type (e.g. non-renewable energy and/or renewable energy). NOTE 2: The e2e tunnel case is limited to case where the tunnel service is deployed in home domain in the present release. 2. Upon receiving the request from the EEC, the EES checks if the EEC is authorized to discover the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. If EAS discovery filters contain energy requirements, the EES checks whether the EAS satisfy the requirements on Energy type supports in the EAS discovery requests. If the EAS fulfil the energy requirements, then EES adds the identified EAS(s) into the list of discovered EAS(s). If UE's location information is not already available, the EES obtains the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.10.3. If EAS discovery filters are provided by the EEC, but it does not contain Application group profile, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location. For the identified EAS(s) if EAS profile indicates the list of associated devices required in order to serve the UE, then EES checks whether the UE has required associated devices with it or not based on AC profile received in EAS discovery request. If the UE has the required associated devices then EES adds the identified EAS(s) into the list of discovered EAS(s). When the bundle EAS information is provided, then - If bundle EAS information includes EAS bundle identifier, the EES identifies all or part of the EAS(s) associated with the same EAS bundle identifier. - If bundle EAS information includes a list of EASIDs, the EES identifies the EASs which are all or part of the EAS bundle. If the EEC indicates that service continuity support is required, when identifying the EAS, the EES shall take the indication which ACR scenarios are supported by the AC, the EEC, the EES and the EAS and which of these are preferred by the AC into consideration. The EES may select one EAS and determine whether to perform application traffic influence for this EAS in advance based on AC's service KPI or EAS’s service KPI in desired response time, when the EAS does not perform traffic influence in advance. If the EES determines to perform application traffic influence for this EAS in advance, then the EES will applies the AF traffic influence of the EAS in the 3GPP Core Network before sending EAS discovery response. If the Prediction expiration time is provided then the EES may determine whether to identify the instantiable but not instantiated EAS as T-EAS based on Prediction expiration time and the predicted EAS deployment time information obtained from ADAES as specified in clause 8.11 of TS 23.436 [28] or from local configured maximum EAS deployment time. The EES determines remaining EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation is in progress from ECSP management system and the predicted EAS deployment time. Furthermore, if EES received the indication which the EAS instantiation is in progress, then the EES determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time information. When EAS discovery filters are not provided, then: - if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location; - EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location), When EAS discovery filters contain Application group profile, the EES checks whether information about common EAS and related Application Group ID is available or not. If the common EAS information related to the Application Group ID is: - not available at the EES, then based on the policy if EES needs to select the common EAS, the EES identifies an EAS for the Application Group ID based on the provided EAS discovery filters such as KPIs, UE-specific service information or the ECSP policy. Furthermore, the EES stores the common EAS information and related Application Group ID. The EES may also subscribe and get the notifications of candidate DNAIs for the UEs of the group as described in 3GPP TS 23.548 [20], 3GPP TS 23.501 [2], the EES identifies the EAS(s) taking the candidate DNAIs into account, e.g., an EAS where its DNAI in the EAS profile is common to all UEs in the group. - available at the EES, then the EES provides information of that EAS as result for EAS discovery, or the EES identifies the EAS based on the provided EAS discovery filters and the UE location when E2E response time is received, furthermore: - when the UE in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE can connect either common EAS or EAS registered to this EES which is UE is in the common EAS service area), the EES identifies either common EAS or EAS registered to this EES for the UE based on the UE location, EAS discovery filters, application group profile, common EAS KPI as specified in table 8.2.5 and EDN information, registered EAS KPI as specified in table 8.2.5 and EDN information, and inter-EAS communication performance of these EASs and checks whether the common EAS or EAS registered to this EES can provide better E2E response time. - When the UE is not in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE is not in the common EAS service area), then the EES identifies candidate EAS(s) for the group from its registered EASs based on UE location, EAS discovery filters and application group profile and checks if the candidate EAS(s) can satisfy the E2E response time considering the response time of the candidate EAS(s), the response time of available common EASs in other EDNs and the inter-EAS communication latency. If the E2E response time cannot be satisfied, the EES will respond the EEC in step 3 indicating such a rejection reason; otherwise, the EES selects a candidate EAS and interacts with the ECS-ER to store it as common EAS as described in clause 8.20.2.3. NOTE 3: The EES can use ADAE analytics service in 3GPP TS 23.436 [28] or local configuration to obtain application performance (e.g. latency) for inter-EAS communication. NOTE 4: If the current EDN has overlapping area (where UE resides) with other EDN and both EDNs has available common EAS which can satisfy E2E response time, which EAS to select can take number of UEs in the group connected to available common EAS into consideration. NOTE 5: The EES may have previously determined and stored the common EAS for Application group ID, or the EES may have received the common EAS selection information for Application group ID during the common EAS announcement procedure. When the ECS-ER is not available and the EES selects the common EAS, the selected common EAS shall be announced to other EES(s) as per procedure specified in clause 8.19. When the ECS-ER is available, if the common EAS information related to the Application Group ID is: - not available at the ECS-ER, the EES identifies one EAS for the group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. The EES may also subscribe and get the notifications of candidate DNAIs for the UEs of the group as described in 3GPP TS 23.548 [20], 3GPP TS 23.501 [2], the EES identifies the EAS(s) taking the candidate DNAIs into account, e.g., an EAS where DNAI in the EAS profile is common to all UEs in the group. - available at the ECS-ER, then the ECS-ER returns the common EAS information to the EES as described in clause 8.20.2.3, or the EES identifies the EAS based on the provided EAS discovery filters and the UE location when E2E response time is received, furthermore: - When the UE is in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE can connect either common EAS or EAS registered to this EES, which is UE is in the common EAS service area), the EES identifies either common EAS or EAS registered to this EES for the UE based on the UE location, EAS discovery filters, application group profile, common EAS KPI as specified in table 8.2.5 and EDN information, registered EAS KPI as specified in table 8.2.5 and EDN information, and inter-EAS communication performance of these EASs and checks whether the common EAS or EAS registered to this EES can provide better E2E response time. - When the UE is not in the overlapping area between the EDNs of common EAS and EAS registered to this EES (e.g. UE is not in the common EAS service area), the EES checks with the ECS-ER about all available common EASs and their corresponding EDNs by using procedure defined in clause 8.20.2.2. If no common EAS is available for the group, EES identifies one EAS for the group and interacts with the ECS-ER to store the common EAS information as described in clause 8.20.2.3. If current EDN already has available common EAS, the EES selects such common EAS considering UE location, EAS discovery filters, application group profile and the required E2E response time. If there are available common EASs in other EDNs, the EES identifies candidate EAS(s) for the group from its registered EASs and checks if the candidate EAS(s) can satisfy the E2E response time considering UE location, EAS discovery filters, application group profile, and the response time of the candidate EAS(s), the response time of available common EASs in other EDNs and the inter-EAS communication latency. If the E2E response time cannot be satisfied, the EES will respond the EEC in step 3 indicating such a rejection reason; otherwise, the EES selects a candidate EAS and interacts with the ECS-ER to store it as common EAS as described in clause 8.20.2.3. NOTE 6: The EES can use ADAE analytics service in 3GPP TS 23.436 [28] or local configuration to obtain application performance (e.g. latency) for inter-EAS communication. NOTE 7: Details of the UE-specific service information and how it is available at the EES is out of scope. NOTE 8: Both steps are evaluated prior to sending a response. Upon receiving the request from the EEC, the EES may also collect edge load analytics from ADAES (as specified in clause 8.8.2 of TS 23.436 [28]) or performance data from OAM to find whether the EAS(s) satisfies the Expected AC service KPIs or the Minimum required AC Service KPIs. Upon receiving the request from the EEC, if the EEC does not indicate EAS Instantiation Triggering Suppress in the EAS Discovery request, the EES may trigger the ECSP management system to instantiate the EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.12. Otherwise, upon receiving the request from the EEC, if the EEC indicates EAS Instantiation Triggering Suppress in the EAS Discovery request and the EES supports such capability, the EES determines not triggering the ECSP management system to instantiate the EAS and may determine Instantiable EAS Information for EAS(s) that are instantiable but not yet instantiated and match the EAS discovery filter IEs. Instantiable EAS Information is provided in the EAS Discovery response and includes the EASID(s) and, for each EASID, the status indicating whether the EAS is instantiated or instantiable but not yet instantiated. If the EEC provides in the EAS discovery request the EAS selection request indicator, the EES selects EAS satisfying the EAS discovery filter or based on other information (e.g. ECSP policy) as described above (if no EAS discovery filter received), and then provides the selected EAS information to the EEC in the discovered EAS list of EAS discovery response. NOTE 9: Without EAS selection request indication, the EES handling is as per R17 procedure. If the tunnel information is received, the EES additionally takes it into consideration in identifying EAS(s). If no tunnel information is received, the EES additionally takes the N6 tunnel (e.g. L2TP) information from 3GPP core network (via NEF user plane path management service as described in 3GPP TS 23.501 [2], clause 5.6.7) or NATed UE IP address (e.g. by local UPF based on local configuration) and EAS IP address information into consideration in identifying EAS(s). For instance, the IP address(es) of identified EAS(s) needs to be topologically close to the IP address of the tunnel server, local UPF (based on NATed UE IP address) or NATed UE for optimal N6 route. 3. If the processing of the request was successful, the EES sends an EAS discovery response to the EEC, which includes information about the discovered EASs and Instantiable EAS Information. For discovered EASs, this includes endpoint information. If the EES perform traffic influence for EAS(s) in step 2, then the discovered EAS(s) is with optimized traffic route. Depending on the EAS discovery filters received in the EAS discovery request, the response may include additional information regarding matched capabilities, e.g. service permissions levels, KPIs, AC locations(s) that the EASs can support, ACR scenarios supported by the EAS, etc., and regarding satisfied energy requirements, e.g., supported energy type. The EAS discovery response may contain a list of EASs and Instantiable EAS Information with EAS instantiation completion time. This list may be based on EAS discovery filters containing a Geographical or Topological Service Area, e.g. a route, included in the EAS discovery request by the EEC. When the discovered EAS is for a certain application group, then the Application Group ID is also included in the response message. If the discovered EAS is registered to another EES, then the EES endpoint of the EES where the discovered EAS is registered is also included in the response message. When the EES accepts the request and determines to trigger the EAS instantiation, then the response may indicate that the EAS instantiation is in progress so that the detailed EAS profile information will be available later. Then the EES determines the remaining EAS instantiation time or EAS instantiation completion time based on the timing receiving EAS instantiation in progress ECSP management system and the predicted EAS deployment time. When EEC receives the EAS instantiation in progress indication, the EEC may send EAS discovery subscription request message if not subscribed yet or send EAS discovery request message later to the EES for obtaining updated EAS information. If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause. If the EEC is not registered with the EES, and ECSP policy requires the EEC to perform EEC registration prior to EAS discovery, the EES shall include an appropriate failure cause in the EAS discovery response indicating that EEC registration is required. If the UE location and predicted/expected UE locations, provided in the EAS discovery request, are outside the Geographical or Topological Service Area of an EAS, then the EES shall not include that EAS in the discovery response. The discovery response may include EAS(s) that cannot serve the UE at its current location if a predicted/expected UE location was provided in the EAS discovery request. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiated (i.e., an EAS profile was provided), the EEC uses the endpoint information for routing of the outgoing application data traffic to EAS(s), as needed, and may provide necessary notifications to the AC(s). The EEC may use the border or overlap between EAS Geographical Service Areas for service continuity purposes. The EEC may cache the EAS information (e.g. EAS endpoint) for subsequent use and avoid the need to repeat step 1. If the Lifetime IE is included in the response, the EEC may cache the EAS information only for the duration specified by the Lifetime IE. Upon receiving the EAS discovery response, if the EEC selects an EAS which is instantiable but not yet instantiated (i.e. an EAS profile is not provided), the EEC sends the EAS information provisioning request indicating the selected EASID as in clause 8.15. If the EAS discovery response message contains the EAS(s) information along with r EAS instantiation completion time which the EAS instantiation in progress, then the EEC determines whether to identify the EAS which instantiation in progress as T-EAS based on Prediction expiration time and remaining EAS instantiation time/EAS instantiation completion time. If the EEC determines to identify the EAS for which instantiation is in-progress, the EEC may retry to discover the EAS and send the EAS discovery request again after waiting for the time as included in the response message, otherwise, the EAS discovery may fail. NOTE 10: Within the duration specified by the Lifetime IE, the cached EAS Profile can be updated (e.g. according to notifications from the EES for changes of EAS information due to EAS status change) or the cached EAS Profile can be invalidated due to new EAS information discovery (e.g. due to UE mobility). The EEC can update or invalidate the cached EAS information (e.g. on PDU Session Release or Modification Command). NOTE 11: The AC can cache the EAS information (e.g. EAS endpoint) for subsequent use. In the case of the cached information needing to be updated or invalidated, the mechanisms for the EEC to notify the AC is up to implementation and is not specified in the current release of the present document. NOTE 12: The EEC can use the EAS information provided by the discovery procedure to perform service continuity planning, for example when ultra-low latency ACR is required. If the EAS discovery request fails, the EEC may resend the EAS discovery request, taking into account the received failure cause. If the failure cause indicated that EEC registration is required, the EEC shall perform an EEC registration before resending the EAS discovery request. NOTE 13: As long as a proper EAS (e.g. considering expected AC service KPIs included in EAS discovery request) is discovered and selected by the EES, EEC of a constraint UE can stop sending EAS discovery to rest candidate EES(s), and provide the selected EAS information to AC.

The information flow for EAS discovery as specified in clause 8.5.3.2 of 3GPP TS 23.558 [aa] is enhanced as follows (new text in bold italics):

#### 8.5.3.2 EAS discovery request Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element. Table 8.5.3.2-1: EAS discovery request Information element; Status; Description. Requestor identifier; M; The ID of the requestor (e.g. EECID). UE Identifier; O; The identifier of the UE (i.e., GPSI). Security credentials; M; Security credentials resulting from a successful authorization for the edge computing service.. EAS discovery filters; O; Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2.. UE location; O; The location information of the UE. The UE location is described in clause 7.3.2.. List of UE IDs; O; List of UE IDs in an Application Group, applicable for S-EAS or S-EES triggered EAS discovery request.. Serving MNO information (NOTE 2); O; The serving MNO information (e.g. MNO name, PLMN ID) which is serving the subscriber.. Target DNAI (NOTE 1); O; Target DNAI information which can be associated with potential T-EAS(s). EEC Service Continuity Support; O; Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR.. EES Service Continuity Support (NOTE 1); O; The IE indicates if the S-EES supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR according to clause 8.8.2.5, which ACR scenario is to be used for the ACR.. EAS Service Continuity Support (NOTE 1); O; The IE indicates if the S-EAS supports service continuity or not. The IE also indicates which ACR scenarios are supported by the S-EAS or, if the EAS discovery is used for an S-EAS decided ACR according to clause 8.8.2.4, which ACR scenario is to be used for the ACR.. EAS Instantiation Triggering Suppress; O; Indicates to the EES that EAS instantiation triggering should not be performed for the current request, and Instantiable EAS Information (e.g. instantiated, instantiable but not be instantiated yet) is to be provided in response.. EAS selection request indicator; O; Indicates the request for EAS selection support from the EES (e.g., for constrained device).. Indication of service continuity planning; O; Indicates that this EAS discovery request is triggered for service continuity planning.. Prediction expiration time; O; The estimated time the UE may reach the Predicted/Expected UE location or EAS service area at the latest. This IE is used by EES as analytics input to get edge load analytics information from ADAES service as described in clause 8.8 of TS 23.436 [28].. Tunnel information; O; It includes service provider ID, the endpoint address (e.g. IP address) of the tunnel server associated with application(s).. NOTE 1: This IE shall not be included when the request originates from the EEC. NOTE 2: This IE shall be included if edge node sharing is used.. Table 8.5.3.2-2: EAS discovery filters Information element; Status; Description. List of AC characteristics (NOTE 1); O; Describes the ACs for which a matching EAS is needed.. > AC profile (NOTE 2); M; AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1.. > Application group profile (NOTE 6); O; Application group profile associated with the AC Profile, as defined in Table 8.2.11-1.. List of EAS characteristics (NOTE 1, NOTE 3); O; Describes the characteristic of required EASs.. > EASID; O; Identifier of the required EAS.. > Application Group ID; O; Application group identifier as defined in 7.2.11.. > EAS content synchronization support; O; Indicates if the EAS content synchronization support is required or not.. > Bundle ID (NOTE 5); O; A list of EASIDs or a bundle ID as described in clause 7.2.10.. > List of EASIDs (NOTE 5); O; A list of EASIDs specific to a particular EAS bundle.. > Bundle type (NOTE 4); O; Type of the EAS bundle as described in clause 7.2.10. > EAS bundle requirements (NOTE 4); O; Requirements associated with the EAS bundle as described in clause 8.2.10.. > EAS provider identifier; O; Identifier of the required EAS provider. > EAS type; O; The category or type of required EAS (e.g. V2X, UAV, application enabler). > EAS schedule; O; Required availability schedule of the EAS (e.g. time windows). > EAS Geographical Service Area (NOTE 6); O; Location(s) (e.g. geographical area, route) where the EAS service should be available.. > EAS Topological Service Area (NOTE 6); O; Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1.. > Service continuity support; O; Indicates if the service continuity support is required or not.. > Service permission level; O; Required level of service permissions e.g. trial, gold-class. > Service feature(s); O; Required service features e.g. single vs. multi-player gaming service. > Energy type; O; Indicates what energy type support is required, e.g. non-renewable energy, renewable energy.. NOTE 1: Either "List of AC characteristics" or "List of EAS characteristics" shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. NOTE 4: When EAS discovery request is sent by the EEC, this IE shall not be included. NOTE 5: “Bundle ID" and "List of EASIDs" shall not both be present. NOTE 6: If application group profile IE is present, the expected group geographic service area IE present in the application group profile is preferentially used. If there is no EAS satisfied the expected group geographic service area or EAS does not provide better E2E response time, then the UE location and other service area IEs are considered..

The information flow of EAS Profile for EAS registration and discovery as specified in clause 8.2.4 of 3GPP TS 23.558 [aa] is enhanced as follows (new text in bold italics):

### 8.2.4 EAS Profile An EAS Profile includes information about an EAS used to describe services and service characteristics offered. NOTE: Information elements in the EAS Profile are provided by the ASP. Table 8.2.4-1: EAS Profile Information element; Status; Description. EASID; M; The identifier of the EAS. EAS Endpoint; M; Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS.. List of EAS bundle information; O; List of EAS bundles to which the EAS belongs and related bundling requirements.. > Bundle ID (NOTE 3); O; Bundle ID as described in clause 7.2.10.. > List of EAS IDs (NOTE 2, NOTE 3); O; List of the EAS IDs of the EASs to be invoked by the EAS for an EAS driven association of EASs.. > Bundle type; M; Type of the EAS bundle as described in clause 7.2.10. > Main EASID; O; Indicate which EAS in a bundle takes the main EAS service role.. > EAS bundle requirements; O; Requirements associated with the EAS bundle as described in clause 8.2.10.. ACID(s); O; Identifies the AC(s) that can be served by the EAS. EAS Provider Identifier; O; The identifier of the ASP that provides the EAS.. Allowed MNO information (NOTE 4); O; Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services. EAS Type; O; The category or type of EAS (e.g. V2X, UAV, application enabler). EAS description; O; Human-readable description of the EAS. EAS Schedule; O; The availability schedule of the EAS (e.g. time windows). EAS Geographical Service Area; O; The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served.. EAS Topological Service Area; O; The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1.. EAS Service KPIs; O; Service characteristics provided by EAS, detailed in Table 8.2.5-1. EAS service permission level; O; Level of service permissions e.g. trial, gold-class supported by the EAS. EAS Feature(s); O; Service features e.g. single vs. multi-player gaming service supported by the EAS. EAS content synchronization support; O; Indicates if the EAS supports content synchronization between EASs.. EAS Service continuity support; O; Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability (e.g. EAS bundle information) of handling bundled EAS coordinate ACR.. EAS Transport layer service continuity support; O; This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation. General context holding time duration (NOTE 1); O; The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC.. List of EAS DNAI(s); O; DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides.. List of N6 Traffic Routing requirements; O; The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI.. EAS Availability Reporting Period; O; The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration.. EAS Status; O; The status of the EAS (e.g. enabled, disabled, overload warning etc.). List of associated devices; O; List of associated devices (e.g. haptic device, joy stick) required along with UE in order to provide service to the user. It includes device type.. Energy type; O; Indicates what energy type is supported by the EAS, e.g. non-renewable energy, renewable energy.. NOTE 1: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). NOTE 2: This IE may be provided when only bundle ID is provided, and the bundle type indicates the proxy bundle. NOTE 3: At least one of the IEs shall be present if EAS bundle information is provided. NOTE 4: For edge node sharing scenario, in order to restrict the access to the subscriber of the partner operator, this IE should only include MNO information of the leading operator.. NOTE: The EAS Transport layer service continuity support can be used in EAS discovery, e.g. as described in 3GPP TS 23.433 [26] for SEALDD server acting as EAS, which can further support the EAS IP replacement function.

Editor’s Note:	How EES gets the information of energy type supported by an EAS is FFS.

### 6.A.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.A.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***