3GPP TSG-SA WG6 Meeting #70	S6-255098

Dallas , US, 17th – 21st November 2025

Source:	China Telecom

Title:	New solution for KI#1 on AIML model storage and deployment on satellite

Spec:	3GPP TR 23.700-002 v0.2.0

Agenda item:	9.12

Document for:	Approval

Contact:	Zhou Zhe <zhouz11@chinatelecom.cn>

Xu Xia <xiaxu@chinatelecom.cn>

1. Introduction

Application enabled layer could enhance AIML service by deploying models on satellites, thereby reducing data transmission burden and latency. It is necessary to study how to deploy AIMLE on satellite and how to storage AIML models on satellite for AIMLE service. Thus, TR 23.700-02 v0.1.0 identifies the following  key issue to be addressed:

Key issue #1: AIML model storage and deployed on satellite

This paper proposes a new solution on KI#1, to store and deploy AIML models on satellite. The information of satellite stored AIML model is provided to AIMLE server and stored in ML repository. When AIMLE client asks for AIMLE model downloading, the AIMLE server retrieve AIML model information in the ML repository according to requirement of AIMLE client. If the needed AIMLE model is deployed far away from the AIMLE client, such situation leads to high latency on AIMLE service. As a result, the AIMLE server may consider retrieving a satellite deployed AIMLE model and the AIMLE client could download the model from the satellite via satellite access. Also, when mutiple AIMLE clients in the coverage of a satellite and need to download the same specific AIML model, they may directly download the required model from the satellite via satellite access.

2. Reason for Change

To support AIML model storage and deployment on satellite.

3. Conclusions

This paper proposes solution on AIML model storage and deployment on satellite.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-002 v0.1.0.

* * * First Change * * * *

5.X	Solution#X: AIML model storage and deployment on satellite

5.X.1	Solution Description

This solution proposes to address the following open issues in KI#1

-	Investigate different deployment options for AIMLAPP when AIML models are deployed on board satellite. Investigate different deployment options for AIMLAPP when AIML models are deployed on satellite and study whether/how to deploy AIMLE on satelllite (e.g. ML repository/ADAES on satellite to perform data processing/interference based on collected remote data).

-	Whether/how could AIML model discovery and storage be optimized.Whether/how to storage AIML models for AIMLE (e.g. Light weight models storaged on remote sensing satellite to process collected remote data, Model transmission and sharing between satellites and terrestrial network).

5.X.1.1 General

Figure 5.X.1.1-1: satellite AIMLE deployment

In this deployment option, ​​the​​ UE with AIMLE client is on the ground or in the air (e.g., ​​a drone​​), requiring AIML models retrieval from AIMLE server. The AIMLE server is deployed on the ground with an ML repository. The space AIMLE component is deployed on ​​a​​ satellite, performing as a VAL server in the satellite EDN containing multiple AIML models. The RAN (e.g., gNB) can be deployed on ​​the​​ ground/sea (e.g., on a ship). The 5GS control plane functions (e.g., AMF, SMF) are deployed on the ground, ​​though they are not shown in the figure for simplicity​​. The satellite EDN can provide the information of AIML models located in satellite EDN to AIMLE server through AIML-S via space. Then the AIMLE server stores satellite deployed AIML models information with satellite EDN information in the ML repository.

Light weight AIML models are deployed on a satellite and their information is provided to AIMLE server on ground. When the AIMLE client in a UE requests the AIMLE server to discovery the proper AIML models, the AIMLE server retrieve AIML model information in the ML repository and discovers the required AIML models.

In the proposed scenario, the discovered AIML models contain those satellite deployed models that satisfy the requirement of the AIMLE client. When the UE is in an area far away from the ML repository, it may bring high latency to the UE’s AIMLE service if the AIMLE client selects the specific AIML model in the ML repository and decides to download it from the ML repository. As a result, the AIML client can choose the satellite stored AIML model instead and directly download it from the satellite EDN.

In another scenario, there are mutiple UEs that request the AIMLE server to discover an AIML model with the same requirement, in order to perform federated learning. When some of the mutiple UEs are deployed in the coverage of one satellite and the satellite EDN contains the required AIML model, the AIMLE clients in the coverage can select the satellite EDN deployed AIML model and directly download it from the satellite.

5.X.1.2	Satellite ML model retrieval

Figure 5.Y.1.2-1 illustrates the procedure of satellite ML model retrieval.

Pre-conditions:

1.	The AIMLE client or VAL server has received information (e.g., URI, IP address) related to the AIMLE Server.

2.	The AIMLE client or VAL server has received security credentials authorizing it to communicate with the AIMLE server.

Figure 5.Y.1.2-1: Satellite ML model retrieval

The corresponding procedure in detail is as follow:

- The AIMLE client sends a ML model retrieval request to the AIMLE server as described in TS 23.482 clause 8.2.2.2.

- Upon receiving the request from the requestor, the AIMLE server performs validation as descrived in TS 23.482 clause 8.2.2.2.

- The AIMLE server sends a ML model retrieval response to the requestor. The response includes an indication of success and the ML model information containing ML model related satellite information.

5.X.1.3	Information Flows

5.X.1.3.1	ML model retrieval response

Table 5.X.1.3.1-1 describes information elements of ML model retrieval response from the AIMLE server to the AIMLE client.

Table 5.X.1.3.1-1: ML model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 2) | Indicates that the ML model retrieval request was successful. |
| >List of ML models | M | List of retrieved ML models; the nested information elements are provided for each ML model in the list. |
| >> ML model identifier | M | The ML model identifier. |
| >> ML model retrieval endpoint (NOTE 1) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| >> ML model related satellite id | M | The identifier of satellite where the ML model is located on. |
| >> ML model (NOTE 1) | O | The ML model containing a retrieved ML model. |
| Failure response | O (NOTE 2) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE 1: At least one of these information elements is needed. NOTE 2: One of the IEs shall be present. |  |  |

5.Y.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

5.Y.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

5.Y.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

* * * End of Changes * * * *