3GPP TSG-SA WG6 Meeting #70	S6-255xxx

Dallas, United States 17th – 21st November 2025	(revision of S6-255176)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0058 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registration and discovery of VAL client as AI/ML member |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI-20 |  |  |  |  |  | Date: |  |  | 2025-10-6 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the exsiting TS, there is decription about AIMLE clients discovery and selection which used for VAL server. However the AIMLE client information is unuseful for VAL server since there is no interface between VAL server and AIMLE clients for anything potentical interation for the usage of the discovered AIMLE client. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correcting the un-useful case about VAL server discove rthe AIMLE client, and Add some text about how to support VAL client discovery in 6.7. - There is no any interafce between VAL server and a seperated AIMLE client entity to support the potential AI/ML task operation requested by VAL server following AIMLE client selection. So rewording the description of 6.12 to make it linked to VAL client intergaration case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect usacase about discoverying AIMLE client for VAL server exists in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7, 6.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************************Begin of the change**********************************

6.7	Support for AIMLE client/VAL client discovery

This functionality enables the VAL to discover available VAL clients for AI/ML operations, such as training or inference. The AIMLE server provides the functionality to select suitable VAL clients that fulfill the discovery criteria.

AIMLE server also support the functionality to select suitable AIMLE clients for participating AI/ML operation required by AIMLE server e.g., HFL training as specified in clause 8.12.

******************************Begin of the change**********************************

6.12	Support VAL client selection subscription and notification

The VAL client selection subscription request and notification to enable VAL Servers to subscribe for monitoring AIML members who meet criteria for performing an AIMLE service, selecting VAL clients and receiving notification when there is an update on the selected and re-selected VAL client’s status when re-selection is performed according to AIML member selection criteria.

******************************Next change**********************************

8.8	VAL client discovery

8.8.1	General

Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable VAL clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered VAL clients to select a set of VAL clients to perform AI/ML operations.

The following clauses specify procedures, information flows, and APIs for AIMLEVAL discovery.

8.8.2	Procedure

8.8.2.1	VAL client discovery

Pre-conditions:

1.	VAL clients that support AI/ML operations have registered with the AIMLE server and included their profiles and optionally a list of supported services by acting as FL member in 8.4.

2.	The AIMLE server can access a ML repository to obtain VAL client profiles and supported services associated with VAL clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends a VAL client discovery request to an AIMLE server to discover a list of VAL clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The request may also include VAL client task capability requirements to discover clients who can perform the AIML tasks like AIML model training/offload/split with compute requirements and task performance preference like green task performance. The AIMLE VAL discovery request includes information as described in Table 8.8.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover VAL clients. If the requestor is authorized, the AIMLE server performs the following to discover VAL clients fulfilling the provided VAL client discovery criteria.

The AIMLE server also obtains candidate VAL clients on UEs from the ML repository.

From the list of candidate VAL clients, the AIMLE server discovers a list of VAL clients that fulfils the discovery criteria based on supported services and VAL client profiles.

The AIMLE server determines whether the discovered VAL clients fulfil the location information in the discovery criteria by using their identifiers to determine their location and includes in the discovery response only those VAL clients that meet the requirements. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the VAL clients which fulfil the location requirement.

If the available VAL clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of VAL clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server may discover the remaining required VAL  clients from other AIMLE servers over AIML-E reference point and include them in the response message.

3.	The AIMLE server sends an VAL client discovery response that includes information in Table 8.8.3.2-1.

If the required number of VAL clients are not included in the response message from the AIMLE server (i.e., the VAL  clients in the response message are less than the required minimum number of VAL  clients), then the AIML service consumer may discover the remaining required VAL clients from other AIML enablement server indicating the required number of VAL  clients. The response may also include information related to supported tasks with guaranteed task KPIs, available AIML tasks service duration, AIML task expected KPI like latency.

8.8.3	Information flows

8.8.3.1	VAL client discovery request

Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure.

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| VAL client discovery criteria | M | Discovery criteria for finding suitable VAL clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of required VAL clients | O | Indicates the requested number of VAL clients to be discovered based on the discovery criteria. |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Service requirement | M | Information about the required service |
| > VAL Service ID | M | VAL Service ID that the client is required to support. This identifies the service associated with the requester. |
| > Service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split. |
| Application layer VAL client capabilities | M | Application layer VAL client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| VAL client task capability requirements | O | Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, Energy-efficient, low costs) |
| VAL client velocity | O | Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static. |
| Location information | O | Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID) to discover the AIMLE clients. |
| VAL client QoS requirements | O | Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients. |

8.8.3.2	VAL client discovery response

Table 8.8.3.2-1 shows the response sent by the AIMLE server to the VAL server for the VA: client discovery procedure.

Table 8.8.3.2-1: AIMLE client discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of VAL client IDs | M | A list of VAL client IDs that matches the AIML discovery criteria. |
| AIML supported tasks | O | Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks. |

8.9	VAL client selection

8.9.1	General

The VAL client selection procedure has two modes of operation: VAL server selection and AIMLE server selection. In VAL server selection, a List of VAL client IDs is provided to form a VAL client set, which can be used for AI/ML operations (e.g., training). In AIMLE server selection, a VAL server provides VAL client selection criteria and number of the required VAL clients for the AIMLE server to select the VAL client set.

The following clauses specify procedures, information flows, and APIs for VAL client selection.

8.9.2	Procedure

8.9.2.1	VAL client selection

Pre-conditions:

1.	VAL clients that support AI/ML operations have registered with the AIMLE server and included their VAL client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain VAL client profiles and supported services associated with the VAL clients.

Figure 8.9.2.1-1: VAL client selection

1.	A VAL server sends an VAL client selection request to an AIMLE server to select VAL clients available for participation in AI/ML operations (e.g., is available and have required data to train an ML model). The VAL client selection request includes information as described in Table 8.9.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select VAL clients.

3.	If the requestor is authorized, the AIMLE server performs VAL client selection based on the provided inputs as described below.

For VAL server selection, the AIMLE server receives a list of VAL client IDs in the request and selects the VAL clients in the list as candidate VAL clients.

For AIMLE server selection, the AIMLE server retrieves a list of clients with AI/ML capabilities from the ML repository. From the list of clients, the AIMLE server selects a list of candidate VAL clients, whose client profiles fulfil the VAL client selection criteria. The AIMLE server may use SEAL (LM service), NEF (e.g., Monitoring Event API), and NWDAF (e.g., UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [2]) capabilities to assist the VAL client selection.

The AIMLE server determines UEs by using their identifiers to determine their location and then selects only those that fulfil the location requirements specified in the VAL client selection criteria. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement.

The AIMLE server may then determine the QoS parameters for the AIML traffic session between the requestor and the candidate VAL client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). 	The AIMLE server determines the application QoS parameters based on the VAL Service ID.

NOTE 1:	To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated VAL client QoS requirements in the VAL client selection criteria provided in step 1.

NOTE 2:	If the available VAL clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required number of VAL clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server can discover the remaining required VAL clients from other AIMLE servers over AIML-E reference point and include them in the VAL client selection response message.

NOTE 3:	The AIMLE server can reuse SEAL group management for any necessary group management.

4.	The AIMLE server performs VAL client participation procedure with each candidate VAL client as described in clause 8.10.

For all candidate VAL clients that agreed to participate in AI/ML operations, the AIMLE server selects the VAL clients and assigns an VAL client set identifier for the selected clients. The VAL client set may then be used for training a ML model.

NOTE 4:	If a required minimum number of VAL clients for AI/ML operation is provided in the VAL client selection request and the number of VAL clients that agreed to participate in AI/ML operations is less than this number, an VAL client set identifier will not be assigned and the status will be set to fail in the VAL client selection response in step 5.

5.	The AIMLE server sends an VAL client selection response that includes information in Table 8.9.3.2-1.

8.9.3	Information flows

8.9.3.1	VAL client selection request

Table 8.9.3.1-1 shows the request sent by a VAL server to an AIMLE server for the VAL client selection procedure.

Table 8.9.3.1-1: VAL client selection request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| List of VAL client IDs | O (NOTE 1) (NOTE 3) | A list of VAL client IDs that was previously discovered for inclusion into an VAL client set. |
| VAL client selection criteria | O (NOTE 2) | Selection criteria for finding suitable VAL clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of required VAL clients | O (NOTE 2) (NOTE 3) | Indicates the requested number of VAL clients to be selected based on selection criteria. |
| Minimum number of required VAL clients | O | Indicates the minimum acceptable number of VAL clients to be selected. |
| NOTE 1: This information element is only present for VAL server selection. NOTE 2: These information elements are only present for AIMLE server selection. NOTE 3: At least one of the information elements are present. |  |  |

8.9.3.2	VAL client selection response

Table 8.9.3.2-1 shows the response sent by the AIMLE server to the VAL server for the VAL client selection procedure.

Table 8.9.3.2-1: VAL client selection response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| VAL client set identifier | O (NOTE 1) | An identifier to associate with the set of VAL clients that the VAL server has provided in the request. The VAL client set can be updated by using this identifier. |
| List of VAL client IDs | O (NOTE 1) (NOTE2) | A list of VAL client IDs that matches the AIML selection criteria. |
| NOTE 1: This is present if the response status is success. NOTE 2: This information element is present for dynamic selection. |  |  |

******************************End of the change**********************************