3GPP TSG-SA WG6 Meeting #70	S6-255532

Dallas, United States 17th – 21st November 2025	(revision of S6-255155)

Source:	Samsung

Title:	Pseudo-CR on AIoT service functional model

Spec:	3GPP TR 23.700-26 V0.2.0

Agenda item:	9.8

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR introduces a functional model for the AIoT service. It proposes to introduce the new functional entities as part of the Vertical application layer since the services exposed by AIoT entities are specific to AIoT applications. Similar to V2X and UASAPP, AIoT services entities should be part of Application enabler layer and not services layer.

2. Reason for Change

This pCR proposes a solution for the Key issue #1 : Enhance Application enablement layer for Ambient IoT services

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-26 V0.2.0.

* * * First Change * * * *

## 7.x	Solution #X: New functional model for AIoT services

### 7.x.1	Solution description

This solution addresses the KI#1. The following procedures introduces the on-network functional model for AIoT services capturing the relevant functional entities and reference points. This solution aligns with the network architecture for AIoT services which specified in clause 4.2 of 3GPP TS 23.369 [3].

#### 7.x.1.1	On-network Functional model for AIoT services

Figure 7.x.1.1-1 illustrates the generic on-network functional model for AIoT service.

Figure 7.x.1.1-1: On-network functional model for AIoT services

The AIoTE server, acting as an AF, could obtain AIoT data information from the 3GPP core network via NEF (over N33 reference point) which is defined in clause 4.5.4 of 3GPP TS 23.369 [3]. If the AIoTE server is a trusted AF, it may interact with AIOTF directly for AIoT related service as defined in clause 4.5.5 of 3GPP TS 23.369 [3]. The AIoTE server may communicate with other SEAL server(s) over SEAL-S reference point.

The VAL server(s) or AIoT application specific server(s) could communicate with the AIoT management server over AIoT-S reference point for requesting the AIoT services and receiving the AIoT data information.

The AIoTE client, acting as a consumer, could also request the AIoT services to the AIoTE server over AIoT-AE reference point, and provide the support for AIoT related functions to the VAL client(s) over AIoT-C reference point.

NOTE: In this release, the AIoTE client only acts as an AIoT service consumer.

#### 7.x.1.2	Functional entities description

AIoTE server (AIoT Enabler Server): Functionalities offered by the AIoTE server are similar to AMS entity described in clause 7.1.1.2.

AIoTE client (AIoT Enabler Client): This entity is similar to AMC entity described in clause 7.1.1.2.

### 7.x.2	Architecture Impacts

This solution proposes a new architecture and functional mode for AIoT services.

### 7.x.3	Corresponding APIs

None

### 7.x.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End Change * * * *