3GPP TSG CN/ETSI SMG3	TDoc NP-99103

Sophia Antipolis, France

01-04 June, 1999

| CHANGE REQUEST No : |  |  |  |  | A591 |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 04.08 |  | Version: |  | 7.0.1 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Technical enhancements and improvements |
| --- | --- |

| Source: | Siemens AG | Date: | 01.06.1999 |
| --- | --- | --- | --- |

| Subject: | Inconsistency in definition of comprehension required IEs |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | The definition of an IE coded as comprehension required is currently defined in GSM 04.07 and 04.08. To simplify the specification and to avoid ambiguous definitions, it is proposed to remove the definition from GSM 04.08. |
| --- | --- |

| Clauses affected: | 8.5; 8.6.1; 8.6.2; 10.5 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications | x |  List of CRs: | CR 04.07 A025 |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: | This CR shall only be approved if CR 04.07 A025 " Inconsistency in definition of comprehension required IEs " is approved. |
| --- | --- |

## 8.5	Non-semantical mandatory information element errors

When on receipt of a message,

-	an "imperative message part" error; or

-	a "missing mandatory IE" error

is diagnosed or when a message containing:

-	a syntactically incorrect mandatory IE; or

-	an IE unknown in the message, but encoded as "comprehension required" (see section GSM 04.07); or

-	an out of sequence IE encoded as "comprehension required" (see section GSM 04.07)

is received,

-	the mobile station shall proceed as follows:

If the message is not one of the messages listed in sections 8.5.1, 8.5.2, and 8.5.3, the mobile station shall ignore the message except for the fact that, if an RR connection exists, it shall return a status message (STATUS, RR STATUS or MM STATUS depending on the protocol discriminator) with cause # 96 "invalid mandatory information". If the message was a GMM message the GMM-STATUS message with cause #98 “Message type not compatible with protocol state” shall be returned. If the message was an SM message the SM-STATUS message with cause # 96 "invalid mandatory information" shall be returned.

-	the network shall proceed as follows:

When the message is not one of the messages listed in section 8.5.3 b), c), d) or e), the network shall either

-	try to treat the message (the exact further actions are implementation dependent), or

-	ignore the message except that it should return a status message (STATUS, RR STATUS, MM STATUS (depending on the protocol discriminator), GMM STATUS, or SM STATUS) with cause # 96 "invalid mandatory information".

next modification

## 8.6	Unknown and unforeseen IEs in the non-imperative message part

### 8.6.1	IEIs unknown in the message

The MS shall ignore all IEs unknown in a message which are not encoded as "comprehension required"(see GSM 04.07).

The network shall take the same approach.

### 8.6.2	Out of sequence IEs

The MS shall ignore all out of sequence IEs in a message which are not encoded as "comprehension required"(see GSM 04.07).

The network should take the same approach.

## 10.5	Other information elements

The different formats (V, LV, T, TV, TLV) and the four categories of information elements (type 1, 2, 3, and 4) are defined in GSM 04.07.

The first octet of an information element in the non-imperative part contains the IEI of the information element. If this octet does not correspond to an IEI known in the message , the receiver shall determine whether this IE is of type 1 or 2 (i.e. it is an information element of one octet length) or an IE of type 4 (i.e. that the next octet is the length indicator indicating the length of the remaining of the information element) (see GSM 04.07).

This allows the receiver to jump over unknown information elements and to analyse any following information elements.

The information elements which are common for at least two of the three protocols Radio Resources management, Mobility Management and Call Control, are listed in section 10.5.1.

The information elements for the protocols Radio Resources management, Mobility Management and Call Control are listed in sections 10.5.2, 10.5.3 and 10.5.4 respectively. Default information element identifiers are listed in annex K.

NOTE:	Different information elements may have the same default information element identifier if they belong to different protocols.

The descriptions of the information element types in sections 10.5.1, 10.5.2, 10.5.3, and 10.5.4 are organized in alphabetical order of the IE types. Each IE type is described in one subsection.

The subsection may have an introduction:

-	possibly explaining the purpose of the IE;

-	possibly describing whether the IE belongs to type 1, 2, 3, 4 or 5;

-	possibly indicating the length that the information element has if it is either type 5 or if it is used in format TV (type 1 and 3) or TLV (type 4).

A figure of the subsection defines the structure of the IE indicating:

-	possibly the position and length of the IEI. (However it depends on the message in which the IE occurs whether the IE contains an IEI.);

-	the fields the IE value part is composed of;

-	possibly the position and length of the length indicator. (However it depends on the IE type whether the IE contains a length indicator or not.);

-	possibly octet numbers of the octets that compose the IE (see clause a) below).

Finally, the subsection contains tables defining the structure and value range of the fields that compose the IE value part. The order of appearance for information elements in a message is defined in section 9.

The order of the information elements within the imperative part of messages has been chosen so that information elements with 1/2 octet of content (type 1) go together in succession. The first type 1 information element occupies bits 1 to 4 of octet N, the second bits 5 to 8 of octet N, the third bits 1 to 4 of octet N + 1 etc. If the number of type 1 information elements is odd then bits 5 to 8 of the last octet occupied by these information elements contains a spare half octet IE in format V.

Where the description of information elements in this Technical Specification contains bits defined to be "spare bits", these bits shall set to the indicated value (0 or 1) by the sending side, and their value shall be ignored by the receiving side. With few exceptions, spare bits are indicated as being set to "0" in GSM 04.08.

The following rules apply for the coding of type 4 information elements:

a)	The octet number of an octet (which is defined in the figure of a subsection) consists of a positive integer, possibly of an additional letter, and possibly of an additional asterisk, see clause f). The positive integer identifies one octet or a group of octets.

b)	Each octet group is a self contained entity. The internal structure of an octet group may be defined in alternative ways.

c)	An octet group is formed by using some extension mechanism. The preferred extension mechanism is to extend an octet (N) through the next octet(s) (Na, Nb, etc.) by using bit 8 in each octet as an extension bit.

The bit value "0" indicates that the octet group continues through to the next octet. The bit value "1" indicates that this octet is the last octet of the group. If one octet (Nb) is present, the preceding octets (N and Na) shall also be present.

In the format descriptions appearing in section 10.5.1 to 10.5.4, bit 8 is marked "0/1 ext" if another octet follows. Bit 8 is marked "1 ext" if this is the last octet in the extension domain.

Additional octets may be defined in later versions of the protocols ("1 ext" changed to "0/1 ext") and equipments shall be prepared to receive such additional octets; the contents of these octets shall be ignored. However the length indicated in sections 9 and 10 only takes into account this version of the protocols.

d)	In addition to the extension mechanism defined above, an octet (N) may be extended through the next octet(s) (N+1, N+2 etc.) by indications in bits 7-1 (of octet N).

e)	The mechanisms in c) and d) may be combined.

f)	Optional octets are marked with asterisks (*).