Source:	Ericsson (Rapporteur GSM 04.08 SM/MM GPRS part)

Topic:	WI,- GPRS

Title:		Summary of 'agreed' Change Requests to GSM 04.08 SM/GMM

Date:		29 May, 1999

Introduction

GSM 04.08 is the Gb interface layer-3 specification used at the radio interface for GPRS network and signalling issues.

GSM 04.08 v6.3.0 R97 was approved by SMG#28 in February 1999, and then v7.0.0 R98 was also released (Note: v7.0.1 is recomended as the base for CR's since v7.0.0 had editorial problems).

GSM 04.08 v6.2.0 was approved by SMG#27 in October 1998.

GSM 04.08 v6.1.0 was approved by SMG#26 in June 1998.

GSM 04.08 v6.0.0 was approved by SMG#25 in March 1998.

This document contains an overview of change requests (CRs) to GSM 04.08 that are related to the GPRS SM/MM part, starting from version 6.0.0 included.

___________________________________________________________________

###### CRs to GSM 04.08 v6.3.0/v7.0.0/v7.0.1 GPRS MM/SM part to be included in v7.1.0

| CR # | Rev | Tdoc N1-99 | Title | Description | SMG3 WPA | SMG3 Plenary | SMG #29 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A701 | 1 | 321 | GPRS Internet Hosted Octet Stream Service (IHOSS) | Codepoints for PDP type PPP and OSP is given in Packet data protocol address IE, and PCO IE is modified accordingly. |  |  |  |
| A621 | 2 | 199 | IE Daylight Saving Time in GMM INFORMATION (This is NITZ but is listed since GMM INF.msg has an IE added for R99)? | By adding a DST parameter to the messages MM INFORMATION and GMM INFORMATION, it is possible for the MS to distinguish between a change in Time Zone and a change in DST. |  |  |  |

###### CRs to GSM 04.08 v6.3.0 GPRS MM/SM part to be included in v6.4.0

| CR # | Rev | Tdoc N1-99 | Title | Description | SMG3 WPA | SMG3 Plenary | SMG #29 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A683 | 0 | 438 | Usage of cause code #10 Implicitely detached | Alignment with 03.60 that SGSN shall reject a RAU if the GMM context does not exist (restart). Cause #10 to be used. | ad-hoc |  |  |
| A681 | 0 | 431 | Improved SM messages error handling for non-semantically mandatory IE errors | At deactivation or request of PDP context the procedure is completed when possible, even if mandatory IE errors occure. | ad-hoc |  |  |
| A679 | 0 | 430 | Deactivation of PDP contexts in case of GPRS detach | Aligning the detach procedure to 03.60, deactivating PDP contexts without peer to peer SM and LLC signalling | ad-hoc |  |  |
| A677 | 0 | 429 | Definition of the PDP address IE | The extension bit in octet 3 is deleted and replaced with spare. | ad-hoc |  |  |
| A675 | 1 | 453 | Abnormal case in the MS when changing cell while trying to attach | New cell update to be performed if a new cell is entered during an ongoing attach procedure, using same random TLLI | ad-hoc |  |  |
| A669 | 0 | 419 | Conditional IE errors | Spesifies the missing error handling for GMM and SM in case of an conditional IE error. | ad-hoc |  |  |
| A665 | 1 | 476 | Correction to PDP-type coding | New coding introduced in PDP Address IE to facilitate sending of empty PDP type | ad-hoc |  |  |
| A659 | 0 | 230 | MS reachable timer | The MS reachable timer stops when READY timer starts, and starts with initial value when READY timer stops/expires. |  |  |  |
| A657 | 1 | 355 | Editorials, and correcting the A&C REQUEST plus INFORMATION messages. | Correcting editorial mistakes and inserting missing Spare half octet IE in A&C REQ., and adjusting layout of GMM INF. msg. |  |  |  |
| A655 | 0 | 265 | Correcting mistakes introduced via CR's 385, 415, 425, 441 incorporation into v6.3.0 | IEI value missing for PDP address in 9.5.2.1 and correcting history part. APN IE reference to 03.03 improved and in 4.2.4.1.2 the option of PLMN-SEARCH is also introduced when max RAU attempt counter value is reached. |  |  |  |
| A653 | 2 | 463 | Reactivation of PDP contexts | At network initiated detach indicating 're-attach required', the MS shall perform attach, but reactivation of PDP contexts are not mandatory any longer (due user action). | ad-hoc |  |  |
| A651 | 4 | 477 | Paging with IMSI | Clarifies the paging for GPRS services with IMSI procedure | ad-hoc |  |  |
| A647 | 0 | 257 | Correction of the SM message collision cases through PDP address. | Collision of network requested and MS initiated PDP context activation detected by same static PDP address is clarified. |  |  |  |
| A637 | 3 | 478 | Initial synchronization after READY timer negotiation | When new READY timer is given by the NW, the MS does cell-update. T3312 starting at value 0 is defined. | ad-hoc |  |  |
| A635 | 0 | 238 | GMM Mobile station identity | As the common Mobile identity IE must be able to carry the P-TMSI for RR purpose (SMG2 CR A476) the GMM Mobile station identity IE is deleted and replaced by Mobile identity IE. |  |  |  |
| A633 | 1 | 261 | Clash between READY timer deactivation and Force to Standby procedure | The Force to Standby has no impact on the READY timer when it is deactivated ,-meaning always running doing cell updates. |  |  |  |
| A631 | 1 | 260 | Maximum length of Access point name and Protocol configuration option IE's | As it is highlighted in Tdoc N1-99142 the maximum length of the APN IE is 100 octets and of the PCO IE it is 250 octets. This CR aligns the definitions given in the appropriate SM message and IE definitions. |  |  |  |
| A629 | 1 | 254 | 'GPRS services and non-GPRS services not allowed':the behavier of the MS. | When this cause is received in ATTACH REJECT the behavier of the MS is specified as for the combined GPRS attach procedure. |  |  |  |
| A587 | 2 | 452 | Gs interface failure | New sub-state GMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM is introduced. For combined procedures when rejected with #16,17 or 22 | ad-hoc |  |  |
| A581 | 0 | 096 | Usage of GMM cause codes | Cause code #7 is introduced in RAU (normal and periodic) while #10 is introduced for combined RAU. Cause #5 (IMEI not accepted) is deleted. |  |  |  |
| A577 | 2 | 262 | Subsequent GMM substate and attach attempt counter | At maximum attempt counter value the substates should be ATTEMPTING -TO-ATTACH and ATTEMPTING-TO-UPDATE instead of PLMN-SEARCH. |  |  |  |
| A573 | 1 | 211 | Completion of error handling for session management | The error handling for session management in case of unknown or unforeseen message types is specified as for CS. |  |  |  |
| A555 | 2 | 464 | New DEREGISTERED 'suspended' sub-state | The state and MS behavior is described | ad-hoc |  |  |
| A553 | 1 | 465 | Clarification of release procedure upon PDP context deactivation | Clarification to release of logical link, meaning local release by both sides without DISC frames | ad-hoc |  |  |
| A527 | 1 | 207 | Description of cause codes in network initiated Detach procedure | Correcting detach type in the network to MS direction, and clarification of MS action on cause codes at DETACH REQUEST when in oper. mode A and B in NMO I. |  |  |  |
| A525 | 0 | 054 | Deactivation of READY timer | When the READY timer is deactivated by the network the MS shall perform cell update whenever a new cell is selected. |  |  |  |
| A523 | 2 | 247 | READY timer handling in SUSPEND state | The READY timer is not affected by transitions to and from the GMM-REGISTERED.SUSPENDED substate, but cellupdates are not performed. |  |  |  |
| A519 | 3 | 258 | Correction to the SM message collision with equal TI to an active PDP context. | Enhancement of TI collision cases to cover the case of an already activated PDP context, deleting detection by comparing contents. Old PDP context is then deleted. |  |  |  |
| A517 | 2 | 245 | Correction of the handling of the Ciphering Key Sequence Number (CKSN) | The ciphering algorithm may be sent in AUTHENTICATION AND CIPHERING REQUEST message during the attach procedure |  |  |  |
| A515 | 4 | 471 | Correction to the handling of subscribed QoS values | QoS IE modified to type 4 with correction of length in SM messages. All 'subscribed' values are valid from MS only else reserved | ad-hoc |  |  |
| A511 | 2 | 226 | Cleaning up on text in general, and especially on attempt counters, PDP access IE length, MS network capability IE and MS identity IE length. | Alignments within 04.08 on attempt counter value as 5 and length on the IE's in different messages.Some editorials and improving the CR403r2 incorpoation into v6.3.0. |  |  |  |
| A509 | 1 | 212 | SM-STATUS msg. Response at unknown or unforeseen message type, and removal of superfluous SM cause in DEACTIVATE ..ACCEPT. | The SM-cause IE is removed from DEACTIVATE PDP CONTEXT ACCEPT message and the ACTIVATE AA PDP CONTEXT ACCEPT message. |  |  |  |
| A505 | 0 | 034 | 'GPRS services not allowed' cause in the ATTACH REJECT (combined case). | When the MS receives an ATTACH REJECT with cause # 7 (GPRS services not allowed) as an answer of a Combined GPRS Attach, then a MS operating in operation mode B shall perform an IMSI attach for non-GPRS services by use of the MM IMSI attach procedure. |  |  |  |
| A481 | 2 | 457 | Improving the user data loss-less inter-SGSN RA update | Replacing V(R) transfer mechanism in the RAU procedure by a Receive N-PDU Numberr transfer mechanism | ad-hoc |  |  |
| A476 | 2 | 356 | P-TMSI is introduced in the paging request messages and in the Mobile identity IE | To clarify that either TMSI or P-TMSI can be used as Mobile identity. |  |  |  |
| A469 | 2 | 182 | Introduction of the A&C reference number IE in the AUTHENTICATION AND CIPHERING REQUEST and RESPONSE messages | To avoid A&C response message to be an answer of wrong A&C request message when eg a change in RA occurs, the Reference number IE ties them together. |  |  |  |
| A467 | 2 | 184 | Multiple ATTACH REQUEST attempts and GMM-common procedures: abnormal cases on the network side. | Corrects the abnormal cases on the network side for the GPRS attach and combined attach procedures. Also that it is not an error for the network to receive subsequent IDENTITY RESPONSE or AUTHENTICATION AND CIPHERING MESSAGES (sub-clause 8.4). Also T3322, T3350, T3360 and T3370 were reduced from 8 to 6 seconds. |  |  |  |
| A465 | 0 | 231 | Conditional IE GPRS CKSN in the Authentication and ciphering request message | Correcting the IEI value of CKSN to 8-. And making CKSN conditional in AUTHENTICATION AND CIPHERING REQUEST message,- always with RAND IE when authentication is requested. |  |  |  |
| A461 | 3 | 271 | Usage of the combined routing area update procedure | A combined RAU shall be performed if either a new LA or RA is entered. A class A MS in NMO I shall perform normal RAU during a CS connection. A combined RA update shall be performed not even if the LA but also if the RA has been changed after the release of the CS connection. |  |  |  |

###### CRs to GSM 04.08 v6.2.0 GPRS MM/SM part to be included in v6.3.0

###### (Status after SMG #28, February ‘99)

| CR # | Rev | Tdoc | Title | Description | SMG3 WPA | SMG3 Plenary | SMG #28 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A501 | 1 | 3A99-226 Revised still use 223 ! | DRX parameter in routing area update message | DRX parameter was sent by the MS only in Attach, but can now be modified in RAU procedure if needed, eg to determine another paging delay. New optional IE in ROUTING AREA UPDATE REQUEST. |  |  |  |
| A499 | 0 | 3A99-222 | MS reaction on cause code #10 (Implicitly detached) | Cause#10 should make the MS perform a new Attach since context is deleted in GSN |  |  |  |
| A487 | 0 | 3A99-123 | GMM | GMM-REGISTERED.LIMITED-SERVICE is introduced as new sub-state in the MS, where cell (re)selection is performed. |  |  |  |
| A485 | 2 | 3A99-214 | Periodic routing area update | T3312 is linked to READY state and shall no more be stopped in SUSPEND state. At expiry the PRAU procedure is delayed if not in GMM-REGIS.NORMAL-SERVICE. |  |  |  |
| A479 | 0 | 3A99-086 | Reset of periodic RAU timer at initiation of Routing Area Update procedure | Timer T3312 is not running in sub-state GMM-REGISTERED.ATTEMPTING-TO-UPDATE, so 4.7.2.2 and 4.7.5 are corrected |  |  |  |
| A477 | 0 | 3A99-085 | Reset of attempt counters at expiry of the timer T3302 | In addition to the lists in 4.7.3 and 4.75 the expiry of T3302 shall now reset the GPRS attach and the RAU attempt counters. |  |  |  |
| A473 | 0 | 3A99-083 | IE GPRS CKSN in the Authentication procedure | 'GPRS ciphering key sequence number' IE is changed from Maandatoy to Optional in the AUTH. AND CIPHERING REQUEST message (when authentication is intended). |  |  |  |
| A459 | 0 | 3A99-068 | Addition of cause 'Invalid transaction identifier value' into SM cause IE | Alignment of chapter 8.3.2 with SM-cause IE, where cause value 81 was missing (Invalid transaction identifier value). |  |  |  |
| A451 | 0 | 3A99-028 | CS connected IE | Alignment with CR A066 in 03.60 about use of combined procedures makes the IE 'CS connected' superfluous, hence deleted. RAU REQ. msg. 'normal' used in CS conn. |  |  |  |
| A449 | 1 | 3A99-225 | READY timer values | READY timer in the network should be shorter than in the MS to avoid data loss in the period of uplink transmission delay. The 'shrink' is a network implementation issue. READY timer default is now 44 sec. |  |  |  |
| A447 | 0 | 3A99-046 | PDP context modification anytime (Based on 6.2.1 ?) | Due to CR A032 in 04.65 a lossless situation for user data is introduced at change of LLC SAP, enabling PDP context modification not only in inter-SGSN RAU's. |  |  |  |
| A443 | 0 | 3A99-045 | NSAPI usage cleanup | Earlier CR A291 introduced Transaction Identifier in SM incomplete. Remaing changes are done in 7.2.2.4 and 8.6.3.2 |  |  |  |
| A435 | 1 | 98AG539 | Handling of authentication error cases in SGSN | After unsuccessful authentication and ciphering procedure, the earlier SGSN detach is changed with keeping the GMM context. |  |  |  |
| A431 | 2 | 3A99-090 | Correction of ciphering algorithm IE | In AUTHENTICATION AND CIPHERING REQUEST message the Ciphering indicator IE is removed,-the IE is also totally removed from 04.08. And Ciphering Algorithm IE is modified with used encryption algorithm for GPRS. |  |  |  |
| A401 | 2 | 98AG541 | Force to Standby procedure | The earlier bidirectional 'force to standby' mechanism is now unidirectional from the network. The network controls when the MS needs to do cell updating after the GMM procedure. |  |  |  |
| A399 | 3 | 98AG540 | Coding of LAC in RAI information element. | Loacation Area Code information had missing code definition,-also aligning RAI definition in 04.08 with the one in 03.03. |  |  |  |
| A397 | 3 | 98AG545 | GPRS GMM Idle Substate behaviour | The MS behavior in GMM-REGISTERED and GMM-DEREGISTERED are clarified in substate ATTEMPTING TO ATTACH,- and for UPDATE-NEEDED in GMM-REG. |  |  |  |
| A441 | 1 | 3A99-087 | State selection in abnormal cases for the GPRS Attach procedure. | Proposal to make optional for the MS to search for a new PLMN in the abnormal cases for the GPRS Attach procedure. |  |  |  |

###### CRs to GSM 04.08  v6.1.1 GPRS MM/SM part to be included in v6.3.0

###### (Status after SMG #28, February ‘99)

| CR # | Rev | Tdoc 98A | Title | Description | SMG3 WPA | SMG3 Plenary | SMG #28 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A429 | 0 | 454 | READY timer clarifications | The criterias for starting the GMM READY timer were ambigous, clarified in 4.7.2.1, 4.7.11 and 6.1.3.4.1. |  |  |  |
| A425 | 1 | 430 | Access Point Name Description (In Tdoc378 it was titled : Length of Activate PDP Context Request message. Using CR#411 ) | Error correction |  |  |  |
| A419 | 1 | 460 | Transfer of the LSA information to the MS | SoLSA functionality introduction to 04.08 to MM INFORMATION message with LSA identifer IE. | Post-poned to January |  3A99-042 | ? |
| A415 | 2 | 458 | Routing area identifier Information Element. (Based on v6.1.1 plus the CRs from SMG#27.) | The IE Routing Area Identifier is changed from optional to mandatory in the messages, ATTACH REQUEST, ATTACH ACCEPT and ROUTING AREA UPDATE ACCEPT. |  |  |  |
| A413 | 1 | 443 | Protocol Configuration Options at PDP context activation failure | Both Activate PDP context reject message and Activate AA PDP context reject message now carries Prot. Conf. Opt. IE when the activation was rejected by the GGSN in order to show the user any incompatibility. |  |  |  |
| A403 | 2 | 428 | Usage of implicit detach (Based on v6.1.1 plus the CR A347 incl. in 6.2.0) | Maintain state GMM-REGISTERED when RAU attempt failed with abnormal causes and the attempt counter has reached its limit instead of the new GMM-DEREGISTERED |  |  |  |
| A395 | 1 | 425 | Reset of attempt counters in GPRS. (Based on v6.1.1 plus the CR A347 incl. in 6.2.0) | Defines when GPRS attach attempt counter and RAU attempt counter are reset, plus introducing reset of Location update attempt counter in combined procedures. |  |  |  |
| A393 | 2 | 436 | GMM Idle state after ab- normal routing area update failure in same RA (Based on v6.1.1 plus the CR A347 incl. in 6.2.0) | When periodic RAU fails caused by abnormal cases, and the RAU attempt counter is less than maximum in the same RA, the MS stays in GMM-REGISTERED.NORMAL-SERVICE and transmits user data |  |  |  |
| A391 | 0 | 368 | GMM Substate selection when returning to GMM DEREGISTERED | A more comprehensive way of defining how to select the proper substate is outlined. |  |  |  |
| A389 | 1 | 423 | Usage of combined procedures | GPRS attach and combined attach plus RAU are changed with respect to network operation modes and MS operation modes. |  |  |  |
| A387 | 1 | 421 | Primary GMM substate selection | The PLMN-SEARCH are entered after max. allowed number on attach attempt counter and RAU attempt counter. In a forbidden PLMN the MS enters LIMITED-SERVICE |  |  |  |
| A358 | 1 | 420 | Corrections to Session Management PDU contents | The network does not return PDP address when MS requested static address, and no PDP address are needed in the REQUEST PDP CONTEXT ACTIVATION REJECT. |  |  |  |
| A381 | 0 | 342 | P-TMSI reallocation procedu. |  | Rejected |  |  |
| A379 | 1 | 440 | Clarification of the relation between P-TMSI and P-TMSI signature. (Based on v6.1.1 plus the CRs from SMG#27.) | P-TMSI signature is assigned by the network in the attach, RAU or P-TMSI reallocation procedures. P-TMSI signature relation to P-TMSI is clarified. |  |  |  |
| A375 | 1 | 444 | Various corrections to GMM. (Based on v6.1.1 plus the CRs from SMG#27.) | Invalid P-TMSI signature may cause initiation of GMM common procedure. RAid is included in ATTACH ACCEPT in 4.7.3.1.3, where IMSI is removed. Modifications to ATTACH COMPLETE and ROUTING AREA UPDATE ACCEPT msg |  |  |  |

###### CRs to GSM 04.08  v6.1.0 GPRS MM/SM part to be included in v6.3.0

###### (Status after SMG #28, February ‘99)

| CR # | Rev | Tdoc 98A | Title | Description | SMG3 WPA | SMG3 Plenary | SMG #28 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A423 | 1 | 390 | SM activation clarification |  | Revised in Tdoc 434 ? |  |  |
| A421 | 0 | 389 | Clarification of states and procedures of GMM handling |  | Revised in Tdoc 433 ? |  |  |
| A417 | 0 | 396 | Radio access capability IE in Attach and RAU request messages | Radio Access Capability IE is now changed to mandatory from optional. |  |  |  |
| A413 duplicated | 1 | 429 | Cause code handling in combined procedures | (Wrong title in the MOM list of docs.) | ?Withdrawn or Rejected |  |  |
| A411 | 0 | 397 | Combined procedures, MSC temporarily not reachable | (The Tdoc429 was never distributed but contained 'CAUSE CODE HANDLING IN COMBINED PROCEDURES' and is mentioned in WPA minutes text as withdrawn?) | Awaiting Tdoc429 ? |  |  |
| A383 | 1 | 419 | Impact of failed authentication on SIM data and update status | At unsuccessful authentication also P-TMSI, P-TMSI signature,GPRS ciphering key sequence number and RAI shall be deleted in the MS. |  |  |  |
| A377 | 1 | 418 | Network initiated detach procedure handling | How to progress when RAU collides with network Detach is here depending on detach type. Also GPRS update status is outlined. |  |  |  |
| A341 | 2 | 394 | Clarification of Paging procedure with IMSI | This CR clarifies the paging with old/new P-TMSI and IMSI at lower layer failure during TMSI reallocation procedure, and the behaviour of the MS when receiving IMSI in a paging message. |  |  |  |

###### CRs to GSM 04.08  v6.1.0 GPRS MM/SM part to be included in v6.2.0

###### (Status after SMG #27, October ‘98)

| CR # | Rev | Tdoc | Title | Description | SMG3 WPA | SMG3 Plenary | SMG #27 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A349 | 1 | A290 | Modification of PDP address IE coding | This CR proposes a modification of the PDP address information element |  |  |  |
| A347 | 1 | A289 | Clarification of the GMM cause code handling | This CR clarifies the GMM cause code handling |  |  |  |
| A343 | 1 | A287 | Various corrections to 04.08 | This CR proposes various corrections as summarised on the CR’s cover page |  |  |  |
| A339 | 1 | A279 | Definition of IEI values for optional information elements | This CR introduces IEI values for IE that optionally occur in GMM or SM messages. |  |  |  |
| A337 | 1 | A283 | MS initiated detach procedure collision handling | This CR fixes an inconsistency in the description of detach procedure collision handling |  |  |  |
| A335 | 3 | A305 | Classmark split | Consequence of: CR A077 to 03.60: Handling of GPRS Classmark and CR A310 to 04.08: GPRS Radio classmark as Radio access capability |  |  |  |
| A329 | 1 | A278 | Deletion of STANDBY timer | Consequence of CR A065 to 03.60: Deletion of Standby timer function |  |  |  |
| A345 | 1 | A288 | SAPI negotiation at AA PDP context activation | At activation of anonymous PDP contexts no QoS mapping to SAPI is available, so this CR describes how to derive a SAPI. |  |  |  |
| A363 | 1 | A291 | QoS correction | Introduction of 'best effort' to mean throughput values, as described in 03.60. |  |  |  |
| A351 | 0 | A204 | Clarification in the negotiation of the READY Timer value | Defines the MS and network behavior with respect to setting READY Timer value in cases not stated earlier. |  |  |  |
| A333 | 0 | A187 | Radio priority level signalling for MO SMS | Consequence of CR A076 to 03.60: Signalling and SMS radio priority levels |  |  |  |
| A331 | 0 | A186 | Usage of combined procedures | Consequence of CR A066 to 03.60: Clarification for usage of combined procedures | Post-poned | _ | _ |

###### CRs to GSM 04.08  v6.0.0 GPRS MM/SM part to be included in v6.1.0

###### (Status after SMG #26, July ‘98)

| CR # | Rev | Tdoc | Title | Description | SMG3 WPA | SMG3 Plenary | SMG #26 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A310 | 3 | P141 | GPRS radio classmark as Radio Access Capability | This CR proposes to separate the radio part and network part of information that is contained in the MS classmark IE | — | rejected |  |
| A302 | 2 | P165 | Correction of the GMM state diagram figures | This CR proposes to correct the GMM state diagrams with respect to the introduction of the implicit detach timer functionality as described in GSM 03.60 | — |  |  |
| A300 | 2 | A130 | Diagram correction | Superseded by CR A302 | withdrawn | — | — |
| A298 | 3 | A173 | SAPI negotiation at PDP context activation and modification | This CR proposes the modifications that are required to align GSM 04.08 with the introduction of SAPI negotiation as described in 03.60 and 04.64 |  |  |  |
| A297 | 3 | P158 | Modification of Timer value IE coding | This CR proposes to modify the coding of the Timer value IE and to rename this IE to Timer IE | — |  |  |
| A296 | 3 | P157 | Alignment of the SM procedure description to GSM 09.60 | This CR proposes the modifications that are required to align the session management procedures in GSM 04.08 with GSM 09.60. | — |  |  |
| A295 | 2 | A163 | Alignment of the description of the suspend/resume operation with GSM 03.60 | This CR proposes the modifications that are required to align GSM 04.08 with the suspend/resume operation as described in GSM 03.60. |  |  |  |
| A294 | 2 | A155 | Radio resource address management by GMM | This CR introduces the different TLLI types defined in GSM 03.03 and clarifies the cases in which they are used. |  |  |  |
| A293 | 3 | P156 | Various corrections of GSM 04.08 related to the GPRS MM part | This CR proposes some editorial corrections as summarised on the CR’s cover page. | — |  |  |
| A292 | 1 | A127 | Various editorial corrections of GSM 04.08 related to GPRS MM and SM. | This CR proposes some editorial improvements as summarised on the CR’s cover page. |  |  |  |
| A291 | 4 | A165 | Addition of transaction identifiers to prevent confusion in GPRS session management signalling exchanges | This CR proposes to use the well established concept of Transaction Identifiers for SM message correlation |  |  |  |
| A290 | 5 | P64 | Addition of the GSM implicit detach timer functionality in the SGSN | This CR proposes the modifications that are required to align GSM 04.08 to the implicit timer functionality as described in GSM 03.60 | — |  |  |