3GPP1 TSG CN WG1 / ETSI STC SMG3 WPA	Tdoc N1-99592

Sophia Antipolis, France	Revision of Tdoc N1-99532

1-4 June, 1999

| CHANGE REQUEST No : |  |  |  |  | A116r2 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 03.60 |  | Version: |  |  | 6.3.1 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  | X |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | EDGE NSS |
| --- | --- |

| Source: | UWCC | Date: | 2 June, 1999 |
| --- | --- | --- | --- |

| Subject: | Tunnelling of Non-GSM Messages |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | To provide a generic mechanism for the exchange of signalling between an MS and a non-GSM MSC/VLR, transparently through the SGSN. This CR requests the addition of the Tunnelling of Messages (TOM) service to 03.60 to support the above function. |
| --- | --- |

| Clauses affected: | 3.2, new 6.10, and 15.2.3. |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications | X | ® List of CRs: | GSM 04.64 A070, and GSM 09.18 A028 |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 3.2	Abbreviations

For the purposes of this specification the following abbreviations apply. Additional applicable abbreviations can be found in GSM 01.04 [1].

TOM	Tunnelling of Messages

The subclause 6.10 CR text is new, and is not underlined in order to enhance ease of reading.

## 6.10	Tunnelling of non-GSM Signalling Messages Function

Tunnelling of Messages (TOM) is an optional protocol layer that uses the LLC unacknowledged mode procedures to tunnel messages between the MS and the SGSN (see GSM 04.64). TOM uses two LLC SAPs for communication between the MS and the SGSN; one for high-priority messages and one for low-priority messages.

Upon receiving a non-GSM signalling message from an MS via the TOM protocol, the SGSN forwards the message to a non-GSM MSC/VLR using the BSSAP+ protocol (see GSM 09.18). The specific Gs interface used by the SGSN is determined by the:

-	RAI associated with the current location of the MS; and

-	information in the TOM protocol header.

Upon receiving a non-GSM signalling message from a non-GSM MSC/VLR via the BSSAP+ protocol, the SGSN forwards the message to a specific MS using the TOM protocol. The specific MS is determined by the SGSN based on the content of the BSSAP+ header.

The signalling plane between an MS and a non-GSM MSC/VLR that uses tunnelling procedures for non-GSM signalling is shown in Figure X:

Figure X: Signalling Plane MS - non-GSM MSC/VLR

### 6.10.1	Uplink Tunnelling of non-GSM Signalling Messages Procedure

The Uplink Tunnelling of non-GSM Signalling Messages procedure is illustrated in Figure Y. Each step is explained in the following list.

Figure Y: Uplink Tunnelling of non-GSM Signalling Messages Procedure

1)	The MS sends a TOM Protocol Envelope (Non-GSM Signalling Message) to the SGSN either in ciphered or clear mode. The TOM protocol header contains information about the application using the TOM facility and any other TOM Protocol Discriminator-specific information. The TOM Protocol Envelope is received on one of the two LLC SAPs used for tunnelling of messages.

2)	The SGSN identifies the non-GSM MSC/VLR to which to forward the non-GSM signalling message. It then sends a BSSAP+ Uplink Tunnel Request (IMSI, SGSN Address, TOM Priority, Cipher, Non-GSM Signalling Message) message to the identified non-GSM MSC/VLR. The Cipher parameter is set to cipher if the TOM Protocol Envelope was received by the LLC layer in ciphered form, otherwise it is set to not cipher. TOM Priority is set to high priority if the TOM Protocol Envelope was received on the high-priority LLC SAP, otherwise it is set to low priority.

### 6.10.2	Downlink Tunnelling of non-GSM Signalling Messages Procedure

The Downlink Tunnelling of non-GSM Signalling Messages procedure is illustrated in Figure Z. Each step is explained in the following list.

Figure Z: DownlinkTunnelling of non-GSM Signalling Messages Procedure

1)	The non-GSM MSC/VLR sends a BSSAP+ Downlink Tunnel Request (IMSI, VLR Address, TOM Priority, Cipher, Non-GSM Signalling Message) message to the SGSN associated with the MS. TOM Priority indicates whether the SGSN shall select the high-priority or low-priority LLC SAP when forwarding the non-GSM signalling message to the MS. Cipher indicates whether or not the SGSN shall cipher the non-GSM signalling message before forwarding it to the MS.

2)	The SGSN sends a TOM Protocol Envelope (Non-GSM Signalling Message) to the MS using the selected LLC SAP.

### 15.2.3	Reliability Class

Data reliability is defined in terms of the residual error rates for the following cases (see GSM 02.60):

-	probability of data loss;

-	probability of data delivered out of sequence;

-	probability of duplicate data delivery; and

-	probability of corrupted data.

The reliability class specifies the requirements of the various network protocol layers. The combinations of the GTP, LLC, and RLC transmission modes support the reliability class performance requirements. The reliability classes are summarised in Table 1.

Table 1: Reliability Classes

| Reliability Class | GTP Mode | LLC Frame Mode | LLC Data Protection | RLC Block Mode | Traffic Type |
| --- | --- | --- | --- | --- | --- |
| 1 | Acknowledged | Acknowledged | Protected | Acknowledged | Non real-time traffic, error-sensitive application that cannot cope with data loss. |
| 2 | Unacknowledged | Acknowledged | Protected | Acknowledged | Non real-time traffic, error-sensitive application that can cope with infrequent data loss. |
| 3 | Unacknowledged | Unacknowledged | Protected | Acknowledged | Non real-time traffic, error-sensitive application that can cope with data loss, GMM/SM, TOM, and SMS. |
| 4 | Unacknowledged | Unacknowledged | Protected | Unacknowledged | Real-time traffic, error-sensitive application that can cope with data loss. |
| 5 | Unacknowledged | Unacknowledged | Unprotected | Unacknowledged | Real-time traffic, error non-sensitive application that can cope with data loss. |
| NOTE: For real-time traffic, the QoS profile also requires appropriate settings for delay and throughput. |  |  |  |  |  |

Each reliability class in combination with the Reordering Required information specifies the target residual error rates for a PDP context. The residual error rate targets are given in GSM 02.60. A PLMN may support only a subset of the reliability classes. Signalling and SMS shall be transferred with reliability class 3.