ETSI SMG3 WPA GPRS ad-hoc	Tdoc N1-99606

Sophia Antipolis, France							    (Revision of Tdoc N1-99560)

May 31, 1999									    (Revision of Tdoc N1-99492)

(Revision of Tdoc N1-99481)

3GPP TSG CN WG1/ETSI SMG3 WPA	(Revision of Tdoc N1-99418)

Oslo, Norway

May 18-21, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A673 R4 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 03-06-1999 |
| --- | --- | --- | --- |

| Subject: | Change of network operation mode |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | This CR clarifies the behaviour of an MS in GPRS operation mode A or B when the RA is changed and the new RA has a different network mode of operation. This CR is based upon GSM 03.60 where the following is stated in section 5.4.5: A GPRS MS can operate in one of three modes of operation. The mode of operation depends on the services that the MS is attached to, i.e., only GPRS or both GPRS and other GSM services, and upon the MS's capabilities to operate GPRS and other GSM services simultaneously. - Class-A mode of operation: The MS is attached to both GPRS and other GSM services, and the MS supports simultaneous operation of GPRS and other GSM services. - Class-B mode of operation: The MS is attached to both GPRS and other GSM services, but the MS can only operate one set of services at a time. In network operation mode III (see subclause "Paging Co-ordination"), an MS that is capable of monitoring only one paging channel at a time cannot operate in class B mode of operation. In this case, such an MS shall revert to class-C mode of operation. - Class-C mode of operation: The MS is exclusively attached to GPRS services. And the following is stated in GSM 03.60, in section 6.9.1: In network mode of operation II and III, whenever an MS determines that it shall perform both an LA update and an RA update, the MS shall perform the LA update first. |
| --- | --- |

| Clauses affected: | 4.7.1.6 (new section) |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

*** First modification ***

#### 4.7.1.6	Change of network mode of operation

Whenever an MS moves to a new RA, the procedures executed by the MS depend on the network mode of operation in the old and new routing area.

In case the MS is in state GMM-REGISTERED or GMM-ROUTING-AREA-UPDATING-INITIATED and is in operation mode:

- A or B (with the exceptions in b and c below), the MS shall execute:

Table x/GSM 04.08: Mode A or B

| Network operation mode change | ## Procedure to execute |
| --- | --- |
| I  II or I  III | Normal Location Update(*), followed by a Normal Routing Area Update |
| II  III or III  II | Normal Location Update (see section 4.2.2), followed by a Normal Routing Area Update |
| II  I or III  I | Combined Routing Area Update with IMSI attach |

- B which reverts to operation mode C in network operation mode III, the MS shall execute:

Table y/GSM 04.08: Mode B which reverts into mode C in network operation mode III

| Network operation mode change | ## Procedure to execute |
| --- | --- |
| I  II | ## Normal Location Update(*), followed by a Normal Routing Area Update |
| I  III or II  III | IMSI Detach (see section 4.3.4), followed by a Normal Routing Area Update |
| II  I or III  I | Combined Routing Area Update with IMSI attach |
| III  II | IMSI attach (see section 4.4.3), followed by a Normal Routing Area Update |

- B which reverts to IMSI attached for CS services only in network operation mode III, the MS shall execute:

Table z/GSM 04.08: Mode B which reverts into IMSI attached for CS services only in network operation mode III

| Network operation mode change | ## Procedure to execute |
| --- | --- |
| I  II | ## Normal Location Update(*), followed by a Normal Routing Area Update |
| I  III | Normal Location Update(*), followed by a GPRS Detach with type indicating “GPRS Detach” |
| II  III | Normal Location Update (see section 4.2.2), followed by a GPRS Detach with detach type indicating “GPRS Detach” |
| II  I | Combined Routing Area Update with IMSI attach |
| III  I | Combined GPRS Attach |
| III  II | Normal Location Update (see section 4.2.2), followed by a Normal GPRS Attach |

(*) Intended to remove the Gs association in the MSC/VLR.

Further details are implementation issues.