3GPP TSG-CN2 #21	Tdoc N2-010972

Cancun, Mexico 	(Revision of N2-010876)
26th – 30th November 2001

| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 23.078 |  |  |  | CR |  |  |  | rev |  |  |  | 1 |  | Current version: |  |  |  |  | 5D.11.1 |  |  |  |
| Proposed change affects: |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | X |
| Title: |  | Corrections to Process CAMEL_ICA_MSC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  | Vodafone Group Plc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  | CAMEL4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 23rd November 2001 |  |  |  |  |  |
| Category: |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change: |  |  | Various errors exist in Process CAMEL_ICA_MSC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | In Process CAMEL_ICA_MSC: - All sheets: Correction of spelling error "pary" to "party" in description - Sheet 1: Removal of Note - Sheet 2, sheet 3: Int_O_Exception input signal is sent to gsmSSF (to left) - Sheet 2: Introduction of Int_Release Call from gsmSSF causing Abort to be sent to VLR - Sheet 3: Wait_For_ICA_Call_Result state renamed to Wait_For_ICA_Call_Result2 to avoid confusion - Sheet 6: Introduction of Disconnect Leg ack In Process CAMEL_ICA_VLR: - Sheet 2: Int_Exception message is changed to Abort |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | Errors remain |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 4.5.6.1 and 4.5.6.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  | Changes since the previous version: Inclusion of Int_Release Call in Wait_For_ICA_Call_Result2 state in Process CAMEL_ICA_MSC sheet 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

**** First Modified Section ****

#### 4.5.6.1	Handling of gsmSCF initiated calls in the MSC

…

Figure 4.82a: Process CAMEL_ICA_MSC (sheet 1)

Figure 4.82b: Process CAMEL_ICA_MSC (sheet 2)

Figure 4.82c: Process CAMEL_ICA_MSC (sheet 3)

Figure 4.82d: Process CAMEL_ICA_MSC (sheet 4)

Figure 4.82e: Process CAMEL_ICA_MSC (sheet 5)

Figure 4.82f: Process CAMEL_ICA_MSC (sheet 6)

…

**** Last Modified Section ****

#### 4.5.6.2	Handling of gsmSCF initiated calls in the VLR

…

Figure 4.88a: Process CAMEL_ICA_VLR (sheet 1)

Figure 4.88b: Process CAMEL_ICA_VLR (sheet 2)

**** End of Document ****