3GPP TSG CN WG2 Meeting #24	N2-020592

Budapest, Hungary,	(Revision of N2-020496)

13th – 17th May 2002

| CR-Form-v5.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29.078 |  |  |  | CR | - |  |  |  rev |  |  |  | 2 |  Current version: |  |  |  |  | d5.7.3 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | Use of Split Leg to create CSID1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Vodafone |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | CAMEL4 |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 15th May 2002 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | Current draft of 23.078 allows the Split Leg operation to create CSID1 if it does not currently exist. Current draft of 29.078 does not allow this. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Alignment of 29.078 with 23.078. Split Leg is used to move a leg from CSID1 to a new call segment, or to move a leg from an existing call segment (not CSID1) to a new CSID1 (if CSID1 does not already exist). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Inconsistency between 23.078 and 29.078 about the use of the Split Leg operation remains. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 11.33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | Changes since previous version: addition of term "target", rewording of precondition 2, removal of precondition 4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

**** Modified Section ****

## 11.33	SplitLeg Procedure

### 11.33.1	General Description

The gsmSCF uses this operation to request the gsmSSF to separate one party from the source Call Segment and place it in a new target Call Segment.

#### 11.33.1.1	Parameters

-	legToBeSplit:
	This parameter indicates the party in the call to be split from the source Call Segment.

-	newCallSegment:
	This parameter indicates the CallSegmentID to be assigned to the newly-created Call Segment.

### 11.33.2	Responding entity (gsmSSF)

#### 11.33.2.1	Normal procedure

gsmSSF preconditions:

- A control relationship exists between the gsmSCF and the gsmSSF.

- The initial Call Segment is either the source Call Segment or the target Call Segment.

- The BCSM for the leg to be split is in the state O_Active, T_Active, O_Mid_Call or T_Mid_Call.

gsmSSF postconditions:

- The gsmSSF performs the necessary actions to separate the specified leg from its original Call Segment and place it in a new target Call Segment.

- The CS_gsmSSF FSM for the new Call Segment transits to the state "Waiting_for_Instructions".

- The CS_gsmSSF FSM for the source Call Segment transits to the state "Waiting_for_Instructions".

- The remaining BCSM instances within the source Call Segment transit to the O_Mid_Call DP or to the T_Mid_Call DP, unless already suspended at a DP. Note that no Mid_Call EDP will be reported for this case.
[CR editor's note" this "Note", is that normative text? In that case, it shall be written in the imperative form.]

- A Return Result shall be sent to the gsmSCF immediately after successful execution of this operation.

#### 11.33.2.2	Error handling

Generic error handling for the operation related errors is described in clause 10, and the TC services which are used for reporting operation errors are described in clause 14.

**** End of Document ****