3GPP TSG-CT3 Meeting #139	C3-250573

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025								revision of C3-250227

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1498 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to NAT mapping information retrieval and removal of ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, ZTE, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - When the immRepInd flag is set to true then in the HTTP POST response the NEF sends back the UeAddrNotif data type which contains the notification correlation id which is duplicate attribute that is sent also in notifId attribute within the UeAddrSubsc data type in the HTTP POST response. - As per TS29.564 (refer clause 6.1.6.2.18 in TS 29.564) the UPF sends the list of NATed IP and port information to the NEF. However, in this TS 29.522 the NEF sends only single entry of the NATed IP and port information in the UeAddrNotif data type to the AF. - The AF provides the remote IP address to the NEF when requesting to retrieve NAT mapping information. This remote IP address shall be an IPv4 or an IPv6 address, NOT an IPv6 prefix. However, the remoteIpAddr attribute in UeAddrInfo has been defined as IpAddr, which can encode an IPv4 address, an IPv6 address or an IPv6 prefix. - TS 29.564 specifies that when the NF service consumer does not provide the remote port number, the UPF may return multiple NATed IP address for the same private IP address but different remote port numbers. However, the event report containing the list of NAT mapping information does not include the remote port number, which entails that the NF service consumer cannot determine which NAT mapping information applies for a given remote port number. The solution of this issue is addressed in CT4#127 with the submitted CR#C4-250266. - The transport protocol information received from the UPF(refer clause 6.1.6.2.19 in TS 29.564) is missing while sending the NAT mapping information to AF. - The remotePortNumber attribute in UeAddrInfo defined as Uinteger in TS 29.522 whereas in TS 29.564 this attribute is defined as Uint16 datatype. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The data type for the immReport attribute is corrected from the UeAddrNotif to the NATMapping data type by referring TS 29.564. Also the presence from optional to conditional is made as this attribute shall be present if the immediate report indication is set to true. - It is clarified that the remote IP address is an IPv4 or an IPv6 address. The remoteIpAddr attribute is replaced by two attributes with Ipv4Addr and Ipv6Addr data types respectively. - It is clarified that the UE IP address is an IPv4 or an IPv6 address. The ueNatIpAddr attribute is replaced by two attributes with Ipv4Addr and Ipv6Addr data types respectively. - The datatype is corrected from Uinteger to Uint16 for port number attributes to have consistency between the received unit from UPF to NEF and sent unit from NEF to AF. - It is clarified that the ueNatAddrInfo is the list of NATed IP and port information that is sent from NEF to AF by changing the attribute name from ueNatAddrInfo to ueNatAddrInfoList with the array type for the NATMapping data structure that is defined in 29.564. - Removed the ENs: Editor's Note: Whether the UE IPv6 address or IPv6 prefix can be provided in uePrivateIpAddr attribute is FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure to return NATed IP address if the remote IP@ is encoded as an IPv6 prefix. The NF service consumer cannot determine which NAT mapping to use when the UPF returns multiple NAT bindings and other inconsistency between the NATed mapping information within the stage-3 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.39.3, 4.4.39.5, 5.35.5.1, 5.35.2.2.4, 5.35.2.2.5, 5.35.2.2.6, 5.35.2.2.7, A.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR provides backward compatible feature updates to the following Open API: TS29522_UEAddress.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

#### 4.4.39.3	Creation of a new subscription to obtain UE Address

In order to create a new subscription to obtain NATed UE public IP address information for a given AF, the AF shall initiate an HTTP POST request message to the NEF for the "UE Address Subscriptions" resource. The HTTP POST request message body shall include the UeAddrSubsc data structure as defined in clause 5.35.2.2.3.3 and clause 5.35.2.2.4.

Upon receipt of the corresponding HTTP POST message, if the AF is authorized by the NEF to obtain the NATed UE public IP address information, the NEF shall discover the UPF implementing e.g., NAT functionality for the provided UE private IP address by invoking Nnrf_NFDiscovery service as defined in 3GPP TS 29.510 [57], then upon receiving the UPF address, the NEF shall further invoke the Nupf_EventExposure_Subscribe service operation to obtain the NATed UE public IP address and port information from the UPF as defined in 3GPP TS 29.564 [61]. If the NEF receives an error response from the NRF or UPF, the NEF shall not create the resource and shall respond to the AF with a proper error status code. If the NEF received within an error response a "ProblemDetails" data structure with a "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

On successful UE Address Subscription resource creation, the NEF shall return an HTTP POST response with an HTTP "201 Created" status code to the AF, including a "Location" header containing the URI of the created "Individual UE Address Subscription" resource and the response body containing a representation of the created resource within the UeAddrSubsc data structure. The NEF shall include the current UE address information received from the UPF within the "immReport" attribute, if available and the "immRepInd" attribute was provided and set to "true" in the corresponding request.

On failure, the NEF shall take proper error handling actions, as specified in clause 5.35.7, and respond to the AF with an appropriate error status code.

# * * * * 2nd Change * * * *

#### 4.4.39.5	Notification for UE Address information

This procedure is used by the NEF to send UE address information notifications to a previously subscribed AF.

When the NEF receives the notification of the NATed UE public IP address information from the UPF, the NEF shall provide a notification by sending an HTTP POST request message to the AF with the request body including the UeAddrNotif data structure and targeting the notification URI provided by the AF during the corresponding UE Address Subscriptions as defined in clause 5.35.4 and clause 5.35.2.2.6.

Upon reception of this notification request, the AF shall acknowledge its successful reception by sending a HTTP POST response message with an HTTP "204 No Content" status code.

On failure, the AF shall take proper error handling actions, as specified in clause 5.35.7, and respond to the NEF with an appropriate error status code.

# * * * * 3rd Change * * * *

#### 5.35.5.1	General

This clause specifies the application data model supported by the UeAddress API. Table 5.35.5.1-1 specifies the data types defined for the UeAddress API.

Table 5.35.5.1-1: UeAddress service specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| UeAddressReq | 5.35.5.2.2 | Represents the parameters to request UE Address information. |  |
| UeAddressInfo | 5.35.5.2.3 | Represents UE Address information. |  |
| UeAddrSubsc | 5.35.5.2.4 | Represents the parameters to subscribe for UE Address information. |  |
| UeAddrInfo | 5.35.5.2.5 | Represents the parameters to subscribe for UE NAT Address information. |  |
| UeAddrNotif | 5.35.5.2.6 | Represents UE Address information. |  |

Table 5.35.5.1-2 specifies data types re-used by the UeAddress API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the UeAddress API.

Table 5.35.5.1-2: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [8] | Represents a GPSI. |
| IpAddr | 3GPP TS 29.571 [8] | Represents UE IP address. |
| Ipv4Addr | 3GPP TS 29.571 [8] | Identifies an IPv4 address. |
| Ipv6Addr | 3GPP TS 29.571 [8] | Identifies an IPv6 address. |
| Link | 3GPP TS 29.122 [4] | Identifies a referenced resource. |
| UeNatMappingInfo | 3GPP TS 29.564 [61] | Represents UE NAT Mapping Information. |
| SupportedFeatures | 3GPP TS 29.571 [8] | Represents the list of supported feature(s) and is used to negotiate the applicability of the optional features. |
| Uinteger | 3GPP TS 29.571 [8] | Unsigned integer. |
| Uri | 3GPP TS 29.122 [4] | Identifies a referenced resource. |

# * * * * 4th Change * * * *

##### 5.35.2.2.4	Type: UeAddrSubsc

Table 5.35.2.2.4-1: Definition of type UeAddrSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | M | 1 | Notification URI for UE address information reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| afId | string | M | 1 | Represents the identifier of the AF that is sending the request. |  |
| ueAddrInfo | UeAddrInfo | C | 0..1 | Contains the UE private address information and IP address information of remote end. It shall be present if the UE NAT address information is subscribed. |  |
| immRepInd | boolean | O | 0..1 | Indicates whether immediate reporting shall be included in the corresponding response if available: - "true": immediate reporting shall be included in the response if availale; - "false"(default): reporting will occur when the event is met. |  |
| immReport | UeNatMappingInfo | C | 0..1 | Contains the UE address information that match this subscription. It shall be included only in the HTTP response of a subscription creation, and only if the "immRepInd" attribute is set to "true" in the corresponding HTTP request and the information is available. (NOTE 1) |  |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the list of Supported features used as described in clause 5.35.6. It shall be present if feature negotiation needs to take place. |  |
| self | Link | C | 0..1 | Identifies the Individual UE Address Subscription resource. Shall be present in the HTTP GET response when reading all the subscriptions for an AF. |  |
| NOTE 1: The UPF may report a list of NATed public IP and port information for the same private UE IP address and the same remote IP address, for different destination port numbers, if no remote port number was received in the subscription within the ueAddrInfo attribute. |  |  |  |  |  |

# * * * * 5th Change * * * *

##### 5.35.2.2.5	Type: UeAddrInfo

Table 5.35.2.2.5-1: Definition of type UeAddrInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uePrivateIpAddr | IpAddr | M | 1 | Identifies a UE private IP Address. |  |
| ipDomain | string | O | 0..1 | The IPv4 address domain identifier. The attribute may only be present if the IPv4 address is provided in the "uePrivateIpAddr" attribute. |  |
| remoteIpv4Addr | Ipv4Addr | C | 0..1 | The IPv4 address of remote end for which the NAT mapping is requested. (NOTE 1) |  |
| remoteIpv6Addr | Ipv6Addr | C | 0..1 | The IPv6 address of remote end for which the NAT mapping is requested. (NOTE 1) |  |
| remotePortNumber | Uint16 | O | 0..1 | Indicates the port number of the remote end for which the NAT mapping is requested. |  |
| NOTE 1: Either the "remoteIpv4Addr" attribute or the "remoteIpv6Addr” attribute shall be included. |  |  |  |  |  |

# * * * * 6th Change * * * *

##### 5.35.2.2.6	Type: UeAddrNotif

Table 5.35.2.2.6-1: Definition of type UeAddrNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifCorrId | string | M | 1 | Notification correlation identifier. |  |
| ueNatAddrInfo | UeNatMappingInfo | M | 1 | Contains the list of NATed public IP address and port Information. The NEF shall return an empty value when there is no UeNatMappingInfo available. (NOTE 1) |  |
| NOTE 1: The UPF may return a list of NATed public IP and port information for the same private UE IP address and the same remote IP address, for different destination port numbers, if no remote port number was received in the subscription. |  |  |  |  |  |

# * * * * 7th Change * * * *

##### 5.35.2.2.7	Void

# * * * * 8th Change * * * *

# A.33	UEAddress API

openapi: 3.0.0

info:

title: 3gpp-ue-address

version: 1.1.0-alpha.1

description: |

API for UE Address service.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.522 V19.1.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/3gpp-ue-address/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 5.2.4 of 3GPP TS 29.122.

paths:

/retrieve:

post:

summary: Request to retrieve UE Address information.

operationId: RetrieveUEAddress

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddressReq'

responses:

'200':

description: >

OK. The UE Address information request is successfully processed, and UE address

information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddressInfo'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

get:

summary: read all of the active subscriptions for the AF.

operationId: ReadAllSubscriptions

tags:

- UE Address Subscriptions

parameters:

- name: supp-feat

in: query

description: Features supported by the NF service consumer

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get all of the active subscriptions for the AF)

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/UeAddrSubsc'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Creates a new subscription resource

operationId: CreateNewSubscription

tags:

- UE Address Subscriptions

requestBody:

description: new subscription creation

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrSubsc'

callbacks:

notification:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrNotif'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Created (Successful creation)

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrSubsc'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'204':

description: >

Successful case. The resource has been successfully created and no additional

content is to be sent in the response message.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: read an active subscription for the AF and the subscription Id

operationId: ReadAnSubscription

tags:

- Individual UE Address information Subscription

parameters:

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

- name: supp-feat

in: query

description: Features supported by the NF service consumer

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK (Successful get the active subscription)

content:

application/json:

schema:

$ref: '#/components/schemas/UeAddrSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Deletes an already existing subscription

operationId: DeleteAnSubscription

tags:

- Individual UE Address Subscription

parameters:

- name: subscriptionId

in: path

description: Identifier of the subscription resource

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

UeAddressReq:

description: Represents the parameters to request the retrieval of UE Address information.

type: object

properties:

afId:

type: string

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- gpsi

UeAddressInfo:

description: Represents UE Address information.

type: object

properties:

ueIpAddrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- ueIpAddrs

UeAddrSubsc:

description: Represents the parameters to subscribe for UE Address information.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

afId:

type: string

ueAddrInfo:

$ref: '#/components/schemas/UeAddrInfo'

immRepInd:

type: boolean

description: >

Indicates whether immediate reporting shall be included in the corresponding response

if

available.

immReport:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/UeNatMappingInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

self:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Link'

required:

- notifUri

- notifId

- afId

UeAddrInfo:

description: Represents the parameters to subscribe for UE NAT Address information.

type: object

properties:

uePrivateIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

ipDomain:

type: string

remoteIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

remoteIpv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

remotePortNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint16'

required:

- uePrivateIpAddr

oneOf:

- required: [remoteIpv4Addr]

- required: [remoteIpv6Addr]

UeAddrNotif:

description: Represents a UE address information.

type: object

properties:

notifCorrId:

type: string

description: Contains the updated notification correlation identifier.

ueNatAddrInfo:

$ref: 'TS29564_Nupf_EventExposure.yaml#/components/schemas/UeNatMappingInfo'

required:

- notifCorrId

- ueNatAddrInfo

# * * * * End of Changes * * * *