3GPP TSG-CT Meeting #107	CP-250199

# Incheon, Korea; 12th – 13th March 2025									Revision of C1-250876

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.186 | CR | 0061 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network initiated standalone IMS DC session setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | China Mobile, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.228 AG.2.2 specifies the procedure of establishing a standalone IMS Data Channel session using IMS DC capability exposure. It is suggested to add the requirement of IMS AS for the above case. In addition, the EN in 9.3.2.2.2.4 can be solved by TS23.228CR1541 agreed by SA2#166AH-E. Editor’s Note: The event about the network intiated media change event is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Specifies the procedure of establishing a standalone IMS Data Channel session using IMS DC capability exposure. - Solve the EN about the event for network initiated DC establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Establishing a standalone IMS Data Channel session using IMS DC capability exposure is not supported in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.2.1A(new), 9.3.2.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23228 CR 1541 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Add dependency SA2 CR. - Add 23.228 AG.2.2 for reference. - Add "with the SDP offer" in the generated request. - Replace "target UE(s)" with "originating and terminating" UE. R2: - Submit to the CT#107 to fix the rev number issue indicated by the MCC. - Add “Huawei, HiSilicon” to the Source. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.2.1A	Network-initiated IMS standalone data channel session setup

Upon receipt a request to setup an IMS standalone data channel session as specified in 3GPP TS 23.228 [3] AG.2.2, the IMS AS shall determine whether the served user is authorized to use IMS data channel as specified in clause 9.3.2.2.1. If the served user is authorized to use data channel, the IMS AS shall select a DCSF and notify the DCSF about the external session create event. Based on the instruction from the DCSF, the IMS AS shall request the MF to allocate data channel media resources, generate the media description for the data channel as specified in clause 9.3.2.2.2.4, generate and send initial INVITE request with SDP offer for the originating and terminating UEs as specified in 3GPP TS 24.229 [9] clause 5.7.3.

Upon receipt of the 18x or 2xx response including SDP answer to the initial INVITE request, the IMS AS shall notify the DCSF about corresponding session event (session establishment progress (i.e. receiving the 183 (Session Progress) response ), session establishment alerting (i.e. receiving the 180 (Ringing) response) or session establishment success (i.e. receiving 200 (OK) response) event) and request the MF to update the media resource.

Upon receipt of a 4xx, 5xx or 6xx response to the initial INVITE request, the IMS AS shall notify the DCSF about session establishment failure, and request the MF to release the data channel media resources.

* * * Next Change * * * *

9.3.2.2.2.4	Network-initiated data channel establishment

If the IMS AS received a request to update an existing IMS session to include a P2A application data channel and/or bootstrap data channel as specified in 3GPP TS 29.175 [18] towards a served UE, the IMS AS shall determine whether the served user is authorized to use IMS data channel as specified in clause 9.3.2.2.1. If the served user is authorized to use data channel, the IMS AS shall select a DCSF and notify the DCSF about the external session update event. Based on the instruction of the DCSF, the IMS AS shall request the MF to allocate data channel media resources and generate the media description for

a)	the bootstrap data channel containing "a=dcmap" attribute line with the subprotocol parameter set to "http" and "stream-id" parameter set to values 0 and 10; or

b)	the P2A application data channel containing "dcmap" attribute in which "stream-id" parameter set to values starting at 1000, "a=3gpp-req-app" attribute line with "endpoint" parameter set to value "server", "c=" line and "m=application" line with the IP address and UDP port number information of termination on the MF towards to the UE, and "a=tlsId", "a=sctp-port", "a=fingerprint" and "a=setup" attribute lines with the information of the termination on the MF towards the UE if the media proxy configuration is HTTP, or the information of the DC AS if the media proxy configuration is UDP; and

generate and send a re-INVITE request to the served UE with an updated SDP offer in which the generated media description of data channel above is added.

Upon reception of the 200 (OK) response to the re-INVITE request in which

-	 the data channel media is accepted, the IMS AS shall notify the DCSF about the media change success event; or

-	 the data channel media is rejected, the IMS AS shall notify the DCSF about the media change failure event.

If the IMS AS receives a request to establish a P2A2P application data channel between two UE in an existing IMS session in which the video/audio media is established, the IMS AS shall follow the procedure above to generate two data channel media descriptions and generate and send re-INVITE request for each UE.

Upon reception of the 200(OK) responses to the re-INVITE request:

-	if both the two UEs accept the data channel media, the IMS AS shall notify the DCSF about the media change success event;

-	if both the two UEs reject the data channel media, the IMS AS shall notify the DCSF about the media change failure event;

-	if one UE accepts the data channel media, and the other UE rejects the data channel media, the IMS AS shall send a re-INVITE request to the UE who accepts the data channel media to close the established data channel, and notify the DCSF about the media change failure.

* * * End of Changes * * * *