3GPP TSG-CT Meeting #108	CP-251191

Prague, Czech Republic; 09th – 10th June 2025	was C4-251389, C4-251035

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0943 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription to QoS Monitoring via SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2025-05-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The QoS Monitoring may be triggered by an NWDAF as specified in 3GPP TS 23.502, clause 4.15.4.5: A consumer of UPF event exposure can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to this consumer. For this event, the interaction between SMF and UPF is over PFCP (TS 29.244 [69]). TS 23.501 [2] describes the QoS monitoring parameters that can be measured by means of QoS monitoring and how to enable the measurements for QoS flows. When the Subscription request for QoS monitoring event indicates that it is for the QoS Flow associated with the default QoS rule, based on local configuration the subscription request may trigger SMF to enable QoS monitoring. NOTE 1: Packets from multiple applications may share the QoS Flow associated with the default QoS Rule which may diminish the relevance of some measurements like data rate. The subscription to QoS monitoring event can target the QoS flows bound to an application by including an Application Identifier. In this case, at subscription request and/or when the PCC rules change, SMF identifies the active PCC Rule that includes a DataCollection_ApplicationIdentifier matching that Application Identifier. SMF enables this consumer (e.g. NWDAF) to receive the QoS Monitoring reports enabled by that PCC Rule. The consumer may indicate that it can receive QoS Flow Performance information for the QoS Flow associated with the default QoS rule if there are no measurements available for the Application Identifier (that is, if no PCC rule is identified). In this case the SMF may instruct the UPF to perform QoS monitoring for the QoS Flow associated with the default QoS rule and include the Indication of QoS Flow associated with the default QoS Rule (see clause 5.8.2.18 of TS 23.501 [2]). In addition, the direct reporting information is also applicable to report TSC Management Information. The table note in 7.5.2.9 should be removed. Therefore, the relevant protocol impact, e.g. QoS Monitoring description needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description for the SRR handling and QoS flow related QoS monitoring and reporting to add the use case when the QoS monitoring and reporting is triggered by subscribing UPF event exposure service via the SMF. Delete a table note "In this release of the specification, the Direct Reporting Information shall only be used for direct reporting of QoS monitoring per QoS flow." in Table 7.5.2.9 which is not relevant anymore. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification leads QoS monitoring and reporting not work when this function is requested by multiple entities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8, 5.39.1, 5.39.2, 5.39.4, 7.5.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: remove "Y" for the new note in clause 5.39.4 and correct the typo "SSR" to "SRR" in clause 5.39.2. Rev2: fix copy-paste errors by removing the extra sentences in clause 5.39.2 and 5.39.4. |  |  |  |  |  |  |  |  |

# *** First Change ***

### 5.2.8	Session Reporting Rule Handling

#### 5.2.8.1	General

The CP function may provision SRR(s) for a PFCP session in a PFCP Session Establishment Request or a PFCP Session Modification Request to request the UP function to detect and report events for a PFCP session that are not related to specific PDRs of the PFCP session or that are not related to traffic usage measurement.

-	Change of 3GPP or non-3GPP access availability, for an MA PDU session (see clause 5.20.4.2).

-	Per QoS flow QoS Monitoring and Reporting (see clause 5.39).

-	Per QoS Flow Traffic Parameter Measurement Report, as specified in clause 5.8.5.11 of 3GPP TS 23.501 [28].

#### 5.2.8.2	Provisioning of Session Reporting Rule in the UP function

##### 5.2.8.2.1	General

When provisioning an SRR, the CP function shall provide control information identifying the events that the UPF is requested to detect and report. If Direct Reporting of QoS monitoring events to a consumer (e.g. to Local NEF or AF) is supported and required, the CP function shall also provide Direct Reporting Information in the SRR (see clause 5.33.5).

The CP function may modify an SRR in the PFCP Session Modification Request message.

The CP function may request the UP function to stop the detection and reporting of specific events by removing the SRR or, if the SRR is used to control different types of events, by updating the SRR with a control information IE for the events to stop with a null length. When so instructed, the UP function shall stop detecting and reporting the corresponding events.

#### 5.2.8.3	Reporting of Session Report to the CP function

##### 5.2.8.3.1	General

When detecting that a provisioned event to report occurs, the UP function shall generate a Session Report for the related SRR and send it to the CP function by initiating the PFCP Session Report procedure, unless Direct Reporting of QoS monitoring events to a consumer (e.g. to Local NEF or AF) applies (see clause 5.33.5).

Upon generating a session report for an SRR towards the CP function or towards the consumer (e.g. the Local NEF or AF) if Direct Reporting of QoS monitoring event applies, the UP function shall continue to apply all the provisioned SRR(s), until getting any further instruction from the CP function.

# *** Next Change ***

## 5.39	QoS flow related QoS monitoring and reporting

### 5.39.1	General

Stage 2 requirements on QoS Monitoring are specified in clause 5.45 of 3GPP TS 23.501 [28] and clause 4.15.4.5 of 3GPP TS 23.502 [29].

QoS monitoring comprises of measurements of QoS parameters for a QoS Flow.

# *** Next Change ***

### 5.39.2	QoS Monitoring Control

The CP function may be requested to apply QoS monitoring and reporting for QoS flows of a PDU session, e.g. in the following scenarios:

-	upon receiving an authorized QoS Monitoring policy for a service data flow as part of a PCC rule as specified in clause 5.45 of 3GPP TS 23.501 [28];

-	upon receiving a subscription request for QoS monitoring event via the Nsmf_EventExposure from a NF service consumer of UPF event exposure service as specified in clause 4.15.4.5 of 3GPP TS 23.502 [29]; or

-	upon receiving a request to start UE level measurements as specified in 3GPP TS 32.422 [35] and 3GPP TS 28.558 [88].

The SMF shall instruct the UPF to perform Per QoS flow QoS monitoring, during a PFCP session establishment or a PFCP session modification procedure, by provisioning a SRR with one or more QoS Monitoring per QoS flow Control Information IEs for one or more QoS flows and for one or more QoS parameters described in clause 5.39.3, if the UPF has indicated support of the corresponding features. When QoS monitoring and reporting is requested by different network functions at the same time for the same PDU session, e.g. NWDAF and (via PCF) AF/NEF, the SMF shall create separate SRRs with the respective QoS monitoring and reporting parameters for each request even when the SRRs are intended for the same QoS flow.

NOTE X:	Provisioning separate SRRs enable the CP function to control and provision separately QoS monitoring and reporting for each request. The Direct Reporting Information contains only one instance of Event Notification URI and the Notification Correlation ID.

NOTE Y:	It is up to the UPF implementation to derive how to perform the QoS measurements for QoS monitoring if multiple SRRs are provisioned for QoS monitoring for the same QoS flow.

The SMF shall provision the following IEs in the QoS Monitoring per QoS flow Control Information IE:

-	one or more QFI IEs indicating the QoS flow(s) for which QoS monitoring is requested;

-	the Requested QoS Monitoring IE indicating one or more QoS parameters to be measured;

-	the Reporting Frequency IE indicating the type of the reporting as "Event Triggered" or "Periodic":

-	If the Reporting Frequence indicates "Event Triggered", a Reporting threshold for each parameter in Requested QoS Monitoring IE and a Minimum Wait Time IE shall be provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports should not be sent by the UPF during the Minimum Wait Time. If measurement results that matches or exceeds the indicated Reporting threshold are received during the Minimum Wait Time, the UPF shall send a report containing the most recent measurement result immmediately after the Minimum Wait Time is over. After that, the UPF shall continue to report a measurement result that matches or exceeds the indicated Reporting threshold.

NOTE z1:	As an implementation option, the UPF can be configured to send subsequent report(s) during the Minimum waiting time, e.g. if the UPF determines that this report is considerably different from the previous report.

NOTE z2:	For Event-Triggered QoS monitoring reporting, the consumer of the QoS monitoring reports can determine that the QoS parameter measurement has fallen below the reporting threshold when it stops receiving reports for a duration equal to the Minimum Wait Time.

-	"Periodic" indicates that the UPF shall send a report each time the measurement period is over.

-	the Minimum Wait Time IE, to indicate the minimum waiting time between two consecutive reports, if the Event Triggered QoS monitoring reporting is required in the reporting frequency;

-	the Measurement Period IE, indicating the period to generate periodic usage reports, periodic QoS monitoring reporting is required in the reporting frequency; and/or

-	the Reporting Suggestion Info IE, i.e. Report urgency indicating whether the event report can be delayed (i.e. it is delay-tolerant) and if so, the Reporting time information defining the last valid reporting time for the UPF to report the detected event. The Reporting time information shall be present if the Report urgency indicates the event report is delay tolerant. Per Reporting suggestion info, the UPF may concatenate several QoS monitoring reports for the same PFCP session in one PFCP Session Report Request (for event reporting over N4) or in one UPF event notification towards the same notification endpoint identified by the notification URI and notification correlation id (for event reporting using the Nupf_Event_Exposure service);

If the UPF supports Direct Reporting of QoS monitoring events (i.e. the DRQOS feature, see clause 8.2.25), the SMF shall include the Direct Reporting Information IE in the SRR, including the Event Notification URI towards which the events shall be sent, and if available, the Notification Correlation ID to be included in these events, if the UPF shall send the reports to a different NF than the SMF (i.e. to the Local NEF or the AF). If the events need to be sent both directly to the consumer (e.g. to the local NEF or AF) and to the SMF, the Direct Reporting Information IE shall also include the Reporting Flags IE with the DUPL flag set to "1"; if so, the UPF shall send the reports to the SMF as well. If the the Direct Reporting Information IE is not provided, the UPF shall send the reports to the SMF.

NOTE z3:	The QoS measurements requested to be measured in a given SRR share the same Direct Reporting Information, including the same Event Notification URI.

The SMF may instruct to measure different QoS parameters of a same QoS flow in a same QoS Monitoring per QoS flow Control Information IE, or in separate QoS Monitoring per QoS flow Control Information IEs if different instructions need to be set in the QoS Monitoring per QoS flow Control Information IE for the different QoS parameters (e.g. different Reporting Frequency or Measurement Period).

The SMF may update the QoS parameters to be measured by an on-going Per QoS flow QoS monitoring by sending a PFCP Modification Request with the Update SRR IE with a modified QoS Monitoring per QoS flow Control Information IE adding or removing QoS parameters to be measured; the SMF shall provide the complete list of QoS Monitoring per QoS flow Control Information IEs, and the UPF shall replace any earlier list by the new received list of QoS Monitoring per QoS flow Control Information IEs. Upon receiving such a PFCP Modification Request message, the UPF shall measure the QoS parameters according to the newly received instructions from the SMF.

The SMF may require the UPF to stop an on-going Per QoS flow QoS monitoring by sending a PFCP Modification Request with the Remove SRR IE, or by sending a PFCP Modification Request with the Update SRR IE within which the previous QoS Monitoring per QoS flow Control IE is removed. Upon receiving such a PFCP Modification Request message, the UPF shall stop the on-going Per QoS flow QoS monitoring.

# *** Next Change ***

### 5.39.4	QoS Monitoring Reporting

The UPF shall report the QoS monitoring result of the QoS flows by sending:

-	QoS Monitoring Report IEs to the SMF in PFCP Session Report Request. Several QoS Monitoring Report IEs may be present to report QoS measuremeents for multiple QoS flows; and/or

-	QoS monitoring events directly to the Local NEF or AF using the UPF Event Exposure service specified in 3GPP TS 29.564 [73]. See clause 5.33.5.

An UPF that supports Direct Reporting of QoS monitoring events to the consumer (e.g. to the Local NEF or AF) shall send the Per QoS Flow Per UE QoS Monitoring events to the SMF if the SRR does not contain the Direct Reporting Information IE, or directy to the consumer (e.g. to the Local NEF or AF) using the notification URI received in the Direct Reporting Information IE, or both if the Direct Reporting Information IE is provided with the DUPL flag set to "1", as specified in clause 5.39.2.

NOTE:	The Event Notification URI and the Notification Correlation ID included in the Direct Reporting Information IE can be an NF other than an AF/NEF, e.g. an NWDAF.

The UPF shall continue to apply all the provisioned SRR(s) and perform the related QoS monitoring measurement(s), until getting any further instruction from the CP function.

# '*** Next Change ***

#### 7.5.2.9	Create SRR IE within PFCP Session Establishment Request

The Create SRR grouped IE shall be encoded as shown in Figure 7.5.2.9-1.

Table 7.5.2.9-1: Create SRR IE within PFCP Session Establishment Request

| Octet 1 and 2 |  |  | Create SRR IE Type = 212 (decimal) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  |  | Length = n |  |  |  |  |  |  |
| Information elements | P | Condition / Comment |  | Appl. |  |  |  |  | IE Type |
|  |  |  |  | Sxa | Sxb | Sxc | N4 | N4mb |  |
| SRR ID | M | This IE shall uniquely identify the SRR among all the SRRs configured for this PFCP session. |  | - | - | - | X | - | SRR ID |
| Access Availability Control Information | C | This IE shall be present if the UPF needs to report when an access type becomes available or not available (see clause 5.20.4.2). |  | - | - | - | X | - | Access Availability Control Information |
| QoS Monitoring per QoS flow Control Information | C | This IE shall be present if per QoS Flow QoS monitoring reporting is triggered. Several IEs within the same IE type may be present to represent a list of QoS Monitoring per QoS flow Control Information for different QoS flows, or for different QoS parameters of a same QoS flow (see clause 5.39.2). |  | - | - | - | X | - | QoS Monitoring per QoS flow Control Information |
| Direct Reporting Information | C | This IE shall be present if the UPF indicated support of the DRQOS feature and if the QoS monitoring events shall be reported by the UPF directly to the consumer (e.g. to the Local NEF or AF), or both to the SMF and the consumer (e.g. to the Local NEF/AF) (see clause 5.33.5). |  | - | - | - | X | - | Direct Reporting Information |
| Traffic Parameter Measurement Control Information | C | This IE shall be present if the SMF requests the UPF to perform Traffic Parameter measurement for one or more QoS Flows. See clauses 5.37 and 5.38.4. Several IEs within the same IE type may be present to represent a list of Traffic Parameter Measurement Control Information for different QoS flows or different traffic parameters. |  | - | - | - | X | - | Traffic Parameter Measurement Control Information |

******Skipped for clarity*******

# *** End of Changes ***