# 3GPP TSG CT Meeting #108	CP-251228

# Prague, CZ, 09 – 10 June, 2025											 (Revision of C3-252416)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.523 | CR | 0116 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE policy delivery outcome handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | UEP19 |  |  |  |  |  | Date: |  |  | 2025-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In CT3#138 the UEP19 work item was agreed with number of C3-245481. Within the work item scope, we identified the issue that when the UE policy is not completely successfully delivered, the current delivery outcome handling is not complete. In CT3#139, we further submitted a discussion paper to present the available information at PCF and one of the possibilities to handle the outcome delivery when the unsuccessfully delivered UE policy information section is received by the PCF. During the discussions over the meeting and offline discussions, companies agreed on that there is a need to implement reporting failure of UE policy authorization/delivery towards the AF, but not agreed on returning the detailed information of the unauthorized UE policies. In this CR, we submit a proposal to complete the UE policy delivery outcome with the support of reporting the failed UE policy type together with unsuccessfully UE policy outcome delivery. 3GPP TS 24.501 clause D.2.1.4 specifies: If the UE could not execute all instructions included in the UE policy section management list IE successfully, the UE shall: a) set the PTI IE to the PTI value received within the MANAGE UE POLICY COMMAND message and encode the results by including: 1) the UPSI associated with the instructions which could not be executed successfully; 2) the failed instruction order set to order of the instruction in the UE policy section management sublist of the received UE policy section management list IE which could not be executed successfully; and 3) the cause of the failure, in the UE policy section management result IE as specified in subclause D.5.3 and include it in a MANAGE UE POLICY COMMAND REJECT message, and b) transport the MANAGE UE POLICY COMMAND REJECT message using the NAS transport procedure as specified in subclause 5.4.5. Upon receipt of the MANAGE UE POLICY COMMAND REJECT message, the PCF shall stop timer T3501. Any instruction that was included in the UE policy section management list IE but not indicatedin the UE policy section management result IE of the received MANAGE UE POLICY COMMAND REJECT message, shall be considered as successfully executed. According to the 3GPP TS 24.501 clause D.5.3.1, the MANAGE UE POLICY COMMAND REJECT message is defined as: Table D.5.3.1.1: MANAGE UE POLICY COMMAND REJECT message content IEI; Information Element; Type/Reference; Presence; Format; Length. PTI; Procedure transaction identity 9.6; M; V; 1. MANAGE UE POLICY COMMAND REJECT message identity; UE policy delivery service message type D.6.1.; M; V; 1. UE policy section management result; UE policy section management result D.6.3; M; LV-E; 11-65533. NOTE: The total length of the MANAGE UE POLICY COMMAND REJECT message content cannot exceed 65535 octets (see Payload container contents maximum length as specified in subclause 9.11.3.39.1).. In 29.525, during the UE policy association response or the PCF updatenofity procedure, the PCF provides the UE policy as a Bytes that follows the "MANAGE UE POLICY COMMAND", as: The (V-)(H-)PCF may combine several policy sections into one "MANAGE UE POLICY COMMAND" message, if the predefined size limit is observed. The following rules apply to policy sections: - The size shall be below the predefined size limit. - The policy section shall only contain complete URSP rule(s), WLANSP rule(s), N3AN node configuration information, V2XP, A2XP, ProSeP and/or RSLPP info content, but no fractions of such rules, configuration information, or info contents. - To ease a subsequent partial update of UE policies, policy sections should only contain a small number of policies, e.g. URSP rule(s), and/or WLANSP rule(s). - The entire content of a policy section shall be provided by a single PLMN. After sending a "MANAGE UE POLICY COMMAND" messages, the (V-)(H-)PCF shall wait for a related confirmation in a "MANAGE UE POLICY COMPLETE" messages or failure indication in a "MANAGE UE POLICY COMMAND REJECT" message. The PCF for UE reports the UE policy delivery results as a Byte that follows the "MANAGE UE POLICY COMPLETE" or "MANAGE UE POLICY COMMAND REJECT". In each Result, it contains the failed instruction order as: 8; 7; 6; 5; 4; 3; 2; 1. UPSC; octet f octet f+1. Failed instruction order; octet f+2 octet f+3. Cause; octet f+4. Figure D.6.3.5: Result PCF is the consumer that send the instruction to the UE as: 8; 7; 6; 5; 4; 3; 2; 1. Instruction contents length; octet d+5 octet d+6. UPSC; octet d+7 octet d+8. UE policy section contents; octet (d+9)* octet k*. Figure D.6.2.5: Instruction 8; 7; 6; 5; 4; 3; 2; 1. UE policy part 1; octet l octet m. UE policy part 2; octet (m+1)* octet n*. …; octet (n+1)* … octet o*. UE policy part N; octet (o+1)* octet p*. Figure D.6.2.6: UE policy section contents 8; 7; 6; 5; 4; 3; 2; 1. UE policy part contents length; octet q octet q+1. 0; 0; 0; 0; UE policy part type; octet q+2. Spare. UE policy part contents; octet q+3 octet r. Figure D.6.2.7: UE policy part Therefore, when PCF receives the failed UPSC and instruction encoded as the "MANAGE UE POLICY COMMAND REJECT" from UE, the PCF can determine the unsuccessfully executed UE policy part type. The UE policy part type may contain UE policy for complete URSP rule(s), WLANSP rule(s), N3AN node configuration information, V2XP, A2XP, ProSeP and/or RSLPP info content as specified in TS 24.501: Therefore, the unsuccessfully executed UE policy part type provided by the UE are available at the PCF. The PCF can report such information from PCF to NEF/AF to indicate the unsuccessfully executed UE policy part types that lead to the unsuccessful UE policy delivery related to the invocation of AF provisioned service parameters. |
| d |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support to report the unsuccessfully executed UE policy part types of the UE policy delivery outcome, together with the unsuccessful UE policy delivery event. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE policy delivery outcome is not complete and not aligned with UE response structure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4.2, 5.6.1, 5.6.2.8, 5.8. A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29523_Npcf_EventExposure.yaml TS29525_Npcf_UEPolicyControl.yaml TS29574_Ndccf_ContextManagement.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 2 for plenary: CT3 agreed CR in CT3#141 C3-252416 incorrectly typed in the OpenAPI as: $ref: 'TS29522_CommonData.yaml#/components/schemas/UePolEventInfo' And now it is revised to $ref: 'TS29522_ServiceParameter.yaml#/components/schemas/UePolEventInfo' . |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** First Change ***

#### 4.2.4.2	Notification about subscribed events

Figure 4.2.4.2-1 illustrates the notification about subscribed events.

Figure 4.2.4.2-1: Notification about subscribed events

If the PCF observes policy control related event(s) for which an NF service consumer has subscribed explicitly as defined in clause 4.2.2 or implicitly when the subscription information is obtained from the UDR for application data, the PCF shall send an HTTP POST request as shown in figure 4.2.4.2-1, step 1, with the "{notifUri}" as request URI containing the value previously provided by the NF service consumer within the corresponding subscription or containing the callback URI provided by the AF to the UDR, and the "PcEventExposureNotif" data structure.

The "PcEventExposureNotif" data structure shall include:

-	The notification correlation ID provided by the NF service consumer during the subscription as "notifId" attribute or obtained from the UDR as specified in 3GPP TS 29.519 [24]; and

-	information about the observed event(s) within the "eventNotifs" attribute that shall contain for each observed event an "PcEventNotification" data structure that shall include:

1.	the Policy Control event as "event" attribute;

2.	for an access type change:

a)	new access type as "accType" attribute;

b)	the new RAT type as "ratType" attribute, if applicable for the notified access type; and

c)	if the "ATSSS" feature is supported:

i.	if it is the first access type report for a PDU session, and both, 3GPP and non-3GPP access information is available, the "addAccessInfo" attribute. The "addAccessInfo" attribute contains the additional access type information, where the access type is encoded in the "accessType" attribute, and the RAT type is encoded in the "ratType" attribute when applicable for the notified access type;

ii.	if it is a subsequent access type change report:

-	if a new access type is added to the MA PDU session, the "addAccessInfo" attribute with the added access type encoded in the "accessType" attribute, and the RAT type encoded in the "ratType" attribute when applicable for the notified access type;

-	if an access type is released in the MA PDU session, the "relAccessInfo" attribute with the released access type encoded in the "accessType" attribute, and the RAT type encoded in the "ratType" attribute when applicable for the notified access type; and

NOTE 1:	For a MA PDU session, if the "ATSSS" feature is not supported by the AF, the PCF includes the "accessType" attribute and the "ratType" attribute with a currently active combination of access type and RAT type (if applicable for the notified access type). When both 3GPP and non-3GPP accesses are available, the PCF includes the information corresponding to the 3GPP access.

d)	for EPC interworking scenarios, the ePDG address as "anGwAddr" attribute, if applicable for the notified access type;

3.	for a PLMN change:

a)	new network identity containing the PLMN Identifier or the SNPN Identifier in the "plmnId" attribute;

NOTE 2:	The SNPN Identifier consists of the PLMN Identifier and the NID.

NOTE 3:	Mobility between non-equivalent SNPNs, and between SNPN and PLMN is not supported. When the UE is operating in SNPN access mode, the trigger reports changes of equivalent SNPNs.

4.	when the feature "AMPoliciesEvents" is supported, for a service area coverage change, the new service area coverage in the "appliedCov" attribute, encoded as specified in 3GPP TS 29.534 [23], clause 4.2.7.4;

NOTE 4:	The service area coverage change event is met and the notification is triggered when the PCF determines the tracking areas where the service is allowed in relation to the NF consumer requested service area coverage. The actual service area coverage for the UE might be larger than the one reported with the service area coverage change event.

5.	for a satellite backhaul category change:

a)	when the "SatelliteBackhaul" feature is supported:

i)	the satellite backhaul category (i.e., "GEO", "MEO", "LEO", or "OTHER_SAT") or the indication of non-satellite backhaul category (i.e., "NON_SATELLITE") in the "satBackhaulCategory" attribute; or

b)	when dynamic satellite backhaul is used by the NG-RAN and the feature "EnSatBackhaulCatChg" is supported:

i)	the dynamic satellite backhaul category (i.e., "DYNAMIC_GEO", "DYNAMIC_MEO", "DYNAMIC_LEO", or "DYNAMIC_OTHER_SAT") in the "satBackhaulCategory" attribute;

6.	when the feature "DeliveryOutcome" is supported, to report the unsuccessful outcome of the UE Policy Delivery related to the invocation of AF provisioned service parameters, the reason of failure within the "delivFailure" attribute. If feature "ExtDeliveryOutcome" is also supported, when the PCF, based on the received "uePolDelResult" attribute from the AMF indicating the UE Policy Delivery result as defined in 3GPP TS 29.525 [11], may determine and include the unsuccessfully executed UE policy detailed information related to the invocation of AF provisioned service parameters within the "uePolEventInfos" attribute;

7.	the identity of the affected UE in the "supi" attribute and, if available, in the "gpsi" attribute;

8.	the time at which the event was observed encoded as "timeStamp" attribute;

9.	if available, and if the feature "ExtendedSessionInformation" is supported, information about the PDU session involved in the reported event in the "pduSessInfo" attribute, that shall include:

a)	the S-NSSAI of the PDU session in the "snssai" attribute;

b)	the DNN of the PDU session in the "dnn" attribute; and

c)	the IPv4 address in the "ueIpv4" attribute and/or the IPv6 prefix in the "ueIpv6" attribute, or the Ethernet MAC address in the "ueMac" attribute; and

if the IPv4 address is included in the "ueIpv4" attribute, may include the IP domain in the "ipDomain" attribute;

10.	if available, and if the feature "ExtendedSessionInformation" is supported, information about the services involved in the reported event in the indicated PDU session in the "repServices" attribute, which may include per identified service:

a)	a list of Ethernet flows in the "servEthFlows" attribute which contains an impacted Ethernet flow number within the "flowNumber" attribute in each EthernetFlowInfo data structure; or

b)	a list of IP flows in the "servIpFlows" attribute which contains an impacted IP flow number within the "flowNumber" attribute in each IpFlowInfo data structure; and/or

c)	an AF application identifier in the "afAppId" attribute.

11.	for an application detection event and if the feature "AppDetection" is supported:

a)	optionally, information about the PDU session involved in the reported event in the "pduSessInfo" attribute; and

NOTE 5:	In the case of subscriptions targeting any UE, the served UE IP address (an Ipv4Addr or Ipv6Prefix or both if available) can be necessary to be provided in the "pduSessionInfo" attribute for the AF to function properly.

b)	the application identifier for which the notification applies in the "appId" attribute.

12.	for an rate limitation information event and if the feature "RateLimitReport" is supported, the authorized session AMBR information for the PDU session in the "rateLimitRepo" attribute.13.	for a signalling information event, if the feature "SignallingInfo" is supported:

a)	the signalling information within the "sigInfos" attribute.

14.	when the feature "PCFServParAuth" is supported, to report the unsuccessful PCF authorization related to the invocation of AF provisioned service parameters, the reason of failure within the "delivFailure" attribute. In addition, if feature "ExtDeliveryOutcome" is also supported, the PCF may include the not successfully authorized UE policy detailed information related to the invocation of AF provisioned service parameters within the "uePolEventInfos" attribute.

If the NF service consumer cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.7.

If the feature "ES3XX" is supported, and the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

Upon successful reception of the HTTP POST request with "{notifUri}" as request URI and a "PcEventExposureNotif" data structure as request body, the NF service consumer shall send a "204 No Content" HTTP response, as shown in figure 4.2.4.2-1, step 2, for a successful processing.

# *** Next Change ***

### 5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Npcf_EventExposure service based interface protocol.

Table 5.6.1-1: Npcf_EventExposure specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EthernetFlowInfo | 5.6.2.6 | Identification of an UL/DL ethernet flow. | ExtendedSessionInformation |
| IpFlowInfo | 5.6.2.7 | Identification of an UL/DL IP flow. | ExtendedSessionInformation |
| PcEvent | 5.6.3.3 | Policy Control Events. |  |
| PcEventExposureNotif | 5.6.2.3 | Describes notifications about Policy Control events that occurred in an Individual Policy Events Subscription resource. |  |
| PcEventExposureSubsc | 5.6.2.2 | Represents an Individual Policy Events Subscription resource. |  |
| PcEventNotification | 5.6.2.8 | Represents the information reported for a Policy Control event. |  |
| PduSessionInformation | 5.6.2.9 | Represents PDU session identification information. | ExtendedSessionInformation AppDetection |
| ReportingInformation | 5.6.2.4 | Represents the type of reporting the subscription requires. |  |
| ServiceIdentification | 5.6.2.5 | Identification of the service to which the subscription applies. | ExtendedSessionInformation |
| SnssaiDnnCombination | 5.6.2.10 | Represents a combination of S-NSSAI and DNN(s). | EneNA |

Table 5.6.1-2 specifies data types re-used by the Npcf_EventExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Npcf_EventExposure service based interface.

Table 5.6.1-2: Npcf_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AccessType | 3GPP TS 29.571 [14] | Access Type. |  |
| AdditionalAccessInfo | 3GPP TS 29.512 [9] | Indicates the combination of additional Access Type and RAT Type for MA PDU session. | ATSSS |
| AfAppId | 3GPP TS 29.514 [12] | AF application Identifier. | ExtendedSessionInformation |
| Ambr | 3GPP TS 29.571 [14] | Contains AMBR. | RateLimitReport |
| AnGwAddress | 3GPP TS 29.514 [12] | Carries the control plane address of the EPC untrusted non-3GPP access network gateway. (NOTE 1) |  |
| ApplicationId | 3GPP TS 29.571 [14] | Application Identifier. | AppDetection |
| DateTime | 3GPP TS 29.571 [14] | Time stamp. |  |
| Dnn | 3GPP TS 29.571 [14] | Identifies a DNN. |  |
| DurationSec | 3GPP TS 29.571 [14] | Seconds of duration. |  |
| EthFlowDescription | 3GPP TS 29.514 [12] | Identifies an ethernet flow description. (NOTE 2) | ExtendedSessionInformation |
| Failure | 3GPP TS 29.522 [25] | Indicates the failure reason for an unsuccessful outcome of the UE Policy Delivery or service parameter authorization. | DeliveryOutcome PCFSerParAuth |
| FlowDescription | 3GPP TS 29.514 [12] | Identifies an IP flow description. | ExtendedSessionInformation |
| Gpsi | 3GPP TS 29.571 [14] | Generic Public Subscription Identifier. |  |
| GroupId | 3GPP TS 29.571 [14] | Identifies a group of UEs. |  |
| MacAddr48 | 3GPP TS 29.571 [14] | Mac Address of the UE. | ExtendedSessionInformation |
| MutingExceptionInstructions | 3GPP TS 29.571 [14] | Contains instructions to be executed upon the occurrence of an event muting exception (e.g. full buffer). |  |
| MutingNotificationsSettings | 3GPP TS 29.571 [14] | Contains setting related to the muting of notifications. |  |
| NfSignallingInfo | 3GPP TS 29.571 [14] | Contains NF signalling information. | SignallingInfo |
| NotificationFlag | 3GPP TS 29.571 [14] | Notification flag. | EneNA |
| NotificationMethod | 3GPP TS 29.508 [15] | Represents the Notification Method. |  |
| PartitioningCriteria | 3GPP TS 29.571 [14] | Used to partition UEs before applying sampling. | EneNA |
| PlmnIdNid | 3GPP TS 29.571 [14] | Identifies the network: the PLMN Identifier or the SNPN Identifier. (NOTE 3) |  |
| RatType | 3GPP TS 29.571 [14] | RAT Type. |  |
| RedirectResponse | 3GPP TS 29.571 [14] | Contains redirection related information. | ES3XX |
| SamplingRatio | 3GPP TS 29.571 [14] | Sampling Ratio. |  |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [14] | Indicates the satellite or non-satellite backhaul category. | SatelliteBackhaul |
| ServiceAreaCoverageInfo | 3GPP TS 29.534 [23] | Service area coverage in terms of tracking area codes and serving network. | AMPoliciesEvents |
| Snssai | 3GPP TS 29.571 [14] | Identifies a S-NSSAI. |  |
| Supi | 3GPP TS 29.571 [14] | Identifies the SUPI of the UE. |  |
| SupportedFeatures | 3GPP TS 29.571 [14] | Used to negotiate the applicability of the optional features defined in clause 5.8. |  |
| TimeWindow | 3GPP TS 29.122 [26] | Represents a Time Window. | SignallingInfo |
| UePolEventInfo | 3GPP TS 29.522 [25] | Event information about the outcome of Service Parameter provisioning requests for the UE policy delivery. | ExtDeliveryOutcome |
| Uinteger | 3GPP TS 29.571 [14] | Unsigned integer. |  |
| NOTE 1: "AnGwAddress" data structure is only used to encode the ePDG address and is only applicable to the 5GS and EPC/E-UTRAN interworking scenario as defined in 3GPP TS 29.512 [9], Annex B. NOTE 2: In order to support a set of MAC addresses with a specific range in the traffic filter, feature MacAddressRange as specified in clause 5.8 shall be supported. NOTE 3: The SNPN Identifier consists of the PLMN Identifier and the NID. |  |  |  |

# *** Next Change ***

#### 5.6.2.8	Type PcEventNotification

Table 5.6.2.8-1: Definition of type PcEventNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | PcEvent | M | 1 | Reported Policy Control event. |  |
| accType | AccessType | C | 0..1 | Access Type. It shall be included when the reported PcEvent is "AC_TY_CH". |  |
| addAccessInfo | AdditionalAccessInfo | O | 0..1 | Indicates the additional combination of Access Type and RAT Type available for MA PDU session. It may be present when the notified event is "AC_TY_CH" and the PDU session is a Multi-Access PDU session. | ATSSS |
| relAccessInfo | AdditionalAccessInfo | O | 0..1 | Indicates the release of a combination of Access Type and RAT Type available for MA PDU session. It may be present when the notified event is "AC_TY_CH" and the PDU session is a Multi-Access PDU session. | ATSSS |
| anGwAddr | AnGwAddress | O | 0..1 | ePDG address. It shall be included if applicable when the reported PcEvent is "AC_TY_CH". |  |
| ratType | RatType | O | 0..1 | RAT Type. It shall be included if applicable when the reported PcEvent is "AC_TY_CH". |  |
| plmnId | PlmnIdNid | C | 0..1 | PLMN Identifier or the SNPN Identifier. It shall be included when the reported PcEvent is "PLMN_CH". (NOTE 1) |  |
| appliedCov | ServiceAreaCoverageInfo | C | 0..1 | The list of applied allowed Tracking Areas for the serving network where the UE is camping. It shall be included when the reported PcEvent is "SAC_CH". | AMPoliciesEvents |
| supi | Supi | C | 0..1 | SUPI of the UE. It shall be present if available. |  |
| gpsi | Gpsi | O | 0..1 | Gpsi shall contain either an External Id or an MSISDN. |  |
| timeStamp | DateTime | M | 1 | Time at which the event is observed. |  |
| pduSessInfo | PduSessionInformation | O | 0..1 | Represents PDU session information related to the observed event. | ExtendedSessionInformation, AppDetection |
| appId | ApplicationId | O | 0..1 | Represents the detected application. | AppDetection |
| repServices | ServiceIdentification | O | 0..1 | Represents service information related to the observed event. | ExtendedSessionInformation |
| satBackhaulCategory | SatelliteBackhaulCategory | C | 0..1 | Indicates the satellite or non-satellite backhaul category of the PDU session. It shall be included when the reported PcEvent is "SAT_CATEGORY_CH". If the "EnSatBackhaulCatChg" feature is supported, the different dynamic satellite backhaul categories may also be provided. | SatelliteBackhaul |
| delivFailure | Failure | C | 0..1 | Indicates the failure reason for an unsuccessful outcome of the UE Policy Delivery or service parameter authorization in the PCF. It shall be included when the reported PcEvent is "UNSUCCESS_UE_POL_DEL_SP". | DeliveryOutcome PCFSerParAuth |
| uePolEventInfos | array(UePolEventInfo) | O | 1..N | Contains the UE policy event information. It may be included when the reported PcEvent is "PARTLY_UNSUCC_UE_POL_DEL_SP". | ExtDeliveryOutcome |
| rateLimitRepo | Ambr | C | 0..1 | Rate limitation information for a PDU session. It shall be included when the reported PcEvent is "RATE_LIMIT_INFO_REPO". | RateLimitReport |
| sigInfos | array(NfSignallingInfo) | C | 1..N | Contains signalling information. It shall be included when the reported event is "SIGNALLING_INFO". | SignallingInfo |
| NOTE 1: The SNPN Identifier consists of the PLMN Identifier and the NID. NOTE 2: The values "DNN_NOT_ALLOWED" and "SNSSAI_NOT_ALLOWED" are only applicable if the feature "PCFSerParAuth" is supported. |  |  |  |  |  |

Editor's note:	The date type for data rate limitation reporting can be updated according to SA2 progress.

# *** Next Change ***

#### 5.6.3.3	Enumeration: PcEvent

The enumeration PcEvent represents the policy control events that can be subscribed. It shall comply with the provisions defined in table 5.6.3.3-1.

Table 5.6.3.3-1: Enumeration PcEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AC_TY_CH | Access Type Change |  |
| PLMN_CH | PLMN Change |  |
| SAC_CH | Service Area Coverage change | AMPoliciesEvents |
| SAT_CATEGORY_CH | Indicates that a change between different satellite backhaul category, or non-satellite backhaul, has been detected. | SatelliteBackhaul |
| SUCCESS_UE_POL_DEL_SP | Indicates about the successful UE Policy delivery related to the invocation of AF provisioned service parameters. | DeliveryOutcome |
| UNSUCCESS_UE_POL_DEL_SP | Indicates about the unsuccessful UE Policy delivery related to the invocation of AF provisioned service parameters. | DeliveryOutcome |
| PARTLY_UNSUCC_UE_POL_DEL_SP | Partially unsuccessful UE Policy Delivery related to the invocation of AF provisioned Service Parameters. | ExtDeliveryOutcome |
| UNSUCCESS_PCF_SERVICE_AUTHORIZATION | Unsuccessful authorization in the PCF to the invocation of AF provisioned Service Parameters. | PCFSerParAuth |
| APPLICATION_START | The start of application traffic has been detected. (NOTE) | AppDetection |
| APPLICATION_STOP | The stop of application traffic has been detected. (NOTE) | AppDetection |
| RATE_LIMIT_INFO_REPO | Indicates reporting of the Rate Limitation Information for the PDU session. | RateLimitReport |
| SIGNALLING_INFO | Information related to signalling is being reported. | SignallingInfo |
| NOTE: The subscription to the "APPLICATION_START" and/or "APPLICATION_STOP" event(s) shall trigger the provisioning of the PCC rule(s) for application detection and control in the SMF as specified in clauses 4.2.2.2, 4.2.2.3 and 4.2.3.2. |  |  |

# *** Next Change ***

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Npcf_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | ExtendedSessionInformation | Indicates the support of additional session information in the subscription and report of policy control event. |
| 2 | MacAddressRange | Indicates the support of a set of MAC addresses with a specific range in the traffic filter. |
| 3 | ATSSS | Indicates the support of the report of the multiple access types of a MA PDU session. |
| 4 | ES3XX | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5]. |
| 5 | AMPoliciesEvents | Indicates the support of the report of changes of service area coverage for a UE. |
| 6 | EneNA | This feature indicates support for the enhancements of network data analytics requirements. |
| 7 | SatelliteBackhaul | Indicates the support of the report of the satellite or non-satellite backhaul category of the PDU session. |
| 8 | DeliveryOutcome | Indicates the support of notifications about the outcome of the UE Policy delivery related to the invocation of AF provisioned service parameters. |
| 9 | ERIR | Indicates the support of immediate report within the subscription response. |
| 10 | EnSatBackhaulCatChg | This feature indicates the support of the report of the dynamic satellite backhaul category of the PDU session. This feature requires the support of SatelliteBackhaul feature. |
| 11 | AppDetection | Indicates the support of Application Traffic Detection Event Exposure. |
| 12 | RateLimitReport | This feature indicates the support of the report of the Authorized Session AMBR as the Rate Limitation Information for the PDU session. |
| 13 | SignallingInfo | This feature indicates the support of signalling information events. |
| 14 | PCFSerParAuth | Indicates the support of notifications about the unsuccessful authorization related to the invocation of AF provisioned service parameters. |
| 15 | ExtDeliveryOutcome | Indicates the support of notifications about the unsuccessfully executed UE policy outcome together with the partially unsuccessful UE policy delivery event related to the invocation of AF provisioned service parameters. This feature requires the support of DeliveryOutcome feature and PCFSerParAuth feature. |

# *** Next Change ***

# A.2	Npcf_EventExposure API

openapi: 3.0.0

info:

version: 1.4.0-alpha.1

title: Npcf_EventExposure

description: |

PCF Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.523 V19.0.0; 5G System; Policy Control Event Exposure Service; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.523/

servers:

- url: '{apiRoot}/npcf-eventexposure/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- npcf-eventexposure

paths:

/subscriptions:

post:

summary: Creates a new Individual Policy Control Events Subscription resource

operationId: PostPcEventExposureSubsc

tags:

- Policy Control Events Subscription (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

headers:

Location:

description: >

Contains the URI of the created individual policy control events subscription

resource, according to the structure

{apiRoot}/npcf-eventexposure/v1/subscriptions/{subscriptionId}

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

PcEventNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureNotif'

responses:

'204':

description: No Content, Notification was succesfull.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: "Reads an existing Individual Policy Control Events Subscription"

operationId: GetPcEventExposureSubsc

tags:

- Individual Policy Control Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Policy Control Event Subscription ID.

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: "Modifies an existing Individual Policy Control Events Subscription "

operationId: PutPcEventExposureSubsc

tags:

- Individual Policy Control Events Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

parameters:

- name: subscriptionId

in: path

description: Policy Control Event Subscription ID.

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PcEventExposureSubsc'

'204':

description: No Content. Resource was succesfully modified.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: "Cancels an existing Individual Policy Control Events Subscription "

operationId: DeletePcEventExposureSubsc

tags:

- Individual Policy Control Events Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Policy Control Event Subscription ID.

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

npcf-eventexposure: Access to the Npcf_EventExposure API.

schemas:

PcEventExposureNotif:

description: >

Represents notifications about Policy Control events related to an Individual

Policy Events Subscription resource.

type: object

properties:

notifId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/PcEventNotification'

minItems: 1

required:

- notifId

- eventNotifs

PcEventExposureSubsc:

description: Represents an Individual Policy Events Subscription resource.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/PcEvent'

minItems: 1

eventsRepInfo:

$ref: '#/components/schemas/ReportingInformation'

groupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

filterDnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

filterSnssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

snssaiDnns:

type: array

items:

$ref: '#/components/schemas/SnssaiDnnCombination'

minItems: 1

filterServices:

type: array

items:

$ref: '#/components/schemas/ServiceIdentification'

minItems: 1

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

tws:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/PcEventNotification'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubs

- notifId

- notifUri

ReportingInformation:

description: Represents the type of reporting that the subscription requires.

type: object

properties:

immRep:

type: boolean

notifMethod:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NotificationMethod'

maxReportNbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

monDur:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

repPeriod:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

sampRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

partitionCriteria:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PartitioningCriteria'

minItems: 1

description: Criteria for partitioning the UEs before applying the sampling ratio.

grpRepTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifFlag:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotificationFlag'

notifFlagInstruct:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MutingExceptionInstructions'

mutingSetting:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MutingNotificationsSettings'

ServiceIdentification:

description: Identifies the service to which the subscription applies.

type: object

properties:

servEthFlows:

type: array

items:

$ref: '#/components/schemas/EthernetFlowInfo'

minItems: 1

servIpFlows:

type: array

items:

$ref: '#/components/schemas/IpFlowInfo'

minItems: 1

afAppId:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId'

# All conditions in allOf must be met

allOf:

# First condition is that servEthFlows and servIpFlows are mutually exclusive

- not:

required: [servEthFlows, servIpFlows]

# Second condition is that at least one the servEthFlows, servIpFlows and afAppId shall be present

- anyOf:

- required: [servEthFlows]

- required: [servIpFlows]

- required: [afAppId]

EthernetFlowInfo:

description: Identifies an UL/DL ethernet flow.

type: object

properties:

ethFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

minItems: 1

maxItems: 2

flowNumber:

type: integer

required:

- flowNumber

IpFlowInfo:

description: Identifies an UL/DL IP flow.

type: object

properties:

ipFlows:

type: array

items:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

minItems: 1

maxItems: 2

flowNumber:

type: integer

required:

- flowNumber

PcEventNotification:

description: Represents the information reported for a Policy Control event.

type: object

properties:

event:

$ref: '#/components/schemas/PcEvent'

accType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

addAccessInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AdditionalAccessInfo'

relAccessInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AdditionalAccessInfo'

anGwAddr:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AnGwAddress'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

satBackhaulCategory:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

appliedCov:

$ref: 'TS29534_Npcf_AMPolicyAuthorization.yaml#/components/schemas/ServiceAreaCoverageInfo'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

pduSessionInfo:

$ref: '#/components/schemas/PduSessionInformation'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

repServices:

$ref: '#/components/schemas/ServiceIdentification'

delivFailure:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/Failure'

uePolEventInfos:

type: array

items:

$ref: 'TS29522_ServiceParameter.yaml#/components/schemas/UePolEventInfo'

minItems: 1

rateLimitRepo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

sigInfos:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSignallingInfo'

minItems: 1

required:

- event

- timeStamp

PduSessionInformation:

description: Represents PDU session identification information.

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

ueIpv4:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

ipDomain:

type: string

ueMac:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MacAddr48'

required:

- snssai

- dnn

oneOf:

- required: [ueMac]

- anyOf:

- required: [ueIpv4]

- required: [ueIpv6]

SnssaiDnnCombination:

description: Represents a combination of S-NSSAI and DNN(s).

type: object

properties:

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnns:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

minItems: 1

# Simple data types and Enumerations

PcEvent:

description: Represents the policy control events that can be subscribed.

anyOf:

- type: string

enum:

- AC_TY_CH

- PLMN_CH

- SAC_CH

- SAT_CATEGORY_CH

- SUCCESS_UE_POL_DEL_SP

- UNSUCCESS_UE_POL_DEL_SP

- UNSUCCESS_PCF_SERVICE_AUTHORIZATION

- APPLICATION_START

- APPLICATION_STOP

- RATE_LIMIT_INFO_REPO

- SIGNALLING_INFO

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

# *** End of Change ***