3GPP TSG-CT WG3 Meeting #141	CP-251289

# Bratislava, Slovakia, 19-23 May 2025                                                     was C3-252612

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1579 | rev | 4 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Nnef_VFLNFDiscovery Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 23.288 (also see the latest agreed S2-2502669), Nnef_VFLNFDiscovery service is supported by NEF to enable an untrusted AF to find a NWDAF as the VFL clients: 12.4 Nnef_VFLNFDiscovery Service 12.4.1 General Service Description: This service is provided by by an NEF towards an untrusted AF acting as VFL server to enable the AF to interact with NWDAFs acting as VFL client. It enables the AF to detect NWDAFs as VFL clients as described in in clause 6.2H.2.1. Editor´s note: Parameters of the service operations are FFS. The above API should be supported in stage-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce of Nnef_VFLNFDiscovery service API |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 requirement not be fulfilled |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.X (new), 5.X.1(new), 5.X.2(new), 5.X.3(new), 5.X.3.1(new), 5.X.3.2(new), 5.X.3.2.1(new), 5.X.3.2.2(new), 5.X.3.3(new), 5.X.3.3.1(new), 5.X.3.3.2(new), 5.X.4(new), 5.X.5(new), 5.X.5.1(new), 5.X.5.2(new), 5.X.5.2.1(new), 5.X.5.2.2(new), 5.X.5.2.3(new), 5.X.5.2.4(new), 5.X.5.3 (new), 5.X.5.3.1 (new), 5.X.5.3.2 (new), 5.X.5.3.3 (new), 5.X.6(new), 5.X.7(new), 5.X.7.1(new), 5.X.7.2(new), 5.X.7.3(new), A.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introdueces a new Nnef_VFLNFDiscovery API and does not impact any yaml file |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev4 - Correct the externalDocs number |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.1	Introduction

The NEF Northbound APIs are a set of APIs defining the related procedures and resources for the interaction between the NEF and the AF.

Tables 5.1-1 summarizes the APIs defined in this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause defined | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| TrafficInfluence | 5.4 | Traffic Influence API | TS29522_TrafficInfluence.yaml | 3gpp-traffic-influence | A.2 |
| NiddConfigurationTrigger | 5.5 | NIDD (Non-IP Data Delivery) Configuration Trigger API | TS29522_NiddConfigurationTrigger.yaml | 3gpp-nidd-configuration-trigger | A.3 |
| AnalyticsExposure | 5.6 | Analytics Exposure API | TS29522_AnalyticsExposure.yaml | 3gpp-analyticsexposure | A.4 |
| 5GLANParameterProvision | 5.7 | 5G LAN Parameter Provision API | TS29522_5GLANParameterProvision.yaml | 3gpp-5glan-pp | A.5 |
| ApplyingBdtPolicy | 5.8 | Applying BDT Policy API | TS29522_ApplyingBdtPolicy.yaml | 3gpp-applying-bdt-policy | A.6 |
| IPTVConfiguration | 5.9 | IPTV Configuration API | TS29522_IPTVConfiguration.yaml | 3gpp-iptvconfiguration | A.7 |
| LpiParameterProvision | 5.10 | LPI (Location Privacy Indicator) Parameter Provision API | TS29522_LpiParameterProvision.yaml | 3gpp-lpi-pp | A.8 |
| ServiceParameter | 5.11 | Service Parameter API | TS29522_ServiceParameter.yaml | 3gpp-service-parameter | A.9 |
| ACSParameterProvision | 5.12 | ACS Parameter Provision API | TS29522_ACSParameterProvision.yaml | 3gpp-acs-pp | A.10 |
| MoLcsNotify | 5.13 | MO LCS Notify API | TS29522_MoLcsNotify.yaml | 3gpp-mo-lcs-notify | A.11 |
| AKMA | 5.14 | AKMA API | TS29522_AKMA.yaml | 3gpp-akma | A.12 |
| TimeSyncExposure | 5.15 | Time Sync Exposure API | TS29522_TimeSyncExposure.yaml | 3gpp-time-sync | A.13 |
| EcsAddressProvision | 5.16 | ECS Address Provision API | TS29522_EcsAddressProvision.yaml | 3gpp-ecs-address-provision | A.14 |
| AMPolicyAuthorization | 5.17 | AM Policy Authorization API | TS29522_AMPolicyAuthorization.yaml | 3gpp-am-policyauthorization | A.15 |
| AMInfluence | 5.18 | AM Influence API | TS29522_AMInfluence.yaml | 3gpp-am-influence | A.16 |
| MBSTMGI | 5.19 | MBS TMGI API | TS29522_MBSTMGI.yaml | 3gpp-mbs-tmgi | A.17 |
| MBSSession | 5.20 | MBS Session API | TS29522_MBSSession.yaml | 3gpp-mbs-session | A.18 |
| EASDeployment | 5.21 | EAS Deployment API | TS29522_EASDeployment.yaml | 3gpp-eas-deployment | A.19 |
| ASTI | 5.22 | ASTI API | TS29522_ASTI.yaml | 3gpp-asti | A.20 |
| DataReporting | 5.23 | DataReporting API | TS29522_DataReporting.yaml | 3gpp-data-reporting | A.21 |
| DataReportingProvisioning | 5.24 | DataReportingProvisioning API | TS29522_DataReportingProvisioning.yaml | 3gpp-data-reporting-provisioning | A.22 |
| UEId | 5.25 | UE ID API | TS29522_UEId.yaml | 3gpp-ueid | A.23 |
| MBSUserService | 5.26 | MBSUserService API | TS29522_MBSUserService.yaml | 3gpp-mb-us | A.24 |
| MBSUserDataIngestSession | 5.27 | MBSUserDataIngestSession API | TS29522_MBSUserDataIngestSession.yaml | 3gpp-mb-ud-ingest | A.25 |
| MSEventExposure | 5.28 | MSEventExposure API | TS29522_MSEventExposure.yaml | 3gpp-ms-event-exposure | A.26 |
| MBSGroupMsgDelivery | 5.29 | MBSGroupMsgDelivery API | TS29522_MBSGroupMsgDelivery.yaml | 3gpp-mbs-group-msg | A.27 |
| DNAIMapping | 5.30 | DNAIMapping API | TS29522_DNAIMapping.yaml | 3gpp-dnai-mapping | A.28 |
| PDTQPolicyNegotiation | 5.31 | PDTQPolicyNegotiation API | TS29522_PDTQPolicyNegotiation.yaml | 3gpp-pdtq-policy-negotiation | A.29 |
| MemberUESelectionAssistance | 5.32 | MemberUESelectionAssistance API | TS29522_MemberUESelectionAssistance.yaml | 3gpp-musa | A.30 |
| GroupParametersProvisioning | 5.33 | Group Parameters Provisioning API | TS29522_GroupParametersProvisioning.yaml | 3gpp-grp-pp | A.31 |
| SliceParamProvision | 5.34 | Network Slice Parameters Provisioning API | TS29522_SliceParamProvision.yaml | 3gpp-slice-pp | A.32 |
| UEAddress | 5.35 | UE Address API | TS29522_UEAddress.yaml | 3gpp-ue-address | A.33 |
| ECSAddress | 5.36 | ECS Address Configuration Information API | TS29522_ECSAddress.yaml | 3gpp-ecs-address | A.34 |
| RSLPPIParametersProvisioning | 5.37 | RSLPPI Parameters Provisioning API | TS29522_RSLPPIParametersProvisioning.yaml | 3gpp-rslppi-pp | A.35 |
| AddressingParamProvision | 5.38 | Addressing Parameters Provisioning API | TS29522_AddressingParamProvision.yaml | 3gpp-addr-pp | A.36 |
| UAVFlightAssistance | 5.39 | UAV Flight Assistance API | TS29522_UAVFlightAssistance.yaml | 3gpp-uav-flight-assistance | A.37 |
| CagInfoParamProvision | 5.40 | CAG Information Parameters Provisioning API | TS29522_CagInfoParamProvision.yaml | 3gpp-caginfo-pp | A.38 |
| RetrieveInfoUAVFlight | 5.41 | RetrieveInfoUAVFlight API | TS29522_RetrieveInfoUAVFlight.yaml | 3gpp-retrieve-uav-flight | A.39 |
| ImsSessionManagement | 5.42 | ImsSessionMangement API | TS29522_ImsSessionManagement.yaml | 3gpp-ims-sm | A.40 |
| ImsEventExposure | 5.43 | ImsEventExposure API | TS29522_ImsEventExposure.yaml | 3gpp-ims-ee | A.41 |
| ImsParamProvision | 5.44 | ImsParamProvision API | TS29522_ImsParamProvision.yaml | 3gpp-ims-pp | A.42 |
| Nnef_VFLNFDiscovery | 5.X | VFLNFDiscovery API | TS29522_VFLNFDiscovery.yaml | 3gpp-vfl-nf-discovery | A.X |

* * * Next Change * * * *

5.X	VFLNFDiscovery API

5.X.1	Introduction

The Nnef_VFLNFDiscovery service shall use the VFLNFDiscovery API.

The API URI of VFLNFDiscovery API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [4], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	"apiRoot" is set as described in clause 5.2.4 in 3GPP TS 29.122 [4].

-	"apiName" shall be set to "3gpp-vfl-nf-discovery".

-	"apiVersion" shall be set to "v1" for the current version defined in the present document.

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [4].

All resource URIs in the clauses below are defined relative to the above API URI.

5.X.2	Resources

There are no resources defined for this API in this release of the specification.

5.X.3	Custom Operations without associated resources

5.X.3.1	Overview

The structure of the custom operation URIs of the VFLNFDiscovery API is shown in Figure 5.X.3.1-1.

Figure 5.X.3.1-1: Custom operation URI structure of the VFLNFDiscovery API

Table 5.X.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 5.X.3.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| DiscoverNwdaf | /discover-nwdaf | POST | Request the discovery of NWDAF that are to act as VFL client(s). |
| ReleaseNwdaf | /release-nwdaf | POST | Request to inform the external NWDAF ID(s) are not used anymore |

5.X.3.2	Operation: DiscoverNwdaf

5.X.3.2.1	Description

The custom operation enables an AF to discover an NWDAF that are to act as VFL client(s).

5.X.3.2.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 5.X.3.2.2-1 and table 5.X.3.2.2-2.

Table 5.X.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NwdafDiscoveryRequest | M | 1 | Represents the parameters to request the NWDAF(s) that are to act as VFL client(s). |

Table 5.X.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NwdafDiscoveryResponse | M | 1 | 200 OK | Successful case: The external NWDAF ID(s) to act as the VFL client(s) are returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.X.3.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.X.3.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

5.X.3.3	Operation: ReleaseNwdaf

5.X.3.3.1	Description

The custom operation enables an AF to inform the external NWDAF ID(s) are not used anymore.

5.X.3.3.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 5.X.3.3.2-1 and table 5.X.3.3.2-2.

Table 5.X.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NwdafReleaseRequest | M | 1 | Represents the parameters to inform the external NWDAF ID(s) are not used anymore. |

Table 5.X.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The requested NWDAF ID(s) are released for the NF. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative NEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [4] |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [4] also apply. |  |  |  |  |

Table 5.X.3.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

Table 5.X.3.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI of the resource located in an alternative NEF. |

5.X.4	Notifications

There are no Notifications defined for this API in this release of the specification.

5.X.5	Data Model

5.X.5.1	General

This clause specifies the application data model supported by the VFLNFDiscovery API. Table 5.X.5.1-1 specifies the data types defined for the VFLNFDiscovery API.

Table 5.X.5.1-1: VFLNFDiscovery specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| NwdafDiscoveryRequest | 5.X.5.2.2 | Represents the parameters to request the NWDAF(s) that are to act as VFL client(s). |  |
| NwdafDiscoveryResponse | 5.X.5.2.3 | Represents the external NWDAF ID(s). |  |
| NwdafReleaseRequest | 5.X.5.2.4 | Represents the external NWDAF ID(s) are not used anymore. |  |
| VFLCapabilityType | 5.X.5.3.3 | Represents the requested VFL capability type. |  |

Table 5.X.5.1-2 specifies data types re-used by the VFLNFDiscovery API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the VFLNFDiscovery API.

Table 5.X.5.1-2: Re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| NwdafEvent | 3GPP TS 29.520 [27] | Represents analytics ID(s). |
| TimeWindow | 3GPP TS 29.122 [4] | Represents the time period of interest. |
| GeographicArea | 3GPP TS 29.572 [34] | Represents the service area of the request. |

5.X.5.2	Structured data types

5.X.5.2.1	Introduction

This clause defines the structured data types to be used in resource representations.

5.X.5.2.2	Type: NwdafDiscoveryRequest

Table 5.X.5.2.2-1: Definition of type NwdafDiscoveryRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| analyticIds | array(NwdafEvent) | M | 1..N | Contains the analytics ID(s). |  |
| reqNfType | FFS | M | FFS | FFS |  |
| vflCapType | VFLCapabilityType | M | 1 | Contains the type of VFL capability. |  |
| vflInterInfo | string | O | 0..1 | Contains a VFL interoperability indicator |  |
| reqFeatureIds | array(string) | O | 1..N | Contains the requested feature ID(s). |  |
| timePeriod | TimeWindow | O | 0..1 | Contains the time period of interest. |  |
| serviceAreas | array(GeographicArea) | O | 1..N | Contains the service area(s) of the request. |  |

Editor’s Note :	It is FFS "reqNfType" attribute refers to type of NF (i.e. NWDAF) or type of NWDAF (e.g. AnLF, MTLF). The name of the attribute can be further updated correspondingly.

5.X.5.2.3	Type: NwdafDiscoveryResponse

Table 5.X.5.2.3-1: Definition of type NwdafDiscoveryResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| exNwdafIds | array(string) | M | 1..N | Contains the list of current external NWDAF ID(s). |  |

5.X.5.2.4	Type: NwdafReleaseRequest

Table 5.X.5.2.4-1: Definition of type NwdafReleaseRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| exNwdafIds | array(string) | M | 1..N | Contains the list of current external NWDAF ID(s). |  |

5.X.5.3	Simple data types and enumerations

5.X.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

5.X.5.3.2	Simple data types

The simple data types defined in table 5.X.5.3.2-1 shall be supported.

Table 5.X.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

5.X.5.3.3	VFLCapabilityType

Table 5.X.5.3.3-1: Enumeration VFLCapabilityType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VFL_CLIENT | VFL client is requested. |  |

5.X.6	Used Features

The optional features listed in table 5.X.6-1 are defined for the VFLNFDiscovery API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [4].

Table 5.X.6-1: Features used by VFLNFDiscovery API

| Feature number | Feature Name | Description |
| --- | --- | --- |

5.X.7	Error handling

5.X.7.1	General

For the VFLNFDiscovery API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [4]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [4] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [4].

In addition, the requirements in the following clauses are applicable for the VFLNFDiscovery API.

5.X.7.2	Protocol Errors

No specific procedures for the VFLNFDiscovery API are specified.

5.X.7.3	Application Errors

The application errors defined for the VFLNFDiscovery API are listed in table 5.X.7.3-1.

Table 5.X.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

* * * Next Change * * * *

A.X	VFLNFDiscovery API

openapi: 3.0.0

info:

title: 3gpp-vfl-nf-discovery

version: 1.0.0-alpha.1

description: |

API for VFL NF Discovery.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.3.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

servers:

- url: '{apiRoot}/3gpp-vfl-nf-discovery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials: []

paths:

/discover-nwdaf:

post:

summary: Request of NWDAF(s) that are to act as VFL client(s).

tags:

- VFL NF discovery

operationId: DiscoverNwdaf

requestBody:

description: Representation of the request of NWDAF that are to act as VFL client(s).

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafDiscoveryRequest'

responses:

'200':

description: >

OK. Successful case. The external NWDAF ID is provisioned.

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafDiscoveryResponse'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/release-nwdaf:

post:

summary: Request to inform the external NWDAF ID(s) are not used anymore.

operationId: ReleaseNwdaf

tags:

- VFL NF discovery release

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NwdafReleaseRequest'

responses:

'204':

description: No Content. Successful case.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

NwdafDiscoveryRequest:

description: >

Represents the parameters to request the NWDAF(s) that are to act as VFL client(s).

type: object

properties:

analyticIds:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NwdafEvent'

minItems: 1

vflInterInfo:

type: string

description: String representing the VFL Interoperability Information.

reqFeatureIds:

type: array

items:

type: string

minItems: 1

timePeriod

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

serviceAreas

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

required:

- analyticId

- vflCapType

NwdafDiscoveryResponse:

description: >

Represents the external NWDAF ID(s) to act as the VFL client(s).

type: object

properties:

exNwdafIds:

type: array

items:

type: string

description: The current external NWDAF identifier.

minItems: 1

required:

- exNwdafIds

NwdafReleaseRequest:

description: >

Represents the parameters to inform the external NWDAF ID(s) are not used anymore.

type: object

properties:

exNwdafIds:

type: array

items:

type: string

description: The current external NWDAF identifier.

minItems: 1

required:

- exNwdafIds

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

VFLCapabilityType:

anyOf:

- type: string

enum:

- VFL_CLIENT

- type: string

description: Represents the VFL capability type to be discovered.

description: |

Represents the VFL capability type to be discovered.

Possible values are:

- VFL_CLIENT: Represents the VFL capability type to be discovered is VFL client.

* * * End of Changes * * * *