# 3GPP TSG CT Meeting #108	CP-251241

# Prague, CZ, 09 – 10 June, 2025											 (Revision of C3-252451)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.523 | CR | 0115 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on data type PcEventExposureSubsc |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2, TEI18_ADEE |  |  |  |  |  | Date: |  |  | 2025-5-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The attribute snssaiDnn of the data type PcEventExposureSubsc was implemented as snssaiDnns in the OpenAPI, considering OpenAPI takes precedence and the fact that the data is defined as an array, the attribute name is incorrectly defined. According to the agreed FASMO criteria 1.(f), the incorrect attribute name is a FASMO change. The attribute was introduced from Re-17. A FASMO correction for the attribute is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change attribute name from snssaiDnn to snssaiDnns |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect attribute name causes incorrect specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2, 4.2.2.3, 5.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has no impact on the OpenAPI. This CR is mirror of Rel-18 CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 2 for plenary: CT3 agreed CR in CT3#141 C3-252451 is revised to include one more change about correcting the attribute snnsaiDnn clause 4.2.2.3. The modification procedure used the same attribute snnsaiDnn incorrectly and considering within the same functionality, we add the missing change in the revision. |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 4.2.2.2	Creating a new subscription

Figure 4.2.2.2-1 illustrates the creation of a subscription.

Figure 4.2.2.2-1: Creation of a subscription

To subscribe to event notifications, the NF service consumer shall send an HTTP POST request with: "{apiRoot}/npcf-eventexposure/v1/subscriptions" as request URI as shown in figure 4.2.2.2-1, step 1, and the "PcEventExposureSubsc" data structure as request body.

The "PcEventExposureSubsc" data structure shall include:

-	identification of the policy events to subscribe as "eventSubs" attribute;

-	indication of the UEs to which the subscription applies via:

a)	identification of a group of UE(s) via a "groupId" attribute; or

b)	identification of any UE by omitting the "groupId" attribute;

-	a URI where to receive the requested notifications as "notifUri" attribute;

-	a Notification Correlation Identifier assigned by the NF service consumer for the requested notifications as "notifId" attribute; and

-	when the feature "AppDetection" is supported, to indicate the specific application(s) for which the application detection policy event report shall occur, the application identifier(s) in the "appIds" attribute and the concerned S-NSSAI and DNN within the "snssaiDnns" attribute.

NOTE:	The subscription to "APPLICATION_START" and/or "APPLICATION_STOP" application detection policy event(s) for one or more application identifiers can impact the SMF/UPF performance. The restriction to a specific combination of S-NSSAI and DNN avoids excessive signalling load.

The "PcEventExposureSubsc" data structure may also include:

-	description of the event reporting information as "eventsRepInfo", which may include:

a)	event notification method (periodic, one time, on event detection) as "notifMethod" attribute;

b)	Maximum Number of Reports as "maxReportNbr" attribute;

c)	Monitoring Duration as "monDur" attribute;

d)	repetition period for periodic reporting as "repPeriod" attribute;

e)	immediate reporting indication as "immRep" attribute;

f)	sampling ratio as "sampRatio" attribute;

g)	group reporting guard time as "grpRepTime" attribute;

h)	partitioning criteria for partitioning the UEs before performing sampling as "partitionCriteria" attribute if the EneNA feature is supported; and/or

i)	a notification flag as "notifFlag" attribute if the EneNA feature is supported;

-	if the supported feature "ExtendedSessionInformation" is supported, to filter the AF sessions for which the policy event report shall occur, the identification of the services one or more AF sessions may belong to as "filterServices" attribute, which may include per service identification:

a)	a list of ethernet flows in the "servEthFlows" attribute; or

b)	a list of IP flows in the "servIpFlows" attribute; and/or

c)	an AF application identifier in the "afAppId" attribute;

-	to filter the DNNs for which the policy event report shall occur, the identification of the DNNs in the "filterDnns" attribute;

-	to filter the S-NSSAIs for which the policy event report shall occur, the identification of the S-NSSAIs in the "filterSnssais" attribute;

-	when the feature "EneNA" is supported, to filter the specific DNN and S-NSSAI combination list for which the policy event report shall occur, the identification of each combination within the "snssaiDnns" attribute; and

-	when the feature "SignallingInfo" is supported, the time windows for which signalling information shall be reported within the "tws" attribute.

If the PCF cannot successfully fulfil the received HTTP POST request due to an internal PCF error or an error in the HTTP POST request, the PCF shall send an HTTP error response as specified in clause 5.7.

Upon successful reception of the HTTP POST request with "{apiRoot}/npcf-eventexposure/v1/subscriptions" as request URI and "PcEventExposureSubsc" data structure as request body, the PCF shall create a new "Individual Policy Events Subscription" resource, store the subscription and send a HTTP "201 Created" response as shown in figure 4.2.2.2-1, step 2. The PCF shall include in the "201 Created" response:

-	a Location header field; and

-	an "PcEventExposureSubsc" data type in the content.

The Location header field shall contain the URI of the created individual application session context resource i.e. "{apiRoot}/npcf-eventexposure/v1/subscriptions/{subscriptionId}".

The "PcEventExposureSubsc" data type in the response content shall contain the representation of the created "Individual Policy Events Subscription".

When the "monDur" attribute is included in the response, it represents a server selected expiry time that is equal or less than a possible expiry time in the request.

When the "immRep" attribute set to true is included in the subscription and the subscribed policy control events are available:

-	if the feature "ERIR" is not supported, the PCF shall immediately notify the NF service consumer with the current available value(s) for the subscribed event(s) using the Npcf_EventExposure_Notify service operation, as described in clause 4.2.4.2.

-	if the feature "ERIR" is supported, the PCF shall immediately notify the NF service consumer with the current available value(s) for the subscribed event(s) within the HTTP "201 Created" response as shown in figure 4.2.2.2-1, step 2. The "PcEventExposureSubsc" data type shall include the corresponding event(s) notification within the "eventNotifs" attribute, as described in clause 4.2.4.2.

When the sampling ratio as the "sampRatio" attribute is included in the subscription without a "partitionCriteria" attribute, the PCF shall select a random subset of UEs among the target UEs according to the sampling ratio and only report the event(s) related to the selected subset of UEs. If the "partitionCriteria" attribute is additionally included, then the PCF shall first partition the UEs according to the value of the "partitionCriteria" attribute and then select a random subset of UEs from each partition according to the sampling ratio and only report the event(s) related to the selected subsets of UEs.

When the group reporting guard time as the "grpRepTime" attribute is included in the subscription, the PCF shall accumulate all the event reports for the target UEs until the group reporting guard time expires. Then the PCF shall notify the NF service consumer using the Npcf_EventExposure_Notify service operation, as described in clause 4.2.4.2.

When the "notifFlag" attribute is included and set to "DEACTIVATE" in the request, the PCF shall mute the event notification and store the available events until the NF service consumer requests to retrieve them by setting the "notifFlag" attribute to "RETRIEVAL" or until a muting exception occurs (e.g. full buffer).

When the feature "AppDetection" is supported, and the "APPLICATION_START" and/or "APPLICATION_STOP" policy event(s) are included in the request, the PCF shall trigger the provisioning of the PCC rule(s) for application detection and control for the application identifier(s) included in the "appIds" attribute as specified in 3GPP TS 29.512 [9], clause 4.2.6.2.11, if not previously provisioned, and for every PDU session of the DNN and S-NSSAI included in the "snssaiDnns" attribute.

When the feature "RateLimitReport" is supported, and the "RATE_LIMIT_INFO_REPO" event is included in the request, the PCF shall determine the Authorized Session AMBR as the rate limitation for the PDU session and report to the NF service consumer in the "rateLimitRepo" attribute in the response.

# *** Next Change ***

#### 4.2.2.3	Modifying an existing subscription

Figure 4.2.2.3-1 illustrates the modification of an existing subscription.

Figure 4.2.2.3-1: Modification of an existing subscription

To modify an existing subscription to event notifications, the NF service consumer shall send an HTTP PUT request with: "{apiRoot}/npcf-eventexposure/v1/subscriptions/{subscriptionId}" as request URI, as shown in figure 4.2.2.3-1, step 1, where "{subscriptionId}" is the subscription correlation ID of the existing subscription. The "PcEventExposureSubsc" data structure is included as request body as described in clause 4.2.2.2.

NOTE 1:	An alternate NF service consumer than the one that requested the generation of the subscription resource can send the PUT.

NOTE 2:	The "notifUri" attribute within the PcEventExposureSubsc data structure can be modified to request that subsequent notifications are sent to a new NF service consumer.

If the PCF cannot successfully fulfil the received HTTP PUT request due to an internal PCF error or an error in the HTTP PUT request, the PCF shall send an HTTP error response as specified in clause 5.7.

If the feature "ES3XX" is supported, and the PCF determines the received HTTP PUT request needs to be redirected, the PCF shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

Upon successful reception of an HTTP PUT request with: "{apiRoot}/npcf-eventexposure/v1/subscriptions/{subscriptionId}" as request URI and "PcEventExposureSubsc" data structure as request body, the PCF shall store the subscription and send an HTTP "200 OK" response with the "PcEventExposureSubsc" data structure as response body or an HTTP "204 No Content" response, as shown in figure 4.2.2.3-1, step 2.

The "PcEventExposureSubsc" data structure in the response content shall contain the representation of the modified "Individual Policy Events Subscription".

When the "monDur" attribute is included in the response, it represents a NF service producer selected expiry time that is equal or less than a possible expiry time received in the request.

When the "immRep" attribute set to true is included in the updated subscription and the subscribed policy control events are available:

-	if the feature "ERIR" is not supported, the PCF shall immediately notify the NF service consumer with the current available value(s) for the subscribed event(s) using the Npcf_EventExposure_Notify service operation, as described in clause 4.2.4.2.

-	if the feature "ERIR" is supported, the PCF shall immediately notify the NF service consumer with the current available value(s) for the subscribed event(s) within the HTTP "200 OK" response as shown in figure 4.2.2.3-1, step 2a. The "PcEventExposureSubsc" data type shall include the corresponding event(s) notification within the "eventNotifs" attribute, as described in clause 4.2.4.2.

When the sampling ratio as the "sampRatio" attribute is included in the subscription without a "partitionCriteria" attribute, the PCF shall select a random subset of UEs among the target UEs according to the sampling ratio and only report the event(s) related to the selected subset of UEs. If the "partitionCriteria" attribute is additionally included, then the PCF shall first partition the UEs according to the value of the "partitionCriteria" attribute and then select a random subset of UEs from each partition according to the sampling ratio and only report the event(s) related to the selected subsets of UEs.

When the group reporting guard time as the "grpRepTime" attribute is included in the subscription, the PCF shall accumulate all the event reports for the target UEs until the group reporting guard time expires. Then the PCF shall notify the NF service consumer using the Npcf_EventExposure_Notify service operation, as described in clause 4.2.4.2.

When the "notifFlag" attribute is included, and set to "DEACTIVATE" in the request, the PCF shall mute the event notification and store the available events until the NF service consumer requests to retrieve them by setting the "notifFlag" attribute to "RETRIEVAL" or until a muting exception occurs (e.g. full buffer); if the "notifFlag" attribute is set to "RETRIEVAL" in the request, the PCF shall send the stored events to the NF service consumer, mute the event notification again and store available events; if the "notifFlag" attribute is set to "ACTIVATE" and the event notifications are muted (due to a previously received "DECATIVATE" value), the PCF shall unmute the event notification, i.e. start sending again notifications for available events.

When the feature "AppDetection" is supported, and the "APPLICATION_START" and/or "APPLICATION_STOP" policy event(s) are included in the request, the PCF shall trigger the provisioning of the PCC rule(s) for application detection and control for the application identifiers indicated within the "appIds" attribute, (if not previously provisioned) and/or the removal application detection and control for the PCC rule(s) (if application detection and control is not required for other use cases) for the removed application identifier(s) as specified in 3GPP TS 29.512 [9], clause 4.2.6.2.11, and for every PDU session of the DNN and S-NSSAI included in the "snssaiDnns" attribute. If the APPLICATION_START" and/or "APPLICATION_STOP" policy event(s) are removed from the subscription, the PCF shall trigger the removal/update of application detection and control for the PCC rule(s) (if application detection and control is not required for other use cases) for the previously subscribed application identifier(s) as specified in 3GPP TS 29.512 [9], clause 4.2.6.2.11, and for every PDU session of the previously subscribed DNN and S-NSSAI.

When the feature "RateLimitReport" is supported, and the "RATE_LIMIT_INFO_REPO" event is included in the request, the PCF shall determine the Authorized Session AMBR as the rate limitation for the PDU session and report to the NF service consumer in the "rateLimitRepo" attribute in the response.

# *** Next Change ***

#### 5.6.2.2	Type PcEventExposureSubsc

Table 5.6.2.2-1: Definition of type PcEventExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubs | array(PcEvent) | M | 1..N | Subscribed Policy Control events. |  |
| eventsRepInfo | ReportingInformation | O | 0..1 | Represents the reporting requirements of the subscription. |  |
| groupId | GroupId | C | 0..1 | Represents an internal group identifier and identifies a group of UEs. It shall be present when the subscription is targeting a Group of UE(s). |  |
| filterDnns | array(Dnn) | O | 1..N | Represents the DNNs for which the policy event report shall apply. Each DNN is a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. If omitted it represents any DNN. |  |
| filterSnssais | array(Snssai) | O | 1..N | Represents the S-NSSAIs for which the policy event report shall apply. If omitted it represents any S-NSSAI. |  |
| snssaiDnns | array(SnssaiDnnCombination) | C | 1..N | Represents the combination list of S-NSSAI and DNN for which the policy event report shall apply. If omitted, it represents any combination. When the feature "AppDetection" is supported, one combination of S-NSSAI and DNN shall be provided for event "APPLICATION_START" or "APPLICATION_STOP". | EneNA, AppDetection |
| filterServices | array(ServiceIdentification) | O | 1..N | Represents the services for which the policy event report shall apply. If omitted, the policy event report shall apply for all the active services. | ExtendedSessionInformation |
| appIds | array(ApplicationId) | C | 1..N | Represents the applications for which the policy event report shall apply. It shall be provided for event "APPLICATION_START" or "APPLICATION_STOP". | AppDetection |
| tws | array(TimeWindow) | O | 1..N | Contains the time windows for which the NF service consumer is requesting to receive signalling information. It may only be provided if the SIGNALLING_INFO event is subscribed. If omitted, the PCF determines the time windows for which the signalling information is reported based on local configuration. | SignallingInfo |
| notifUri | Uri | M | 1 | Notification URI for Policy Control event reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| eventNotifs | array(PcEventNotification) | C | 1..N | Represents the Policy Control Events to be reported in the Npcf_EvenExposure_Subscribe response. It shall be present in the resource creation/update response when the "eventsRepInfo" attribute includes the "immRep" attribute set to true. Otherwise, it shall be omitted. | ERIR |
| suppFeat | SupportedFeatures | C | 0..1 | This IE represents a list of Supported features used as described in clause 5.8. Shall be present in the HTTP POST request/response. (NOTE) |  |
| NOTE: In the HTTP request, it represents the set of features supported by the NF service consumer. In the HTTP response, it represents the set of features supported by both the NF service consumer and the PCF. |  |  |  |  |  |

# *** End of Changes ***