3GPP TSG-CT WG4 Meeting #130	C4-253569

# Göteborg, Sweden; 25th – 29th August 2025				revision of C4-253551

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 1477 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VlanTag correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_VLANSUB |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The vlanTagAllowed attribute within DnnConfiguration contains an array of vlanTagValues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Type VlanTag is renamed to VlanTagValue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.9, 6.1.6.2.127, 6.1.6.2.128, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29503_Nudm_SDM.yaml TS29504_Nudr_DR.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nudm_SDM service API.

Table 6.1.6.1-1: Nudm_SDM specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| Nssai | 6.1.6.2.2 | Network Slice Selection Assistance Information |
| SdmSubscription | 6.1.6.2.3 | A subscription to notifications |
| AccessAndMobilitySubscriptionData | 6.1.6.2.4 | Access and Mobility Subscription Data |
| SmfSelectionSubscriptionData | 6.1.6.2.5 | SMF Selection Subscription Data |
| DnnInfo | 6.1.6.2.6 | Data Network Name and associated information (LBO roaming allowed flag) |
| SnssaiInfo | 6.1.6.2.7 | S-NSSAI and associated information (DNN Info) |
| SessionManagementSubscriptionData | 6.1.6.2.8 | User subscribed session management data |
| DnnConfiguration | 6.1.6.2.9 | User subscribed data network configuration |
| PduSessionTypes | 6.1.6.2.11 | Default/allowed session types for a data network |
| SscModes | 6.1.6.2.12 | Default/allowed SSC modes for a data network |
| SmsSubscriptionData | 6.1.6.2.13 |  |
| SmsManagementSubscriptionData | 6.1.6.2.14 | SMS Management Subscription Data |
| SubscriptionDataSets | 6.1.6.2.15 |  |
| UeContextInSmfData | 6.1.6.2.16 | UE Context In SMF Data |
| PduSession | 6.1.6.2.17 |  |
| IdTranslationResult | 6.1.6.2.18 | SUPI that corresponds to a given GPSI |
| ModificationNotification | 6.1.6.2.21 |  |
| IpAddress | 6.1.6.2.22 | IP address (IPv4, or IPv6, or IPv6 prefix) |
| UeContextInSmsfData | 6.1.6.2.23 |  |
| SmsfInfo | 6.1.6.2.24 |  |
| AcknowledgeInfo | 6.1.6.2.25 |  |
| SorInfo | 6.1.6.2.26 | Steering Of Roaming Information |
| SharedData | 6.1.6.2.27 | Subscription Data shared by multiple UEs |
| PgwInfo | 6.1.6.2.28 | Information about the DNNs/APNs and PGW-C+SMF FQDNs used in interworking with EPS |
| TraceDataResponse | 6.1.6.2.29 | Contains Trace Data or a shared data Id identifying shared Trace Data |
| SteeringContainer | 6.1.6.2.30 |  |
| SdmSubsModification | 6.1.6.2.31 | Modification instruction for a subscription to notifications |
| EmergencyInfo | 6.1.6.2.32 | Information about emergency session |
| UpuInfo | 6.1.6.2.33 | UE Parameters Update Information |
| GroupIdentifiers | 6.1.6.2.34 |  |
| NiddInformation | 6.1.6.2.35 | Non-IP Data Delivery information |
| CagData | 6.1.6.2.36 |  |
| CagInfo | 6.1.6.2.37 |  |
| AdditionalSnssaiData | 6.1.6.2.38 | Additional information specific to a slice |
| VnGroupData | 6.1.6.2.39 |  |
| AppDescriptor | 6.1.6.2.40 |  |
| AppPortId | 6.1.6.2.41 | Application Port Id |
| LcsPrivacyData | 6.1.6.2.42 |  |
| Lpi | 6.1.6.2.43 |  |
| UnrelatedClass | 6.1.6.2.44 |  |
| PlmnOperatorClass | 6.1.6.2.45 |  |
| ValidTimePeriod | 6.1.6.2.46 |  |
| LcsMoData | 6.1.6.2.47 |  |
| EcRestrictionDataWb | 6.1.6.2.48 | Enhance Coverage Restriction Data |
| ExpectedUeBehaviourData | 6.1.6.2.49 | Expected UE Behaviour Data |
| SuggestedPacketNumDl | 6.1.6.2.52 | Suggested Number of Downlink Packets |
| FrameRouteInfo | 6.1.6.2.54 | Frame Route Information |
| SorUpdateInfo | 6.1.6.2.55 |  |
| EnhancedCoverageRestrictionData | 6.1.6.2.56 | Enhanced Coverage Restriction Data |
| EdrxParameters | 6.1.6.2.57 | eDRX Parameters |
| PtwParameters | 6.1.6.2.58 | Paging Time Window Parameters |
| ExternalUnrelatedClass | 6.1.6.2.62 |  |
| AfExternal | 6.1.6.2.63 |  |
| LcsClientExternal | 6.1.6.2.64 |  |
| LcsClientGroupExternal | 6.1.6.2.65 |  |
| ServiceTypeUnrelatedClass | 6.1.6.2.66 |  |
| UeId | 6.1.6.2.67 |  |
| DefaultUnrelatedClass | 6.1.6.2.68 |  |
| ContextInfo | 6.1.6.2.69 | Contains the HTTP Headers received by the NFs |
| UeContextInAmfData | 6.1.6.2.70 |  |
| V2xSubscriptionData | 6.1.6.2.71 | V2X Subscription Data |
| LcsBroadcastAssistanceTypesData | 6.1.6.2.72 | LCS Broadcast Assistance Data Types |
| DatasetNames | 6.1.6.2.73 | Data Set Names |
| PlmnRestriction | 6.1.6.2.74 |  |
| ProseSubscriptionData | 6.1.6.2.76 | ProSe Subscription Data |
| AerialUeSubscriptionInfo | 6.1.6.2.78 | Aerial UE Subscription Information |
| SmSubsData | 6.1.6.2.79 |  |
| ExtendedSmSubsData | 6.1.6.2.80 |  |
| AmfInfo | 6.1.6.2.81 | AMF information |
| ProSeAllowedPlmn | 6.1.6.2.82 | Contains the allowed PLMN information for ProSe Service |
| ImmediateReport | 6.1.6.2.83 | UDM SDM Immediate Report |
| MbsSubscriptionData | 6.1.6.2.84 | 5MBS Subscription Data |
| UcSubscriptionData | 6.1.6.2.85 | User Consent Subscription Data |
| UeContextInSmfDataSubFilter | 6.1.6.2.86 | UE Context In Smf Data Subscription Filter |
| UeIdentifiers | 6.1.6.2.87 | List of UE identifiers |
| SupiInfo | 6.1.6.2.88 | List of the SUPIs |
| TimeSyncData | 6.1.6.2.89 | Time Synchronization Data |
| TimeSyncSubscriptionData | 6.1.6.2.90 |  |
| AfRequestAuthorization | 6.1.6.2.91 |  |
| TimeSyncServiceId | 6.1.6.2.92 |  |
| LcsSubscriptionData | 6.1.6.2.93 | LCS Subscription Data |
| ConditionalCagInfo | 6.1.6.2.94 |  |
| RangingSlPosSubscriptionData | 6.1.6.2.97 | Ranging/SL positioning Subscription Data |
| RangingSlPosPlmn | 6.1.6.2.98 | Contains the allowed PLMN information for Ranging/SL positioning Service |
| AppSpecificExpectedUeBehaviourData | 6.1.6.2.99 | Application specific UE Behaviour Data |
| ExpectedUeBehaviourThreshold | 6.1.6.2.100 | Expected UE Behaviour Threshold in SdmSubscription |
| A2xSubscriptionData | 6.1.6.2.101 | A2X Subscription Data |
| RangingSlPosQos | 6.1.6.2.102 | Ranging/SL Positioning QoS |
| MbsrOperationAllowed | 6.1.6.2.103 | MBSR Operation Allowed Information |
| DnnLadnServiceAreas | 6.1.6.2.105 | LADN Service Areas for DNNs |
| DnnLadnServiceArea | 6.1.6.2.106 | LADN ServiceArea for a DNN |
| GptpAllowedInfo | 6.1.6.2.107 | AF Authorization Info for gPTP |
| AstiAllowedInfo | 6.1.6.2.108 | AF Authorization Info for ASTI |
| MbsrLocationInfo | 6.1.6.2.109 | MBSR operation location information |
| MbsrTimeInfo | 6.1.6.2.110 | MBSR operation time information |
| RecurTime | 6.1.6.2.111 | Recurring time |
| RangingSlPrivacyData | 6.1.6.2.112 | Ranging/SL positioning Privacy Data |
| Rslppi | 6.1.6.2.113 | Ranging/SL positioning Privacy Indication |
| RangingSlUnrelatedClass | 6.1.6.2.114 | Call/Session unrelated Classes for Ranging/SL positioning |
| RangingSlPlmnOperatorClass | 6.1.6.2.115 | PLMN Operator Class for Ranging/SL positioning |
| RangingSlDefaultUnrelatedClass | 6.1.6.2.116 | Default Call/Session unrelated Class subscription for Ranging/SL positioning for unidentified value added LCS clients or AFs |
| RangingSlExternalUnrelatedClass | 6.1.6.2.117 | Call/Session unrelated Class subscriptions for Ranging/SL positioning for identified value added LCS Clients or AFs |
| RangingSlAfExternal | 6.1.6.2.118 | List of Call/session Unrelated Class for ranging and sidelink positioning identified by AF in the external LCS client list |
| RangingSlLcsClientExternal | 6.1.6.2.119 | List of Call/session Unrelated Class for ranging and sidelink positioning identified by LCS client in the external LCS client list |
| RangingSlAppIDUnrelatedClass | 6.1.6.2.120 | Call/Session unrelated Class subscriptions for ranging and sidelink positioning service for identified service type for UE |
| AdditionalSmfSelectionInfo | 6.1.6.2.121 | Additional parameters for SMF selection in target PLMN |
| OdbExemptedDnnInfo | 6.1.6.2.122 | ODB Exempted DNN Information |
| OdbExemptedConditions | 6.1.6.2.123 | ODB Exempted Conditions |
| GpsiInformation | 6.1.6.2.124 | List of the GPSIs |
| UpfFunctionalityData | 6.1.6.2.125 | UPF Functionnality Data |
| UpfFunctionalityDataWithPriority | 6.1.6.2.126 | UPF Functionnality Data ordered by priority |
| VlanTagValue | 6.1.6.2.127 | VLAN Tag Value |
| VlanTagHandling | 6.1.6.2.128 | VLAN Tag Handling Information |
| TrajectorySegment | 6.1.6.2.129 | A Segment in Assigned Trajectory |
| DefaultDnnIndicator | 6.1.6.3.2 |  |
| LboRoamingAllowed | 6.1.6.3.2 |  |
| UeUsageType | 6.1.6.3.2 |  |
| MpsPriorityIndicator | 6.1.6.3.2 |  |
| McsPriorityIndicator | 6.1.6.3.2 |  |
| 3GppChargingCharacteristics | 6.1.6.3.2 | 3GPP Charging Characteristics |
| MicoAllowed | 6.1.6.3.2 |  |
| SmsSubscribed | 6.1.6.3.2 |  |
| SharedDataId | 6.1.6.3.2 |  |
| IwkEpsInd | 6.1.6.3.2 | Interworking with EPS Indication |
| SecuredPacket | 6.1.6.3.2 |  |
| UpuRegInd | 6.1.6.3.2 |  |
| ExtGroupId | 6.1.6.3.2 |  |
| NbIoTUePriority | 6.1.6.3.2 |  |
| CodeWord | 6.1.6.3.2 |  |
| AfId | 6.1.6.3.2 |  |
| LcsClientId | 6.1.6.3.2 |  |
| SorTransparentContainer | 6.1.6.3.2 |  |
| UpuTransparentContainer | 6.1.6.3.2 |  |
| DataSetName | 6.1.6.3.3 |  |
| PduSessionContinuityInd | 6.1.6.3.7 |  |
| LocationPrivacyInd | 6.1.6.3.8 |  |
| PrivacyCheckRelatedAction | 6.1.6.3.9 |  |
| LcsClientClass | 6.1.6.3.10 |  |
| LcsMoServiceClass | 6.1.6.3.11 |  |
| OperationMode | 6.1.6.3.12 |  |
| SorUpdateIndicator | 6.1.6.3.13 | SoR Update Indicator |
| CodeWordInd | 6.1.6.3.14 |  |
| MdtUserConsent | 6.1.6.3.15 | MDT User Consent |
| SharedDataTreatmentInstruction | 6.1.6.3.16 |  |
| GpsiType | 6.1.6.3.17 | Type of GPSI (MSISDN or External-ID) |
| AerialUeIndication | 6.1.6.3.18 | Indication on whether Aerial service for UE is allowed or not. |
| ProseDirectAllowed | 6.1.6.3.19 | Contains the ProSe Services that can be authorised by the given PLMN identity. |
| UcPurpose | 6.1.6.3.20 | Purpose of the user consent |
| UserConsent | 6.1.6.3.21 | User Consent |
| NsacAdmissionMode | 6.1.6.3.22 | NSAC Admission Mode |
| PruInd | 6.1.6.3.23 | PRU Indicator |
| AreaUsageInd | 6.1.6.3.24 | Area Usage Indication |
| RangingSlPosAllowed | 6.1.6.3.28 | Contains the Ranging/SL positioning Services that can be authorised by the given PLMN identity. |
| ExpectedUeBehaviourDataset | 6.1.6.3.29 | Expected UE Behaviour Dataset Names |
| UpLocRepIndAf | 6.1.6.3.30 | Report periodic or triggered location events via user plane to an LCS Client or AF |
| RecurType | 6.1.6.3.31 | Recurring Type |
| RangingSlPrivacyInd | 6.1.6.3.32 |  |
| RangingSlPrivacyCheckRelatedAction | 6.1.6.3.33 |  |
| VlanTagType | 6.1.6.3.34 | VLAN Tag Type |

Table 6.1.6.1-2 specifies data types re-used by the Nudm_SDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudm_SDM service API.

Table 6.1.6.1-2: Nudm_SDM re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Dnn | 3GPP TS 29.571 [7] | Data Network Name with Network Identifier only; this type is used as key in a map of: - DnnConfigurations; see clause 6.1.6.2.8; - EpsIwkPgws; see clause 6.2.6.2.2; - ExpectedUeBehaviourData; see clause 6.1.6.2.8; |
| DurationSec | 3GPP TS 29.571 [7] | Time value in seconds |
| ProblemDetails | 3GPP TS 29.571 [7] | Common data type used in response bodies |
| Snssai | 3GPP TS 29.571 [7] | Single NSSAI |
| Uri | 3GPP TS 29.571 [7] | Uniform Resource Identifier |
| Gpsi | 3GPP TS 29.571 [7] | Generic Public Subscription Identifier |
| RatType | 3GPP TS 29.571 [7] | Radio Access Technology Type |
| Area | 3GPP TS 29.571 [7] |  |
| AreaCode | 3GPP TS 29.571 [7] |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [7] |  |
| CoreNetworkType | 3GPP TS 29.571 [7] |  |
| SupportedFeatures | 3GPP TS 29.571 [7] | see 3GPP TS 29.500 [4] clause 6.6 |
| PlmnId | 3GPP TS 29.571 [7] | PLMN Identity |
| PduSessionType | 3GPP TS 29.571 [7] |  |
| SubscribedDefaultQos | 3GPP TS 29.571 [7] | Subscribed Default QoS |
| Ambr | 3GPP TS 29.571 [7] |  |
| AmbrRm | 3GPP TS 29.571 [7] |  |
| SliceMbrRm | 3GPP TS 29.571 [7] |  |
| PduSessionId | 3GPP TS 29.571 [7] | PduSessionId is used as key in a map of PduSessions; see clause 6.1.6.2.16. |
| NfInstanceId | 3GPP TS 29.571 [7] |  |
| Supi | 3GPP TS 29.571 [7] |  |
| RfspIndexRm | 3GPP TS 29.571 [7] |  |
| SscMode | 3GPP TS 29.571 [7] |  |
| Ipv4Addr | 3GPP TS 29.571 [7] |  |
| Ipv6Addr | 3GPP TS 29.571 [7] |  |
| Ipv6Prefix | 3GPP TS 29.571 [7] |  |
| SorMac | 3GPP TS 29.509 [24] |  |
| SteeringInfo | 3GPP TS 29.509 [24] |  |
| AckInd | 3GPP TS 29.509 [24] |  |
| CounterSor | 3GPP TS 29.509 [24] |  |
| UpuMac | 3GPP TS 29.509 [24] |  |
| UpuData | 3GPP TS 29.509 [24] |  |
| UpuAckInd | 3GPP TS 29.509 [24] |  |
| CounterUpu | 3GPP TS 29.509 [24] |  |
| TraceData | 3GPP TS 29.571 [7] | Trace control and configuration parameters |
| NotifyItem | 3GPP TS 29.571 [7] |  |
| UpSecurity | 3GPP TS 29.571 [7] |  |
| ServiceName | 3GPP TS 29.510 [19] |  |
| OdbPacketServices | 3GPP TS 29.571 [7] |  |
| GroupId | 3GPP TS 29.571 [7] | This type is also used as key of a map in attributes: - vnGroupInfo and sharedVnGroupDataIds; see clause 6.1.6.2.4, 6.1.6.2.8, 6.1.6.2.27; |
| DateTime | 3GPP TS 29.571 [7] |  |
| DayOfWeek | 3GPP TS 29.571 [7] |  |
| MonthOfYear | 3GPP TS 29.571 [7] |  |
| CagId | 3GPP TS 29.571 [7] |  |
| StnSr | 3GPP TS 29.571 [7] | Session Transfer Number for SRVCC |
| CMsisdn | 3GPP TS 29.571 [7] | Correlation MSISDN |
| OsId | 3GPP TS 29.519 [33] |  |
| Uint16 | 3GPP TS 29.571 [7] |  |
| RgWirelineCharacteristics | 3GPP TS 29.571 [7] |  |
| GeographicArea | 3GPP TS 29.572 [34] |  |
| LcsServiceType | 3GPP TS 29.572 [34] |  |
| ScheduledCommunicationTime | 3GPP TS 29.571 [7] | Scheduled Communication Time |
| LocationArea | 6.5.6.2.10 |  |
| DataRestorationNotification | 6.2.6.2.25 | Contains identities representing those UEs potentially affected by a data-loss event at the UDR |
| StationaryIndication | 3GPP TS 29.571 [7] | Stationary Indication |
| TrafficProfile | 3GPP TS 29.571 [7] | Traffic Profile |
| ScheduledCommunicationType | 3GPP TS 29.571 [7] | Scheduled Communication Type |
| BatteryIndication | 3GPP TS 29.571 [7] | Battery Indication |
| AcsInfo | 3GPP TS 29.571 [7] | ACS Information |
| IPv4AddrMask | 3GPP TS 29.571 [7] |  |
| Fqdn | 3GPP TS 29.571 [7] | Fully Qualified Domain Name |
| NefId | 3GPP TS 29.510 [19] |  |
| PatchResult | 3GPP TS 29.571 [7] |  |
| NrV2xAuth | 3GPP TS 29.571 [7] |  |
| LteV2xAuth | 3GPP TS 29.571 [7] |  |
| ProseServiceAuth | 3GPP TS 29.571 [7] |  |
| BitRate | 3GPP TS 29.571 [7] |  |
| MdtConfiguration | 3GPP TS 29.571 [7] |  |
| Uint64 | 3GPP TS 29.571 [7] |  |
| WirelineArea | 3GPP TS 29.571 [7] |  |
| WirelineServiceAreaRestriction | 3GPP TS 29.571 [7] |  |
| NfGroupId | 3GPP TS 29.571 [7] |  |
| EcsServerAddr | 3GPP TS 29.571 [7] |  |
| RedirectResponse | 3GPP TS 29.571 [7] | Response body of the redirect response message |
| Bytes | 3GPP TS 29.571 [7] | Binary data encoded as a base64 character string |
| SpatialValidityCond | 3GPP TS 29.571 [7] |  |
| AccessType | 3GPP TS 29.571 [7] | Access Type |
| NsSrg | 3GPP TS 29.571 [7] | NSSRG value |
| MbsSessionId | 3GPP TS 29.571 [7] | 5MBS Session Id |
| RoamingRestrictions | 3GPP TS 29.571 [7] | Roaming Restrictions related information |
| PlmnIdNid | 3GPP TS 29.571 [7] |  |
| Uinteger | 3GPP TS 29.571 [7] | Unsigned Integer |
| Tai | 3GPP TS 29.571 [7] | Tracking Area Identity |
| TemporalValidity | 3GPP TS 29.514 [68] | Temporal Validity contains start time and stop time |
| LMFIdentification | 3GPP TS 29.572 [34] | LMF Identification |
| LpHapType | 3GPP TS 29.518 [36] | Low Power and High Accuracy Positioning type |
| RangingSlPosAuth | 3GPP TS 29.571 [7] |  |
| ApplicationId | 3GPP TS 29.571 [7] | Application ID |
| ClockQualityAcceptanceCriterion | 3GPP TS 29.571 [7] | Clock Quality Acceptance Criterion |  |
| ClockQualityDetailLevel | 3GPP TS 29.571 [7] | Clock Quality Detail Level |  |
| QmcConfigInfo | 3GPP TS 29.571 [7] | Configuration information for signaling-based activation of QMC (Quality of Experience Measurements Collection) |  |
| NrA2xAuth | 3GPP TS 29.571 [7] |  |  |
| LteA2xAuth | 3GPP TS 29.571 [7] |  |  |
| Accuracy | 3GPP TS 29.572 [34] | Indicates value of accuracy |  |
| ResponseTime | 3GPP TS 29.572 [34] | Indicates acceptable delay of location request |  |
| MinorLocationQoS | 3GPP TS 29.572 [34] | Minor Location QoS |  |
| LcsQosClass | 3GPP TS 29.572 [34] | Specifies LCS QoS class |  |
| FlowInfo | 3GPP TS 29.122 [45] | Contains the flow information. |  |
| UeLevelMeasurementsConfiguration | 3GPP TS 29.571 [7] | 5GC UE level measurements configuration |  |
| MonitoringSuspension | 6.4.6.2.25 |  |  |
| OpConfiguredCapability | 3GPP TS 29.571 [7] | Operator Configurable UPF Capabilities |  |
| UpfPacketInspectionFunctionality | 3GPP TS 29.571 [7] | UPF Packet Inspection Functionality |  |
| MtcProviderInformation | 3GPP TS 29.571 [7] | MTC Provider Information |  |
| EnergySavingIndicator | 3GPP TS 29.571 [7] | Energy Saving Indicator |  |

* * * Next Change * * * *

##### 6.1.6.2.9	Type: DnnConfiguration

Table 6.1.6.2.9-1: Definition of type DnnConfiguration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pduSessionTypes | PduSessionTypes | M | 1 | Default/Allowed session types |
| sscModes | SscModes | M | 1 | Default/Allowed SSC modes |
| iwkEpsInd | IwkEpsInd | O | 0..1 | Indicates whether interworking with EPS and EPC/ePDG is subscribed: true: Subscribed; false: Not subscribed; If this attribute is absent it means not subscribed. |
| 5gQosProfile | SubscribedDefaultQos | O | 0..1 | 5G QoS parameters associated to the session for a data network |
| sessionAmbr | Ambr | O | 0..1 | The maximum aggregated uplink and downlink bit rates to be shared across all Non-GBR QoS Flows in each PDU Session. |
| 3gppChargingCharacteristics | 3GppChargingCharacteristics | O | 0..1 | Subscribed charging characteristics data associated to the session for a data network. (NOTE 1) |
| staticIpAddress | array(IpAddress) | O | 1..2 | Subscribed static IP address(es) of the IPv4 and/or IPv6 type. |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane. |
| pduSessionContinuityInd | PduSessionContinuityInd | O | 0..1 | When present, this IE shall indicate how to handle a PDU Session when UE the moves to or from NB-IoT. If this attribute is absent it means that Local policy shall be used. |
| niddNefId | NefId | O | 0..1 | Indicates the identity of the NEF to be selected for NIDD service for this DNN. |
| niddInfo | NiddInformation | O | 0..1 | When present, this IE shall indicate information used for SMF-NEF Connection. |
| redundantSessionAllowed | boolean | O | 0..1 | Indicates whether redundant PDU Sessions are allowed: true: Allowed; false: Not allowed; If this attribute is absent it means not allowed. |
| acsInfo | AcsInfo | O | 0..1 | When present, this IE shall include the ACS information for the 5G-RG as defined in BBF TR-069 [42] or in BBF TR-369 [43]. |
| ipv4FrameRouteList | array(FrameRouteInfo) | O | 1..N | List of Frame Route information of IPv4, see clause 5.6.14 of 3GPP TS 23.501 [2]. |
| ipv6FrameRouteList | array(FrameRouteInfo) | O | 1..N | List of Frame Route information of IPv6, see clause 5.6.14 of 3GPP TS 23.501 [2]. |
| atsssAllowed | boolean | O | 0..1 | Indicates whether this DNN supports ATSSS, i.e. whether Multi-Access PDU session is allowed to this DNN. true: Allowed; false (default): Not allowed; If this attribute is absent it means this DNN does not allow ATSSS. |
| secondaryAuth | boolean | O | 0..1 | Indicates whether DN-AAA based secondary authentication and authorization is needed. true: required. false: not required. If absent, it indicates that DN-AAA based secondary authentication is not required by subscription data, but it still may be required by local policies at the SMF. (NOTE 2) |
| uavSecondaryAuth | boolean | O | 0..1 | Indicates whether SBI-based secondary authentication and authorization for Uncrewed Aerial Vehicles (UAV) is needed. true: required. false or absent: not required. |
| dnAaaIpAddressAllocation | boolean | O | 0..1 | Indicates whether the SMF is required to request the UE IP address from the DN-AAA server for PDU Session Establishment. true: required false: not required If absent, it indicates that the request by SMF of the UE IP address from the DN-AAA server is not required by subscription data, but it still may be required by local policies at the SMF. (NOTE 2) |
| dnAaaAddress | IpAddress | O | 0..1 | The IP address of the DN-AAA server used for secondary authentication and authorization. (NOTE 2) |
| additionalDnAaaAddresses | array(IpAddress) | O | 1..N | Additional IP addresses of the DN-AAA server used for secondary authentication and authorization. (NOTE 2) |
| dnAaaFqdn | Fqdn | O | 0..1 | The FQDN of the DN-AAA server used for secondary authentication and authorization. (NOTE 2) |
| iptvAccCtrlInfo | string | O | 0..1 | The IPTV access control information used in IPTV access procedure, see clause 7.7.1.1.2 of 3GPP TS 23.316 [37]. |
| ipv4Index | IpIndex | O | 0..1 | Indicates the "IP Index" (i.e. information that identifies an address pool or an external server) to be sent to the SMF for allocation of an IPv4 address to the UE, for this DNN configuration. |
| ipv6Index | IpIndex | O | 0..1 | Indicates the "IP Index" (i.e. information that identifies an address pool or an external server) to be sent to the SMF for allocation of an IPv6 address to the UE, for this DNN configuration. |
| ecsAddrConfigInfo | EcsAddrConfigInfo | O | 0..1 | ECS Address Configuration Information Parameters. See 3GPP TS 23.502 [3] |
| additionalEcsAddrConfigInfos | array(EcsAddrConfigInfo) | O | 1..N | This IE may be present when ecsAddrConfigInfo IE is present. When present, this IE shall contain the additional ECS Address Configuration Information Parameters. See 3GPP TS 23.548 [60] |
| sharedEcsAddrConfigInfoId | SharedDataId | O | 0..1 | Identifier of shared data indicating an ECS Configuration Information. |
| additionalSharedEcsAddrConfigInfoIds | array(SharedDataId) | O | 1..N | This IE may be present when sharedEcsAddrConfigInfoId IE is present. When present, this IE shall contain the identifiers of shared data indicating the additional ECS Address Configuration Information. |
| easDiscoveryAuthorized | boolean | O | 0..1 | Indicates whether the UE is authorized to use 5GC assisted EAS discovery via EASDF. true: Authorized; false (default): Not authorized; See 3GPP TS 23.548 [60] |
| onboardingInd | boolean | O | 0..1 | Indicates whether the UE is allowed to use this DNN for onboarding. UE subscription allows, or does not allow the UE to access the PLMN as the Onboarding Network using PLMN credentials (see clause 5.30.2.10.4.4 in 3GPP TS 23.501 [2]). - false (default): not restricted to onboarding only; - true: allowed for onboarding only. |
| aerialUeInd | AerialUeIndication | O | 0..1 | This IE shall indicate Aerial service for the UE is allowed or not allowed. |
| subscribedMaxIpv6PrefixSize | integer | O | 0..1 | maximum size of the IPv6 prefix that may be allocated for the PDU Session. See 3GPP TS 23.316 [37] clause 4.6.2.3. |
| hrSboAuthorized | boolean | O | 0..1 | Indicates whether Home Routed Session Breakout (HR-SBO) is authorized (see 3GPP TS 23.548 [60] clause 6.7.2.2) true: Authorized; false: Not authorized; If this attribute is absent it means not authorized. |
| requiredUpfFunctionList | array(UpfFunctionalityData) | O | 1..N | Indicates the required UPF functionalities for the PDU Session, which shall be used to select an UPF supporting all the required UPF functionalities. |
| prefUpfFunctionList | array(UpfFunctionalityDataWithPriority) | O | 1..N | Indicates the preferred UPF functionalities for the PDU Session and its priority, which should be used to select preferentially an UPF supporting some or all preferred UPF functionalities with higher priority. |
| vlanTagAllowed | array(VlanTagValue) | O | 1..N | This IE may be present when pduSessionTypes is set to ETHERNET. When present, this IE shall indicate the VLAN tag values allowed for the PDU Session, i.e. the ethernet packets (of the PDU session) shall be only allowed when the outer tag (which is the one closest to the Ethernet header) is set to one of the allowed vlan tag values. (NOTE 4) |
| vlanTagHandlingInfo | array(VlanTagHandling) | O | 1..N | This IE may be present when pduSessionTypes is set to ETHERNET. When present, this IE provide instructions for VLAN tag handling in the UPF, e.g. the C-VLAN tag to be inserted or removed, S-TAG to be inserted or removed (see clause 5.6.10.2 of 3GPP TS 23.501 [2]). (NOTE 3) |
| localOffloadingMngtInd | boolean | O | 0..1 | Indicates whether Local Offloading Management is allowed. true: Allowed; false: Not allowed; default: false. |
| NOTE 1: When present, this attribute shall take precedence over the "3gppChargingCharacteristics" attribute in the SessionManagementSubscriptionData level. NOTE 2: These attributes shall be consistent with the information received on the 5GVnGroupData (see clause 6.5.6.2.7), in the Nudm_PP API. If both FQDN and IP addresses are provided, the IP addresses should be preferred to target the DN-AAA server. NOTE 3: In this release of the specification, VlanTagHandlingInfo shall contain at most 2 elements in the array corresponding to a S-TAG and/or a C-TAG to be inserted into the ethernet header of UL traffic and to be removed from the ethernet header of DL traffic. NOTE 4: In this release of the specification, the maximal number of allowed Vlan Tag values is 16. |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.127	Type: VlanTagValue

# Table 6.1.6.2.127-1: Definition of type VlanTagValue

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| vlanTagValue | string | M | 1 | The vlan tag value which contains: 3 bits PCP followed by 1 bit DEI followed by 12 bits VID encoded in hexadecimal format. pattern: ^[A-Fa-f0-9]{4}$ See IEEE 802.1Q [74]. |

* * * Next Change * * * *

##### 6.1.6.2.128	Type: VlanTagHandling

# Table 6.1.6.2.128-1: Definition of type VlanTagHandling

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| vlanTag | VlanTagValue | M | 1 | This IE shall indicate a VLAN tag value to be handled. |
| vlanTagType | VlanTagType | O | 0..1 | When present, this IE indicate the type of VLAN Tag |

* * * Next Change * * * *

# A.2	Nudm_SDM API

openapi: 3.0.0

*************text not shown for clarity*************

DnnConfiguration:

description: Contains DNN Configuration

type: object

required:

- pduSessionTypes

- sscModes

properties:

pduSessionTypes:

$ref: '#/components/schemas/PduSessionTypes'

sscModes:

$ref: '#/components/schemas/SscModes'

iwkEpsInd:

$ref: '#/components/schemas/IwkEpsInd'

5gQosProfile:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SubscribedDefaultQos'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

3gppChargingCharacteristics:

$ref: '#/components/schemas/3GppChargingCharacteristics'

staticIpAddress:

type: array

items:

$ref: '#/components/schemas/IpAddress'

minItems: 1

maxItems: 2

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

pduSessionContinuityInd:

$ref: '#/components/schemas/PduSessionContinuityInd'

niddNefId:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NefId'

niddInfo:

$ref: '#/components/schemas/NiddInformation'

redundantSessionAllowed:

type: boolean

acsInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AcsInfo'

ipv4FrameRouteList:

type: array

items:

$ref: '#/components/schemas/FrameRouteInfo'

minItems: 1

ipv6FrameRouteList:

type: array

items:

$ref: '#/components/schemas/FrameRouteInfo'

minItems: 1

atsssAllowed:

type: boolean

default: false

secondaryAuth:

type: boolean

uavSecondaryAuth:

type: boolean

default: false

dnAaaIpAddressAllocation:

type: boolean

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

additionalDnAaaAddresses:

type: array

items:

$ref: '#/components/schemas/IpAddress'

minItems: 1

dnAaaFqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

iptvAccCtrlInfo:

type: string

ipv4Index:

$ref: '#/components/schemas/IpIndex'

ipv6Index:

$ref: '#/components/schemas/IpIndex'

ecsAddrConfigInfo:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/EcsAddrConfigInfo'

additionalEcsAddrConfigInfos:

type: array

items:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/EcsAddrConfigInfo'

minItems: 1

sharedEcsAddrConfigInfo:

$ref: '#/components/schemas/SharedDataId'

additionalSharedEcsAddrConfigInfoIds:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

minItems: 1

easDiscoveryAuthorized:

type: boolean

default: false

onboardingInd:

type: boolean

default: false

aerialUeInd:

$ref: '#/components/schemas/AerialUeIndication'

subscribedMaxIpv6PrefixSize:

type: integer

hrSboAuthorized:

type: boolean

default: false

requiredUpfFunctionList:

type: array

items:

$ref: '#/components/schemas/UpfFunctionalityData'

minItems: 1

prefUpfFunctionList:

type: array

items:

$ref: '#/components/schemas/UpfFunctionalityDataWithPriority'

minItems: 1

vlanTagAllowed:

type: array

items:

$ref: '#/components/schemas/VlanTagValue'

minItems: 1

vlanTagHandlingInfo:

type: array

items:

$ref: '#/components/schemas/VlanTagHandling'

minItems: 1

localOffloadingMngtInd:

type: boolean

default: false

*************text not shown for clarity*************

VlanTagValue:

description: VLAN Tag Value

type: object

properties:

vlanTagValue:

type: string

pattern: '^[A-Fa-f0-9]{4}$'

required:

- vlanTagValue

VlanTagHandling:

type: object

description: VLAN Tag Handling information

properties:

vlanTag:

$ref: '#/components/schemas/VlanTagValue'

vlanTagType:

$ref: '#/components/schemas/VlanTagType'

required:

- vlanTag

*************text not shown for clarity*************

* * * End of Change * * * *