3GPP TSG-CT Meeting #109	CP-252187

# Beijing, P.R. China; 15th – 16th September 2025								Revision of C4-253367

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.564 | CR | 0141 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Bundling Event Reports of Multiple Subscriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia, Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Work item code: | PAIDC_UPF |  |  |  |  |  | Date: |  |  | 2025-09-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Solution #7 (Bundling event reports of different subscriptions) of TR 29.889 has been agreed for normative work (see clause 8.1 of TR 29.889). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Nupf_EventExposure service is enhanced to enable the UPF to bundle event reports of multiple subscriptions in a same Notify request message according to the solution #7 of TR 29.889. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UPF sends event reports in Notify request messages per subscription, causing massive signaling and processing for reporting the measurements from all PDU sessions related with different subscriptions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 5.2.2.2.3, 5.2.2.3.2, 5.2.2.3.X (new), 6.1.5.2.2, 6.1.6.2.2, 6.1.6.2.11, 6.1.6.X (new), 6.1.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29564_Nupf_EventExposure.yaml TS29574_Ndccf_ContextManagement.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1 : China Mobile is added as co-source. One new note is added in clause 5.2.2.3.X. Rev. 2: Correction of one OpenAPI error found during the implementation of the CT4 agreed CR: ExtNotificationData: oneOf: - $ref: '#/components/schemas/NotificationData' - type: array items: type: NotificationData minItems: 2 is corrected to ExtNotificationData: oneOf: - $ref: '#/components/schemas/NotificationData' - type: array items: $ref: '#/components/schemas/NotificationData' minItems: 2 |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 5.2.2.2.2	Creation of a subscription

An NF Service Consumer shall invoke the Subscribe service operation towards the UPF to create a subscription to monitor at least one UPF event. The NF Service Consumer may subscribe to multiple events in a subscription. A subscription may be associated with one UE's PDU session or with any UE.

The NF Service Consumer shall request to create a new subscription by using the HTTP method POST with the URI of the subscriptions collection, see clause 6.1.3.2.

Figure 5.2.2.2.2-1 Subscription creation

1.	The NF Service Consumer shall send a POST request to create a subscription resource in the UPF. The content of the POST request shall contain a representation of the individual subscription resource to be created.

The NF Service Consumer shall include the following information in the HTTP message body:

-	NF ID, indicating the identity of the network function instance creating the subscription;

-	Target of Event Reporting, indicating the target(s) to be monitored, i.e.

-	a specific PDU Session of a UE identified with a (5GC assigned) UE IP address for an IP PDU session type;

-	a specific PDU Session of a UE identified with a SUPI (and S-NSSAI/DNN) for a non-IP PDU session type if the SMF allows the UPF to receive the SUPI associated with a N4 session based on the local configuration; or

-	any UE (identified by the "anyUE" flag);

-	List of UPF events requested to be subscribed;

-	Type of measurement, for UPF events supporting multiple types of measurement, e.g. for a subscription to the UserDataUsageMeasures event;

-	Event Reporting Mode, indicating how the events shall be reported (One-time Report, Periodic Report or Continuous Report);

-	UPF event consumer notification URI, indicating the address where to send the event notifications generated by the subscription;

-	Public IPv4 or IPv6 address of the remote end, for a subscription to "UE_NAT_MAPPING_INFO" event; and

-	the bundledEventNotifyUri set to the URI to be used by the UPF for sending notification requests including bundled event reports of multiple UPF event subscriptions (see clause 5.2.2.3.X), if the bundlingAllowed IE is set to true and the NF service consumer provides a distinct eventNotifyUri per UPF event subscription.

The NF Service Consumer may include the following information in the HTTP message body:

-	The S-NSSAI and/or the DNN of PDU sessions to which the subscription applies;

-	either one or more Application ID(s) or traffic filters identifying the traffic to be monitored by the subscription;

-	Granularity of Measurement, indicating that the granularity of the required measurements is per PDU Session, per data flow or per application;

-	Reporting period, defining the period for periodic reporting;

-	Maximum number of reports, defining the maximum number of reports after which the event subscription ceases to exist;

-	Expiry time, suggested by the NF Service Consumer representing the time up to which the subscription is desired to be kept active and the time after which the subscribed event(s) shall stop generating reports;

-	Reporting suggestion information, i.e. Report urgency indicating whether the event report can be delayed (i.e. it is delay-tolerant) and if so, the Reporting time information defining the last valid reporting time for the UPF to report the detected event;

-	Deactivate notification flag, indicating that the notification of the available events shall be muted until the event consumer NF (e.g. NWDAF or DCCF) provides the retrieval notification flag to retrieve the stored events;

-	Immediate Report Flag per event, indicating an immediate report to be generated with the current event status;

-	Notification Correlation ID, indicating the correlation identity to be signalled in the event notifications generated by the subscription;

-	Sampling ratio, defining the random subset of PDU sessions among target PDU sessions, in which case the UPF shall only report the event(s) related to the selected subset of PDU sessions;

-	partitioningCriteria, defining the criteria for partitioning PDU sessions before applying the sampling ratio;

-	Muting Exception Instructions, which specify instructions to apply to the subscription and the stored events when an exception occurs at the UPF while the event is muted (e.g., the buffer of stored event reports is full, or the number of stored event reports exceeds a certain number), if the EEMM feature is supported (see clause 6.1.8);

-	Port number of the remote end, for a subscription to "UE_NAT_MAPPING_INFO" event;

-	IP domain, for a subscription to "UE_NAT_MAPPING_INFO" event;

-	the subTerminationReportInd IE set to true to indicate a request to send a notification when the subscription is terminated;

-	the remainingDataReports IE per subscribed event requesting to discard or to send the remaining data for the related event to the NF service consumer, if the subscription is terminated at the UPF (e.g. when the PFCP session of the PDU session that is the target of the subscription is released) and the UPF has not yet reported the collected data for the related event to the consumer;

-	the bundlingAllowed IE set to true to allow the UPF to bundle event reports generated for this subscription with event reports generated for other subscriptions in a same Nupf_EventExposure_Notify request message (see clause 5.2.2.3.X), if the UPF supports the BERMS feature (see clause 6.1.8); and/or

-	the bundleId IE containing a Bundle Identifier identifying the subscriptions whose event reports may be bundled together (see clause 5.2.2.3.X), if the bundlingAllowed IE is set to true and if the NF service consumer only allows to bundle event reports of specific subscriptions, e.g. if the NWDAF wishes to only bundle event reports generated for a same analytic service request.

2a.	On success (i.e. if the request is accepted), the UPF shall include a HTTP Location header to provide the location of the newly created resource (subscription) together with the status code 201 in the response message indicating that the requested resource is created.

If the NF Service Consumer has included more than one events in the event subscription and some of the events cannot be subscribed, the UPF shall accept the request and provide the successfully subscribed event(s) in the CreatedEventSubscription.

If the NF Service Consumer has included the Immediate Report Flag with the value true in the event subscription, and if the current status of the events subscribed are available, the UPF shall include the current status of the events subscribed in the response. Otherwise, the UPF shall generate reports for the events and notify the NF service consumer using the Nupf_EventExposure_Notify service operation. If the events with the Immediate Report Flag set to true are subscribed via an SMF, the notification shall be sent to the actual NF service consumer directly, i.e. the current status of the events subscribed shall not be included in the subscription creation response.

If the NF Service Consumer has set the event reporting option to ONE_TIME and if the UPF has included the current status of the events subscribed in the response, then the UPF shall not do any subsequent event notification for the corresponding events.

The response, based on operator policy and taking into account the expiry time included in the request, may contain the expiry time, as determined by the UPF, after which the subscription becomes invalid. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the UPF. The UPF shall not provide the same expiry time for many subscriptions in order to avoid all of them expiring and recreating the subscription at the same time. If the expiry time is not included in the response, the NF Service Consumer shall consider the subscription to be valid without an expiry time.

If the sampling ratio ("sampRatio") attribute is included in the subscription without a partitioningCriteria, the UPF shall select a random subset of PDU sessions among target PDU sessions according to the sampling ratio and only report the event(s) related to the selected subset of PDU sessions. If the partitioningCriteria attribute is also included along with sampling ratio, the UPF shall apply the sampling ratio on the group of PDU sessions determined according to the partitioning criteria.

If the "notifFlag" attribute is included and set to "DEACTIVATE" in the request by e.g. the NWDAF or DCCF, the UPF shall mute the event notification and store the available events. Additionally, if the UPF supports the EEMM feature (see clause 6.1.8) and if the NF service consumer includes event muting instructions in the request, the UPF should evaluate the received event muting instructions against to local actions (if configured) and, if the subscription creation request is accepted, the UPF may indicate the following information to the NF service consumer in the response:

-	the maximum number of notifications that the UPF expects to be able to store for the subscription;

-	an estimate of the duration for which notifications can be buffered.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3.

If the UPF supports the EEMM features (see clause 6.1.8), the NF service consumer sets the "notifFlag" attribute to "DEACTIVATE" and event muting instructions in the request, but the UPF cannot accept the received instructions, the UPF may reject the request with a 403 Forbidden response and the application error "MUTING_EXC_INSTR_NOT_ACCEPTED".

For a subscription request targeting a PDU session, if the UPF cannot find a unique PDU session due to no DNN and/or S-NSSAI being received in the request, the UPF shall reject the request with a 403 Forbidden response and the application error "REJECTION_DUE_TO_NO_DNN_SNSSAI" (see clause 4.4.1.2 of 3GPP TS 23.502 [3]).

* * * Next Change * * *

##### 5.2.2.2.3	Modification of a subscription

The service operation is invoked by a NF Service Consumer, towards the UPF, when it needs to modify an existing subscription previously created at the UPF.

The NF Service Consumer shall modify the subscription by using the HTTP method PATCH with the URI of the individual subscription resource (see clause 6.1.3.3) to be modified.

Figure 5.2.2.2.3-1: Modification of a subscription

1.	The NF service consumer shall send a PATCH request to the resource representing a subscription. The modification may be for updating the list of subscribed events, the event report options, the event reports bundling parameters  and/or the NF Instance Identity of the NF owning the subscription.

NOTE X:	An NF service consumer can change a subscription, e.g. adding a bundlingAllowed IE to allow the corresponding event reports being bundled together with other subscriptions as described in clause 5.2.2.3.X, or changing the eventNotifyUri, bundledEventNotifyUri and/or bundleId; such changes affect only the subscription being modified.

NOTE Y:	The NF service consumer can receive (for some short time after changing event reports bundling parameters of a subscription) event reports bundled together with other subscriptions if the NF service consumer removes the bundlingAllowed IE from the subscription, or event reports bundled together with other subscriptions having the same old bundleId if the NF service consumer changes the bundleId.

2a.	On success, the request is accepted, and all the modification instructions in the PATCH request have been implemented, the UPF shall respond with "204 No Content".

2b.	On success, the request is accepted, but some of the modification instructions in the PATCH request have been discarded, the UPF shall respond with "200 OK" including PatchResult to indicate the failed modifications.

2c.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-3.

* * * Next Change * * *

##### 5.2.2.3.2	UPF notifying events of a subscription

Figure 5.2.2.3.2-1: UPF notifying events of a subscription

1.	The UPF shall send a POST request to the eventNotificationUri as provided by the SMF during the provisioning of Session Reporting Rule (see clause 7.5.2.9 of 3GPP TS 29.244 [15]) or received in the subscription creation as specified in clause 5.2.2.2.2.

2a.	Upon success, the NF Service Consumer responds with "204 No Content".

If the notification is sent to report the subscription termination (i.e. the event type is set to "SUBSCRIPTION_TERMINATION"), the NF service consumer shall additionally terminate the subscription locally.

2b.	On failure or redirection:

-	If the NF Service Consumer does not consider the "eventNotificationUri" as a valid notification URI, the NF Service Consumer shall return "404 Not Found" status code with the ProblemDetails IE providing details of the error.

-	In the case of redirection, the NF service consumer shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NF service consumer endpoint.

* * * Next Change * * *

##### 5.2.2.3.X	UPF notifying bundled event reports of multiple subscriptions

The procedure specified in this clause is optional to support for the UPF and the NF Service Consumer. It may be used if both the UPF and the NF Service Consumer support the Bundling Event Reports of Multiple Subscriptions (BERMS) feature (see clause 6.1.8).

If the UPF supports the BERMS feature, the UPF may bundle event reports of multiple UPF event subscriptions within a same Nupf_EventExposure_Notify request message, for subscriptions:

- received with the bundlingAllowed IE set to true (see clause 5.2.2.2.2);

- having the same eventNotifyUri (for subscriptions received without the bundledEventNotifyUri IE) or having the same bundledEventNotifyUri (for subscriptions received with the bundledEventNotifyUri IE); and

- having the same bundleId value, for subscriptions received with the bundleId IE.

The UPF may bundle event reports of multiple UPF event subscriptions (having the same eventNotifyUri or bundledEventNotifyUri as described above) received with the bundlingAllowed IE set to true and without a bundleId IE.

NOTE X:	The UPF cannot bundle event reports of UPF event subscriptions received with a bundleId IE together with event reports of UPF event subscriptions received without a bundleId IE.

The UPF shall not bundle event reports generated for a subscription received without the bundlingAllowed IE set to true, with other event reports of other suscriptions.

When bundling of event reports of multiple subscriptions is allowed and possible, the UPF implementation shall consider the following aspects:

- bundling should not add delay for delay critical events, i.e. the UPF may bundle delay critical events which happen nearly at the same time; and

NOTE Z:	How the UPF determines whether events happen nearly at the same time is implementation specific.

- bundling for delay tolerant events may happen as long as the bundled events are sent before their last allowed reporting time (indicated by the reportingTimeInfo IE).

The UPF shall notify bundled event reports of multiple subscriptions as described in Figure 5.2.2.3.X-1.

NOTE Y:	The procedure specified in clause 5.2.2.3.2 applies when the UPF notifies event reports of a single subscription. In this case, the UPF sends the event reports to the eventNotifyUri of the subscription, regardless of whether a bundledEventNotifyUri was provided or not for the subscription.

Figure 5.2.2.3.X-1: UPF notifying bundled event reports of multiple subscriptions

1.	The UPF shall send a POST request to the bundledEventNotifyUri (if the bundledEventNotifyUri IE was received for the subscription as specified in clause 5.2.2.2.2), otherwise to the eventNotifyUri (i.e. if no bundledEventNotifyUri IE was received for the subscription).

The content of the POST request shall include one NotificationData for each subscription for which events are reported. If the POST request is sent to the bundledEventNotifyUri, each NotificationData shall further include the eventNotifyUri IE set to the eventNotifyUri of the subscription for which events are reported.

2a.	Upon success, the NF Service Consumer shall respond with "204 No Content".

If a given NotificationData is sent to report a subscription termination (i.e. the event type is set to "SUBSCRIPTION_TERMINATION"), the NF service consumer shall additionally terminate the corresponding subscription locally.

2b.	On failure or redirection:

-	If the NF Service Consumer does not consider the bundledEventNotifyUri or the eventNotifyUri as a valid notification URI, the NF Service Consumer shall return the "404 Not Found" status code with the ProblemDetails IE providing details of the error.

-	In the case of redirection, the NF service consumer shall return a 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NF service consumer endpoint.

* * * Next Change * * *

#### 6.1.5.2	Event Notification

##### 6.1.5.2.1	Description

The Event Notification is used by the UPF to report one or several observed Events to a NF service consumer that has subscribed to such notifications.

##### 6.1.5.2.2	Target URI

The POST method shall be used for Event Notification and the URI shall be:

-	the Event Notification URI provided by the SMF during the provisioning of

-	Session Reporting Rule, see clause 5.33.5 of 3GPP TS 29.244 [15];

-	Forwarding Action Rule (see clause 5.41.2 of 3GPP TS 29.244 [15]); or

-	TSC Management Information within PFCP Session Modification Request, see clause 5.26.3.2 of 3GPP TS 29.244 [15]; or

-	the Event Notification URI (i.e. eventNotifyURI) or the bundledEventNotifyUri provided by the NF Service Consumer during creation of the subscription as specified in clauses 5.2.2.2.2, 5.2.2.3.2 and 5.2.2.3.X.

NOTE:	When sending an Event Notification to an alternative NF service consumer, e.g. when NF set is deployed, the UPF replaces the authority and callback URI Prefix (if any) part of the eventNotifyURI or bundledEventNotifyUri. See clause 6.5.3.2 of 3GPP TS 29.500 [4].

Resource URI: {eventNotificationUri or bundledEventNotifyUri}

Support of URI query parameters is specified in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Support of request data structures is specified in table 6.1.5.2.2-2, and support of response data structures and response codes is specified in table 6.1.5.2.2-3.

Table 6.1.5.2.2-2: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ExtNotificationData | M | 1 | Representation of: - Event reports notification of a subscription; or - Event reports notifications of multiple subscriptions, when using the BERMS feature (see clause 5.2.2.3.X). |

Table 6.1.5.2.2-3: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents a successful notification of the event. |
| ProblemDetails | O | 0..1 | 404 Not Found | If the NF Service Consumer considers the "eventNotificationUri or bundledEventNotifyUri " and/or "Notification Correlation ID" is not recognized, the NF Service Consumer shall return "404 Not Found" status code |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.5.2.2-4: Headers supported by the 307 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF instance ID towards which the notification is redirected |

Table 6.1.5.2.2-5: Headers supported by the 308 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of the NF service consumer instance to which the request should be sent. For the case, when a request is redirected to the same target resource via a different SCP, see clause 6.10.9.1 in 3GPP TS 29.500 [4]. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF instance ID towards which the notification is redirected |

* * * Next Change * * *

##### 6.1.6.2.2	Type: NotificationData

Table 6.1.6.2.2-1: Definition of type NotificationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationItems | array(NotificationItem) | M | 1..N | List of NotificationItem, whereby each entry shall correspond to a report for one subscribed event per PDU session. |  |
| correlationId | string | C | 0..1 | The UPF shall include this attribute in the notification if the "Notification Correlation ID" IE was received via N4 interface (see clause 7.5.2.9 of 3GPP TS 29.244 [15]) or if the notifyCorrelationId IE was received in the Nupf_EventExposure Subscribe request. When present, it shall be set to the notification correlation ID received via the N4 or Nupf interface. |  |
| achievedSampRatio | SamplingRatio | O | 0..1 | This IE may be included for an event subscription for any UE to indicate the ratio of the random subset of target PDU sessions achieved by the UPF. |  |
| eventNotifyUri | Uri | C | 0..1 | This IE shall be present if the Nupf_EventExposure_Notify request message is sent to the URI received in the bundledEventNotifyUri IE. When present, it shall be set to the URI received in the eventNotifyUri IE of the subscription for which the notification data is sent. (NOTE) | BERMS |
| NOTE: This IE may be used by the receiver of the notification message to correlate the NotificationData with the related events subscription, e.g. if the NF service consumer provides distinct eventNotifyUri for each subscription and if it does not use the correlationId for correlating the notification data with the subscription. |  |  |  |  |  |

* * * Next Change * * *

##### 6.1.6.2.11	Type: UpfEventSubscription

Table 6.1.6.2.11-1: Definition of type UpfEventSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventList | array(UpfEvent) | M | 1..N | This IE shall describe the events requested to be subscribed in a subscription request or the events successfully subscribed for this subscription in a subscription response. |  |
| eventNotifyUri | Uri | M | 1 | This IE shall identify the recipient address of the notifications sent by the UPF for this subscription. |  |
| notifyCorrelationId | string | M | 1 | This IE shall contain the notification correlation ID. The UPF shall include this notification correlation ID in the notifications. The value of this IE shall be unique per subscription for a given NF service consumer. |  |
| eventReportingMode | UpfEventMode | M | 1 | This IE shall describe how the reports of the event shall be generated. |  |
| nfId | NfInstanceId | M | 1 | This IE shall indicate the instance identity of the network function creating the subscription. |  |
| ueIpAddress | IpAddr | C | 0..1 | The IE shall be present if the event subscription targets one specific UE's PDU session, for an IP PDU session type. When present, the IE shall indicate the IP address of the UE's PDU Session. (NOTE 1) |  |
| supi | Supi | C | 0..1 | The IE shall be present if the event subscription targets one specific UE's PDU session, for a non-IP PDU session type, and the UPF is allowed by local SMF configuration to receive the SUPI associated with a N4 session. (NOTE 1, NOTE 2) |  |
| anyUe | boolean | C | 0..1 | This IE shall be present if the event subscription targets any UE. When present, it shall be set as follows: true: the subscription applies to any UE. false (default): the subscription applies to a specific UE. (NOTE 1) |  |
| dnn | Dnn | C | 0..1 | Data Network Name. This IE shall be present when the subscription is for "UE_NAT_MAPPING_INFO" event and DNNs are configured with overlapping UE private address range(s) across different data networks. It may be present otherwise. |  |
| snssai | Snssai | O | 0..1 | A single Network Slice Selection Assistance Information. |  |
| bundlingAllowed | boolean | O | 0..1 | This IE may be present and set to true to allow the UPF to bundle event reports generated for this subscription with event reports generated for other subscriptions in a same Nupf_EventExposure_Notify request message. The presence of this IE with the value false shall be prohibited. | BERMS |
| bundleId | Uint32 | O | 0..1 | This IE may be present if bundlingAllowed is set to true and if the NF service consumer only allows to bundle event reports of specific (but not all) subscriptions. When present, this IE shall contain a Bundle Identifier identifying the subscriptions whose event reports may be bundled together. | BERMS |
| bundledEventNotifyUri | Uri | C | 0..1 | This IE shall be present if bundlingAllowed is set to true and the NF service consumer provides a distinct eventNotifyUri per UPF event subscription. When present, this IE shall indicate the URI to be used by the UPF for sending notification requests with bundled event reports. | BERMS |
| NOTE 1: Either information about a single UE (i.e. ueIpAddress or supi) or anyUe set to true shall be included. NOTE 2: UPF event Exposure targeting a UE is not supported for non-IP PDU session types, when the UPF is not allowed by local SMF configuration to receive the SUPI associated with a N4 session. |  |  |  |  |  |

* * * Next Change * * *

#### 6.1.6.X	Data types describing alternative data types or combinations of data types

##### 6.1.6.X.1	Enumeration: ExtNotificationData

Table 6.1.6.X.1-1: Definition of type ExtNotificationData as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| NotificationData | 1 | Notification Data of a subscription |
| array(NotificationData) | 2..N | List of Notification Data of multiple subscriptions, when using the BERMS feature |

* * * Next Change * * *

### 6.1.8	Feature negotiation

The optional features listed in table 6.2.8-1 are defined for the Nupf_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |
| 1 | EEMM | O | Event Exposure Muting Mechanism A UPF supporting this feature shall support the handling of event muting exception instructions as specified in clause 6.2.7.2 of 3GPP TS 23.288 [17]. |
| X | BERMS | O | Bundling Event Reports of Multiple Subscriptions A NF Service Consumer and a UPF supporting this feature shall support UPF notifying bundled event reports of multiple subscriptions (see clause 5.2.2.3.X). |

* * * Next Change * * *

# A.2	Nupf_EventExposure API

openapi: 3.0.0

[…]

paths:

/ee-subscriptions:

post:

summary: Nupf_EventExposure Subscribe service Operation

operationId: CreateSubscription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CreateEventSubscription'

responses:

'201':

description: Successful creation of an UPF Event Subscription

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nupf-ee/<apiVersion>/ee-subscriptions/{subscriptionId}'

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/CreatedEventSubscription'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

eeNotification:

'{eventNotificationUri}':

# The URI in {eventNotificationUri} is provided via N4 interface during provisioning of Session Reporting Rule or in the Nupf_EventExposure Subscribe request.

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ExtNotificationData'

responses:

'204':

description: No Content, Notification was succesfull

'307':

description: Temporary Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'The URI pointing to the resource located on the redirect target NF service consumer'

required: true

schema:

type: string

'308':

description: Permanent Redirect

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RedirectResponse'

headers:

Location:

description: 'The URI pointing to the resource located on the redirect target NF service consumer'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

[…]

NotificationData:

description: the list of NotificationItems

type: object

required:

- notificationItems

properties:

notificationItems:

type: array

items:

$ref: '#/components/schemas/NotificationItem'

minItems: 1

correlationId:

type: string

achievedSampRatio:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SamplingRatio'

eventNotifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

[…]

UpfEventSubscription:

description: UPF Event Subscription

type: object

properties:

eventList:

type: array

items:

$ref: '#/components/schemas/UpfEvent'

minItems: 1

eventNotifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

type: string

eventReportingMode:

$ref: '#/components/schemas/UpfEventMode'

nfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ueIpAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

anyUe:

type: boolean

default: false

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

bundlingAllowed:

type: boolean

enum:

- true

bundleId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

bundledEventNotifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

required:

- eventList

- eventNotifyUri

- notifyCorrelationId

- eventReportingMode

- nfId

[…]

HandlingOfPayloadHeader:

description: Handling of Payload Header status

type: object

properties:

headerInfo:

type: string

headerValueBefore:

type: string

headerAction:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/HeaderHandlingAction'

headerValueAfter:

type: string

required:

- headerInfo

- headerAction

ExtNotificationData:

oneOf:

- $ref: '#/components/schemas/NotificationData'

- type: array

items:

$ref: '#/components/schemas/NotificationData'

minItems: 2

[…]

* * * End of Changes * * *