3GPP TSG-CT Meeting #109	CP-252234

# Beijing, P.R. China; 15th – 16th September 2025								(was CP-252221)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.041 | CR | 0269 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of support for PWS over satellite NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Incorporated, Nokia, T-Mobile USA, Novamint, Thales, Viasat, EchoStar |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_ARCH-CT |  |  |  |  |  | Date: |  |  | 2025-09-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA1 has added requirements for support for PWS over satellite NG-RAN to TS 22.268 for Rel-17 (see S1-244777) in a Category F CR under 5GSAT work item. CT1 needs to align and provide stage 2 supports for the SA1 requirements. A Satellite NG-RAN may be deployed by a PLMN identified by either a country-specific MCC or a shared MCC (9xx). When a shared MCC is used, the UE may not be able to correctly derive the duplication detection timer, since the timer is derived based on the MCC of the serving PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for PWS over satellite NG-RAN to the scope of TS 23.041. Clarify that the UE may not be able to correctly derive the duplication detection time when the MCC of the current PLMN is a shared MCC 9xx |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PWS feature will not be supported over satellite NG-RAN. The user experience will be negatively affected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Corrected Source to TSG from “C1” to “Qualcomm Incorporated, Nokia, T-Mobile USA, Novamint, Thales, Viasat, EchoStar” |  |  |  |  |  |  |  |  |

*** first change ***

# 1	Scope

The present document describes the Cell Broadcast short message service (CBS) for GSM and UMTS.

For GSM it defines the primitives over the Cell Broadcast Centre - Base Station System (CBC-BSS) interface and the message formats over the Base Station System - Mobile Station (BSS-MS) interface for Teleservice 23 as specified in 3GPP TS 22.003 [2].

For UMTS it defines the interface requirements for the Cell Broadcast Centre – UMTS Radio Network System (RNS) interface and the radio interface requirements for UMTS Radio Acces Networks to support CBS as specified in 3GPP TS 22.003 [2].

The present document also describes the Public Warning System (PWS) for GSM, UMTS, E-UTRAN, NG-RAN and satellite NG-RAN, see 3GPP TS 22.268 [28].

*** second change ***

# 2	General description

The CBS service is analogous to the Teletex service offered on television, in that like Teletex, it permits a number of unacknowledged general CBS messages to be broadcast to all receivers within a particular region. CBS messages are broadcast to defined geographical areas known as cell broadcast areas. These areas may comprise of one or more cells, or may comprise the entire PLMN or SNPN. Individual CBS messages will be assigned their own geographical coverage areas by mutual agreement between the information provider and the PLMN or SNPN operator. CBS messages may originate from a number of Cell Broadcast Entities (CBEs), which are connected to the Cell Broadcast Centre. CBS messages are then sent from the CBC to the cells, in accordance with the CBS's coverage requirements.

A CBS page comprises of 82 octets, which, using the default character set, equates to 93 characters. Other Data Coding Schemes may also be used, as described in 3GPP TS 23.038 [3]. Up to 15 of these pages may be concatenated to form a CBS messagee. Each page of such CBS message will have the same message identifier (indicating the source of the message), and the same serial number. Using this information, the MS/UE is able to identify and ignore re-broadcasts of already received messages.

CBS messages are broadcast cyclically by the cell at a frequency and for a duration agreed with the information provider. The frequency at which CBS messages are repeatedly transmitted will be dependent on the information that they contain; for example, it is likely that dynamic information such as road traffic information, will require more frequent transmission than weather information. The repetition period will also be affected by the desire for CBS messages to be received by high speed mobiles which rapidly traverse cells.Reception of CBS messages for an MS/UE is not a requirement if it is connected in the CS domain. It should be possible for an MS/UE to receive messages if it is connected in the PS domain and no data is currently transmitted.

| CS-Domain | CS-Connected | CS-Idle | CS-Idle |
| --- | --- | --- | --- |
| PS-Domain | - | PS-Idle | PS-Connected |
| Reception of CBS Message | Not possible | Possible | Depends on RRC mode |

NOTE:	In case the UE is in CS-Idle and PS-Connected Mode it depends on the Radio Resource Control State whether reception of CBS messages is possible. The relevant states are described in 3GPP TS 25.331 [16].

GSM only [CBS messages may be broadcast on two different cell broadcast channels, which are characterized by different QoS. A MS is always able to read the basic channel (see 3GPP TS 45.002 [8]). The reading of the extended channel may collide with other tasks of the MS. Therefore the probability of receiving a CBS message on the extended channel is smaller than on the basic channel. The reading of the extended channel for MSs is optional. The scheduling on the channels will be done independently].

To permit mobiles to selectively display only those CBS messages required by the MS/UE user, CBS messages are assigned a message class which categorises the type of information that they contain and the language (Data Coding Scheme) in which the CBS message has been compiled. Through the use of appropriate MMI, the user is then able to ignore message types that he does not wish to receive, e.g. advertising information or messages in an unfamiliar language.

A network may be able to remotely activate mobile terminals in order to enable them to receive CBS messages, according to regulatory requirements (see 3GPP TS 25.331 [16]).

PWS provides a service that allows the network to distribute warning messages on behalf of public authority. PWS enables the distribution of ETWS, CMAS (aka WEA), KPAS and EU-Alert warning messages in GSM, UMTS, E-UTRAN, and NG-RAN.

Some of the PWS warning message distribution mechanisms are access technology specific, but some CBS procedures and related message structures are common for GSM and UMTS, and some CBS procedures and related message structures are common for E-UTRAN and NG-RAN.

The language-independent content mapped to an event or a disaster can be included in a warning message that is transparently passed from CBC to UEs. UEs with user interface which support the ePWS language-independent content functionality can display the entire warning message that they receive.

UEs with no user interface which support the ePWS disaster characteristics functionality can derive the characteristics of a disaster from the message identifier of a received warning message.

Requirements applicable for NG-RAN are also applicable for satellite NG-RAN, unless otherwise specified.

***third and last change ***

## 8.2	Duplication Detection Function

The MS/UE uses a common duplication detection function for all messages received in GSM, UMTS, E-UTRAN and NG-RAN.

In a PLMN or SNPN, upon reception of a new message, the MS/UE shall perform duplication detection on the messages. Those messages that are received from the same PLMN or SNPN, in the certain time period specified by the duplication detection time are subject to duplication detection. The MS/UE shall not perform duplication detection on messages whose duplication detection time has elapsed. The value of the duplication detection time to be used by the MS/UE shall be derived from the MCC of the current PLMN or SNPN as follows:

-	If the message was received from a PLMN and the MCC of the PLMN = 440 or = 441 (Japan), duplication detection time shall be 1 hour;

-	For all other cases, duplication detection time shall be 24 hours.

NOTE 0:	In the present version of the present document, the UE in Japan incorrectly derives the duplication detection time when the MCC of the current PLMN is 9xx (shared MCC). In that case, a received message can be incorrectly considered as a duplicate of a message received earlier and be ignored.

If the messages are received from the same PLMN or SNPN, then:

1)	the MS/UE shall check whether the Serial Number associated with the Message Identifier of the new message matches the Serial Number of any of those messages with the same Message Identifier that have been received and displayed to the user and that are subject to the duplication detection; and

2)	the MS/UE may check other criteria for detecting duplicates. An example of such a criterion is whether the actual contents of the two messages are the same.

NOTE 1:	Criterion 2 allows the UE to present more than 1024 different messages received within the duplication detection time period and will prevent those messages with the same Serial Number but with different message content to be rejected as duplicates.

If criterion 1 is fulfilled and any implemented additional checks (as described in criterion 2) are also met, then the MS/UE shall consider the new message as duplicated and shall ignore it. If the Geographical Scope is not PLMN/SNPN wide the validity of the Serial Number may be considered as described in clause 9.4.1.2.1.

If the messages are received from different PLMNs or SNPNs, then (as described in clause 9.4.1.2.1) "the CBS message is only relevant to the PLMN or SNPN in which it is broadcast, so any change of PLMN (including a change to another PLMN which is an ePLMN) or SNPN means the CBS message is "new"". Therefore, the MS/UE is not required to conduct duplication detection on messages received from different PLMNs/SNPNs.

NOTE 2:	A duplicate message can only be received once by the user in a PLMN/SNPN during a duplication detection time. When the UE is in a different PLMN /SNPN then receiving a duplicate message once by the user, that was received earlier in a different PLMN/SNPN, can be tolerated.

For ETWS, duplicate message detection shall be performed independently for primary and secondary notifications.

*** no more changes ***