3GPP TSG-CT1 Meeting #157	C1-256708

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.186 | CR | 0102 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update in UE procedures for Standalone IMS DC sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is specified in TS 23.228 [AC.10.2.2] and [AC.10.2.3] that the terminating UE may not alert the user and tear down the entire IMS session if it did not successfully download the DC Application. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Rearranged the paragraphs in Clause 8.1.2 for better readability. Change 2: Added procedure for the terminating UE when the DC Application is not successfully downloaded while establishing application DC sessions in standalone case. Change 3: Editorial corrections in clause 8.1.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 alignment will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2, 9.3.3.1.6.2A, 9.3.3.1.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

### 8.1.2	Support of Standalone IMS Data Channel Session

If the originating UE has downloaded the data channel application, application data channel can be established together with bootstrap data channel during standalone IMS data channel session establishment.

If the originating UE did not download the data channel application, the UE shall establish an IMS session with only the bootstrap data channel towards a PSI, download the application through the established bootstrap data channel, and then establish a standalone IMS session with the associated application data channel and required bootstrap data channels towards peer UE; or initiate an IMS session with only the bootstrap data channel towards peer UE, and download the application through the established bootstrap data channel then add the associated application data channel during standalone IMS data channel session establishment.

If the terminating UE received the INVITE request to set up the bootstrap and application data channel simultaneously, the UE shall identify the data channel application in the application binding information (provided within the "a=3gpp-req-app" SDP attribute) is available or not, and:

1)	if available, it shall follow the procedure in clause 9.3.3.1.6.3 to establish the requested bootstrap data channel and application data channel; or

2)	if not available, it shall accept the bootstrap data channel and reject the application data channel to indicate that the data channel application is desired to be downloaded and download the application data channel through the established bootstrap data channel.

# *** Next Change***

###### 9.3.3.1.6.2A	Adding application data channel during standalone IMS data channel session establishment

Upon receipt of the initial INVITE request with the SDP offer with only the media description for the bootstrap data channel and the UE accepts the the bootstrap data channel, the UE shall return a 183 (Session Progress) response to the initial INVITE request with the SDP answer generated as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1].

NOTE 1:	When receiving the incoming INVITE request, the UE can alert the user that there is a standalone data channel establishing based on the UE's implementation for user consent.

Upon receipt of the UPDATE request with the updated SDP offer containing the established bootstrap data channel and a requested application data channel with the associated DC application binding information (provided within the "a=3gpp-req-app" SDP attribute) according to 3GPP TS 26.114 [4] to establish an application data channel, the UE shall determine whether the data channel application has been downloaded or not. If the data channel application:

- has not been downloaded, the UE shall not:

1)	download the application data channel application;

2)	alert the user and return the 180 (Ringing) once the data channel application is downloaded;

3)	generate the SDP answer that contains the media description for the application data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and return the 200(OK) response to the UPDATE request; and

4)	return 200 (OK) response to the INVITE request; or

b)	has been downloaded, the UE shall:

1)	alert the user and return the 180 (Ringing);

2)	generate the SDP answer that contains media descriptions for both the bootstrap data channel and application data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and return the 200(OK) response to the UPDATE request; and

3)		return 200 (OK) response to the INVITE request.

NOTE 2:	Based on the UE's implementation for user consent, the UE can alert the user that data channel is downloaded.

If the UE did not successfully download the data channel application, the UE shall reject the INVITE or re-INVITE or UPDATE request with a suitable 4xx response code. In this case, the UE shall not alert the terminating user.

# *** Next Change***

###### 9.3.3.1.6.3	Combined standalone bootstrap data channel and application data channel establishment

Upon receipt of the initial INVITE request with the SDP offer with the media description for both the bootstrap data channel and the application data channel associated with the application binding information in "a=3gpp-req-app" line, the UE shall determine whether the data channel application has been downloaded or not. If the data channel application:

a)	has not been downloaded, the UE:

1)	shall generate SDP answer for the data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] accepting the bootstrap data channel and rejecting the application data channel by setting the port number of m=line of application data channel to zero, indicate the data channel application is desired to be downloaded according to 3GPP TS 23.228 [3] clause AC.10.2.3 and return a 183 (Session Progress) response with the generated SDP answer to the initial INVITE request;

NOTE 1:	When receiving the incoming INVITE request, the UE can alert the user that there is a standalone data channel establishing based on the UE's implementation for user consent.

2)	download the application data channel application;

3)	 upon receipt of the UPDATE request with the updated SDP offer to establish the corresponding application data channel,

-	alert the user and return the 180 (Ringing) once the data channel application is downloaded;

-	generate the SDP answer as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and return the 200(OK) response to the UPDATE request;

-	return 200 (OK) to the INVITE request; or

b)	has been downloaded, the UE shall:

1)	generate the SDP answer that contains both the bootstrap data channel and application data channel as per 3GPP TS 26.114 [4] and IETF RFC 8864 [1] and retrun the 183 (Session Process) response to the network;

2)	alert the user and return the 180 (Ring) response; and

3)	 return the 200(OK) response to the INVITE request.

NOTE 2:	Based on the UE's implementation for user consent, the UE can alert the user that data channel is downloaded.

If the UE did not successfully download the data channel application, the UE shall reject the INVITE or re-INVITE or UPDATE request with a suitable 4xx response code. In this case, the UE shall not alert the terminating user.

# *** End of Changes***