3GPP TSG-CT1 Meeting #157	C1-256711

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6758 | rev | 1 | Current version: | 19.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Abnormal scenarios related to HTTP signing/verification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is specified in TS 23.228[AF.3], TS 24.229[5.7.1.25.3, 5.10.10.2] that the terminating IMS network (i.e., S-CSCF, IMS AS or IBCF) may reject the initial INVITE or MESSAGE request if the RCD verification is unsuccessful. The handling of the INVITE or MESSAGE request in other abnormal scenarios such as when the AS for verification is not reachable have to be addressed in stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Text indentation proposed in C1-255380 is not correctly implemented in TS 24.229 [Clause 5.10.10.2]. Hence, it is corrected here for clarity. Change 2: Clause 5.10.10.2 specifies that upon sending a verificationRequest over the Ms reference point, if the RCD verification is unsuccessful e.g., the outcome of the RCD verification is equal to the value "RCD-Validation-Failed" or the AS for verification is not reachable, the IBCF acting as as entry point may based on the policy reject the initial INVITE or MESSAGE request with a suitable SIP response code. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Abnormal cases related to HTTP verification will not be completely addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was C1-256294 |  |  |  |  |  |  |  |  |

# *** First Change ***

#### 5.10.10.2	Procedures for an IBCF acting as an entry point

When receiving an initial INVITE, re-INVITE or MESSAGE request containing one or more SIP Identity header fields, the IBCF shall determine the information (originating identity, diverting identities, contents of the Resource-Priority, Priority and Call-Info header fields) to be verified by decoding the Identity header fields containing a PASSporT SHAKEN JSON Web Token and/or a PASSporT rph JSON Web Token with an optional PASSporT sph JSON Web Token and/or a PASSporT rcd JSON Web Token. The IBCF uses the Identity header fields to:

1)	build and send a verificationRequest, specified in annex V, to an AS for verification over the Ms reference point; and

2)	shall upon receiving an HTTP 200 (OK) response to the above request, use:

-	the verstat claim from this response to

populate the "verstat" tel URI parameter associated with the originating identity and add this parameter to the verified identity in the SIP From header field or the SIP P-Asserted-Identity header field in the forwarded SIP request. Additionally, if the HTTP 200 (OK) response included verification results for the diverting identities, the IBCF shall based on local policy add the "verstat" tel URI parameter to the verified diverting identities in the History-Info header field if this field is available;

-	the verstatRcd claim from this response to include in the Call-Info header field containing the RCD info in the forwarded initial INVITE request or MESSAGE request the "verified" parameter set to value "true" as defined in RFC 9796 [303] if the outcome of RCD verification is equal to the value "RCD-Validation-Passed";

-	the verstatPriority claim from this response to populate the Priority-Verstat header field associated with the Resource-Priority header field and with the header field value "psap-callback" of the Priority header field (if present) and include the Priority-Verstat header field in the forwarded SIP request; and

-	the verifyResults from this response, if present, to store any of the PASSporT verification failure parameters shown in Table V.2.6.2-4.

If the RCD verification is unsuccessful e.g., the outcome of the RCD verification is equal to the value "RCD-Validation-Failed" or the AS for verification is not reachable, then based on the policy, the IBCF may reject the initial INVITE request or MESSAGE request with suitable 4xx or 5xx or 6xx response code.

Based on local policy, the IBCF may populate for each reported Identity header field verification error a Reason header field in the next provisional or final response of the INVITE or MESSAGE request, where the Reason header field protocol value is set to "STIR", as specified in RFC 9410 [294] and RFC 9366 [296], and the "cause" header field parameter contains the stored "reasonCode" value. Additionally, the IBCF may include the "ppi" header field parameter containing the failing PASSporT.

NOTE 1:	Multiple Reason header fields with the protocol value set to "STIR" are not supported in the present document.

Based on local policy, the IBCF may verify that the validated claims returned in the validClaims parameter of the verification response authorize the associated SIP header field values.

NOTE 2:	For sessions originating in another domain, only one of the following entities needs to be configured to verify the Identity header field for the resource priority: the IBCF or the AS. Which functional entity inserts the Identity header field verification is subject to network configuration and local policy.

NOTE 3:	For sessions originating in another domain, only one of the following entities needs to be configured to verify the Identity header field for the RCD info: the S-CSCF, the IBCF or the AS. Which functional entity inserts the Identity header field verification is subject to network configuration and local policy.

# *** End of Changes ***