3GPP TSG-CT6 Meeting #124	C6-250655

# Dallas, United States, 18th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0831 | rev | - | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Geographical location discovery related test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT6 |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R18 |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a descriptive error in Step 2 of th eGeographical location discovery test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | "PROACTIVE COMMAND : Geographical Location Request 1.1.1" shall be changed to "PROACTIVE COMMAND PENDING: Geographical Location Request 1.1.1" . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description of Step 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 27.22.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************************start of changes ********************************

27.22.15	Geographical location discovery

27.22.15.1	Definition and applicability

See clause 3.2.2.

27.22.15.2	Conformance requirement

The ME shall support the Geographical Location Request Command and Geographical Location Reporting Command as defined in:

-	TS 31.111 [15] clause 6.4.40, clause 6.6.40 and clause 7.10.

27.22.15.3	Test purpose

To verify that the ME performs the proactive command Geographical Location Request and the envelope command Geographical Location Reporting allows the UICC to request and receive the current geographical location information from the ME when the ME is equipped with a positioning feature and it is enabled (e.g. autonomous GPS, Assisted GPS or Assisted GNSS).

27.22.15.4	Method of test

27.22.15.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the NG-SS.

The NG-RAN parameters of the NG-SS are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

The elementary files are coded as the default NG-RAN UICC .

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME is equipped with a positioning feature and it is enabled.

Test environment shall be set up to provide the needed input to the UE, in order for the UE to derive its location, if the UE uses Geolocation header for providing its geographical location. This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [46], if supported by the UE. Otherwise, or in addition any other suitable method may also be used.

27.22.15.4.2	Procedure

Expected Sequence 1.1 (Geographical location discovery, Preferred GAD shapes is Ellipsoid point with altitude, NG-RAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: Geographical Location Request 1.1.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: Geographical Location Request 1.1.1 |  |
| 5 | ME UICC | TERMINAL RESPONSE: Geographical Location Request 1.1.1 | [normal ending] |
| 6 | ME UICC | ENVELOPE Geographical Location Reporting 1.1.1 | [The ME provides the GAD shapes of Ellipsoid point with altitude ] |

PROACTIVE COMMAND: Geographical Location Request 1.1.1

Logically:

Command details

Command number:	1

Command type:	Geographical Location Request

Command qualifier:	0

Device identities

Source device:	UICC

Destination device:	ME

Geographical Location Parameters

Horizontal accuracy: horizontal accuracy not specified / best effort

Vertical coordinate:  vertical coordinate is requested, (i.e. 3D location fix is preferred) but accuracy is not specified (best effort)

Velocity:                   Horizontal velocity requested

Preferred GAD shapes: Ellipsoid point with altitude

Preferred NMEA sentences: RMC

Preferred Maximum Response Time: 2^7 seconds

Coding:

| BER-TLV: | D0 | 11 | 81 | 03 | 01 | 16 | 00 | 82 | 02 | 81 | 82 | 76 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 06 | 81 | 81 | 01 | 08 | 01 | 07 |  |  |  |  |  |

TERMINAL RESPONSE: Geographical Location Request 1.1.1

Logically:

Command details

Command number:	1

Command type:	Geographical Location Request

Command qualifier:	0

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 16 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: Geographical Location Reporting 1.1.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

GAD shapes

GAD shape:	Degrees of latitude, Degrees of longitude and Altitude 8038D13852BC9A0071 (as an example)

Velocity:        Horizontal Velocity  005B0006 (as an example)

Coding:

| BER-TLV: | DD | 15 | 82 | 02 | 82 | 81 | 77 | 0F | 09 | Note1 | 04 | Note2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

NOTE 1:	The GAD shape values are present. The values of these nine bytes shall not be verified.

NOTE 2:	The Velocity values are present. The values of these four bytes shall not be verified.

****************************end of changes ********************************