3GPP TSG-CT Meeting #110	CP-253209

Baltimore, United States, 8th - 9th Dec 2025

Source:	Huawei, HiSilicon, vivo, CATT, China Unicom, SIA

Title:	Discussion on the Working Agreement for the data collection from LMF

Document for:	Discussion

Agenda Item:	19.39

Work Item / Release:	AIML_CN / Rel-19

- Introduction

A Working Agreement was declared by the CT3 Chair during CT3#144 meeting on the agreement of C3-255506 (CR#0135 to TS 29.574, "Resolve the Editor’s Note for the LMF data subscription indication"). This WA was challenged by a company, this issue will hence be discussed during CT#110 plenary meeting and a voting will take place.

This discussion paper aims hence at providing an overview of the issue and clarifications on the situation and technical details of this Working Agreement.

- Discussion

2.1 Background and Status during CT3#144 meeting

When specifying the support of LMF retrieval of a trained AI/ML Model for positioning from the NWDAF (MTLF), consensus was not reached in SA2 regarding whether the DCCF could act as a consumer of the Nlmf_DataExposure service (see Table 8.3.1-1 of TS 23.273).

Due to no consensus in stage 2, CT3 agreed a CR as CP-243300 in December 2024 with an Editor’s Note that whether the DCCF can be a consumer of the LMF API, conditioned on SA2 agreeing on introducing this requirement in their corresponding specifications. CT3 later sent out an LS (as C3-251653) to SA2 in April 2025 seeking clarifications on this issue.

In addition, CT4 followed the stage 2 requirement and as specified in Figure 4-1 of TS 29.572, the DCCF is not one of the consumers of the LMF per Nlmf_DataExposure service.

Per the LS reply (as S2-2507650) from SA2 in October 2025, it clearly states that stage 2 "did not yet conclude on whether the DCCF can be used to collect data using the Nlmf_DataExposure service". Accordingly and as this was the last meeting before the stage 3 OpenAPI/code freeze for Rel-19, in order to align with stage 2, a CR as C3-255506 was submitted by Huawei in November CT3#144 meeting to solve the Editor’s Note by not introducing the DCCF as one of the consumers of Nlmf_DataExposure service.

Only a single company objected to this proposal without any valid technical reasons, while the proposal is widely supported by the majority of companies. To move forward and ensure the completion of the AIML_CN work, a Working Agreement was declared by the CT3 Chair for the agreement of C3-255506.

It is also important to note that only data collection by the NWDAF (MTLF) is currently supported as per clause 6.22 of TS 23.273 (see in particular clause 6.22.4). It does not have any impacts or side-effects on the overall LMF-based AI/ML Positioning functionality of Rel-19. The data collection for the LMF based AI/ML Positioning works well and can be deployed with no problems and no short-comings, e.g., by using the data collection by the NWDAF (MTLF).

2.2 Status after CT3#144 meeting

The Working Agreement from CT3 has been challenged and a new proposed CR from the objecting company in CP-253195 is directly submitted as a company contribution to the plenary. This contribution has not been seen in CT3 WG and is not really a compromise as per the following:

-	The "lmfEventInd" attribute within DccfEvent data type defined in Ndccf_DataManagement API (to support the DCCF as the consumer of Nlmf_DataExposure service) is kept while the corresponding optional feature is removed, which does not make sense. This would make the situation worse and the misalignment with stage 2 wider, as the attribute becomes now part of the baseline of Rel-19 (implementations have to support it) and is not anymore optional.

-	The following NOTE 2 is added for this "lmfEventInd" attribute. This NOTE is incorrect and unclear because:

NOTE 2:	The "lmfEventInd" attribute is not applicable for the Ndccf_DataManagement and Nnwdaf_DataManagement APIs in this release of the specification, and it is included here only for usage by other APIs.

1)	The entire NOTE has no stage 2 requirement.

2)	There are many APIs impacted by the DccfEvent data type (see the screenshot below from the "Data Type Finder" tool of 3GPP Forge, showing the list of APIs in which this DccfEvent data structure, and the attributes within it, are reused), which makes it even more dangerous and incorrect to support anything within it that has no corresponding stage 2 requirements and analysis in stage 3, e.g., this attribute is not applicable to the VFL related APIs defined in TS 29.530.

Hence, the new proposal from the objecting company is technically incorrect, unclear and will worsen the misalignment with stage 2.

Figure. The list of the APIs impacted by the DccfEvent data type

Based on the above, the authors of this paper strongly recommend to follow the stage 2 requirement and 3GPP working process and move forward with the CT3 Working Agreement in C3-255506.

- Proposal

It is hence proposed to agree on the working agreement on C3-255506 to have alignment between Stage 2 and Stage 3.