3GPP TSG-CT Meeting #110	CP-253210

# Baltimore, US; 08th – 09th December 2025								(revision of C4-255268)

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 29.175 | CR | 0106 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Audio/video support in the MDC2 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.228 clause AA.2.4.3, for the avatar communication, the parameter of rendering mode is defined. The value of this parameter can be set to network centric with DC AS rendering. In this case, the audio/video media will be transmitted to DC AS as specified in the step 25b of procedure in clause AC.11.3.3 in TS 23.228. While, currently, the audio/video media can only be anchored to MF, it is not supported to transmit the audio/video media to DC AS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the media instruction to support to transmit audio/video media to DC AS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DC AS preforms media transcoding is not supported for avatar. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.1; 6.2.6.2.3; 6.2.6.2.x; A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new feature to the following APIs: TS29175_Nimsas_MediaControl.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: use separate endpoint for audio and video media and the OpenAPI accordingly. R2: add the missing ":” in the yaml |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nimsas_MediaControl service based interface protocol.

Table 6.2.6.1-1: Nimsas_MediaControl specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| ArMediaSpecification | 6.2.6.2.5 | The AR media specification information needed for AR communication services from application layer. |
| AudioVideoReNegotiationInd | 6.2.6.2.7 | The audio/video media re-negotiation indication information to connect the UE's audio/video media stream to MF. |
| DcMediaSpecification | 6.2.6.2.4 | The data channel media specification information needed for data channel media stream from application layer. |
| InterworkingInstruction | 6.2.6.2.8 | The interworking instruction of data channel transcoding. |
| Mdc2EndpointInfo | 6.2.6.2.6 | The MDC2 media endpoint information. |
| Mdc2AudioVideoEndpoint | 6.2.6.2.x | The endpoint information of DC AS for the video media in MDC2 interface. |
| MediaInstruction | 6.2.6.3.3 | The media instruction for handling a specific media flow. |
| MediaInstructionData | 6.2.6.2.2 | The media instruction data for a specific session. |
| MediaInstructions | 6.2.6.2.3 | The media instructions for a specific media flow. |
| MediaConnSide | 6.2.6.3.4 | The side that the audio or video media need to be connected to MF. E.g., both the originating and terminating side. |
| TranscodeMode | 6.2.6.3.5 | The mode of data channel transcoding. |
| SendRecvMode | 6.2.6.3.6 | The send and recv mode of the media stream. |

Table 6.2.6.1-2 specifies data types re-used by the Nimsas_MediaControl service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nimsas_MediaControl service based interface.

Table 6.2.6.1-2: Nimsas_MediaControl re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| DcStream | 3GPP TS 29.571 [16] | Data Channel mapping and configuration information. |
| MdcEndpoint | 3GPP TS 29.571 [16] | The MDC1 and MDC2 Endpoint information. |
| MaxMessageSize | 3GPP TS 29.571 [16] | Maximum SCTP user message size. |
| Mdc2Protocol | 3GPP TS 29.176 [17] | Transport layer protocols for MDC2 interface. |
| MediaId | 3GPP TS 29.571 [16] | IMS media flow identity. |
| MediaProxy | 3GPP TS 29.571 [16] | Media proxy configuration applicable to the media flow. |
| MediaResourceType | 3GPP TS 29.571 [16] | IMS media resource type. |
| ReplaceHttpUrl | 3GPP TS 29.571 [16] | A list of replacement HTTP URL per stream ID. |
| SessionId | 3GPP TS 29.571 [16] | IMS session identity. |
| Uri | 3GPP TS 29.571 [16] | URI |
| BdcUsedBy | 3GPP TS 29.571 [16] | The party using the bootstrap data channel. |
| AppBindingInfo | 3GPP TS 29.571 [16] | The application binding information of the application data channel. |
| QosHintInfo | 6.1.6.2.7 | The QoS hint information of the media. |
| AvatarMedia | 3GPP TS 29.176 [17] | Avatar media specification. |
| Endpoint | 3GPP TS 29.571 [16] | Endpoint information. |
| AudioVideoMedia | 3GPP TS 29.571 [16] | The description of audio and video media. |

* * * Next Change * * * *

6.2.6.2.3	Type: MediaInstructions

Table 6.2.6.2.3-1: Definition of type MediaInstructions

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mediaId | MediaId | M | 1 | The media ID uniquely identifies this media item within the list and used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field shall be null for instructions related to originating new media or for data channel media components de-multiplexed from a data channel multiplexing media which do not reuse the media ID of that data channel multiplexing media. |
| associatedMediaId | MediaId | C | 0..1 | The identifier of the original multiplexed media stream which is de-multiplexed. It shall be included only when the mediaId is null, mediaInstruction is set to "ORIGINATE" and the media stream identified by the value of this attribute is de-multiplexed. |
| mediaResourceType | MediaResourceType | M | 1 | The mediaResourceType identify the Media Resource types the Media instruction is intended for. |
| mediaInstruction | MediaInstruction | C | 0..1 | It shall be included if the mediaResourceType is set to "DC". When present, mediaInstruction includes instructions to the producer (IMS AS) for handling the media. |
| dcMediaSpecification | DcMediaSpecification | C | 0..1 | dcMediaSpecification includes the description of additional media specification information which are needed for data channel media stream from application layer. It shall be contained if the mediaResourceType is set to "DC". |
| arMediaSpecification | ArMediaSpecification | C | 0..1 | arMediaSpecification includes the description of additional media specification information which are needed for AR communication services from application layer. It shall be contained if the mediamedia is used for AR communication. |
| mediaProcessingUrl | Uri | C | 0..1 | The mediaProcessingURL indicates the address where MF receive service-related media instructions. This field shall be contained in the response when originating a new media. |
| audioVideoReNegotiationInd | AudioVideoReNegotiationInd | C | 0..1 | Represents the audio/video media re-negotiation indication information to connect the UE's audio/video media stream to MF which is required by the application data channel. It shall be included if the mediaResourceType is set to "AUDIO" or "VIDEO". |
| mdc2AudioEndpoint | Mdc2AudioVideoEndpoint | C | 0..1 | Represents the audio media endpoint information in MDC2 interface. It shall be included when the attribute renderingMode within the avatarMedia is set to "NET_CENTRIC_DCAS". |
| mdc2VideoEndpoint | Mdc2AudioVideoEndpoint | C | 0..1 | Represents the video media endpoint information in MDC2 interface. It shall be included when the attribute renderingMode within the avatarMedia is set to "NET_CENTRIC_DCAS". |
| avatarMedia | AvatarMedia | O | 0..1 | Represents the additional media specifications needed for avatar communication services. |

* * * Next Change * * * *

6.2.6.2.x	Type: Mdc2AudioVideoEndpoint

Table 6.2.6.2.x-1: Definition of type Mdc2AudioVideoEndpoint

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mdc2AVEndpointDcAs | Endpoint | C | 0..1 | Represents the MDC2 endpoint information on DC AS for audio or video media. It shall be included in the POST request message for the media control. |
| mdc2AVEndpointMf | Endpoint | C | 0..1 | Represents the MDC2 endpoint information on MF for audio or video media. It shall be included in the successful POST response message for the media control API. |
| audioVideoMedia | AudioVideoMedia | C | 0..1 | Represents the description of audio or video media. It shall be included in the successful POST response message for the media control API. |

* * * Next Change * * * *

# A.3	Nimsas_MediaControl API

openapi: 3.0.0

info:

title: 'IMS AS MediaControl Service'

version: 1.1.1-alpha.3

description: |

Nimsas_MediaControl Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

{Not shown for clarity}

MediaInstructions:

description: The media instructions for a specific media flow.

type: object

required:

- mediaId

- mediaResourceType

properties:

mediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

associatedMediaId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaId'

mediaResourceType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaResourceType'

mediaInstruction:

$ref: '#/components/schemas/MediaInstruction'

dcMediaSpecification:

$ref: '#/components/schemas/DcMediaSpecification'

arMediaSpecification:

$ref: '#/components/schemas/ArMediaSpecification'

mediaProcessingUrl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

audioVideoReNegotiationInd:

$ref: '#/components/schemas/AudioVideoReNegotiationInd'

mdc2AudioEndpoint:

$ref: '#/components/schemas/Mdc2AudioVideoEndpoint'

mdc2VideoEndpoint:

$ref: '#/components/schemas/Mdc2AudioVideoEndpoint'

avatarMedia:

$ref: 'TS29176_Nmf_MRM.yaml#/components/schemas/AvatarMedia'

DcMediaSpecification:

description: The data channel media specification information.

type: object

required:

- streams

- mediaProxyConfig

properties:

mediaProxyConfig:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MediaProxy'

replaceHttpUrls:

description: >

Contains a list of replacement HTTP URLs. The streamId

attribute within the ReplaceHttpUrl data type is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ReplaceHttpUrl'

minProperties: 1

mdc1EndpointDcsf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

mdc1EndpointMf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

mdc2EndpointInfo:

$ref: '#/components/schemas/Mdc2EndpointInfo'

streams:

description: >

Contains a data channel mapping and configuration information. The streamId

attribute within the DcStream data type is the key of the map.

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DcStream'

minProperties: 1

bdcUsedBy:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BdcUsedBy'

appBindingInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AppBindingInfo'

qosHintInfo:

$ref: 'TS29175_Nimsas_SessionEventControl.yaml#/components/schemas/QosHintInfo'

interworkingInfo:

$ref: '#/components/schemas/InterworkingInstruction'

dcIwkInd:

type: boolean

default: false

ArMediaSpecification:

description: The AR media specification information.

type: object

required:

- mediaProcessingSpec

properties:

mediaProcessingSpec:

type: string

Mdc2EndpointInfo:

description: Represents the negotiated MDC2 media endpoint information.

type: object

properties:

mdc2Protocol:

$ref: 'TS29176_Nmf_MRM.yaml#/components/schemas/Mdc2Protocol'

mdc2EndpointDcAs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

mdc2EndpointMf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MdcEndpoint'

AudioVideoReNegotiationInd:

description: The audio/video media re-negotiation indication information.

type: object

required:

- reNegotiation

properties:

reNegotiation:

type: boolean

mediaConnSide:

$ref: '#/components/schemas/MediaConnSide'

mediaDirection:

$ref: '#/components/schemas/SendRecvMode'

InterworkingInstruction:

description: The interworking instruction of data channel transcoding.

type: object

required:

- transcodeMode

properties:

transcodeMode:

$ref: '#/components/schemas/TranscodeMode'

addTransInfo:

type: string

Mdc2AudioVideoEndpoint:

description: The audio or video media endpoint information in MDC2 interface.

type: object

properties:

mdc2AVEndpointDcAs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

mdc2AVEndpointMf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Endpoint'

audioVideoMedia:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AudioVideoMedia'

{Not shown for clarity}

* * * End of Changes * * * *