3GPP TSG-CT WG4 Meeting #119	C4-235686

# Chicago, US; 13th– 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.573 | CR | 0153 | rev | 5 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N32-f related error determined upon receipt of an N32-f request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Nokia, Nokia Shanghai Bell, Ericsson, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, Roaming5G |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In CR#1789 to TS 33.501(SP-231202), it is approved that 1) SEPP can send error messages to intermediaries over the N32-f so that the error messages become visible to intermediaries. 2) intermediaries can send N32-f error messages to SEPP with an empty ReformattedData IE and appropriate error code. It is needed to capture the above change into TS 29.573. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Describe the N32 call flows for Error message originated by Roaming Intermediary or pSEPP upon receipt of an N32-f request. Defines new application errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with Stage2, i.e.SA3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.C.2(new), 5.X.C.2.1(new), 5.X.C.2.2(new), 6.2.4.2.2, 6.2.4.2.2, 6.2.5.2.XA, 6.2.5.2.XB, 6.2.6.3, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution introduces backward compatible new features to the OpenAPI file of JOSEProtectedMessageForwarding API. Clause 5.X.C is introduced in CR0149. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1) -add the work item code, TEI18 -add (New) and clauses affected -add “A” and “B” in order to distinguish two roaming intermediaries in the Figure 5.X.C.2-1 -change the message name on step9 in the Figure 5.X.C.2-1 -change the descrption on step5 5.X.C.2-1 -add “A” and “B” to distinguish two intermediaries in the Figure 5.X.C.3-1 -change the message name on step11 in the Figure 5.X.C.3-1 -align the number of step between the callflow and the descriptions -change the descrption on step7 -remove CLEAR_EXPECTED_IE_IN_ENCRYPTED and ENCRYPTED_EXPECTED_IE_IN_CLEAR from -add POLICY_MISMATCH to Table 6.2.6.3-1 Rev2) -correct the clause number 5.X.C.2, 5.X.C.2.1, 5.X.C.2.2, -correct the clause number Figure 5.X.C.2.1-1, Figure 5.X.C.2.2-1 -remove the text "or receiving SEPP" from Table 6.2.4.2.2-2 -change the descpription on step5 and 6 respectively in 5.X.C.2.1 -add Ericsson as a co-source campany Rev3) -correct the style of the new attributes in 6.2.5.2.XA and 6.2.5.2.XB -correct the indent in OpenAPI -add Vodafone as a co-source company Rev4) - correct the indent in OpenAPI (the last line) Rev5) - correct the indent in OpenAPI |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.X.C.2	N32-f related error determined upon receipt of an N32-f request

5.X.C.2.1	Error message originated by Roaming Intermediary via N32-f

Figure 5.X.C.2.1-1: Error message originated by Roaming Intermediary via N32-f

1.	The cSEPP receives a service request (HTTP request) message from cNF.

2.	The cSEPP sends an N32-f request using PRINS security (i.e. JOSE protected message) to forward the service request message to the pSEPP.

3.	The Roaming Intermediary detects an N32-f related error and returns an N32-f error response, e.g. "403 Forbidden" response, with the ProblemDetails data providing details on the N32-f related error for the cSEPP. If the error is due to an encryption policy mismatch, the ProblemDetails may include the invalidParams attribute indicating which IEs were received ciphered when they were expected to be received in clear, and vice-versa. The N32-f error response may additionally contain a suggested status code (e.g. "504 Gateway Timeout") and a suggested application error (e.g. "TARGET_PLMN_NOT_REACHABLE") that the Roaming Intermediary suggests the cSEPP to forward in the error response to the cNF, if the cSEPP cannot or does not attempt to re-send the N32-f request taking into account the N32-f error information.

4.	If the cSEPP cannot or does not attempt to re-send the N32-f request taking into account the N32-f error information, the cSEPP sends an error response to the cNF. The cSEPP may use the suggested status code and/or suggested application error for the error response sent to the cNF (e.g. the cSEPP may send a "504 Gateway timeout" response with the cause "TARGET_PLMN_NOT_REACHABLE" in the ProblemDetails).

5. 	Alternatively, the cSEPP may re-send the N32-f request taking into account the N32-f error information that was received from the Roaming Intermediary. For instance, if the cSEPP receives an error message with the application error "POLICY_MISMATCH", the cSEPP may change the data type encryption policy to 'Parameter shall be encrypted' or 'Parameter shall not be encrypted', if this is allowed by local policies, and if necessary, re-negotiate the data type encryption policy with the peer SEPP. After that, the cSEPP may re-send the N32-f Request based on the updated data type encryption policy to the Roaming Intermediary.

6-9.	The rest of procedures are processed accordingly.

5.X.C.2.2	Error message originated by pSEPP on N32-f (and optionally N32-c)

Figure 5.X.C.2.2-1: Error message originated by pSEPP via N32-f (and optionally N32-c)

1. 	The cSEPP receives a service request (HTTP request) message from cNF.

2.	The cSEPP sends an N32-f request using PRINS security (i.e. JOSE protected message) to forward the service request message to the pSEPP.

3.	The pSEPP detects an N32-f related error and returns an N32-f error response, e.g. "403 Forbidden" response, with the ProblemDetails data providing details on the N32-f related error for the cSEPP. If the error is due to an encryption policy mismatch, the ProblemDetails may include the invalidParams attribute indicating which IEs were received ciphered when they were expected to be received in clear, and vice-versa. The N32-f error response may additionally contain a suggested status code (e.g. "504 Gateway Timeout") and a suggested application error (e.g. "TARGET_PLMN_NOT_REACHABLE") that the pSEPP suggests the cSEPP to forward in the error response to the cNF, if the cSEPP cannot or does not attempt to re-send the N32-f request taking into account the N32-f error information.

4.	The pSEPP may also send an N32-c request (HTTP POST request) towards the cSEPP with the content containing the N32-f error information that is to be reported (see clause 5.2.5).

5.	The cSEPP shall return the status code "204 No Content" as the response to the N32-f Error Reporting. (see clause 5.2.5)

6.	If the cSEPP cannot or does not attempt to re-send the N32-f request taking into account the N32-f error information, the cSEPP sends an error response to the cNF. The cSEPP may use the suggested status code and/or suggested application error for the error response to the cNF (e.g. the cSEPP may send a"504 Gateway timeout" response with the cause "TARGET_PLMN_NOT_REACHABLE" in the ProblemDetails).

7.	Alternatively, the cSEPP may re-send the N32-f request taking into account the N32-f error information that was received from the pSEPP. For instance, if the cSEPP receives an error message with the application error "POLICY_MISMATCH", the cSEPP may change the data type encryption policy to 'Parameter shall be encrypted' or 'Parameter shall not be encrypted', if this is allowed by local policies, and if necessary, re-negotiate the data type encryption policy with the peer SEPP. After that, the cSEPP may re-send the N32-f Request based on the updated data type encryption policy to the Roaming Intermediary.

8-11.	The rest of procedures are processed accordingly.

* * * Next Change * * * *

6.2.4.2.2	Operation Definition

This operation shall support the request data structures and response codes specified in tables 6.2.4.2.2-1 and 6.2.4.2.2-2.

Table 6.2.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| N32fReformattedReqMsg | M | 1 | This IE shall contain the reformatted HTTP/2 message comprising the plain text part, encrypted information, meta data and modification chain information. See clause 6.2.5.2.2. |

Table 6.2.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| N32fReformattedRspMsg | M | 1 | 200 OK | This represents the successful processing of the reformatted JOSE protected message at the responding SEPP. The responding SEPP shall provide the reformatted and JOSE protected content of the corresponding HTTP/2 response message received from the NF service producer, or the HTTP/2 notification response message received from the NF service consumer. |
| ProblemDetailsMsgForwarding | O | 0..1 | 403 Forbidden | When the receiving SEPP fails to process the reconstructed message due to PLMN ID or SNPN ID verification failure, the "cause" attribute shall be set to "PLMNID_MISMATCH" or "SNPNID_MISMATCH". When the receiving SEPP receives HTTP requests over N32-f with purpose, marked using 3gpp-Sbi-Interplmn-Purpose header as specified in 3GPP TS 29.500 [4], that does not match with any of the purposes exchanged via the Security Capability Negotiation procedure, then the "cause" attribute shall be set to "REQUESTED_PURPOSE_NOT_ALLOWED". When the receiving SEPP fails to process the reconstructed message due to the n32fContextId is unknown, the "cause" attribute shall be set to "CONTEXT_NOT_FOUND". When the receiving SEPP fails to process the reconstructed message, and the error is reported by N32f error reporting procedure as specified in clause 5.2.5, the "cause" attribute shall be set to "UNSPECIFIED". When the Roaming Intermediary or receiving SEPP receives HTTP requests over N32-f and detects an encryption policy mismatch, e.g. protected IEs are not ciphered, or unprotected IEs are ciphered, the "cause" attribute shall be set to "POLICY_MISMATCH". In this case, the ProblemDetails may include the invalidParams attribute indicating which IEs were received ciphered when they were expected to be received in clear, and vice-versa, with the reason attribute for each invalid parameter set to "Parameter shall be encrypted" if the IE was sent without confidentiality protection" or “Parameter shall not be encrypted" if the IE was sent with confidential protection. When the Roaming Iintermediary or receiving SEPP receives HTTP requests, but the N32 connection cannot be setup due to contractual reasons, the "cause" attribute shall be set to "NO_CONNECTION_DUE_TO_CONTRACT". When the Roaming Intermediary receives HTTP requests but the N32 connection cannot be setup due to a connectivity issue, the "cause" attribute shall be set to "NO_CONNECTION_DUE_TO_CONNECTIVITY". When the Roaming Intermediary receives HTTP requests over N32-f but the message was not delivered due to contractual reasons, the "cause" attribute shall be set to "MSG_NOT_DELIVERED_DUE_TO_CONTRACT". |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] other than those specified in the table above also apply, with a ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.2.4.2.2-3: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| 3gpp-Sbi-Message-Priority | string | O | 0..1 | 3gpp-Sbi-Message-Priority header, defined in 3GPP TS 29.500 [4]. |

Table 6.2.4.2.2-4: Headers supported by 200 Response Code on this endpoint

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| 3gpp-Sbi-Message-Priority | string | O | 0..1 | 3gpp-Sbi-Message-Priority header, defined in 3GPP TS 29.500 [4]. |

* * * Next Change * * * *

6.2.5.2.XA	Type: ProblemDetailsMsgForwarding

Table 6.2.5.2.XA-1: Definition of type ProblemDetailsMsgForwarding as a list of to be combined data types

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| ProblemDetails | 1 |  |  |
| AdditionInfoMsgForwarding | 1 |  |  |

6.2.5.2.XB	Type: AdditionInfoMsgForwarding

Table 6.2.5.2.XB-1: Definition of type AdditionInfoMsgForwarding

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| suggestedStatusCode | integer | O | 0..1 | When present, this IE shall indicate a status code that is suggested to be sent to cNF if the cSEPP cannot or does not resend the N32-f request taking into account the N32-f error information. |
| suggestedProblemDetails | ProblemDetails | O | 0..1 | When present, this IE shall indicate suggested ProblemDetails to be sent to cNF if the cSEPP cannot or does not resend the N32-f request taking into account the N32-f error information. |

* * * Next Change * * * *

6.2.6.3	Application Errors

The application errors defined for the JOSE protected message forwarding API on N32-f are listed in Table 6.2.6.3-1.

Table 6.2.6.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| PLMNID_MISMATCH | 403 Forbidden | The PLMN ID in the Bearer token carried in the "Authorization" header of the reconstructed message does not match the PLMN ID of the N32-f context. |
| SNPNID_MISMATCH | 403 Forbidden | The SNPN ID in the Bearer token carried in the "Authorization" header of the reconstructed message does not match the SNPN ID of the N32-f context. |
| REQUESTED_PURPOSE_NOT_ALLOWED | 403 Forbidden | The purpose indicated in 3gpp-Sbi-Interplmn-Purpose header as specified in 3GPP TS 29.500 [4] of the reconstructed message does not match with any of the purposes exchanged via the Security Capability Negotiation procedure. |
| CONTEXT_NOT_FOUND | 403 Forbidden | The n32fContextId is unknown in the receiving SEPP. |
| UNSPECIFIED | 403 Forbidden | The receiving SEPP fails to process the reconstructed message, and the error is reported by N32f error reporting procedure as specified in clause 5.2.5. |
| POLICY_MISMATCH | 403 Forbidden | The encryption policy verification on the received N32-f message has failed, e.g. protected IEs are not ciphered, or unprotected IEs are ciphered. |
| NO_CONNECTION_DUE_TO_CONTRACT | 403 Forbidden | The message failed to be delivered as N32 connection cannot be setup due to contractual reasons. |
| NO_CONNECTION_DUE_TO_CONNECTIVITY | 403 Forbidden | The message failed to be delivered as N32 connection cannot be setup due to a connectivity issue. |
| MSG_NOT_DELIVERED_DUE_TO_CONTRACT | 403 Forbidden | The message was not delivered due to contractual reasons. |

Editor's note: the list of application errors to be added for the support of roaming intermediaries is FFS.

* * * Next Change * * * *

A.3	JOSE Protected Message Forwarding API on N32-f

openapi: 3.0.0

info:

version: '1.3.0-alpha.1'

title: 'JOSE Protected Message Forwarding API'

description: |

N32-f Message Forwarding Service.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

[…]

paths:

/n32f-process:

post:

summary:  N32-f Message Forwarding

tags:

- N32-f Forward

operationId: PostN32fProcess

parameters:

- name: Content-Encoding

in: header

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

- name: Accept-Encoding

in: header

description: Accept-Encoding, described in IETF RFC 7231

schema:

type: string

- name: 3gpp-Sbi-Message-Priority

in: header

description: 3gpp-Sbi-Message-Priority, defined in 3GPP TS 29.500

schema:

type: string

requestBody:

description: Custom operation N32-f Message Forwarding

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/N32fReformattedReqMsg'

responses:

'200':

description: OK (Successful forwarding of reformatted message over N32-f)

content:

application/json:

schema:

$ref: '#/components/schemas/N32fReformattedRspMsg'

headers:

Accept-Encoding:

description: Accept-Encoding, described in IETF RFC 7694

schema:

type: string

Content-Encoding:

description: Content-Encoding, described in IETF RFC 7231

schema:

type: string

3gpp-Sbi-Message-Priority:

description: 3gpp-Sbi-Message-Priority, defined in 3GPP TS 29.500

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ProblemDetailsMsgForwarding'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

[…]

EncodedHttpHeaderValue:

description: HTTP header value or index to the HTTP header value

oneOf:

- type: string

- $ref: '#/components/schemas/IndexToEncryptedValue'

ProblemDetailsMsgForwarding:

allOf:

- $ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

- $ref: '#/components/schemas/AdditionInfoMsgForwarding'

AdditionInfoMsgForwarding:

description: Problem Details extensions for N32-f message forwarding

properties:

suggestedStatusCode:

type: integer

suggestedProblemDetails:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

* * * End of Changes * * * *