3GPP TSG-CT Meeting #102	CP-233111

# Edinburgh, GB, 11th – 12th December 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0072 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling EAS Bundle in EAS Discovery service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP TS 23.558 CR#0247(see S6-231074) agreed in SA6#53, proposed the EAS Bundle feature as part of EEC Register, ECS Service Provisioning and EAS Discovery procedures. 3GPP TS 24.558 CR#0031 addressed EAS Bundle in EEC Register, in this contribution EAS Bundle in EAS Discovery procedures will be addressed. TS 24.558 needs to be aligned with the above SA6 changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update EAS Discovery procedures with EAS Bundle feature. Backward compatibility analysis: This CR does not introduce any backward compatible issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with the stage 2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1, 6.3.5.1, 6.3.5.2.7, 6.3.7, A.3. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 5.3.2.1	Introduction

The service operations defined for Eees_EASDiscovery API are shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the Eees_EASDiscovery API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eees_EASDiscovery_EasDiscRequest | This service operation is used by the EEC to request for one-time EAS discovery information. | EEC |
| Eees_EASDiscovery_Subscribe | This service operation is used by the EEC to request to subscribe to EAS discovery information reporting. | EEC |
| Eees_EASDiscovery_Notify | This service operation is used by the EES to notify a previously subscribed EEC on EAS discovery information. | EES |
| Eees_EASDiscovery_UpdateSubscription | This service operation is used by the EEC to update an existing subscription to EAS discovery information reporting. | EEC |
| Eees_EASDiscovery_Unsubscribe | This service operation is used by the EEC to delete an existing subscription to EAS discovery information reporting. | EEC |

* * * Next Change * * * *

#### 6.3.5.1	General

This clause specifies the application data model supported by the Eees_EASDiscovery API.

Table 6.3.5.1-1 specifies the data types defined specifically for the Eees_EASDiscovery API service.

Table 6.3.5.1-1: Eees_EASDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ACCharacteristics | 6.3.5.2.11 |  |  |
| DiscoveredEas | 6.3.5.2.8 |  |  |
| EasCharacteristics | 6.3.5.2.7 |  |  |
| EASDiscEventIDs | 6.3.5.3.3 |  |  |
| EasDiscoveryFilter | 6.3.5.2.6 |  |  |
| EasDiscoveryNotification | 6.3.5.2.5 |  |  |
| EasDiscoveryReq | 6.3.5.2.2 |  |  |
| EasDiscoveryResp | 6.3.5.2.3 |  |  |
| EasDiscoverySubscription | 6.3.5.2.4 |  |  |
| EasDiscoverySubscriptionPatch | 6.3.5.2.12 |  |  |
| EasDynamicInfoFilter | 6.3.5.2.9 |  |  |
| EasDynamicInfoFilterData | 6.3.5.2.10 |  |  |
| RequestorId | 6.3.5.2.13 |  |  |

Table 6.3.5.1-2 specifies data types re-used by the Eees_EASDiscovery API service.

Table 6.3.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACProfile | clause 6.2.5.2.3 |  |  |
| ACRScenario | 3GPP TS 29.558 [4] |  |  |
| DateTime | 3GPP TS 29.122 [3] |  |  |
| Dnai | 3GPP TS 29.571 [5] |  |  |
| DurationSec | 3GPP TS 29.122 [3] |  |  |
| EASBundleInfo | 3GPP TS 29.558 [4] | Represents EAS bundle information. | EdgeApp_2 |
| EASCategory | 3GPP TS 29.558 [4] | Represents the EAS type. |  |
| EASInstantiationInfo | 3GPP TS 29.558 [4] |  | EdgeApp_2 |
| EASProfile | 3GPP TS 29.558 [4] |  |  |
| EndPoint | 3GPP TS 29.558 [4] |  |  |
| Gpsi | 3GPP TS 29.571 [5] | Used to identify a UE. |  |
| LocationArea5G | 3GPP TS 29.122 [3] |  |  |
| LocationInfo | 3GPP TS 29.122 [3] |  |  |
| RouteToLocation | 3GPP TS 29.571 [5] |  |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [3] | Represents the scheduled communication time. | EdgeApp_2 |
| SupportedFeatures | 3GPP TS 29.571 [5] |  |  |
| TimeWindow | 3GPP TS 29.122 [3] |  |  |
| Uinteger | 3GPP TS 29.571 [5] | Unsigned Integer, i.e. only value 0 and integers above 0 are permissible. | EdgeApp_2 |
| Uri | 3GPP TS 29.122 [3] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] |  |  |

* * * Next Change * * * *

##### 6.3.5.2.7	Type: EasCharacteristics

Table 6.3.5.2.7-1: Definition of type EasCharacteristics

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| easId | string | O | 0..1 | The application identifier of the EAS, e.g. FQDN, URI. |  |
| easProvId | string | O | 0..1 | EAS provider identifier |  |
| appGrpId | string | O | 0..1 | The application group identifier, identifying a group of UEs using the same application service. | EdgeApp_2 |
| easSyncInd | boolean | O | 0..1 | Indicates whether the synchronization between the EASs is required. "true": the EAS synchronization is required. "false" (default): the EAS synchronization is not required. | EdgeApp_2 |
| stdEasType | EASCategory | O | 0..1 | The EAS type with the 3GPP standardized value set. (NOTE 2) |  |
| easType | string | O | 0..1 | The EAS type with flexible value set between the EEC and the EAS. |  |
| easSched | TimeWindow | O | 0..1 | EAS availability schedule |  |
| svcArea | LocationArea5G | O | 0..1 | Service availability area (geographical and topological) |  |
| easSvcContinuity | array(ACRScenario) | O | 1..N | The ACR scenarios required by the EAS for service continuity. If this attribute is not present, then the EAS does not require to support service continuity. |  |
| svcPermLevel | string | O | 0..1 | Service permissions level |  |
| svcFeats | array(string) | O | 1..N | Service features |  |
| easBundleInfo | EASBundleInfo | O | 0..1 | Represents the EAS bundle information. | EdgeApp_2 |
| NOTE 1: Must include at least one optional IE. NOTE 2: The "stdEasType" attribute and the "easType" attribute are mutually exclusive. Either one of them may be provided. The same attribute should be used when this data type is conveyed over the EDGE-1 and EDGE-3 interfaces (i.e. for the Eees_EASRegistration and the Eees_EASDiscovery APIs). |  |  |  |  |  |

* * * Next Change * * * *

### 6.3.7	Feature negotiation

General feature negotiation procedures are described in clause 6.1. Table 6.3.7-1 lists the supported features for Eees_EASDiscovery API.

Table 6.3.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 6.1. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 6.1. This feature requires that the Notification_test_event feature is also supported. |
| 3 | enNB1 | This feature indicates the support of the support of enhancements to this northbound API in Rel-18. |
| 4 | EdgeApp_2 | This feature indicates support of the enhancements for the Enabling Edge Applications. Within this feature the following enhancements are covered: - support of constrained devices for Edge (e.g. support of the EEC with Reduced Capabilities); - support of the EAS instantiation triggering; - support of the EAS synchronization; - support of EAS bundle information; and - obtaining edge load analytics information. |

* * * Next Change * * * *

# A.3	Eees_EASDiscovery API

openapi: 3.0.0

info:

title: Eees_EASDiscovery

description: |

API for EAS Discovery.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.1.0-alpha.3"

externalDocs:

description: >

3GPP TS 24.558 V18.2.0 Enabling Edge Applications; Protocol specification.

url: https://www.3gpp.org/ftp/Specs/archive/24_series/24.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eees-easdiscovery/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.1 of 3GPP TS 24.558

paths:

/subscriptions:

post:

description: Creates a new individual EAS discovery subscription.

operationId: CreateEASDiscSub

tags:

- EAS Discovery Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

responses:

'201':

description: >

Created. A new Individual EAS Discovery Subscription resource was successfully

created.

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoveryNotification'

responses:

'204':

description: No Content (The receipt of the Notification is acknowledged).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

description: >

Updates an existing individual EAS discovery subscription identified by the subscriptionId.

operationId: UpdateIndEASDiscSub

tags:

- Individual EAS Discovery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual EAS discovery subscription resource.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

responses:

'200':

description: >

OK. The individual EAS discovery subscription resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

'204':

description: No Content (updated successfully).

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: >

Deletes an existing individual EAS discovery subscription identified by the subscriptionId.

operationId: DeleteIndEASDiscSub

tags:

- Individual EAS Discovery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual EAS discovery subscription resource.

required: true

schema:

type: string

responses:

'204':

description: >

An individual EAS discovery subscription resource deleted successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: >

Partial update an existing EAS Discovery Subscription resource identified by a

subscriptionId.

operationId: ModifyIndEASDiscSub

tags:

- Individual EAS Discovery Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual EAS discovery subscription resource.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscriptionPatch'

responses:

'200':

description: >

OK (An individual EAS discovery subscription resource updated successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoverySubscription'

'204':

description: No Content (modified successfully).

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/eas-profiles/request-discovery:

post:

description: >

Provides EAS information requested by the service consumer (i.e. EEC, EAS or EES).

operationId: GetEASDiscInfo

tags:

- EAS Profiles (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoveryReq'

responses:

'200':

description: >

OK (The requested EAS discovery information was returned successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/EasDiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

EasDiscoveryReq:

description: EAS discovery request information.

type: object

properties:

requestorId:

$ref: '#/components/schemas/RequestorId'

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

easDiscoveryFilter:

$ref: '#/components/schemas/EasDiscoveryFilter'

eecSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

eesSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EES supports service continuity or not, also indicates which ACR

scenarios are supported by the EES.

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EAS supports service continuity or not, also indicates which ACR

scenarios are supported by the EAS.

locInf:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

easTDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

easSelSupInd:

type: boolean

description: >

Indicates if the EEC requires the EAS selection support from the EES (e.g., for

constrained device). The default value false indicates the EAS selection is not

required from the EES.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

easIntTrigSup:

type: boolean

description: >

Indicates to the EES whether the EAS instantiation triggering should be performed for

the current request. The default value false indicates the EAS instantiation triggering

should not be performed. The true value indicate the EAS instantiation triggering should

be performed.

predictExpTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- requestorId

EasDiscoveryResp:

description: EAS discovery response.

type: object

properties:

discoveredEas:

type: array

items:

$ref: '#/components/schemas/DiscoveredEas'

description: List of EAS discovery information.

easInstInfos:

type: object

additionalProperties:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EASInstantiationInfo'

minProperties: 1

description: >

Contains the EAS instantiation information for each discovered EAS returned within

the discoveredEas attribute.

edgeLoadAnalytics:

type: object

additionalProperties:

$ref: '#/components/schemas/EdgeLoadAnalytic'

minProperties: 1

description: >

Contains the statistical analytics data and predictive analytics data for each

discovered application server. The key of the map shall be the EAS ID to which

the provided analytics data within the map value relates.

required:

- discoveredEas

EasDiscoverySubscription:

description: Represents an Individual EAS Discovery Subscription resource.

type: object

properties:

eecId:

type: string

description: Represents a unique identifier of the EEC.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

easEventType:

$ref: '#/components/schemas/EASDiscEventIDs'

easDiscoveryFilter:

$ref: '#/components/schemas/EasDiscoveryFilter'

easDynInfoFilter:

$ref: '#/components/schemas/EasDynamicInfoFilter'

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by Subscriber to request the EES to send a test notification. Set to false

or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

easIntTrigSup:

type: boolean

description: >

Indicates to the EES whether the EAS instantiation triggering should be performed for

the current request. The default value false indicates the EAS instantiation triggering

should not be performed. The true value indicate the EAS instantiation triggering should

be performed.

eecTriggerRequest:

type: boolean

description: >

Indicates to the EES whether the application triggering is required by the EEC.

Default value false indicates the application triggering is not required.

required:

- eecId

- easEventType

EasDiscoveryNotification:

description: Notification of EAS discovery information.

type: object

properties:

subId:

type: string

description: >

Identifier of the individual service provisioning subscription for which the service

provisioning notification is delivered.

eventType:

$ref: '#/components/schemas/EASDiscEventIDs'

discoveredEas:

type: array

items:

$ref: '#/components/schemas/DiscoveredEas'

minItems: 1

description: List of EAS discovery information.

easInstInfos:

type: object

additionalProperties:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EASInstantiationInfo'

minProperties: 1

description: >

Contains the EAS instantiation information for each discovered EAS returned within

the "discoveredEas" attribute.

edgeLoadAnalytics:

type: object

additionalProperties:

$ref: '#/components/schemas/EdgeLoadAnalytic'

minProperties: 1

description: >

Contains the statistical analytics data and predictive analytics data for each

discovered application server. The key of the map shall be the EAS ID to which

the provided analytics data within the map value relates.

required:

- subId

- eventType

- discoveredEas

EasDiscoveryFilter:

description: Represents the EAS characteristics.

type: object

properties:

acChars:

type: array

items:

$ref: '#/components/schemas/ACCharacteristics'

minItems: 1

description: AC description for which an EAS is needed.

easChars:

type: array

items:

$ref: '#/components/schemas/EasCharacteristics'

minItems: 1

description: Required EAS chararcteristics.

EasCharacteristics:

description: Represents the EAS chararcteristics.

type: object

properties:

easId:

type: string

description: EAS application identifier.

appGrpId:

type: string

description: >

Application group identifier, identifying a group of UEs using the same

application service.

easSyncInd:

type: boolean

description: >

Indicates whether the synchronization between the EASs is required. The

default value false indicates the EAS synchronization is not required.

easProvId:

type: string

description: EAS provider identifier.

stdEasType:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASCategory'

easType:

type: string

description: EAS type with the flexible value set.

easSched:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

svcArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

svcPermLevel:

type: string

description: Service permissions level.

svcFeats:

type: array

items:

type: string

minItems: 1

description: Service features.

easBundleInfo:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

not:

required: [stdEasType, easType]

DiscoveredEas:

description: Represents an EAS discovery information.

type: object

properties:

eas:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASProfile'

lifeTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- eas

EasDynamicInfoFilter:

description: Represents EAS dynamic information changes filter.

type: object

properties:

dynInfoFilter:

type: array

items:

$ref: '#/components/schemas/EasDynamicInfoFilterData'

minItems: 1

description: List of EAS dynamic information required by the EEC per EAS.

required:

- dynInfoFilter

EasDynamicInfoFilterData:

description: Represents an EAS dynamic information.

type: object

properties:

eecId:

type: string

description: The application identifier of the EAS, e.g. FQDN, URI.

easStatus:

type: boolean

description: Notify if EAS status changed.

easAcIds:

type: boolean

description: Notify if list of AC identifiers changed.

easDesc:

type: boolean

description: Notify if EAS description changed.

easPt:

type: boolean

description: Notify if EAS endpoint changed.

easEndPoint:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

easFeature:

type: boolean

description: Notify if EAS feature changed.

easSchedule:

type: boolean

description: Notify if EAS schedule changed.

svcArea:

type: boolean

description: Notify if EAS service area changed.

svcKpi:

type: boolean

description: Notify if EAS KPIs changed.

svcCont:

type: boolean

description: Notify if EAS supported ACR changed.

required:

- eecId

ACCharacteristics:

description: Represents EAS dynamic information changes filter.

type: object

properties:

acProf:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

required:

- acProf

EASDiscEventIDs:

anyOf:

- type: string

enum:

- EAS_AVAILABILITY_CHANGE

- EAS_DYNAMIC_INFO_CHANGE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: >

Possible values are

- EAS_AVAILABILITY_CHANGE: Represents the EAS availability change event.

- EAS_DYNAMIC_INFO_CHANGE: Represents the EAS dynamic information change event.

EasDiscoverySubscriptionPatch:

description: Represents an Individual EAS Discovery Subscription resource.

type: object

properties:

easDiscoveryFilter:

$ref: '#/components/schemas/EasDiscoveryFilter'

easDynInfoFilter:

$ref: '#/components/schemas/EasDynamicInfoFilter'

easSvcContinuity:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

easEventType:

$ref: '#/components/schemas/EASDiscEventIDs'

RequestorId:

description: Represents identifier of the requestor.

type: object

properties:

eesId:

type: string

description: The identifier of the EES (e.g. S-EES).

easId:

type: string

description: The application identifier of the EAS (e.g. S-EAS), e.g. FQDN, URI.

eecId:

type: string

description: The identifier of the EEC.

oneOf:

- required: [eesId]

- required: [easId]

- required: [eecId]

EdgeLoadAnalytic:

description: >

Contains the statistical analytics data and predictive analytics data for each

discovered application server.

type: object

properties:

easId:

type: string

description: The application identifier of the EAS, e.g. FQDN, URI.

predictData:

$ref: '#/components/schemas/PredictiveData'

statisticData:

$ref: '#/components/schemas/StatisticalData'

required:

- easId

PredictiveData:

description: >

Contains the predictive analytics data for each discovered EAS service status

(e.g. EAS schedule, EAS status) change.

type: object

properties:

scheds:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

status:

type: string

description: Indicates the EAS status (e.g. Enabled, Disabled etc.).

StatisticalData:

description: >

Contains the statistical analytics data (e.g. number of times the client received

expected performance from the EAS).

type: object

properties:

numRecPerf:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

* * * End of Changes * * * *