3GPP TSG-CT Meeting #102	CP-233112

# Edinburgh, GB, 11th – 12th December 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.558 | CR | 0071 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling EAS Bundle in ECS Service Provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 3GPP TS 23.558 CR#0247(see S6-231074) agreed in SA6#53, proposed the EAS Bundle feature as part of EEC Register, ECS Service Provisioning and EAS Discovery procedures. 3GPP TS 24.558 CR#0031 addressed EAS Bundle in EEC Register, in this contribution EAS Bundle in ECS Service Provisioning will be addressed. TS 24.558 needs to be aligned with the above SA6 changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update ECS Service provisioning service with EAS Bundle feature. Backward compatibility analysis: This CR does not introduce any backward compatible issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with the stage 2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.1, 8.1.5.1, 8.1.5.2.9, 8.1.7, B.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.2.2.1	Introduction

The service operation defined for Eecs_ServiceProvisioning API is shown in the table 7.2.2.1-1.

Table 7.2.2.1-1: Operations of the Eecs_ServiceProvisioning API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Eecs_ServiceProvisioning_Request | This service operation is used by the EEC to request for one-time service provisioning information. | EEC |
| Eecs_ServiceProvisioning_Subscribe | This service operation is used by the EEC to subscribe to ECS for reporting of service provisioning information. | EEC |
| Eecs_ServiceProvisioning_Notify | This service operation is used by the ECS to notify the EEC about the service provisioning information. | ECS |
| Eecs_ServiceProvisioning_UpdateSubscription | This service operation is used by the EEC to update its subscription at ECS for reporting of service provisioning information. | EEC |
| Eecs_ServiceProvisioning_Unsubscribe | This service operation is used by the EEC to remove its subscription from ECS for reporting of service provisioning information. | EEC |

* * * Next Change * * * *

#### 8.1.5.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 of 3GPP TS 29.558 [4] apply to this API.

Table 8.1.5.1-1 specifies the data types defined specifically for the Eecs_ServiceProvisioning API service.

Table 8.1.5.1-1: Eecs_ServiceProvisioning API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ConnectivityInfo | 8.1.5.2.5 |  |  |
| ECSServProvReq | 8.1.5.2.2 |  |  |
| ECSServProvResp | 8.1.5.2.3 |  |  |
| ECSServProvSubscription | 8.1.5.2.4 | Represents the service provisioning subscription. |  |
| ECSServProvSubscriptionPatch | 8.1.5.2.10 |  |  |
| EDNConfigInfo | 8.1.5.2.7 |  |  |
| EDNConInfo | 8.1.5.2.8 |  |  |
| EESInfo | 8.1.5.2.9 |  |  |
| ServProvNotification | 8.1.5.2.6 | Service provisioning information notification from ECS to EEC. |  |

Table 8.1.5.1-2 specifies data types re-used by the Eecs_ServiceProvisioning API service.

Table 8.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ACProfile | clause 6.2.5.2.3 |  |  |
| ACRScenario | 3GPP TS 29.558 [4] |  |  |
| DateTime | 3GPP TS 29.122 [3] |  |  |
| Dnai | 3GPP TS 29.571 [5] |  |  |
| Dnn | 3GPP TS 29.571 [5] |  |  |
| EASBundleInfo | 3GPP TS 29.558 [4] | Represents EAS bundle information. | EdgeApp_2 |
| EASInstantiationInfo | 3GPP TS 29.558 [4] |  |  |
| EndPoint | 3GPP TS 29.558 [4] |  |  |
| Gpsi | 3GPP TS 29.571 [5] | Used to identify the UE. |  |
| LocationArea5G | 3GPP TS 29.122 [3] |  |  |
| LocationInfo | 3GPP TS 29.122 [3] | The location information related to the UE. |  |
| PlmnIdNid | 3GPP TS 29.571 [5] | Identifies the network: PLMN Identifier or the SNPN Identifier (the PLMN Identifier and the NID). |  |
| Snssai | 3GPP TS 29.571 [5] |  |  |
| SupportedFeatures | 3GPP TS 29.571 [5] | Used to negotiate the applicability of optional features. |  |
| Uri | 3GPP TS 29.122 [3] |  |  |
| WebsockNotifConfig | 3GPP TS 29.122 [3] |  |  |

* * * Next Change * * * *

##### 8.1.5.2.9	Type: EESInfo

Table 8.1.5.2.9-1: Definition of type EESInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eesId | string | M | 1 | The identifier of the EES |  |
| endPt | EndPoint | M | 1 | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |  |
| easIds | array(string) | O | 1..N | The list of application identifiers of the Edge Application Servers registered or expected to be registered with the EES, e.g. FQDN, URI. |  |
| ecspInfo | string | O | 0..1 | String representing the EES Provider (such as ECSP Information) |  |
| svcArea | LocationArea5G | O | 0..1 | The list of geographical and topological areas that the EES serves. EECs in the UE that are outside the area shall not be served. |  |
| dnais | array(Dnai) | O | 0..1 | Represents list of Data network access identifier |  |
| eesSvcContSupp | array(ACRScenario) | O | 0..1 | The ACR scenarios supported by the EES for service continuity. If this attribute is not present, then the EEC does not support service continuity. |  |
| eecRegConf | boolean | M | 1 | Indicates whether the EEC is required to register on the EES to use edge services or not |  |
| easInstInfos | array(EASInstantiationInfo) | O | 1..N | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet) | EdgeApp_2 |
| easBundleInfo | array(EASBundleInfo) | O | 1..N | Represents a list of EAS bundles to which the EAS belongs.(NOTE) | EdgeApp_2 |
| NOTE: In case of ECS Service Provisioning response, the "easBdlReqs" and "mainEasId" attributes shall not be present within the "easBundleInfo" attribute. |  |  |  |  |  |

* * * Next Change * * * *

### 8.1.7	Feature negotiation

General feature negotiation procedures are described in clause 7.8 of 3GPP TS 29.558 [4]. Table 8.1.7-1 lists the supported features for Eecs_ServiceProvisioning API.

Table 8.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | Notification_test_event | Testing of notification connection is supported according to clause 7.6 of 3GPP TS 29.558 [4]. |
| 2 | Notification_websocket | The delivery of notifications over Websocket is supported according to clause 7.6 of 3GPP TS 29.558 [4]. This feature requires that the Notification_test_event feature is also supported. |
| 3 | enNB1 | This feature indicates the support of enhancements to this northbound API in Rel-18. |
| 4 | EdgeApp_2 | This feature indicates support of the enhancements for the Enabling Edge Applications. Within this feature the following enhancements are covered: - support of enhanced EES service differentiation; - support of Edge computing in SNPN; - support of EAS bundle information; and - support of EAS instantiation. |

* * * Next Change * * * *

# B.1	Eecs_ServiceProvisioning

openapi: 3.0.0

info:

title: Eecs_ServiceProvisioning

version: "1.1.0-alpha.3"

description: |

API for ECS Service Provisioning.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 24.558 V18.2.0 Enabling Edge Applications; Protocol specification.

url: https://www.3gpp.org/ftp/Specs/archive/24_series/24.558/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/eecs-serviceprovisioning/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.558

paths:

/subscriptions:

post:

description: >

Creates a new subscription in ECS in order to be notified of provisioning data

changes of interest.

operationId: CreateServProvSub

tags:

- Service Provisioning Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ECSServProvSubscription'

callbacks:

notificationDestination:

'{request.body#/notificationDestination}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServProvNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: >

Individual ECS Service Provisioning Subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ECSServProvSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

description: >

Updates an existing individual service provisioning subscription identified

by the subscriptionId.

operationId: UpdateIndServProvSub

tags:

- Individual Service Provisioning Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual service provisioning subscription.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ECSServProvSubscription'

responses:

'200':

description: >

OK (The individual service provisioning subscription matching the subscriptionId

was modified successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/ECSServProvSubscription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: >

Deletes an existing individual service provisioning subscription identified by

the subscriptionId.

operationId: DeleteIndServProvSub

tags:

- Individual Service Provisioning Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual service provisioning subscription.

required: true

schema:

type: string

responses:

'204':

description: >

The individual service provisioning subscription matching the subscriptionId is

deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: >

Partially updates an existing individual service provisioning subscription identified

by the subscriptionId.

operationId: ModifyIndServProvSub

tags:

- Individual Service Provisioning Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual service provisioning subscription.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ECSServProvSubscriptionPatch'

responses:

'200':

description: >

OK (The individual service provisioning subscription matching the subscriptionId

was modified successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/ECSServProvSubscription'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request:

post:

summary: Request service provisioning information.

operationId: RequestServProv

tags:

- Request Service Provisioning

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ECSServProvReq'

responses:

'200':

description: >

OK (The requested service provisioning information was returned successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/ECSServProvResp'

'204':

description: >

No Content (The requested service provisioning information does not exist).

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ECSServProvReq:

description: ECS service provisioning request information.

type: object

properties:

eecId:

type: string

description: Represents a unique identifier of the EEC.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

description: Information about services the EEC wants to connect to.

eecSvcContSupp:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which

ACR scenarios are supported by the EEC.

connInfo:

type: array

items:

$ref: '#/components/schemas/ConnectivityInfo'

description: List of connectivity information for the UE.

locInf:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

ecspIds:

type: array

items:

type: string

minItems: 1

description: Indicates to the ECS which EES providers are preferred by the EEC.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eecId

ECSServProvResp:

description: ECS service provisioning response information.

type: object

properties:

ednCnfgInfo:

type: array

items:

$ref: '#/components/schemas/EDNConfigInfo'

minItems: 1

description: List of EDN configuration information.

required:

- ednCnfgInfo

ECSServProvSubscription:

description: Represents an individual service provisioning subscription resource.

type: object

properties:

eecId:

type: string

description: Represents a unique identifier of the EEC.

ueId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

description: Information about services the EEC wants to connect to.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

eecSvcContSupp:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which

ACR scenarios are supported by the EEC.

connInfo:

type: array

items:

$ref: '#/components/schemas/ConnectivityInfo'

description: List of connectivity information for the UE.

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by Subscriber to request the ECS to send a test notification. Set to

false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

ecspIds:

type: array

items:

type: string

minItems: 1

description: Indicates to the ECS which EES providers are preferred by the EEC.

eecTriggerRequest:

type: boolean

description: >

Indicates to the ECS, whether the application triggering is required by the EEC.

Default value false indicates the application triggering is not required.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eecId

ServProvNotification:

description: Represents notification information of a service provisioning Event.

type: object

properties:

subId:

type: string

description: >

Identifier of the individual service provisioning subscription for which the service

provisioning notification is delivered.

ednCnfgInfo:

type: array

items:

$ref: '#/components/schemas/EDNConfigInfo'

minItems: 1

description: List of EDN configuration information.

required:

- subId

- ednCnfgInfo

ConnectivityInfo:

description: Represents the connectivity information for the UE.

type: object

properties:

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

ssId:

type: string

description: Identifies the SSID of the access point to which the UE is attached.

EDNConfigInfo:

description: Represents the EDN configuration information.

type: object

properties:

ednConInfo:

$ref: '#/components/schemas/EDNConInfo'

eess:

type: array

items:

$ref: '#/components/schemas/EESInfo'

minItems: 1

description: Contains the list of EESs of the EDN.

lifeTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

required:

- ednConInfo

- eess

EDNConInfo:

description: Represents an EDN connection information.

type: object

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

ednTopoSrvArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

EESInfo:

description: Represents EES information.

type: object

properties:

eesId:

type: string

description: Identity of the EES.

endPt:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

easIds:

type: array

items:

type: string

description: >

Application identities of the Edge Application Servers registered

with the EES.

ecspInfo:

type: string

description: Represents an ECSP Information.

svcArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

description: Represents list of Data network access identifiers.

eesSvcContSupp:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EES supports service continuity or not, also indicates which ACR

scenarios are supported by the EES.

eecRegConf:

type: boolean

description: >

Indicates whether the EEC is required to register on the EES to use edge services

or not.

easInstInfos:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/EASInstantiationInfo'

minItems: 1

description: >

The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be

instantiated yet).

easBundleInfo:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASBundleInfo'

required:

- eesId

- eecRegConf

ECSServProvSubscriptionPatch:

description: >

Represents modifications to an individual service provisioning subscription resource.

type: object

properties:

acProfs:

type: array

items:

$ref: 'TS24558_Eees_EECRegistration.yaml#/components/schemas/ACProfile'

description: Information about services the EEC wants to connect to.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

eecSvcContSupp:

type: array

items:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario'

description: >

Indicates if the EEC supports service continuity or not, also indicates which ACR

scenarios are supported by the EEC.

connInfo:

type: array

items:

$ref: '#/components/schemas/ConnectivityInfo'

description: List of connectivity information for the UE.

* * * End of Changes * * * *