3GPP TSG-CT Meeting #102	CP-233116

# Edinburgh, GB, 11th – 12th December 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0432 | rev | 6 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding RSC in the U2N relay discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2023-11-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The below has been specified in TS 33.503 v17.4.0 clause 6.1.3.2.2.1: 1. Announcing UE sends a Discovery Request message containing the Restricted ProSe Application User ID (RPAUID) to the 5G DDNMF in its HPLMN in order to get the ProSe Code to announce and to get the associated security material. In addition, the Announcing UE shall include its PC5 UE security capability that contains the list of supported ciphering algorithms by the UE in the Discovery Request message. For 5G ProSe UE-to-Network Relay discovery, the 5G ProSe UE-to-Network Relay plays the role of the Announcing UE and sends a Relay Discovery Key Request instead of a Discovery Request. The Relay Discovery Key Request message includes the Relay Service Code (RSC) and the 5G ProSe UE-to-Network Relay's PC5 security capability. 4. The 5G DDNMF in the HPLMN of the Announcing UE returns the ProSe Restricted Code and the corresponding Code-Sending Security Parameters, along with the CURRENT_TIME and MAX_OFFSET parameters. The Code-Sending Security Parameters provide the necessary information for the Announcing UE to protect the transmission of the ProSe Restricted Code and are stored with the ProSe Restricted Code. The Announcing UE takes the same actions with CURRENT_TIME and MAX_OFFSET as described for the Announcing UE in step 4 of clause 6.1.3.1 of the present document. The 5G DDNMF in the HPLMN of the Announcing UE shall include the chosen PC5 ciphering algorithm in the Discovery Response message. The 5G DDNMF determines the chosen PC5 ciphering algorithm based on the ProSe Restricted Code and the received PC5 UE security capability in step 1. The UE stores the chosen PC5 ciphering algorithm together with the ProSe Restricted Code. In addition, the 5G DDNMF in the HPLMN of the Announcing UE may associate the ProSe Restricted Code with the PC5 security policies and include the PC5 security policies in the Discovery Response message. For 5G ProSe UE-to-Network Relay discovery, a Relay Discovery Key Response is used instead of the Discovery Response, and the RSC is used instead of the ProSe Restricted Code. It can be observed that the RSC is used in the interaction between UE and DDNMF. The same for model A and model B per remote UE and relay UE. Besides, there is the following so the RSC is also needed over PC8 interface. 5G ProSe UE-to-Network Relay discovery is different from 5G ProSe Restricted Direct Discovery. In 5G ProSe UE-to-Network Relay discovery, the discovery security materials are provided by the PKMF for RSC(s) representing user-plane based security procedure, and by the DDNMF or the PCF for RSC(s) with Control Plane Security Indicator set representing control-plane based security procedure. The 5G ProSe UE-to-Network Relay discovery procedures described in clause 6.1.3.2.2.1 and clause 6.1.3.2.2.2 apply with adjustment when 5G DDNMF or 5G PKMF is used for 5G ProSe UE-to-Network Relay discovery. This CR proposes to add the RSC over PC3a and PC8 as a madatory IE, as requested by SA3 in their LS (S3-234992). Then the network provides the RSC specific security parameters, instead of a list of RSCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the RSC in the U2N relay discovery security material provision procedure as a mandatory IE. 2. Remove the list of the RSC specific security parameters from the network to the UE. Only one RSC specific security parameters. 3. Add XML schema for U2N relay security parameters over PC3a and PC8. Backward compatibility analysis: Not Backward compatible. The legacy network cannot decode the RSC received from the UE implementing this CR. The network implementing this CR expects the mandatory RSC but cannot receive it from the legacy UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 requirement is not satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.10.2.2.2, 8.2.10.2.2.3, 8.2.10.2.2.4, 8.2.12.2.2.2, 8.2.12.2.2.3, 8.2.12.2.2.4, 8.2.12.2.2.4, 10.5.3, 10.6.3, 10.6.4.2, 10.6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | Rev 4: change the BC analysis from not backward compatible to backward compatible. Rev 5: change the RSC as a mandatory IE and mark this CR as not backward compatible. Rev 6: correct the CR number. |  |  |  |  |  |  |  |  |

***** First change *****

8.2.10.2.2.2	5G ProSe UE-to-network relay discovery security parameters request procedure initiation

The UE shall initiate the 5G ProSe UE-to-network relay discovery security parameters request procedure:

a)	if the UE is authorized to act as a 5G ProSe remote UE and uses the security procedure over user plane as specified in 3GPP TS 33.503 [34]:

1)	when the UE has no 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE and the UE is in NG-RAN coverage; or

2)	after expiration of timer T5094, when in NG-RAN coverage or when entering NG-RAN coverage; or

b)	if the UE is authorized to act as a 5G ProSe UE-to-network relay UE and uses the security procedure over user plane as specified in 3GPP TS 33.503 [34]:

1)	when the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE and the UE is in NG-RAN coverage; or

2)	after expiration of timer T5095, when in NG-RAN coverage or when entering NG-RAN coverage.

The UE shall initiate the 5G ProSe UE-to-network relay discovery security parameters request procedure by sending a PROSE_SECURITY_PARAM_REQUEST message with the <UNR-discovery-security-parameters-request> element. In the <UNR-discovery-security-parameters-request> element, the UE:

a)	shall include a new transaction ID;

b)	shall indicate whether the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE, the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE or both;

c)	shall include the PC5 UE security capabilities indicating ciphering algorithms supported by the UE;

d)	if the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE, may include a list of PLMN identities of the visited PLMNs;

e)	may indicate the requested model indicating the model of the 5G ProSe UE-to-network relay discovery over PC5 interface for which security parameters are requested, set to "model A" or "model B"; and

f)	shall include the relay service code indicating the connectivity service to be discovered.

NOTE:	If the requested model is not included in the PROSE_SECURITY_PARAM_REQUEST message, security parameters are requested for both model A and model B of the 5G ProSe UE-to-network relay discovery over PC5 interface.

Figure 8.2.10.2.2.2.1 illustrates the interaction of the UE and the 5G PKMF in the 5G ProSe UE-to-network relay discovery security parameters request procedure.

Figure 8.2.10.2.2.2.1: 5G ProSe UE-to-network relay discovery security parameters request procedure

***** Next change *****

8.2.10.2.2.3	5G ProSe UE-to-network relay discovery security parameters request procedure accepted by the 5G PKMF

Upon receiving a PROSE_SECURITY_PARAM_REQUEST message with the <UNR-discovery-security-parameters-request> element, if:

a)	the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE only and the PROSE_SECURITY_PARAM_REQUEST message is received over a TLS tunnel established by a UE authorized to act as a 5G ProSe remote UE;

b)	the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE only and the PROSE_SECURITY_PARAM_REQUEST message is received over a TLS tunnel established by a UE authorized to act as a 5G ProSe UE-to-network relay UE; or

c)	the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE and the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE and the PROSE_SECURITY_PARAM_REQUEST message is received over a TLS tunnel established by a UE authorized to act as a 5G ProSe UE-to-network relay UE and authorized to act as a 5G ProSe remote UE;

the 5G PKMF shall send a PROSE_SECURITY_PARAM_RESPONSE message containing a <UNR-discovery-security-parameters-accept> element. In the <UNR-discovery-security-parameters-accept> element, the 5G PKMF:

a)	shall include the transaction ID set to the value of the transaction ID received in the PROSE_SECURITY_PARAM_REQUEST message;

b)	if the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE:

1)	shall include the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE. In the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE, the 5G PKMF:

A)	shall include the expiration timer of the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE; and

B)	for the received relay service code for which the UE is authorized to act as a 5G ProSe remote UE:

i)	if the requested model is not indicated in the PROSE_SECURITY_PARAM_REQUEST message or is set to "model A", may include the code-receiving security parameters for model A containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask;

ii)	if the requested model is not indicated in the PROSE_SECURITY_PARAM_REQUEST message or is set to "model B", may include the code-receiving security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask and the code-sending security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask; and

iii)	shall include the selected ciphering algorithm; and

2)	may include the PC5 security policies for the received relay service code for 5G ProSe remote UE. In the PC5 security policies for the received relay service code for 5G ProSe remote UE, the 5G PKMF:

A)	for the received relay service code for which the UE is authorized to act as a 5G ProSe remote UE:

i)	shall include the PC5 security policies;

c)	if the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE:

1)	shall include the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE. In the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE, the 5G PKMF:

A)	shall include the expiration timer of the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE; and

B)	for the received relay service code for which the UE is authorized to act as a 5G ProSe UE-to-network relay UE:

i)	if the requested model is not indicated in the PROSE_SECURITY_PARAM_REQUEST message or is set to "model A", may include the code-sending security parameters for model A containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask;

ii)	if the requested model is not indicated in the PROSE_SECURITY_PARAM_REQUEST message or is set to "model B", may include the code-receiving security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask and the code-sending security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask; and

iii)	shall include the selected ciphering algorithm; and

2)	may include the PC5 security policies for the received relay service code for 5G ProSe UE-to-network relay UE. In the PC5 security policies for the received relay service code for 5G ProSe UE-to-network relay UE, the 5G PKMF:

A)	for the received relay service code for which the UE is authorized to act as a 5G ProSe UE-to-network relay UE:

i)	shall include the PC5 security policies; and

d)	shall include the current time set to the current UTC-based time at the 5G PKMF and the max offset.

If the UE requests the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE, the UE identity in the PROSE_SECURITY_PARAM_REQUEST message indicates a UE authorized to act as a 5G ProSe remote UE and the PROSE_SECURITY_PARAM_REQUEST message contains a list of PLMN identities of the visited PLMNs, the 5G PKMF shall include in the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE any information received from 5G PKMFs of potential 5G ProSe UE-to-network relay UEs which can serve the UE.

***** Next change *****

8.2.10.2.2.4	5G ProSe UE-to-network relay discovery security parameters request procedure completion by the UE

Upon receipt of the PROSE_SECURITY_PARAM_RESPONSE message with the <UNR-discovery-security-parameters-accept>, if the transaction ID contained in the <UNR-discovery-security-parameters-accept> element matches the value sent by the UE in a PROSE_SECURITY_PARAM_REQUEST message with the <UNR-discovery-security-parameters-request> element, the UE:

a)	if the PROSE_SECURITY_PARAM_RESPONSE message contains the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE:

1)	shall store the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE, shall stop timer T5094, if running and shall start timer T5094 with the value of the expiration timer indicated in the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE; and

2)	if the PC5 security policies for the relay service code for 5G ProSe remote UE are received, shall store the PC5 security policies for the relay service code for 5G ProSe remote UE;

b)	if the PROSE_SECURITY_PARAM_RESPONSE message contains the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE:

1)	shall store the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE, shall stop timer T5095, if running and shall start timer T5081 with the value of the expiration timer indicated in the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE; and

2)	if the PC5 security policies for the relay service code for 5G ProSe UE-to-network relay UE are received, shall store the PC5 security policies for the relay service code for 5G ProSe UE-to-network relay UE; and

c)	shall set a ProSe clock (see 3GPP TS 33.503 [34]) to the value of the received current time parameter and store the received max offset.

***** Next change *****

8.2.12.2.2.2	5G ProSe UE-to-network relay discovery security material request procedure initiation

The UE shall initiate the 5G ProSe UE-to-network relay discovery security material request procedure:

a)	if the UE is authorized to act as a 5G ProSe remote UE and uses the security procedure over control plane as specified in 3GPP TS 33.503 [34]:

1)	when the UE has no 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE and the UE is in NG-RAN coverage; or

2)	after expiration of timer T5078, when in NG-RAN coverage or when entering NG-RAN coverage; or

b)	if the UE is authorized to act as a 5G ProSe UE-to-network relay UE and uses the security procedure over control plane as specified in 3GPP TS 33.503 [34]:

1)	when the UE has no 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE and the UE is in NG-RAN coverage; or

2)	after expiration of timer T5079, when in NG-RAN coverage or when entering NG-RAN coverage.

The UE shall initiate the 5G ProSe UE-to-network relay discovery security material request procedure by sending a PROSE_SECURITY_MATERIAL_REQUEST message with the <UNR-discovery-security-parameters-request> element. In the <UNR-discovery-security-parameters-request> element, the UE:

a)	shall include a new transaction ID;

b)	shall indicate whether the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE, the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE or both;

c)	shall include the PC5 UE security capabilities indicating ciphering algorithms supported by the UE;

d)	if the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE, may include a list of PLMN identities of the visited PLMNs;

e)	may indicate the requested model indicating the model of the 5G ProSe UE-to-network relay discovery over PC5 interface for which security parameters are requested, set to "model A" or "model B"; and

f)	shall include the relay service code indicating the connectivity service to be discovered.

NOTE:	If the requested model is not included in the PROSE_SECURITY_MATERIAL_REQUEST message, security parameters are requested for both model A and model B of the 5G ProSe UE-to-network relay discovery over PC5 interface.

Figure 8.2.12.2.2.2.1 illustrates the interaction of the UE and the 5G DDNMF in the 5G ProSe UE-to-network relay discovery security material request procedure.

Figure 8.2.12.2.2.2.1: 5G ProSe UE-to-network relay discovery security material request procedure

***** Next change *****

8.2.12.2.2.3	5G ProSe UE-to-network relay discovery security material request procedure accepted by the 5G DDNMF

Upon receiving a PROSE_SECURITY_MATERIAL_REQUEST message with the <UNR-discovery-security-parameters-request> element, if:

a)	the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE only and the PROSE_SECURITY_MATERIAL_REQUEST message is received over a TLS tunnel established by a UE authorized to act as a 5G ProSe remote UE;

b)	the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE only and the PROSE_SECURITY_MATERIAL_REQUEST message is received over a TLS tunnel established by a UE authorized to act as a 5G ProSe UE-to-network relay UE; or

c)	the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE and the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE and the PROSE_SECURITY_MATERIAL_REQUEST message is received over a TLS tunnel established by a UE authorized to act as a 5G ProSe UE-to-network relay UE and authorized to act as a 5G ProSe remote UE;

the 5G DDNMF shall send a PROSE_SECURITY_MATERIAL_RESPONSE message containing a <UNR-discovery-security-parameters-accept> element. In the <UNR-discovery-security-parameters-accept> element, the 5G DDNMF:

a)	shall include the transaction ID set to the value of the transaction ID received in the PROSE_SECURITY_MATERIAL_REQUEST message;

b)	if the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE:

1)	shall include the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE. In the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE, the 5G DDNMF:

A)	shall include the expiration timer of the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE; and

B)	for the received relay service code for which the UE is authorized to act as a 5G ProSe remote UE:

i)	if the requested model is not indicated in the PROSE_SECURITY_MATERIAL_REQUEST message or is set to "model A", may include the code-receiving security parameters for model A containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask;

ii)	if the requested model is not indicated in the PROSE_SECURITY_MATERIAL_REQUEST message or is set to "model B", may include the code-receiving security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask and the code-sending security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask; and

iii)	shall include the selected ciphering algorithm; and

2)	may include the PC5 security policies for the received relay service code for 5G ProSe remote UE. In the PC5 security policies per relay service code for 5G ProSe remote UE, the 5G DDNMF:

A)	for the received relay service code for which the UE is authorized to act as a 5G ProSe remote UE:

i)	shall include the PC5 security policies;

c)	if the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE:

1)	shall include the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE. In the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE, the 5G DDNMF:

A)	shall include the expiration timer of the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE; and

B)	for the received relay service code for which the UE is authorized to act as a 5G ProSe UE-to-network relay UE:

i)	if the requested model is not indicated in the PROSE_SECURITY_MATERIAL_REQUEST message or is set to "model A", may include the code-sending security parameters for model A containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask;

ii)	if the requested model is not indicated in the PROSE_SECURITY_MATERIAL_REQUEST message or is set to "model B", may include the code-receiving security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask and the code-sending security parameters for model B containing one or more of DUSK, DUIK and DUCK with associated encrypted bitmask; and

iii)	shall include the selected ciphering algorithm; and

2)	may include the PC5 security policies for the received relay service code for 5G ProSe UE-to-network relay UE. In the PC5 security policies for the received relay service code for 5G ProSe UE-to-network relay UE, the 5G DDNMF:

A)	for the received relay service code for which the UE is authorized to act as a 5G ProSe UE-to-network relay UE:

i)	shall include the PC5 security policies; and

d)	shall include the current time set to the current UTC-based time at the 5G DDNMF and the max offset.

If the UE requests the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE, the UE identity in the PROSE_SECURITY_MATERIAL_REQUEST message indicates a UE authorized to act as a 5G ProSe remote UE and the PROSE_SECURITY_MATERIAL_REQUEST message contains a list of PLMN identities of the visited PLMNs, the 5G DDNMF shall include in the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE any information received from 5G DDNMFs of potential 5G ProSe UE-to-network relay UEs which can serve the UE.

***** Next change *****

8.2.12.2.2.4	5G ProSe UE-to-network relay discovery security material request procedure completion by the UE

Upon receipt of the PROSE_SECURITY_MATERIAL_RESPONSE message with the <UNR-discovery-security-parameters-accept>, if the transaction ID contained in the <UNR-discovery-security-parameters-accept> element matches the value sent by the UE in a PROSE_SECURITY_MATERIAL_REQUEST message with the <UNR-discovery-security-parameters-request> element, the UE:

a)	if the PROSE_SECURITY_MATERIAL_RESPONSE message contains the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE:

1)	shall store the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE, shall stop timer T5078, if running and shall start timer T5078 with the value of the expiration timer indicated in the 5G ProSe UE-to-network relay discovery security material for 5G ProSe remote UE; and

2)	if the PC5 security policies for the relay service code for 5G ProSe remote UE are received, shall store the PC5 security policies for the relay service code for 5G ProSe remote UE;

b)	if the PROSE_SECURITY_MATERIAL_RESPONSE message contains the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE:

1)	shall store the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE, shall stop timer T5079, if running and shall start timer T5079 with the value of the expiration timer indicated in the 5G ProSe UE-to-network relay discovery security material for 5G ProSe UE-to-network relay UE; and

2)	if the PC5 security policies for the relay service code for 5G ProSe UE-to-network relay UE are received, shall store the PC5 security policies for the relay service code for 5G ProSe UE-to-network relay UE; and

c)	shall set a ProSe clock (see 3GPP TS 33.503 [34]) to the value of the received current time parameter and store the received max offset.

***** Next change *****

10.5.3	XML schema

Implementations in compliance with the present document shall implement the XML schema defined below for messages used in 5G ProSe direct discovery procedures over PC3a interface.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns="urn:3GPP:ns:5GProSe:Discovery:2021"

elementFormDefault="qualified"

targetNamespace="urn:3GPP:ns:5GProSe:Discovery:2021">

<xs:annotation>

<xs:documentation>

Info for 5G ProSe Discovery Control Messages Syntax

</xs:documentation>

</xs:annotation>

<!-- Complex types defined for parameters with complicated structure -->

<xs:complexType name="AppID-info">

<xs:sequence>

<xs:element name="OS-ID">

<xs:simpleType>

<xs:restriction base="xs:hexBinary">

<xs:length value="16"/>

</xs:restriction>

</xs:simpleType>

</xs:element>

<xs:element name="OS-App-ID" type="xs:string"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PLMN-info">

<xs:sequence>

<xs:element name="mcc" type="xs:integer"/>

<xs:element name="mnc" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PLMN-list-type">

<xs:sequence>

<xs:element name="PLMN" type="PLMN-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscFilter-info">

<xs:sequence>

<xs:element name="ProSe-Application-Code" type="xs:hexBinary"/>

<xs:element name="ProSe-Application-Mask" type="xs:hexBinary" maxOccurs="unbounded"/>

<xs:element name="TTL-timer-T5064" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchingFilter-info">

<xs:sequence>

<xs:element name="Code" type="xs:hexBinary"/>

<xs:element name="Mask" type="xs:hexBinary" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DUCK-info">

<xs:sequence>

<xs:element name="discovery-user-confidentiality-key" type="xs:hexBinary"/>

<xs:element name="encrypted-bitmask" type="xs:hexBinary"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscFilter-info">

<xs:sequence>

<xs:element name="filter" type="MatchingFilter-info" maxOccurs="unbounded"/>

<xs:element name="TTL-timer-T5066" type="xs:integer"/>

<xs:element name="RPAUID" type="xs:string" minOccurs="0" />

<xs:element name="metadata-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedCodeSuffixRange-info">

<xs:sequence>

<xs:element name="beginning-suffix-code" type="xs:hexBinary" />

<xs:element name="ending-suffix-code" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMonitoringUpdate-info">

<xs:sequence>

<xs:element name="updated-filter" type="RestrictedDiscFilter-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedAnnouncingUpdate-info">

<xs:sequence>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary" />

<xs:element name="validity-timer-T5062" type="xs:integer" />

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MonitoringUpdate-info">

<xs:sequence>

<xs:element name="updated-filter" type="DiscFilter-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="AnnouncingUpdate-info">

<xs:sequence>

<xs:element name="ProSe-Application-Code" type="xs:hexBinary" />

<xs:element name="validity-timer-T5060" type="xs:integer" />

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="Update-Option-info">

<xs:choice>

<xs:element name="update-info-restricted-announce" type="RestrictedAnnouncingUpdate-info" />

<xs:element name="update-info-restricted-monitor" type="RestrictedMonitoringUpdate-info" />

<xs:element name="update-info-open-annnounce" type="AnnouncingUpdate-info" />

<xs:element name="update-info-open-monitor" type="MonitoringUpdate-info"/>

<xs:element name="anyExt" type="anyExtType" />

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="Restricted-Code-Option-info">

<xs:choice>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary" />

<xs:element name="ProSe-Response-Code" type="xs:hexBinary" />

<xs:element name="anyExt" type="anyExtType" />

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

</xs:complexType>

<xs:complexType name="Subquery-info">

<xs:sequence>

<xs:element name="ProSe-Rquery-Code" type="xs:hexBinary" />

<xs:element name="response-filter" type="MatchingFilter-info" maxOccurs="unbounded"/>

<xs:element name="validity-timer-T5070" type="xs:integer"/>

<xs:element name="code-sending-security-parameter" type="Restricted-Security-info" />

<xs:element name="code-receiving-security-parameter" type="Restricted-Security-info" minOccurs="0" />

<xs:element name="RPAUID" type="xs:string" minOccurs="0" />

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="Restricted-Security-info">

<xs:sequence>

<xs:element name="DUSK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUIK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUCK" type="DUCK-info" minOccurs="0" />

<xs:element name="MIC-check-indicator" type="xs:boolean" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="ApplicationCodeSuffixRange-info">

<xs:sequence>

<xs:element name="beginning-suffix-code" type="xs:hexBinary" />

<xs:element name="ending-suffix-code" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="ProSeApplicationCodeACE-info">

<xs:sequence>

<xs:element name=" ProSe-Application-Code-Prefix" type="xs:hexBinary" />

<xs:element name=" ProSe-Application-Code-Suffix-Range" type="ApplicationCodeSuffixRange-info" maxOccurs="unbounded" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PC5-Security-Policies-info">

<xs:sequence>

<xs:element name="signalling-integrity-protection-policy" type="xs:integer"/>

<xs:element name="signalling-ciphering-policy" type="xs:integer"/>

<xs:element name="user-plane-integrity-protection-policy" type="xs:integer"/>

<xs:element name="user-plane-ciphering-policy" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="requested-for-type">

<xs:sequence>

<xs:element name="remote-UE" type="empty-type" minOccurs="0"/>

<xs:element name="UNR" type="empty-type" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PC5-security-policies-type">

<xs:sequence>

<xs:element name="signalling-ciphering-policy" type="xs:integer"/>

<xs:element name="user-plane-integrity-protection-policy" type="xs:integer"/>

<xs:element name="user-plane-ciphering-policy" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="security-parameters-for-remote-UE-type">

<xs:sequence>

<xs:element name="expiration-timer" type="xs:integer"/>

<xs:element name="code-receiving-security-parameters-for-model-A" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-receiving-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-sending-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="security-parameters-for-UNR-type">

<xs:sequence>

<xs:element name="expiration-timer" type="xs:integer"/>

<xs:element name="code-sending-security-parameters-for-model-A" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-receiving-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-sending-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="code-sending-or-receiving-security-parameters-type">

<xs:sequence>

<xs:element name="DUSK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUIK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUCK" type="DUCK-type" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DUCK-type">

<xs:sequence>

<xs:element name="discovery-user-confidentiality-key" type="xs:hexBinary"/>

<xs:element name="encrypted-bitmask" type="xs:hexBinary"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- Complex types defined for transaction-level -->

<xs:complexType name="AnnounceRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Application-Code" type="xs:hexBinary" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ProSe-Application-Code-ACE" type="ProSeApplicationCodeACE-info" minOccurs="0"/>

<xs:element name="validity-timer-T5060" type="xs:integer" minOccurs="0" />

<xs:element name="discovery-key" type="xs:hexBinary" minOccurs="0" />

<xs:element name="discovery-entry-ID" type="xs:integer" minOccurs="0" />

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MonitorRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="discovery-filter" type="DiscFilter-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="discovery-entry-ID" type="xs:integer" minOccurs="0" />

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscReq-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="command" type="xs:integer"/>

<xs:element name="ProSe-Application-ID" type="xs:string"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="discovery-entry-ID" type="xs:integer" minOccurs="0" />

<xs:element name="Requested-Timer" type="xs:integer" minOccurs="0" />

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="Announcing-PLMN-ID" type="PLMN-info" minOccurs="0" />

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscReq-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="command" type="xs:integer"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="discovery-type" type="xs:integer"/>

<xs:element name="PC5-UE-ciphering-algorithm-capability" type="xs:integer"/>

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0"/>

<xs:element name="announcing-type" type="xs:integer" minOccurs="0"/>

<xs:element name="application-level-container" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="discovery-model" type="xs:integer" minOccurs="0"/>

<xs:element name="Announcing-PLMN-ID" type="PLMN-info" minOccurs="0" />

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="Requested-Timer" type="xs:integer" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedAnnounceRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="ProSe-Restricted-Code-Suffix-Range" type="RestrictedCodeSuffixRange-info" minOccurs="0"/>

<xs:element name="validity-timer-T5062" type="xs:integer" minOccurs="0"/>

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0" />

<xs:element name="code-sending-security-parameter" type="Restricted-Security-info" />

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="on-demand-announcing-enabled-indicator" type="xs:boolean" minOccurs="0" />

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMonitorRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="restricted-discovery-filter" type="RestrictedDiscFilter-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="ACE-enabled-indicator" type="xs:integer" minOccurs="0" />

<xs:element name="application-level-container" type="xs:hexBinary"/>

<xs:element name="code-receiving-security-parameter" type="Restricted-Security-info" minOccurs="0" />

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscovereeRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Response-Code" type="xs:hexBinary" />

<xs:element name="query-filter" type="MatchingFilter-info" maxOccurs="unbounded"/>

<xs:element name="validity-timer-T5068" type="xs:integer"/>

<xs:element name="code-sending-security-parameter" type="Restricted-Security-info" />

<xs:element name="code-receiving-security-parameter" type="Restricted-Security-info" minOccurs="0" />

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedDiscovererRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="subquery-result" type="Subquery-info" minOccurs="1" maxOccurs="unbounded"/>

<xs:element name="code-sending-security-parameter" type="Restricted-Security-info" />

<xs:element name="code-receiving-security-parameter" type="Restricted-Security-info" minOccurs="0" />

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="PC5-security-policies" type="xs:PC5-Security-Policies-info" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RejectRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="PC3a-control-protocol-cause-value" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="UE-RejectRsp-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="PC3a-control-protocol-cause-value" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchRep-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-PC5-discovery-message" type="xs:hexBinary"/>

<xs:element name="Monitored-PLMN-ID" type="PLMN-info"/>

<xs:element name="VPLMN-ID" type="PLMN-info" minOccurs="0"/>

<xs:element name="UTC-based-counter" type="xs:hexBinary"/>

<xs:element name="Metadata-flag" type="xs:boolean"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMatch-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="discovery-type" type="xs:integer"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="Restricted-Code-Discovered" type="Restricted-Code-Option-info" minOccurs="0"/>

<xs:element name="ProSe-PC5-discovery-message" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="UTC-based-counter" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="Metadata-flag" type="xs:boolean" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchAck-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="ProSe-Application-ID" type="xs:string"/>

<xs:element name="validity-timer-T5072" type="xs:integer"/>

<xs:element name="UTC-based-counter" type="xs:hexBinary"/>

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="metadata-index-mask" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="match-report-refresh-timer-T5074" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="RestrictedMatchAck-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="application-identity" type="AppID-info"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="validity-timer-T5076" type="xs:integer"/>

<xs:element name="UTC-based-counter" type="xs:hexBinary"/>

<xs:element name="metadata" type="xs:string" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="match-report-refresh-timer-T5077" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="MatchReject-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="PC3a-control-protocol-cause-value" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscUpdateReq-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="update-info" type="Update-Option-info" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DiscUpdateRsp-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="AnnouncingAlertReq-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:element name="RPAUID" type="xs:string"/>

<xs:element name="discovery-entry-ID" type="xs:integer"/>

<xs:element name="ProSe-Restricted-Code" type="xs:hexBinary"/>

<xs:element name="ProSe-Restricted-Code-Suffix-Range" type="RestrictedCodeSuffixRange-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="AnnouncingAlertRsp-info">

<xs:sequence>

<xs:element name="DDNMF-transaction-ID" type="xs:integer"/>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PKMFAddrReq-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PKMFAddrRsp-info">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="PKMF-address" type="xs:string"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="UNR-discovery-security-parameters-request-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="requested-for" type="requested-for-type"/>

<xs:element name="PC5-UE-security-capabilities" type="xs:integer"/>

<xs:element name="relay-service-code" type="xs:integer"/>

<xs:element name="VPLMN-list" type="PLMN-list-type" minOccurs="0"/>

<xs:element name="discovery-model" type="xs:integer" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="UNR-discovery-security-parameters-accept-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="security-parameters-for-remote-UE" type="security-parameters-for-remote-UE-type"  minOccurs="0"/>

<xs:element name="PC5-security-policies-for-remote-UE" type="PC5-Security-Policies-info" minOccurs="0"/>

<xs:element name="security-parameters-for-UNR" type="security-parameters-for-UNR-type"  minOccurs="0"/>

<xs:element name="PC5-security-policies-for-UNR" type="PC5-Security-Policies-info" minOccurs="0"/>

<xs:element name="Current-Time" type="xs:dateTime"/>

<xs:element name="Max-Offset" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- Complex types defined for Message-level -->

<xs:complexType name="prose-direct-discovery-request">

<xs:sequence>

<xs:element name="discovery-request" type="DiscReq-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-discovery-request" type="RestrictedDiscReq-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="network-initiated-transaction-method" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-response">

<xs:sequence>

<xs:element name="Current-Time" type="xs:dateTime"/>

<xs:element name="Max-Offset" type="xs:integer"/>

<xs:element name="response-announce" type="AnnounceRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="response-monitor" type="MonitorRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-announce-response" type="RestrictedAnnounceRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-monitor-response" type="RestrictedMonitorRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-discoveree-response" type="RestrictedDiscovereeRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-discoverer-response" type="RestrictedDiscovererRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="response-reject" type="RejectRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="network-initiated-transaction-method" type="xs:integer"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-update-request">

<xs:sequence>

<xs:element name="discovery-update-request" type="DiscUpdateReq-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-update-response">

<xs:sequence>

<xs:element name="response-update" type="DiscUpdateRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="response-reject" type="UE-RejectRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-match-report">

<xs:sequence>

<xs:element name="match-report" type="MatchRep-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-match" type="RestrictedMatch-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-match-report-ack">

<xs:sequence>

<xs:element name="Current-Time" type="xs:dateTime"/>

<xs:element name="match-ack" type="MatchAck-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="match-reject" type="MatchReject-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="restricted-match-ack" type="RestrictedMatchAck-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-announcing-alert-request">

<xs:sequence>

<xs:element name="announcing-alert-request" type="AnnouncingAlertReq-info" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-direct-discovery-announcing-alert-response">

<xs:sequence>

<xs:element name="announcing-alert-response" type="AnnouncingAlertRsp-info" maxOccurs="unbounded"/>

<xs:element name="response-reject" type="UE-RejectRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-5gpkmf-address-request">

<xs:sequence>

<xs:element name="PKMF-address-request" type="PKMFAddrReq-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-5gpkmf-address-response">

<xs:sequence>

<xs:element name="PKMF-address-response" type="PKMFAddrRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="PKMF-address-reject" type="RejectRsp-info" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-security-material-request">

<xs:sequence>

<xs:element name="UNR-discovery-security-parameters-request" type="UNR-discovery-security-parameters-request-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="prose-security-material-response">

<xs:sequence>

<xs:element name="UNR-discovery-security-parameters-accept" type="UNR-discovery-security-parameters-accept-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="UNR-discovery-security-parameters-reject" type="reject-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--  extension allowed -->

<xs:complexType name="DiscMsgExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--  XML attribute for any future extensions  -->

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<!--  Top level discovery message definition  -->

<xs:element name="prose-discovery-message">

<xs:complexType>

<xs:choice>

<xs:element name="DISCOVERY_REQUEST" type="prose-direct-discovery-request"/>

<xs:element name="DISCOVERY_RESPONSE" type="prose-direct-discovery-response"/>

<xs:element name="MATCH_REPORT" type="prose-direct-discovery-match-report"/>

<xs:element name="MATCH_REPORT_ACK" type="prose-direct-discovery-match-report-ack"/>

<xs:element name="DISCOVERY_UPDATE_REQUEST" type="prose-direct-discovery-update-request"/>

<xs:element name="DISCOVERY_UPDATE_RESPONSE" type="prose-direct-discovery-update-response"/>

<xs:element name="ANNOUNCING_ALERT_REQUEST" type="prose-direct-discovery-announcing-alert-request"/>

<xs:element name="ANNOUNCING_ALERT_RESPONSE" type="prose-direct-discovery-announcing-alert-response"/>

<xs:element name="PROSE_5GPKMF_ADDRESS_REQUEST" type="prose-5gpkmf-address-request"/>

<xs:element name="PROSE_5GPKMF_ADDRESS_RESPONSE" type="prose-5gpkmf-address-response"/>

<xs:element name="PROSE_SECURITY_MATERIAL_REQUEST" type="prose-security-material-request"/>

<xs:element name="PROSE_SECURITY_MATERIAL_RESPONSE" type="prose-security-material-response"/>

<xs:element name="message-ext" type="DiscMsgExtType"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

</xs:complexType>

</xs:element>

</xs:schema>

An entity receiving the XML body ignores any unknown XML element and any unknown XML attribute.

***** Next change *****

10.6.3	XML schema

Implementations in compliance with the present document shall implement the XML schema defined below for messages used in 5G ProSe security procedures over PC8 interface.

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns="urn:3GPP:ns:5GProSe:Security:2022"

elementFormDefault="qualified"

targetNamespace="urn:3GPP:ns:5GProSe:Security:2022">

<xs:annotation>

<xs:documentation>

Info for 5G ProSe Security Control Messages Syntax

</xs:documentation>

</xs:annotation>

<xs:complexType name="empty-type"/>

<xs:complexType name="requested-for-type">

<xs:sequence>

<xs:element name="remote-UE" type="empty-type" minOccurs="0"/>

<xs:element name="UNR" type="empty-type" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PLMN-type">

<xs:sequence>

<xs:element name="mcc" type="xs:integer"/>

<xs:element name="mnc" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PLMN-list-type">

<xs:sequence>

<xs:element name="PLMN" type="PLMN-type" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="model-type">

<xs:sequence>

<xs:element name="model-A" type="empty-type" minOccurs="0"/>

<xs:element name="model-B" type="empty-type" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="DUCK-type">

<xs:sequence>

<xs:element name="discovery-user-confidentiality-key" type="xs:hexBinary"/>

<xs:element name="encrypted-bitmask" type="xs:hexBinary"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="code-sending-or-receiving-security-parameters-type">

<xs:sequence>

<xs:element name="DUSK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUIK" type="xs:hexBinary" minOccurs="0" />

<xs:element name="DUCK" type="DUCK-type" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="security-parameters-for-remote-UE-type">

<xs:sequence>

<xs:element name="expiration-timer" type="xs:integer"/>

<xs:element name="code-receiving-security-parameters-for-model-A" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-receiving-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-sending-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="security-parameters-for-UNR-type">

<xs:sequence>

<xs:element name="expiration-timer" type="xs:integer"/>

<xs:element name="code-sending-security-parameters-for-model-A" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-receiving-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="code-sending-security-parameters-for-model-B" type="code-sending-or-receiving-security-parameters-type" minOccurs="0"/>

<xs:element name="selected-PC5-ciphering-algorithm" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PC5-security-policies-type">

<xs:sequence>

<xs:element name="signalling-ciphering-policy" type="xs:integer"/>

<xs:element name="user-plane-integrity-protection-policy" type="xs:integer"/>

<xs:element name="user-plane-ciphering-policy" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- Complex types defined for transaction-level -->

<xs:complexType name="UNR-discovery-security-parameters-request-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="requested-for" type="requested-for-type"/>

<xs:element name="PC5-UE-security-capabilities" type="xs:integer"/>

<xs:element name="relay-service-code" type="xs:integer"/>

<xs:element name="VPLMN-list" type="PLMN-list-type" minOccurs="0"/>

<xs:element name="model" type="model-type" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="UNR-discovery-security-parameters-accept-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="security-parameters-for-remote-UE" type="security-parameters-for-remote-UE-type"  minOccurs="0"/>

<xs:element name="PC5-security-policies-for-remote-UE" type="PC5-security-policies-type" minOccurs="0"/>

<xs:element name="security-parameters-for-UNR" type="security-parameters-for-UNR-type"  minOccurs="0"/>

<xs:element name="PC5-security-policies-for-UNR" type="PC5-security-policies-type" minOccurs="0"/>

<xs:element name="Current-Time" type="xs:dateTime"/>

<xs:element name="Max-Offset" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PRUK-request-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="UP-PRUK-ID" type="xs:string" minOccurs="0" />

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PRUK-accept-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="UP-PRUK-ID" type="xs:string"/>

<xs:element name="UP-PRUK" type="xs:hexBinary"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="key-request-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="relay-service-code" type="xs:integer"/>

<xs:element name="SUCI" type="xs:string" minOccurs="0"/>

<xs:element name="UP-PRUK-ID" type="xs:string" minOccurs="0"/>

<xs:element name="Knrp-freshness-parameter-1" type="xs:hexBinary"/>

<xs:element name="HPLMN" type="PLMN-type" minOccurs="0"/>

<xs:element name="AUTS" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="RAND" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="key-accept-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="UP-PRUK-ID" type="xs:string"/>

<xs:element name="Knrp" type="xs:hexBinary"/>

<xs:element name="Knrp-freshness-parameter-2" type="xs:hexBinary"/>

<xs:element name="GPI" type="xs:hexBinary" minOccurs="0"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="reject-type">

<xs:sequence>

<xs:element name="transaction-ID" type="xs:integer"/>

<xs:element name="PC8-control-protocol-cause-value" type="xs:integer"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- Complex types defined for Message-level -->

<xs:complexType name="PROSE_SECURITY_PARAM_REQUEST-type">

<xs:sequence>

<xs:element name="UNR-discovery-security-parameters-request" type="UNR-discovery-security-parameters-request-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PROSE_SECURITY_PARAM_RESPONSE-type">

<xs:sequence>

<xs:element name="UNR-discovery-security-parameters-accept" type="UNR-discovery-security-parameters-accept-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="UNR-discovery-security-parameters-reject" type="reject-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PROSE_PRUK_REQUEST-type">

<xs:sequence>

<xs:element name="PRUK-request" type="PRUK-request-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PROSE_PRUK_RESPONSE-type">

<xs:sequence>

<xs:element name="PRUK-accept" type="PRUK-accept-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="PRUK-reject" type="reject-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PROSE_KEY_REQUEST-type">

<xs:sequence>

<xs:element name="key-request" type="key-request-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="PROSE_KEY_RESPONSE-type">

<xs:sequence>

<xs:element name="key-accept" type="key-accept-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="key-reject" type="reject-type" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--  extension allowed -->

<xs:complexType name="DiscMsgExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!--  XML attribute for any future extensions  -->

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

<!--  Top level Security Message definition  -->

<xs:element name="prose-security-message">

<xs:complexType>

<xs:choice>

<xs:element name="PROSE_SECURITY_PARAM_REQUEST" type="PROSE_SECURITY_PARAM_REQUEST-type"/>

<xs:element name="PROSE_SECURITY_PARAM_RESPONSE" type="PROSE_SECURITY_PARAM_RESPONSE-type"/>

<xs:element name="PROSE_PRUK_REQUEST" type="PROSE_PRUK_REQUEST-type"/>

<xs:element name="PROSE_PRUK_RESPONSE" type="PROSE_PRUK_RESPONSE-type"/>

<xs:element name="PROSE_KEY_REQUEST" type="PROSE_KEY_REQUEST-type"/>

<xs:element name="PROSE_KEY_RESPONSE" type="PROSE_KEY_RESPONSE-type"/>

<xs:element name="message-ext" type="DiscMsgExtType"/>

<xs:any namespace="##other" processContents="lax"/>

</xs:choice>

</xs:complexType>

</xs:element>

</xs:schema>

An entity receiving the XML body ignores any unknown XML element and any unknown XML attribute.

***** Next change *****

10.6.4.2	Semantics of <PROSE_SECURITY_PARAM_REQUEST> element

The <PROSE_SECURITY_PARAM_REQUEST> element contains:

a)	zero or more <UNR-discovery-security-parameters-request> elements which contain transactions sent from the UE to the 5G PKMF;

b)	zero or one <anyExt> element containing elements defined in future releases;

c)	zero or more elements from other namespaces defined in future releases; and

d)	zero or more attributes defined in future releases.

The <UNR-discovery-security-parameters-request> contains:

a)	a <transaction-ID> element containing the parameter defined in clause 11.6.2.1;

b)	a <requested-for> element;

c)	a <PC5-UE-security-capabilities> element containing the parameter defined in clause 11.6.2.4;

d)	a <relay-service-code> element containing the parameter defined in clause 11.6.2.10;

e)	zero or one <VPLMN-list> element;

f)	zero or one <model> element;

g)	zero or one <anyExt> element containing elements defined in future releases;

h)	zero or more elements from other namespaces defined in future releases; and

i)	zero or more attributes defined in future releases.

The <requested-for> element contains:

a)	the following:

1)	a <remote-UE> element indicating that the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE are requested;

2)	a <UNR> element indicating that the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE are requested; or

3)	both;

b)	zero or one <anyExt> element containing elements defined in future releases;

c)	zero or more elements from other namespaces defined in future releases; and

d)	zero or more attributes defined in future releases.

The <VPLMN-list> element contains:

a)	one or more <PLMN> elements;

b)	zero or one <anyExt> element containing elements defined in future releases;

c)	zero or more elements from other namespaces defined in future releases; and

d)	zero or more attributes defined in future releases.

The <PLMN> element contains:

a)	an <mcc> elements containing the parameter defined in clause 11.6.2.5;

b)	an <mnc> elements containing the parameter defined in clause 11.6.2.6;

c)	zero or one <anyExt> element containing elements defined in future releases;

d)	zero or more elements from other namespaces defined in future releases; and

e)	zero or more attributes defined in future releases.

The <model> element contains:

a)	a <model-A> elements indicating that security parameters are requested for 5G ProSe UE-to-network relay discovery over PC5 interface model A or an <model-B> elements indicating that security parameters are requested for 5G ProSe UE-to-network relay discovery over PC5 interface model B;

b)	zero or one <anyExt> element containing elements defined in future releases;

c)	zero or more elements from other namespaces defined in future releases; and

d)	zero or more attributes defined in future releases.

***** Next change *****

10.6.4.3	Semantics of <PROSE_SECURITY_PARAM_RESPONSE> element

The <PROSE_SECURITY_PARAM_RESPONSE> element sent from the 5G PKMF to the UE contains:

a)	zero or more <UNR-discovery-security-parameters-accept> elements which contain accepted transactions;

b)	zero or more <UNR-discovery-security-parameters-reject> elements which contain rejected transactions;

c)	zero or one <anyExt> element containing elements defined in future releases;

d)	zero or more elements from other namespaces defined in future releases; and

e)	zero or more attributes defined in future releases.

The <UNR-discovery-security-parameters-accept> contains:

a)	a <transaction-ID> element containing the parameter defined in clause 11.6.2.1;

b)	zero or one <security-parameters-for-remote-UE> element indicating the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe remote UE;

c)	zero or one <PC5-security-policies-for-remote-UE> element indicating the PC5 security policies for 5G ProSe remote UE;

d)	zero or one <security-parameters-for-UNR> element indicating the 5G ProSe UE-to-network relay discovery security parameters for 5G ProSe UE-to-network relay UE;

e)	zero or one <PC5-security-policies-for-UNR> element indicating the PC5 security policies for 5G ProSe UE-to-network relay UE;

f)	a <Current-Time> element containing the parameter defined in clause 11.6.2.7;

g)	a <Max-Offset> element containing the parameter defined in clause 11.6.2.8;

h)	zero or one <anyExt> element containing elements defined in future releases;

i)	zero or more elements from other namespaces defined in future releases; and

j)	zero or more attributes defined in future releases.

The <security-parameters-for-remote-UE> element and the <security-parameters-for-UNR> element contains:

a)	an <expiration-timer> element containing the parameter defined in clause 11.6.2.9;

b)	optionally a <code-receiving-security-parameters-for-model-A> element;

c)	optionally a <code-receiving-security-parameters-for-model-B> element;

d)	optionally a <code-sending-security-parameters-for-model-B> element;

e)	a <selected-PC5-ciphering-algorithm> element containing the parameter defined in clause 11.6.2.11;

f)	zero or one <anyExt> element containing elements defined in future releases;

g)	zero or more elements from other namespaces defined in future releases; and

h)	zero or more attributes defined in future releases.

The <code-receiving-security-parameters-for-model-A> element, the <code-sending-security-parameters-for-model-A> element, the <code-receiving-security-parameters-for-model-B> element and the <code-sending-security-parameters-for-model-B> element contain:

a)	optionally a <DUSK> element containing the parameter defined in clause 11.6.2.12;

b)	optionally a <DUIK> element containing the parameter defined in clause 11.6.2.13;

c)	optionally a <DUCK> element;

d)	zero or one <anyExt> element containing elements defined in future releases;

e)	zero or more elements from other namespaces defined in future releases; and

f)	zero or more attributes defined in future releases.

The <DUCK> element contains:

a)	a <discovery-user-confidentiality-key> element containing the parameter defined in clause 11.6.2.14;

b)	an <encrypted-bitmask> element containing the parameter defined in clause 11.6.2.15;

c)	zero or one <anyExt> element containing elements defined in future releases;

d)	zero or more elements from other namespaces defined in future releases; and

e)	zero or more attributes defined in future releases.

The <PC5-security-policies-for-remote-UE> element and the <PC5-security-policies-for-UNR> element contain:

a)	a <signalling-ciphering-policy> element containing the parameter defined in clause 11.6.2.17;

b)	a <user-plane-integrity-protection-policy> element containing the parameter defined in clause 11.6.2.18;

c)	a <user-plane-ciphering-policy> element containing the parameter defined in clause 11.6.2.19;

d)	zero or one <anyExt> element containing elements defined in future releases;

e)	zero or more elements from other namespaces defined in future releases; and

f)	zero or more attributes defined in future releases.

The <UNR-discovery-security-parameters-reject> element contains:

a)	a <transaction-ID> element containing the parameter defined in clause 11.6.2.1;

b)	a <PC8-control-protocol-cause-value> element containing the parameter defined in clause 11.6.2.20;

c)	zero or one <anyExt> element containing elements defined in future releases;

d)	zero or more elements from other namespaces defined in future releases; and

e)	zero or more attributes defined in future releases.

***** End of changes *****