3GPP TSG-CT Meeting #102	CP-233304

# Edinburgh, United Kingdom, 11th Dec 2023 - 12th Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0731 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to C6-230681 for TC 27.22.14.2 (Seq 2.3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Comprion GmbH, Apple |  |  |  |  |  |  |  |  |  |
| Work item code: | UEConTest_R17 |  |  |  |  |  | Date: |  |  | 2023-12-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The proposed implementation in the agreed CR C6-230681 is not considering that the device could have registered to NG-RAN cell 1 before the ENVELOPE: EVENT DOWNLOAD – Location Status 2.3.0 is expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Step 6b to handle ENVELOPE: EVENT DOWNLOAD - Location Status (no service) in case the device hasn’t already successfully registered to the network is made optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices successfully registered at the time step 6b is executed will send an unexpected ENVELOPE: EVENT DOWNLOAD - Location Status and therefore unfairly fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 27.22.14.2 Expected Sequence 2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is a correction on top of C6-230681 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Is revision of CP-233223 to correct the WI code |  |  |  |  |  |  |  |  |

[…]

***** start of changes *****

#### 27.22.14.2	Steering of Roaming via DL NAS TRANSPORT message

[…]

Expected Sequence 2.3: (Steering of Roaming via DL NAS TRANSPORT message with "Acknowledgement requested" and REFRESH command [Steering of Roaming])

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | NG-SS | NG-RAN Cell 1 and NG-RAN Cell 2 transmit BCCH. |  |
| 2 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 2.3.1 | If programmable non-removable UICC with a Test Applet is used (as defined in clause 27.0), the TERMINAL RESPONSE cannot be verified by the Test Applet and that the LOCATION STATUS Event has been successfully registered in the device after step 5 is implicitly verified at steps 8 and 19. |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 2.3.1 |  |
| 6a | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 2.3.1 |  |
| 6b | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 2.3.0 | This step is optional and applies in case the ME did not successfully register to NG-RAN cell 1 yet. |
| 7 | ME NG-SS | The ME successfully registers to NG-RAN cell 1 |  |
| 8 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 2.3.1 |  |
| 9 | NG-SS ME | DL NAS TRANSPORT message 2.3.1 | SOR header with: - ACK set to "acknowledgement requested" - List Type set to "secured packet" |
| 10 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 2.3.1 | the ME shall pass the message transparently to the UICC using the ENVELOPE (SMS-PP DOWNLOAD) command as specified in TS 31.111 [15] clause 7.1.1.1a |
| 11 | UICC ME | SW1/SW2 '91 XX' |  |
| 12 | ME NG-SS | ME sends to NG-SS an acknowledgement in the Payload container IE of an UL NAS TRANSPORT message with Payload type IE set to "Steering of Roaming transparent container". | Note: this step can be performed in parallel or any point after step 13 and before step 18. [SOR transparent container 2.3.1 with Acknowledgement] |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: REFRESH 2.3.1 [Steering of Roaming] |  |
| 15 | ME → UICC | Update of EFFPLMN | [Deletion of the entry with PLMN 254/003] |
| 16 | ME UICC | TERMINAL RESPONSE: REFRESH 2.3.1 |  |
| 17 | UICC ME | Proactive UICC session is terminated |  |
| 18 | ME NG-SS | The ME successfully registers to NG-RAN cell 2 within 6 minutes | Note: The ME might have registered to the Cell 2 before this step |
| 19 | ME → UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 2.3.2 | PLMN MCC/MNC 254/003, Normal service |

***** end of changes *****