3GPP TSG CT4 Meeting #66bis	C4-141965

# Sophia-Antipolis, FR; 20th – 24th October 2014							   Revision of C4-141695

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.334 | CR | 0039 | rev | 7 | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of RTP transport multiplexing (Iq, stage 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | RTCP-MUX |  |  |  |  |  |  | Date: |  | 2014-10-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | An UE may request usage of RTP transport multiplexing (via SIP signalling). When RTP transport multiplexing is supported by the IMS-AGW, then the IMS-ALG needs to control RTP transport multiplexing during the creation of H.248 termination. The transport multiplexed mode is optional, the existing transport unmultiplexed mode is mandatory. In order to add optional support for RTP transport multiplexing, following aspects need to be addressed (in clause 5.9): - Multiplexing mode: separate paragraphs for - "transport unmultiplexed mode" and - "transport multiplexed mode"; - Symmetry assumptions in case of multiplexed mode: only symmetrical cases are considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of optional RTP transport multiplexing capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An UE would have to use the default of unmultiplexed RTP / RTCP, which double the number of required UDP transport connections, and could reduce the likelihood of successful NAT traversal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 5.9, 6.2.9, 8.2, 8.3, 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*******

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228:  "IP Multimedia Subsystem (IMS), stage 2".

[3]	3GPP TS 29.334: "IMS Application Level Gateway (IMS-ALG) – IMS Access Gateway (IMS-AGW) Iq interface, stage 3".

[4]	IETF RFC 2663: "IP Network Address Translator (NAT) Terminology and Considerations".

[5]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[6]	IETF RFC 3556: "Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth".

[7]	IETF RFC 3605: "Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP)".

[8]	3GPP TS 23.205: "Bearer independent circuit-switched core network; Stage 2".

[9]	ITU-T Recommendation H.248.1 (05/2002): "Gateway Control Protocol: Version 2" including the Corrigendum1 for Version 2 (03/04).

[10]	IETF RFC 2216: "Network Element Service Template".

[11]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on SIP and SDP".

[12]	3GPP TS 33.328: "IMS Media Plane Security".

[13]	IETF RFC 4568: "Session Description Protocol (SDP) Security Descriptions for Media Streams".

[14]	IETF RFC 3711: "The Secure Real-time Transport Protocol (SRTP)".

[15]	IETF RFC 5124: "Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF)".

[16]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[17]	IETF RFC 6679: "Explicit Congestion Notification (ECN) for RTP over UDP".

[18]	3GPP TS 23.237: "IP Multimedia subsystem (IMS) Service Continuity; Stage 2".

[19]	3GPP TS 24.237: "IP Multimedia subsystem (IMS) Service Continuity; Stage 3".

[20]	3GPP TS 29.162: "Interworking between the IM CN subsystem and IP networks".

[21]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[22]	3GPP TS 22.153: "Multimedia Priority Service".

[23]	IETF RFC 5285: "A General Mechanism for RTP Header Extensions".

[24]	IETF RFC 6236: "Negotiation of Generic Image Attributes in the Session Description Protocol (SDP)".

[25]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[26]	IETF RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[27]	IETF RFC 4583: "Session Description Protocol (SDP) Format for Binary Floor Control Protocol (BFCP) Streams".

[28]	IETF RFC 5246: "The Transport Layer Security (TLS) Protocol Version 1.2".

[29]	IETF RFC 793: "Transmission Control Protocol – DARPA Internet Program – Protocol Specification".

[30]	IETF RFC 4145: "TCP-Based Media Transport in the Session Description Protocol (SDP)".

[31]	IETF RFC 4582: "The Binary Floor Control Protocol (BFCP)".

[32]	IETF RFC 6347: "Datagram Transport Layer Security Version 1.2".

[33]	IETF draft-ietf-mmusic-udptl-dtls-07: "UDP Transport Layer (UDPTL) over Datagram Transport Layer Security (DTLS)".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[34]	IETF draft-schwarz-sdp-for-gw-02: "SDP codepoints for gateway control".

Editor's note: The above document cannot be formally referenced until it is published as an RFC.

[35]	GSM Association RCC.07: "Rich Communication Suite 5.1 Advanced Communications Services and Client Specification, Version 2.0, 03 May 2013".

[36]	GSM Association RCC.07: "Rich Communication Suite 5.1 Advanced Communications Services and Client Specification, Version 3.0, 25 September 2013".

[37]	IETF RFC 4572: "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

[38]	ITU-T Recommendation H.248.84 (07/2012): "Gateway control protocol: NAT-traversal for peer-to-peer services".

[39]	IETF RFC 5245: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[40]	IETF RFC 5389: "Session Traversal Utilities for NAT (STUN)".

[41]	IETF RFC 5766: "Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)".

[42]	IETF RFC 5763: "Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS)".

[43]	IETF RFC 5764: "Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP)".

[44]	3GPP TS 24.371: "Web Real-Time Communications (WebRTC) client access to the IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[45]	IETF RFC 6135: "An Alternative Connection Model for the Message Session Relay Protocol (MSRP)".

[46]	OMA-TS-CPM_Conversation_Function-V2_0-20130926-D: "CPM Conversation Functions".

Editor´s Notes: Spec is not yet public. Reference to be updated once OMA makes new version public.

[x1]	IETF RFC 5761: "Multiplexing RTP Data and Control Packets on a Single Port".

* * * Next Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

End-to-access edge security: media protection extending between an IMS UE and the first IMS core network node in the media path without being terminated by any intermediary node.

End-to-end security: media protection between two IMS UEs without being terminated by any intermediary node.

Full ICE: The full implementation of the Interactive Connectivity Establishment (ICE) specified in IETF RFC 5245 [39].

ICE lite: The lite implementation of the Interactive Connectivity Establishment (ICE) specified in IETF RFC 5245 [39].

Local (near-end) NAPT control: the operation of providing network address mapping information and NAPT policy rules to a near-end NAT in the media flow.

NAT-PT/NAPT-PT: see definition in 3GPP TS 23.228 [2].

NAPT control and NAT traversal: controls network address translation for both near-end NA(P)T and far-end NA(P)T

Network Address Translation (NA(P)T): see definition in 3GPP TS 23.228 [2].

Remote (far-end) NAT traversal: the operation of adapting the IP addresses so that the packets in the media flow can pass through a far-end (remote) NAT.

RTP / RTCP transport multiplexing: a single IP transport (L4) port for RTP and RTCP packets.

TLS-client: the entity that initiates a TLS session establishment to a server (see IETF RFC 5246 [28]).

TLS-server: the entity that responds to requests for TLS session establishment from clients (see IETF RFC 5246 [28]).

TLS endpoint: either a TLS-client or a TLS-server.

Convention:

Wherever the term NAT is used in this specification, it may be replaced by NA(P)T or NA(P)T-PT.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.237 [18] apply:

Access Leg

Access Transfer Control Function (ATCF)

Access Transfer Gateway (ATGW)

Remote Leg

Target Access Leg

Source Access Leg

* * * Next Change * * * *

## 5.9	Handling of RTCP streams

The IMS-ALG and the IMS-AGW shall support control via the Iq interface of the specific RTCP behaviour associated to an RTP flow.

When the IMS-ALG requests the IMS-AGW to reserve transport addresses/resources for an RTP flow, the IMS-ALG should also request the IMS-AGW to reserve resources for the corresponding RTCP flow, but may alternatively request the IMS-AGW not to reserve resources for the corresponding RTCP flow. When the IMS-ALG requests the IMS-AGW to reserve transport addresses/resources for a non-RTP flow, the IMS-ALG shall not request the IMS-AGW to reserve resources for an RTCP flow.

To request the IMS-AGW to reserve resources for an RTCP flow, the IMS ALG shall provide the RTCP handling information element with a value indicating that resources for RTCP shall be reserved.

To request the IMS-AGW not to reserve resources for an RTCP flow, the IMS ALG shall either provide the RTCP handling information element with a value indicating that resources for RTCP shall not be reserved or omit the RTCP handling information element.

If the IMS-AGW receives the indication to reserve RTCP resources, the IMS-AGW shall allocate a local port with even number for an RTP flow also reserve the consecutive local port with odd number for the associated RTCP flow, and it shall send and be prepared to receive RTCP.

If the IMS-AGW receives the indication to not reserve RTCP resources, or if it does not receive any indication at all, it shall not allocate an RTCP port when allocating a port for an RTP flow. The IMS-AGW shall not send any RTCP packets and shall silently discard any received RTCP packets.

When RTCP resources are requested, the IMS-ALG may also specify:

- the remote RTCP port, and optionally the remote address, where to send RTCP packets; if not specified, the IMS-AGW shall send RCTP packets to the port contiguous to the remote RTP port;

- bandwidth allocation requirements for RTCP, if the RTCP bandwidth level for the session is different than the default RTCP bandwidth as specified in RFC 3556 [6].

NOTE:	In line with the recommendations of RFC 3605 [7], separate address or non-contiguous RTCP port numbers will not be allocated by the IMS-ALG / IMS-AGW.

The IMS-AGW shall return an error if it can not allocate the requested RTCP resources.

RTCP port allocation for RTP/RTCP transport multiplexed traffic

The previous procedure describing the default case of transport unmultiplexed scenarios may be extended for the transport multiplexed mode by addition of the RTP/RTCP transport multiplexing information element to indicate to the IMS-AGW that RTP and RTCP traffic shall be multiplexed on a single port (as described in IETF RFC 5761 [x1]). The RTP/RTCP transport multiplexing information element may only be sent to the IMS-AGW in combination with the RTCP handling information element with the value indicating that resources for RTCP shall be reserved. The support of these procedures is optional for the IMS ALG and the IMS-AGW. The IMS-ALG shall only use these procedures when knowing support at IMS-AGW side (e.g., via configuration management).

The usage is conditional, given by following restrictions:

1)	The transport multiplexed mode may be only supported for terminations at the access network side of the IMS-AGW.

2)	The transport multiplexed mode shall be only enabled for the local connection endpoint when agreed via SIP SDP offer/answer negotiation (using the "a=rtcp-mux" SDP attribute, see IETF RFC 5761 [x1]) with the remote endpoint.

3)	When transport multiplexed mode is agreed with the remote endpoint, then it may be applied in both traffic directions.

NOTE:	The last two conditions enforce a symmetrical usage of RTP/RTCP transport multiplexing in the related network domain (here the access network).

* * * Next Change * * * *

### 6.2.9	Handling of RTCP streams

This procedure is identical to that of subclause 6.2.1 apart from the IMS-ALG optionally requesting the IMS-AGW to allocate or not allocate RTCP resources. Additionally if RTCP handling is requested, the IMS ALG may include:

-	the explicit RTCP transport address information element;

-	the RTP/RTCP transport multiplexing information element; and

-	the bandwidth allocation for RTCP,

as described in subclause 5.9.

* * * Next Change * * * *

## 8.2	Reserve and Configure AGW Connection Point

This procedure is used to reserve multimedia-processing resources for the Iq interface connection.

Table 8.2.1: Procedures between IMS-ALG and IMS-AGW: Reserve and Configure AGW Connection Point

| Procedure | Initiated | Information element name | Information element required | Information element description |
| --- | --- | --- | --- | --- |
| Reserve and Configure AGW Connection Point | IMS-ALG | Context/Context Request | M | This information element indicates the existing context or requests a new context for the bearer termination. |
|  |  | Emergency Call Indicator | O | This information element identifies the call as emergency call that requires a preferential handling. |
|  |  | Priority information | O | This information element requests the IMS-AGW to apply priority treatment for the terminations and bearer connections in the specified context. |
|  |  | Bearer Termination Request | M | This information element indicates the existing bearer termination or requests a new bearer termination for the bearer to be established. |
|  |  | Local IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall be prepared to receive user data. May be excluded (i.e. "-" is used in SDP m-line) if no transcoding or media related functions are required. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. |
|  |  | ReserveValue | O | This information element indicates if multiple local resources are to be reserved. This information element shall be included if a speech codec and auxiliary payload types are configured. |
|  |  | Remote IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall send data. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. May be excluded (i.e. "-" is used in SDP m-line) if no transcoding or media related functions are required. |
|  |  | Local Connection Address Request | M | This information element requests an IP address and port number(s) on the IMS-AGW that the remote end can send user plane data to. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Remote Connection Address | M | This information element indicates the remote IP address and port number(s) that the IMS-AGW can send user plane data to. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Notify termination heartbeat | M | This information element requests termination heartbeat indications. This information element shall be included when requesting a new bearer termination. Otherwise the information element is optional. |
|  |  | Notify Released Bearer | O | This information element requests a notification of a released bearer. |
|  |  | Latching Requirement | O | This information element indicates that the IMS-AGW should (re)latch onto the address of received media packets to determine the corresponding destination address. |
|  |  | IP Realm Identifier | O | This information element indicates the IP realm of the bearer termination. |
|  |  | Remote Source Address Filtering | O | This information element indicates that remote source address filtering is required. |
|  |  | Remote Source Address Mask | C | This information element provides information on the valid remote source addresses. This is required if a range of remote source address filtering is required. |
|  |  | Remote Source Port Filtering | O | This information element indicates that remote source port filtering is required. |
|  |  | Remote Source Port | C | This information element identifies the valid remote source port. This may be included if remote source port filtering is included. (NOTE 1) |
|  |  | Remote Source Port Range | C | This information element identifies a range of valid remote source ports. This may be included if remote source port filtering is included. (NOTE 1) |
|  |  | Traffic Policing Required | O | This information element indicates that policing of the media flow is required. |
|  |  | Peak Data Rate | O | This information element may be present if Policing is required and specifies the permissible peak data rate for a media stream. (NOTE 2).. |
|  |  | Sustainable Data Rate | O | This information element may be present if Policing is required and specifies the permissible sustainable data rate for a media stream. (NOTE 2). |
|  |  | Delay Variation Tolerance | O | This information element may be present if Policing on Peak Data Rate is required and specifies the maximum expected delay variation tolerance for the corresponding media stream. |
|  |  | Maximum Burst Size | C | This information element shall be present if Policing on Sustainable Data Rate is required and specifies the maximum expected burst size for the corresponding media stream. |
|  |  | DiffServ Code Point | O | This information element indicates a specific DiffServ code point to be used in the IP header in packets sent on the bearer termination. |
|  |  | DiffServ Tagging Behaviour | O | This information element indicates whether the Diffserv code point in the IP header in packets sent on the bearer termination shall be copied from the received value or set to a specific value. |
|  |  | Media Inactivity Detection Required | O | This information element indicates that detection of inactive media flows is required. |
|  |  | Media Inactivity Detection Time | C | This information element may be present if Inactive Media Detection is required and specifies the Inactivity Detection time. |
|  |  | Media Inactivity Detection Direction | C | This information element may be present if Inactive Media Detection is required and specifies the Inactivity Detection direction. |
|  |  | RTCP handling | O | This information element is present if IMS-ALG wants explicitly control the reservation of RTCP resources by the IMS-AGW. |
|  |  | RTP/RTCP transport multiplexing | C | This information element requests the IMS-AGW to apply the RTP/RTCP transport multiplexing. This information element may only be present if RTCP handling is required. |
|  |  | Local cryptographic SDES attribute | C | This information element is present if IMS-ALG wants that the media is encrypted and/or integrity protected by the IMS-AGW (NOTE 3). It indicates the SDES local cryptographic parameters such as key(s) |
|  |  | Remote cryptographic SDES attribute | C | This information element is present if IMS-ALG wants that the media is decrypted, and/or integrity checked by the IMS-AGW (NOTE 3). It indicates the SDES remote cryptographic parameters such as key(s) |
|  |  | ECN Enable | O | This information element requests the IMS-AGW to apply ECN procedures. |
|  |  | ECN Initiation Method | C | This information element specifies the ECN Initiation method and requests the IMS-AGW to perform IP header settings as an ECN endpoint, or indicates that ECN bits shall be passed transparently. It may be included only if ECN is enabled. |
|  |  | Notify ECN Failure Event | C | This information element requests a notification if a ECN failure occurs due to ECN. It may only be supplied if ECN is enabled and the IMS-AGW acts as ECN endpoint. |
|  |  | Extended RTP Header for CVO | O | This information element requests the IMS-AGW to pass on the CVO extended RTP header as defined by IETF RFC 5285 [23]. |
|  |  | Generic Image Attributes | O | This information element indicates image attributes (e.g. image size) as defined by IETF RFC 6236 [24]. |
|  |  | Local certificate fingerprint Request | O | This information element is present if the IMS-ALG wants that the media is decrypted, and/or integrity protected by the IMS-AGW (NOTE 3). It requests the IMS-AGW to provide a local certificate fingerprint. |
|  |  | Remote certificate fingerprint | O | This information element is present if the IMS-ALG wants that the media is decrypted, and/or integrity checked by the IMS-AGW (NOTE 3). It indicates the remote certificate fingerprint. |
|  |  | Establish (D)TLS session | O | This information element requests the IMS-AGW to take the (D)TLS client role and to initiate the establishment of the (D)TLS session. (NOTE 3) |
|  |  | Notify (D)TLS session establishment Failure Event | O | This information element requests a notification if a (D)TLS session establishment failure occurs. (NOTE 3) |
|  |  | TCP State-aware Handling Indicator and Setup Direction | C | This information element indicates that TCP state-aware handing is required and indicates the TCP setup direction. It may only be included if the IMS-AGW supports TCP state-aware handling. |
|  |  | Discard Incoming TCP Connection Establishment Requests Indicator | C | This information element indicates whether incoming TCP connection establishment requests (TCP SYN) shall be discarded. It may only be included if the IMS-AGW supports TCP state-aware handling and discarding incoming TCP connection establishment requests. |
|  |  | Forward Incoming TCP Connection Establishment Requests Indicator | C | This information element indicates for a given termination to use the incoming TCP connection establishment request (TCP SYN) at that termination as a trigger for sending a TCP connection establishment request at the interconnected termination in the same context. It may only be included if the IMS-AGW supports TCP state-aware handling and the Forward Incoming TCP Connection Establishment Requests Indicator. |
|  |  | Notify TCP Connection Establishment Failure Event | C | This information element requests a notification if a TCP connection establishment failure occurs. It may only be included if the IMS-AGW supports TCP state-aware handling. |
|  |  | STUN server request | O | This information element is present if IMS-ALG requests the IMS-AGW to answer STUN connectivity checks for ICE. |
|  |  | ICE Connectivity Check | C | This information element requests the IMS-AGW to perform ICE connectivity check as defined by IETF RFC 5245 [39]. It is only applicable for full ICE. |
|  |  | Notify ICE Connectivity Check Result | C | This information element requests a notification of ICE connectivity check result. It is only applicable for full ICE. |
|  |  | Notify New Peer Reflexive Candidate | C | This information element requests a notification of new peer reflexive candidate was discovered during a connectivity check. It is only applicable for full ICE. |
|  |  | ICE password request | O | This information element is present if IMS-ALG requests an ICE password. |
|  |  | ICE Ufrag request | O | This information element is present if IMS-ALG requests an ICE ufrag. |
|  |  | ICE host candidate request | O | This information element is present if IMS-ALG requests an ICE host candidate. |
|  |  | ICE received candidate | O | This information element is present if IMS-ALG indicates a received candidate for ICE. |
|  |  | ICE received password | O | This information element is present if IMS-ALG indicates a received password for ICE. |
|  |  | ICE received Ufrag | O | This information element is present if IMS-ALG indicates a received Ufrag for ICE. |
|  |  | MSRP Path | O | This information element is present for application-aware MSRP Interworking. It provides the path information that the IMS-AGW shall insert in the MSRP layer "To-Path" Information element. |
|  |  | Application-aware MSRP interworking request | O | This information element is present if IMS-ALG requests the IMS-AGW to perform application-aware MSRP Interworking. |
| Reserve and Configure AGW Connection Point Ack | IMS-AGW | Context | M | This information element indicates the context where the command was executed. |
|  |  | Bearer Termination | M | This information element indicates the bearer termination where the command was executed. |
|  |  | Local IP Resources | C | This information element indicates the resource(s) for which the IMS-AGW shall be prepared to receive user data. This IE shall be present if it was contained in the request. If the IE was not contained in the request, it may be present in the reply. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. |
|  |  | Remote IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall send data. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. |
|  |  | Local Connection Address | M | This information element indicates the IP address and port number(s) the IMS-AGW shall receive user plane data from IMS. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Remote Connection Address | O | This information element indicates the remote IP address and port number(s) that the IMS-AGW can send user plane data to. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Local cryptographic SDES attribute | C | This information element may be present only if it was contained in the request. It indicates the SDES local cryptographic parameters such as key(s) |
|  |  | Remote cryptographic SDES attribute | C | This information element may be present only if it was contained in the request. It indicates the SDES remote cryptographic parameters such as key(s) |
|  |  | Local certificate fingerprint | C | This information element may be present only if the Local certificate fingerprint Request was contained in the request. It indicates the local certificate fingerprint. (NOTE 3) |
|  |  | ICE password | C | This information element shall be present only if it was contained in the request. It indicates the ICE password assigned by the IMS-AGW. |
|  |  | ICE Ufrag | C | This information element shall be present only if it was contained in the request. It indicates the ICE Ufrag assigned by the IMS-AGW. |
|  |  | ICE host candidate | C | This information element shall be present only if it was contained in the request. It indicates the ICE host candidate assigned by the IMS-AGW. |
|  |  | ICE lite indication | C | This information element shall be present only if an ICE host candidate request was contained in the request, and the IMS-AGW supports ICE lite, but not full ICE. It indicates that the IMS-AGW only supports ICE lite. |
| NOTE 1: Remote Source Port and Remote Source Port Range are mutually exclusive. NOTE 2: One of those IEs shall at least be present when policing is required. NOTE 3: This IE may only be present for access network terminations. |  |  |  |  |

* * * Next Change * * * *

## 8.3	Reserve AGW Connection Point Procedure

This procedure is used to reserve local connection addresses and local resources in IMS-AGW.

Table 8.3.1: Procedures between IMS-ALG and IMS-AGW: Reserve AGW Connection Point

| Procedure | Initiated | Information element name | Information element required | Information element description |
| --- | --- | --- | --- | --- |
| Reserve AGW Connection Point | IMS-ALG | Context /Context Request | M | This information element indicates the existing context or requests a new context for the bearer termination. |
|  |  | Emergency Call Indicator | O | This information element identifies the call as emergency call that requires a preferential handling. |
|  |  | Priority information | O | This information element requests the IMS-AGW to apply priority treatment for the terminations and bearer connections in the specified context. |
|  |  | Bearer Termination Request | M | This information element requests a new bearer termination |
|  |  | Local IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall be prepared to receive user data. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. May be excluded (i.e. "-" is used in SDP m-line) if no transcoding or media related functions are required. |
|  |  | ReserveValue | O | This information element indicates if multiple local resources are to be reserved. This information element shall be included if a speech codec and auxiliary payload types are configured. |
|  |  | Local Connection Address Request | M | This information element requests an IP address and port number(s) on the IMS-AGW that the remote end can send user plane data to. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Notify termination heartbeat | M | This information element requests termination heartbeat indications. |
|  |  | Notify Released Bearer | O | This information element requests a notification of a released bearer. |
|  |  | Latching Requirement | O | This information element indicates that the IMS-AGW should (re)latch onto the address of received media packets to determine the corresponding destination address. |
|  |  | IP Realm Identifier | O | This information element indicates the IP realm of the bearer termination. |
|  |  | Remote Source Address Filtering | O | This information element indicates that remote source address filtering is required. |
|  |  | Remote Source Address Mask | C | This information element provides information on the valid remote source addresses. This is required if a range of remote source address filtering is required. |
|  |  | Remote Source Port Filtering | O | This information element indicates that remote source port filtering is required. |
|  |  | Remote Source Port | C | This information element identifies the valid remote source port. This may be included if remote source port filtering is included. (NOTE 1) |
|  |  | Remote Source Port Range | C | This information element identifies a range of valid remote source ports. This may be included if remote source port filtering is included. (NOTE 1) |
|  |  | Policing Required | O | This information element indicates that policing of the media flow is required. |
|  |  | Peak Data Rate | O | This information element may be present if Policing is required and specifies the permissible peak data rate for a media stream. (NOTE 2). |
|  |  | Sustainable Data Rate | O | This information element may be present if Policing is required and specifies the permissible sustainable data rate for a media stream. (NOTE 2). |
|  |  | Delay Variation Tolerance | O | This information element may be present if Policing on Peak Data Rate is required and specifies the maximum expected delay variation tolerance for the corresponding media stream. |
|  |  | Maximum Burst Size | C | This information element shall be present if Policing on Sustainable Data Rate is required and specifies the maximum expected burst size for the corresponding media stream. |
|  |  | DiffServ Code Point | O | This information element indicates a specific DiffServ code point to be used in the IP header in packets sent on the bearer termination. |
|  |  | DiffServ Tagging Behaviour | O | This information element indicates whether the Diffserv code point in the IP header in packets sent on the bearer termination should be copied from the received value or set to a specific value. |
|  |  | Media Inactivity Detection Required | O | This information element indicates that detection of inactive media flows is required. |
|  |  | Media Inactivity Detection Time | C | This information element may be present if Inactive Media Detection is required and specifies the Inactivity Detection time. |
|  |  | Media Inactivity Detection Direction | C | This information element may be present if Inactive Media Detection is required and specifies the Inactivity Detection direction. |
|  |  | RTCP handling | O | This information element is present if IMS-ALG wants explicitly control the reservation of RTCP resources by the IMS-AGW. |
|  |  | RTP/RTCP transport multiplexing | C | This information element requests the IMS-AGW to apply the RTP/RTCP transport multiplexing. This information element may only be present if RTCP handling is required. |
|  |  | Local cryptographic SDES attribute | C | This information element is present if IMS-ALG wants that the media is encrypted and/or integrity protected by the IMS-AGW (NOTE 3). It indicates the SDES local cryptographic parameters such as key(s). |
|  |  | ECN Enable | O | This information element requests the IMS-AGW to apply ECN procedures. |
|  |  | ECN Initiation Method | C | This information element specifies the ECN Initiation method and requests the IMS-AGW to perform IP header settings as an ECN endpoint, or indicates that ECN bits shall be passed transparently. It may be included only if ECN is enabled. |
|  |  | Notify ECN Failure Event | C | This information element requests a notification if a ECN failure occurs due to ECN. It may only be supplied if ECN is enabled and the IMS-AGW acts as ECN endpoint. |
|  |  | Extended RTP Header for CVO | O | This information element requests the IMS-AGW to pass on the CVO extended RTP header as defined by IETF RFC 5285 [23]. |
|  |  | Generic Image Attributes | O | This information element indicates image attributes (e.g. image size) as defined by IETF RFC 6236 [24]. |
|  |  | Local certificate fingerprint Request | O | This information element is present if the IMS-ALG wants that the media is decrypted, and/or integrity protected by the IMS-AGW (NOTE 4). It requests the IMS-AGW to provide a local certificate fingerprint. |
|  |  | Establish (D)TLS session | O | This information element requests the IMS-AGW to take the (D)TLS client role and to initiate the establishment of the (D)TLS session. (NOTE 4) |
|  |  | Notify (D)TLS session establishment Failure Event | O | This information element requests a notification if a (D)TLS session establishment failure occurs. (NOTE 4) |
|  |  | TCP State-aware Handling Indicator and Setup Direction | C | This information element indicates that TCP state-aware handing is required and indicates the TCP setup direction. It may only be included if the IMS-AGW supports TCP state-aware handling. |
|  |  | Discard Incoming TCP Connection Establishment Requests Indicator | C | This information element indicates whether incoming TCP connection establishment requests (TCP SYN) shall be discarded. It may only be included if the IMS-AGW supports TCP state-aware handling and discarding incoming TCP connection establishment requests. |
|  |  | Forward Incoming TCP Connection Establishment Requests Indicator | C | This information element indicates for a given termination to use the incoming TCP connection establishment request (TCP SYN) at that termination as a trigger for sending a TCP connection establishment request at the interconnected termination in the same context. It may only be included if the IMS-AGW supports TCP state-aware handling and the Forward Incoming TCP Connection Establishment Requests Indicator. |
|  |  | Notify TCP Connection Establishment Failure Event | C | This information element requests a notification if a TCP connection establishment failure occurs. It may only be included if the IMS-AGW supports TCP state-aware handling. |
|  |  | ICE password request | O | This information element is present if IMS-ALG requests an ICE password. |
|  |  | ICE Ufrag request | O | This information element is present if IMS-ALG requests an ICE ufrag. |
|  |  | ICE host candidate request | O | This information element is present if IMS-ALG requests an ICE host candidate. |
|  |  | STUN server request | O | This information element is present if IMS-ALG requests the IMS-AGW to answer STUN connectivity checks for ICE. |
|  |  | Application-aware MSRP interworking request | O | This information element is present if IMS-ALG requests the IMS-AGW to perform application-aware MSRP Interworking. |
| Reserve AGW Connection Point Ack | IMS-AGW | Context | M | This information element indicates the context where the command was executed. |
|  |  | Bearer Termination | M | This information element indicates the bearer termination where the command was executed. |
|  |  | Local IP Resources | C | This information element indicates the resource(s) for which the IMS-AGW shall be prepared to receive user data. This IE shall be present if it was contained in the request. If the IE was not contained in the request, it may be present in the reply. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. |
|  |  | Local Connection Address | M | This information element indicates the IP address and port number(s) the IMS-AGW shall receive user plane data from IMS. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Local cryptographic SDES attribute | C | This information element may be present only if it was contained in the request. It indicates the SDES local cryptographic parameters such as key(s) |
|  |  | Local certificate fingerprint | C | This information element may be present only if the Local certificate fingerprint Request was contained in the request. It indicates the local certificate fingerprint. (NOTE 4) |
|  |  | ICE password | C | This information element shall be present only if it was contained in the request. It indicates the ICE password assigned by the IMS-AGW. |
|  |  | ICE Ufrag | C | This information element shall be present only if it was contained in the request. It indicates the ICE Ufrag assigned by the IMS-AGW. |
|  |  | ICE host candidate | C | This information element shall be present only if it was contained in the request. It indicates the ICE host candidate assigned by the IMS-AGW. |
|  |  | ICE lite indication | C | This information element shall be present only if an ICE host candidate request was contained in the request, and the IMS-AGW supports ICE lite, but not full ICE. It indicates that the IMS-AGW only supports ICE lite. |
| NOTE 1: Remote Source Port and Remote Source Port Range are mutually exclusive. NOTE 2: One of those IEs shall at least be present when policing is required. NOTE 3: This IE may only be present for access network terminations, and only if the IMS-ALG includes only one SDES crypto attribute in the SDP sent towards the served UE. NOTE 4: This IE may only be present for access network terminations. |  |  |  |  |

* * * Next Change * * * *

## 8.4	Configure AGW Connection Point Procedure

This procedure is used to select or modify multimedia-processing resources for the Iq interface connection.

Table 8.4.1: Procedures between IMS-ALG and IMS-AGW: Configure AGW Connection Point

| Procedure | Initiated | Information element name | Information element required | Information element description |
| --- | --- | --- | --- | --- |
| Configure AGW Connection Point | IMS-ALG | Context | M | This information element indicates the context for the bearer termination. |
|  |  | Priority information | O | This information element shall be present if the priority information needs to be modified, it may be present otherwise. |
|  |  | Bearer Termination | M | This information element indicates the existing bearer termination. |
|  |  | Local IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall be prepared to receive user data. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. May be excluded (i.e. "-" is used in SDP m-line) if no transcoding or media related functions are required. |
|  |  | Remote IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall send data. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. May be excluded (i.e. "-" is used in SDP m-line) if no transcoding or media related functions are required. |
|  |  | Local Connection Address | O | This information element indicates the IP address and port number(s) on the IMS-AGW that the IMS user can send user plane data to. For terminations supporting video any combination of video, audio and messaging may contain multiple addresses. |
|  |  | Remote Connection Address | O | This information element indicates the remote IP address and port number(s) that the IMS-AGW can send user plane data to. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Latching Requirement | O | This information element indicates that the IMS-AGW should (re)latch onto the address of received media packets to determine the corresponding destination address. |
|  |  | IP Realm Identifier | O | This information element indicates the IP realm of the bearer termination. (NOTE 3) |
|  |  | Remote Source Address Filtering | O | This information element indicates that remote source address filtering is required. |
|  |  | Remote Source Address Mask | C | This information element provides information on the valid remote source addresses. This is required if a range of remote source address filtering is required. |
|  |  | Remote Source Port Filtering | O | This information element indicates that remote source port filtering is required. |
|  |  | Remote Source Port | C | This information element identifies the valid remote source port. This may be included if remote source port filtering is included. (NOTE 1) |
|  |  | Remote Source Port Range | C | This information element identifies a range of valid remote source ports. This may be included if remote source port filtering is included. (NOTE 1) |
|  |  | Policing Required | O | This information element indicates that policing of the media flow is required. |
|  |  | Peak Data Rate | O | This information element may be present if Policing is required and specifies the permissible peak data rate for a media stream. (NOTE 2). |
|  |  | Sustainable Data Rate | O | This information element may be present if Policing is required and specifies the permissible sustainable data rate for a media stream. (NOTE 2). |
|  |  | Delay Variation Tolerance | O | This information element may be present if Policing on Peak Data Rate is required and specifies the maximum expected delay variation tolerance for the corresponding media stream. |
|  |  | Maximum Burst Size | C | This information element shall be present if Policing on Sustainable Data Rate is required and specifies the maximum expected burst size for the corresponding media stream. |
|  |  | DiffServ Code Point | O | This information element indicates a specific DiffServ code point to be used in the IP header in packets sent on the bearer termination. |
|  |  | DiffServ Tagging Behaviour | O | This information element indicates whether the Diffserv code point in the IP header in packets sent on the bearer termination should be copied from the received value or set to a specific value. |
|  |  | Media Inactivity Detection Required | O | This information element indicates that detection of inactive media flows is required. |
|  |  | Media Inactivity Detection Time | C | This information element may be present if Inactive Media Detection is required and specifies the Inactivity Detection time. |
|  |  | Media Inactivity Detection Direction | C | This information element may be present if Inactive Media Detection is required and specifies the Inactivity Detection direction. |
|  |  | RTCP handling | O | This information element is present if IMS-ALG wants explicitly control the reservation of RTCP resources by the IMS-AGW. |
|  |  | RTP/RTCP transport multiplexing | C | This information element requests the IMS-AGW to apply the RTP/RTCP transport multiplexing. This information element may only be present if RTCP handling is required. |
|  |  | Local cryptographic SDES attribute | C | This information element is present if IMS-ALG wants that the media is encrypted and/or integrity protected by the IMS-AGW (NOTE 4). It indicates the SDES local cryptographic parameters such as key(s). |
|  |  | Remote cryptographic SDES attribute | C | This information element is present if IMS-ALG wants that the media is decrypted, and/or integrity checked by the IMS-AGW (NOTE 4). It indicates the SDES remote cryptographic parameters such as key(s). |
|  |  | ECN Enable | O | This information element requests the IMS-AGW to apply ECN procedures. |
|  |  | ECN Initiation Method | C | This information element specifies the ECN Initiation method and requests the IMS-AGW to perform IP header settings as an ECN endpoint, or indicates that ECN bits shall be passed transparently. It may be included only if ECN is enabled. |
|  |  | Notify ECN Failure Event | C | This information element requests a notification if a ECN failure occurs due to ECN. It may only be supplied if ECN is enabled and the IMS-AGW acts as ECN endpoint. |
|  |  | Extended RTP Header for CVO | O | This information element requests the IMS-AGW to pass on the CVO extended RTP header as defined by IETF RFC 5285 [23]. |
|  |  | Generic Image Attributes | O | This information element indicates image attributes (e.g. image size) as defined by IETF RFC 6236 [24]. |
|  |  | Remote certificate fingerprint | O | This information element is present if the IMS-ALG wants that the media is decrypted, and/or integrity checked by the IMS-AGW (NOTE 4). It indicates the remote certificate fingerprint. |
|  |  | Establish (D)TLS session | O | This information element requests the IMS-AGW to take the (D)TLS client role and to initiate the establishment of the (D)TLS session. (NOTE 4) |
|  |  | Release (D)TLS session | O | This information element requests the IMS-AGW to release the (D)TLS session. (NOTE 4) |
|  |  | Notify (D)TLS session establishment Failure Event | O | This information element requests a notification if a TLS session establishment failure occurs. (NOTE 4) |
|  |  | TCP State-aware Handling Indicator and Setup Direction | C | This information element indicates that TCP state-aware handing is required and indicates the TCP setup direction. It may only be included if the IMS-AGW supports TCP state-aware handling. |
|  |  | Discard Incoming TCP Connection Establishment Requests Indicator | C | This information element indicates whether incoming TCP connection establishment requests (TCP SYN) shall be discarded. It may only be included if the IMS-AGW supports TCP state-aware handling and discarding incoming TCP connection establishment requests. |
|  |  | Forward Incoming TCP Connection Establishment Requests Indicator | C | This information element indicates for a given termination to use the incoming TCP connection establishment request (TCP SYN) at that termination as a trigger for sending a TCP connection establishment request at the interconnected termination in the same context. It may only be included if the IMS-AGW supports TCP state-aware handling and the Forward Incoming TCP Connection Establishment Requests Indicator. |
|  |  | Send TCP Connection Establishment Requests Indicator | C | This information element indicates for a given termination to send a TCP connection establishment request (TCP SYN). It may only be included if the IMS-AGW supports TCP state-aware handling. |
|  |  | Notify TCP Connection Establishment Failure Event | C | This information element requests a notification if a TCP connection establishment failure occurs. It may only be included if the IMS-AGW supports TCP state-aware handling. |
|  |  | ICE Connectivity Check | C | This information element requests the IMS-AGW to perform ICE connectivity check as defined by IETF RFC 5245 [39]. It is only applicable for full ICE. |
|  |  | Notify ICE Connectivity Check Result | C | This information element requests a notification of ICE connectivity check result. It is only applicable for full ICE. |
|  |  | Notify New Peer Reflexive Candidate | C | This information element requests a notification of new peer reflexive candidate was discovered during a connectivity check. It is only applicable for full ICE. |
|  |  | Additional ICE Connectivity Check | C | This information element requests the IMS-AGW to perform additional ICE connectivity check as defined by IETF RFC 5245 [39]. It is only applicable for full ICE. |
|  |  | ICE received candidate | O | This information element is present if IMS-ALG indicates a received candidate for ICE. |
|  |  | ICE received password | O | This information element is present if IMS-ALG indicates a received password for ICE. |
|  |  | ICE received Ufrag | O | This information element is present if IMS-ALG indicates a received Ufrag for ICE. |
|  |  | MSRP Path | O | This information element is present for application-aware MSRP Interworking. It provides the path information that the IMS-AGW shall insert in the MSRP layer "To-Path" Information element. |
| Configure AGW Connection Point Ack | IMS-AGW | Context | M | This information element indicates the context where the command was executed. |
|  |  | Bearer Termination | M | This information element indicates the bearer termination where the command was executed. |
|  |  | Local IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall be prepared to receive user data For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. |
|  |  | Remote IP Resources | O | This information element indicates the resource(s) for which the IMS-AGW shall send data. For terminations supporting any combination of video, audio and messaging this IE shall contain separate resources per stream. |
|  |  | Local Connection Address | O | This information element indicates the IP address and port number(s) on the IMS-AGW that the IMS user can send user plane data to. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Remote Connection Address | O | This information element indicates the remote IP address and port number(s) that the IMS-AGW can send user plane data to. For terminations supporting any combination of video, audio and messaging this may contain multiple addresses. |
|  |  | Local cryptographic SDES attribute | C | This information element may be present only if it was contained in the request. It indicates the SDES local cryptographic parameters such as key(s) |
|  |  | Remote cryptographic SDES attribute | C | This information element may be present only if it was contained in the request. It indicates the SDES remote cryptographic parameters such as key(s) |
| NOTE 1: Remote Source Port and Remote Source Port Range are mutually exclusive. NOTE 2: One of those IEs shall at least be present when policing is required. NOTE 3: Additional streams may be added by the Configure AGW Connection Point procedure. The additional streams shall then carry the same IP Realm Identifier as the very first Stream. NOTE 4: This IE may only be present for access network terminations. |  |  |  |  |

Editor's Note :	The details of how the transparent indication included in ECN Control is subject of stage 3 specification. It also needs to be determined if this indication is needed on both incoming and outgoing terminations.

* * * End of Changes * * * *