3GPP TSG CT Meeting #75	CP-170222

Dubrovnik, Croatia; 06th – 07th March 2017	(was C1-170964)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 2767 | rev | 4 | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Light Connection support over NAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,Vodafone, Qualcomm Incorporated, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon,Vodafone, Qualcomm Incorporated, Intel, Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LIGHT_CON-CT |  |  |  |  |  |  | Date: |  | 2017-02-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A Rel-14 CT WID under the LTE_LIGHT_CON-CT acronym was approved by CT#74 plenary at their December’s meeting (2016) which identifies potential NAS protocol impacts due to the Light connection for LTE feature. As analyzed and evaluated in the discussion papers in C1-170103 and C1-170766, the Light Connection (LC) causes some impacts on existing NAS protocols at both the UE and MME sides. Hence, CT1 need to update TS 24.301 to allow that the Light Connection feature can work properly. Revision 3 (for CT#75): Fixed terminology in order to make the specification consistent (“operates on its home PLMN (HPLMN) or equivalent home PLMN (EHPLMN)” -> “the attach/tracking area updating procedure is initiated in the UE’s HPLMN or EHPLMN”, and (“operates on a visited PLMN” -> “the attach/tracking area updating procedure is initiated in a VPLMN”) in subclause 5.5.1.2.2 and 5.5.3.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to update the specification so that the necessary updates to NAS protocols in order to support the Light Connection for LTE feature are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of NAS support of Light connection for LTE, and therefore the feature is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.5.1.2.2, 5.5.3.2.2, 9.9.3.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 23.401 CR#3196 TS 24.368 CR#0025 TS 31.102 CR#0720 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The CR was endorsed as "technically correct" by CT1 at their last meeting (#102; in Dubrovnik). |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

The term "mobile station" (MS) in the present document is synonymous with the term "user equipment" (UE) as defined in 3GPP TR 21.905 [1].

1x CS fallback capable UE: A UE that uses a CS infrastructure for a voice call and other CS-domain services by falling back to cdma2000® 1x access network if the UE is served by E-UTRAN when a CS service is requested.

Aggregate maximum bit rate: The maximum bit rate that limits the aggregate bit rate of a set of non-GBR bearers of a UE. Definition derived from 3GPP TS 23.401 [10].

APN based congestion control: Congestion control in session management where the network can reject session management requests from UEs or deactivate PDN connections when the associated APN is congested.

Attached for emergency bearer services: A UE is attached for emergency bearer services if it has only a PDN connection for emergency bearer services established.

Chosen PLMN: The same as selected PLMN as specified in 3GPP TS 23.122 [6].

Control plane CIoT EPS optimization: signalling optimizations to enable efficient transport of user data (IP, non-IP or SMS) over control plane via the MME including optional header compression of IP data.

User plane CIoT EPS optimization: signalling optimizations to enable efficient transport of user data (IP or non-IP) over the user plane.

UE supporting CIoT EPS optimizations: A UE that supports control plane CIoT EPS optimization or user plane CIoT EPS optimization and one or more other CIoT EPS optimizations when the UE is in S1 mode.

Attached for EPS services with CP-CIoT EPS optimization: A UE supporting CIoT EPS optimizations is attached for EPS services, and control plane CIoT EPS optimization along with one or more other CIoT EPS optimizations have been accepted by the network.

Attached for EPS services with User plane CIoT EPS optimization: A UE supporting CIoT EPS optimizations is attached for EPS services, and user plane CIoT EPS optimization along with one or more other CIoT EPS optimizations have been accepted by the network.

Attached for EPS services with CIoT EPS optimization: A UE is attached for EPS services with CP-CIoT EPS optimization or attached for EPS services with user plane CIoT EPS optimization.

CS fallback cancellation request: A request received from the MM sublayer to cancel a mobile originating CS fallback.

CS fallback capable UE: A UE that uses a CS infrastructure for a voice call and other CS-domain services by falling back to A/Gb or Iu mode if the UE is served by E-UTRAN when a CS service is requested.

CSG cell: A cell in which only members of the CSG can get normal service. Depending on local regulation, the CSG cell can provide emergency bearer services also to subscribers who are not member of the CSG. Definition derived from 3GPP TS 23.401 [10].

CSG ID: A CSG ID is a unique identifier within the scope of one PLMN defined in 3GPP TS 23.003 [2] which identifies a Closed Subscriber Group (CSG) in the PLMN associated with a cell or group of cells to which access is restricted to members of the CSG.

CSG selection: A UE supporting CSG selection selects CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs. Definition derived from 3GPP TS 23.122 [6].

Dedicated bearer: An EPS bearer that is associated with uplink packet filters in the UE and downlink packet filters in the PDN GW where the filters only match certain packets. Definition derived from 3GPP TS 23.401 [10].

Default bearer: An EPS bearer that gets established with every new PDN connection. Its context remains established throughout the lifetime of that PDN connection. A default EPS bearer is a non-GBR bearer. Definition derived from 3GPP TS 23.401 [10].

Emergency EPS bearer context: A default EPS bearer context which was activated with request type "emergency" or "handover of emergency bearer services", or any dedicated EPS bearer context associated to this default EPS bearer context.

EMM context: An EMM context is established in the UE and the MME when an attach procedure is successfully completed.

EMM-CONNECTED mode: A UE is in EMM-CONNECTED mode when a NAS signalling connection between UE and network is established. The term EMM-CONNECTED mode used in the present document corresponds to the term ECM-CONNECTED state used in 3GPP TS 23.401 [10].

EMM-IDLE mode: A UE is in EMM-IDLE mode when no NAS signalling connection between UE and network exists or when RRC connection suspend has been indicated by lower layers. The term EMM-IDLE mode used in the present document corresponds to the term ECM-IDLE state used in 3GPP TS 23.401 [10].

EPS security context: In the present specification, EPS security context is used as a synonym for EPS NAS security context specified in 3GPP TS 33.401 [19].

EPS services: Services provided by PS domain. Within the context of this specification, EPS services is used as a synonym for GPRS services in 3GPP TS 24.008 [13].

Evolved packet core network: The successor to the 3GPP Release 7 packet-switched core network, developed by 3GPP within the framework of the 3GPP System Architecture Evolution (SAE).

Evolved packet system: The evolved packet system (EPS) or evolved 3GPP packet-switched domain consists of the evolved packet core network and the evolved universal terrestrial radio access network. Definition derived from 3GPP TS 23.401 [10].

GBR bearer: An EPS bearer that uses dedicated network resources related to a guaranteed bit rate (GBR) value, which are permanently allocated at EPS bearer establishment/modification. Definition derived from 3GPP TS 23.401 [10].

General NAS level mobility management congestion control: The type of congestion control that is applied at a general overload or congestion situation in the network, e.g. lack of processing resources.

Group specific session management congestion control: Type of congestion control at session management level that is applied to reject session management requests from UEs belonging to a particular group when one or more group congestion criteria as specified in 3GPP TS 23.401 [10] are met.

Highest ranked ACDC category: The ACDC category with the lowest value as defined in 3GPP TS 24.105 [35].

Initial NAS message: A NAS message is considered as an initial NAS message, if this NAS message can trigger the establishment of a NAS signalling connection. For instance, the ATTACH REQUEST message is an initial NAS message.

IPv4v6 capability: Capability of the IP stack associated with a UE to support a dual stack configuration with both an IPv4 address and an IPv6 address allocated.

Kilobit: 1000 bits.

Last Visited Registered TAI: A TAI which is contained in the TAI list that the UE registered to the network and which identifies the tracking area last visited by the UE.

Linked Bearer Identity: This identity indicates to which default bearer the additional bearer resource is linked.

LIPA PDN connection: A PDN connection, for which the default EPS bearer context or default PDP context was activated with an APN authorized to use LIPA. The network authorizes an APN for using LIPA based on the subscription profile (see 3GPP TS 29.272 [16C]) and subsequently the network considers this PDN connection a LIPA PDN connection.

Lower layer failure: A failure reported by the AS to the NAS that cannot be corrected on AS level. When the AS indicates a lower layer failure to NAS, the NAS signalling connection is not available.

Mapped EPS security context: A mapped security context to be used in EPS. Definition derived from 3GPP TS 33.401 [19].

Mapped GUTI: A GUTI which is mapped from a P-TMSI and an RAI allocated previously by an SGSN. Mapping rules are defined in 3GPP TS 23.003 [2]. Definition derived from 3GPP TS 23.401 [10].

Megabit: 1,000,000 bits.

Message header: A standard L3 message header as defined in 3GPP TS 24.007 [12].

MME area: An area containing tracking areas served by an MME.

MO MMTEL voice call is started: the MO-MMTEL-voice-started indication was received from upper layers (see 3GPP TS 24.173 [13E]) and after reception of the MO-MMTEL-voice-started indication, the MO-MMTEL-voice-ended indication has not been received.

MO MMTEL video call is started: the MO-MMTEL-video-started indication was received from upper layers (see 3GPP TS 24.173 [13E]) and after reception of the MO-MMTEL-video-started indication, the MO-MMTEL-video-ended indication has not been received.

MO SMSoIP is started: the MO-SMSoIP-attempt-started indication was received from upper layers (see 3GPP TS 24.341 [15D]) and after reception of the MO-SMSoIP-attempt-started indication, the MO-SMSoIP-attempt-ended indication has not been received.

NAS level mobility management congestion control: Congestion control mechanism in the network in mobility management. "NAS level mobility management congestion control" consists of "subscribed APN based congestion control" and "general NAS level mobility management congestion control".

NAS signalling connection: A peer to peer S1 mode connection between UE and MME. A NAS signalling connection consists of the concatenation of an RRC connection via the "LTE-Uu" interface and an S1AP connection via the S1 interface. Additionally, for the purpose of optimized handover or idle mode mobility from cdma2000® HRPD access to E-UTRAN (see 3GPP TS 23.402 [11]), the NAS signalling connection can consist of a concatenation of an S101-AP connection and a signalling tunnel over a cdma2000® HRPD access network.

NOTE 1:	cdma2000® is a registered trademark of the Telecommunications Industry Association (TIA-USA).

NAS signalling connection recovery: A mechanism initiated by the NAS to restore the NAS signalling connection on indication of "RRC connection failure" by the lower layers.

Native GUTI: A GUTI previously allocated by an MME. Definition derived from 3GPP TS 23.401 [10].

Non-access stratum protocols: The protocols between UE and MSC or SGSN that are not terminated in the UTRAN, and the protocols between UE and MME that are not terminated in the E-UTRAN. Definition derived from 3GPP TR 21.905 [1].

Non-emergency EPS bearer context: Any EPS bearer context which is not an emergency EPS bearer context.

Non-EPS services: Services provided by CS domain. Within the context of this specification, non-EPS services is used as a synonym for non-GPRS services in 3GPP TS 24.008 [13]. A UE which camps on E-UTRAN can attach to both EPS services and non-EPS services.

Non-GBR bearer: An EPS bearer that uses network resources that are not related to a guaranteed bit rate (GBR) value. Definition derived from 3GPP TS 23.401 [10].

PDN address: An IP address assigned to the UE by the Packet Data Network Gateway (PDN GW).

PDN connection for emergency bearer services: A PDN connection for which the default EPS bearer context or default PDP context was activated with request type "emergency" or "handover of emergency bearer services".

Plain NAS message: A NAS message with a header including neither a message authentication code nor a sequence number.

Persistent EPS bearer context: either a non-emergency EPS bearer context representing a GBR bearer with QoS equivalent to QoS of teleservice 11 and where there is a radio bearer associated with that context, or an emergency EPS bearer context where there is a radio bearer associated with that context.

NOTE 2:	An example of a persistent EPS bearer context is a non-emergency EPS bearer context with QCI = 1 where there is a radio bearer associated with that context.

Procedure Transaction Identity: An identity which is dynamically allocated by the UE for the UE requested ESM procedures. The procedure transaction identity is released when the procedure is completed.

RAT-related TMSI: When the UE is camping on an E-UTRAN cell, the RAT-related TMSI is the GUTI; when it is camping on a GERAN or UTRAN cell, the RAT-related TMSI is the P-TMSI.

Registered PLMN: The PLMN on which the UE is registered. The identity of the registered PLMN is provided to the UE within the GUTI.

Relay node: A network element in the E-UTRAN, wirelessly connected to an eNode B and providing relaying function to UEs served by the E-UTRAN. Definition derived from 3GPP TS 23.401 [10].

Removal of eCall only mode restriction: All the limitations as described in 3GPP TS 22.101 [46] for the eCall only mode do not apply any more.

The label (S1 mode only) indicates that this subclause or paragraph applies only to a system which operates in S1 mode, i.e. with a functional division that is in accordance with the use of an S1 interface between the radio access network and the core network. The S1 mode includes WB-S1 mode and NB-S1 mode. In a multi-access system this case is determined by the current serving radio access network.

In NB-S1 mode: Indicates this paragraph applies only to a system which operates in NB-S1 mode. For a multi-access system this case applies if the current serving radio access network provides access to network services via E-UTRA by NB-IoT (see 3GPP TS 36.300 [20], 3GPP TS 36.331 [22], 3GPP TS 36.306 [44]).

In WB-S1 mode: Indicates this paragraph applies only to a system which operates in WB-S1 mode. For a multi-access system this case is applies if the system operates in S1 mode, but not in NB-S1 mode.

SCEF PDN Connection: A PDN connection established between the UE and the Service Capability Exposure Function (SCEF) for transmitting the UE's non-IP data related to a specific application.

SGi PDN Connection: A PDN connection established between the UE and the Packet Gateway (P-GW) for transmitting the UE's IP or non-IP data related to a specific application.

S101 mode: Applies to a system that operates with a functional division that is in accordance with the use of an S101 interface. For the definition of the S101 reference point, see 3GPP TS 23.402 [11].

SIPTO at the local network PDN connection: A PDN connection, for which the default EPS bearer context or default PDP context was activated with an APN authorized to use SIPTO at the local network and it was activated such that the traffic of the PDN connection will be using an L-GW. The network authorizes an APN for using SIPTO at the local network based on the subscription profile (see 3GPP TS 29.272 [16C]) and subsequently the network considers this PDN connection a SIPTO at the local network PDN connection.

SIPTO at the local network PDN connection with a collocated L-GW: A SIPTO at the local network PDN connection which is established to a L-GW function collocated with the (H)(e)NodeB. The core-network entity (i.e. the MME or the SGSN) can be aware of whether the SIPTO at the local network PDN connection with a collocated L-GW is used when the PDN connection is established.

SIPTO at the local network PDN connection with a stand-alone GW: A SIPTO at the local network PDN connection which is established to a stand-alone GW (with collocated L-GW and S-GW). The core-network entity (i.e. the MME or the SGSN) can be aware of whether the SIPTO at the local network PDN connection with a stand-alone GW is used when the PDN connection is established.

"SMS only": A subset of services which includes only Short Message Service. A UE camping on E-UTRAN can attach to both EPS services and "SMS only".

SMS over NAS: refers to SMS in MME or SMS over SGs.

SMS over S102: refers to SMS which uses 1xCS procedures in EPS as defined in 3GPP TS 23.272 [9].

Subscribed APN based congestion control: Congestion control in mobility management where the network can reject attach requests from UEs with a certain APN in the subscription.

TAI list: A list of TAIs that identify the tracking areas that the UE can enter without performing a tracking area updating procedure. The TAIs in a TAI list assigned by an MME to a UE pertain to the same MME area.

Traffic flow aggregate: A temporary aggregate of packet filters that are included in a UE requested bearer resource allocation procedure or a UE requested bearer resource modification procedure and that is inserted into a traffic flow template (TFT) for an EPS bearer context by the network once the UE requested bearer resource allocation procedure or UE requested bearer resource modification procedure is completed.

UE configured for dual priority: A UE which provides dual priority support is configured for NAS signalling low priority and also configured to override the NAS signalling low priority indicator (see 3GPP TS 24.368 [15A], 3GPP TS 31.102 [17]).

UE configured to use AC11 – 15 in selected PLMN: A UE configured with at least one access class in the range 11-15 on the USIM, and the access class is applicable in the selected PLMN according to 3GPP TS 22.011 [1A].

UE's availability for voice calls in the IMS: The indication of this availability or non-availability is provided by the upper layers of the UE as specified in 3GPP TS 24.229 [13D] in the annex relevant to the IP-Connectivity Access Network in use or determined in the NAS layer, as specified in subclause 4.3.1. If availability is indicated, the UE uses the IM CN Subsystem and can terminate or originate requests for SIP sessions including an audio component with codecs suited for voice.

UE's usage setting: This is a UE setting that indicates whether the UE has preference for voice services over data services or vice-versa. If a UE has preference for voice services, then the UE's usage setting is "voice centric". If a UE has preference for data services, then the UE's usage setting is "data centric". A UE whose setting is "data centric" may still require access to voice services. A UE whose setting is "voice centric" may still require access to data services. This definition is derived from 3GPP TS 23.221 [8A] and it applies to voice capable UEs.

UE using EPS services with control plane CIoT EPS optimization: A UE that is attached for EPS services with the control plane CIOT EPS optimization accepted by the network.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.401 [10] apply:

# Cellular IoT (CIoT)

# DCN-ID

# eCall only mode

# NarrowBand-IoT

# Dedicated core network

# PDN connection

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.272 [9] apply:

# CS fallback

# SMS in MME

# SMS over SGs

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.682 [11A] apply:

# SCEF

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.008 [13] apply:

# A/Gb mode

# Access domain selection

# Default PDP context

# Extended idle-mode DRX cycle

# Iu mode

# Power saving mode

# PS signalling connection

# RR connection

# TFT

For the purposes of the present document, the following terms and definitions given in 3GPP TS 33.102 [18] apply:

# UMTS security context

For the purposes of the present document, the following terms and definitions given in 3GPP TS 33.401 [19] apply:

# Current EPS security context

# Full native EPS security context

# KASME

# K'ASME

# Mapped security context

# Native EPS security context

# Non-current EPS security context

# Partial native EPS security context

# Data via MME

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.122 [6] apply:

# Country

# EHPLMN

# Home PLMN

# Shared Network

# Suitable Cell

# Visited PLMN

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.216 [8] apply:

# SRVCC

vSRVCC

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.011 [1A] apply:

# Extended Access Barring

# Application specific Congestion control for Data Communication (ACDC)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.003 [10] apply:

# Local Home Network Identifier

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.303 [31] apply:

# ProSe direct communication

# ProSe direct discovery

# ProSe UE-to-Network Relay

# For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.161 [36] apply:

# Multi-access PDN connection

# NBIFOM

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.167 [45] apply:

# eCall over IMS

* * * * Next Change * * * *

##### 5.5.1.2.2	Attach procedure initiation

In state EMM-DEREGISTERED, the UE initiates the attach procedure by sending an ATTACH REQUEST message to the MME, starting timer T3410 and entering state EMM-REGISTERED-INITIATED (see example in figure 5.5.1.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411.

The UE shall include the IMSI in the EPS mobile identity IE in the ATTACH REQUEST message if the selected PLMN is neither the registered PLMN nor in the list of equivalent PLMNs and:

-	the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17]; or

-	the UE is in NB-S1 mode.

For all other cases, the UE shall handle the EPS mobile identity IE in the ATTACH REQUEST message as follows:

If the UE supports neither A/Gb mode nor Iu mode:

-	the UE shall include in the ATTACH REQUEST message a valid GUTI together with the last visited registered TAI, if available. In addition, the UE shall include Old GUTI type IE with GUTI type set to "native GUTI". If there is no valid GUTI available, the UE shall include the IMSI in the ATTACH REQUEST message.

If the UE supports A/Gb mode or Iu mode or both:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE 1:	The mapping of the P-TMSI and the RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI".

-	If the TIN is deleted and

-	the UE holds a valid GUTI, the UE shall indicate the GUTI in the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "native GUTI";

-	the UE does not hold a valid GUTI but holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the EPS mobile identity IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE; or

-	the UE does not hold a valid GUTI, P-TMSI or RAI, the UE shall include the IMSI in the EPS mobile identity IE.

-	Otherwise the UE shall include the IMSI in the EPS mobile identity IE.

If the UE is attaching for emergency bearer services and does not hold a valid GUTI, P-TMSI or IMSI as described above, the IMEI shall be included in the EPS mobile identity IE.

If the UE supports A/Gb mode or Iu mode or if the UE needs to indicate its UE specific DRX parameter to the network, the UE shall include the UE specific DRX parameter in the DRX parameter IE in the ATTACH REQUEST message.

NOTE 2:	The UE specific DRX parameter is not used by the E-UTRAN for paging from NB-IoT cells (see 3GPP TS 23.401 [10] and 3GPP TS 36.304 [21]).

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the ATTACH REQUEST message.

If the UE supports SRVCC to GERAN/UTRAN, the UE shall set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported".

If the UE supports vSRVCC from S1 mode to Iu mode, then the UE shall set the H.245 after handover capability bit to "H.245 after SRVCC handover capability supported" and additionally set the SRVCC to GERAN/UTRAN capability bit to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported" in the ATTACH REQUEST message.

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

If the UE supports ProSe direct discovery, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct discovery bit to "ProSe direct discovery supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports ProSe direct communication, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct communication bit to "ProSe direct communication supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports acting as a ProSe UE-to-network relay, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe UE-to-network relay bit to "acting as a ProSe UE-to-network relay supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports NB-S1 mode or Non-IP PDN type, then the UE shall support the extended protocol configuration options IE.

If the UE supports the extended protocol configuration options IE, then the UE shall set the ePCO bit to "extended protocol configuration options supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE is in NB-S1 mode, then the UE shall set the control plane CIoT EPS optimization bit to "control plane CIoT EPS optimization supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE is in NB-S1 mode, supports NB-S1 mode only, and requests to attach for EPS services and "SMS only", the UE shall indicate the SMS only requested bit to "SMS only" in the additional update type IE and shall set the EPS attach type IE to "EPS attach" in the ATTACH REQUEST message.

If the UE supports CIoT EPS optimizations, it shall indicate in the UE network capability IE of the ATTACH REQUEST message whether it supports EMM-REGISTERED without PDN connection.

If the UE supports S1-U data transfer and multiple user plane radio bearers (see 3GPP TS 36.306 [44], 3GPP TS 36.331 [22]) in NB-S1 mode, then the UE shall set the Multiple DRB support bit to "Multiple DRB supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports V2X communication over PC5, then the UE shall set the V2X PC5 bit to "V2X communication over PC5 supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports light connection and the attach procedure is initiated in the UE’s HPLMN or EHPLMN, then the UE shall set the LC bit to "light connection supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports light connection, the attach procedure is initiated in a VPLMN and the UE is configured to enable light connection in visited PLMN (see the "LightConnection" leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17]), then the UE shall set the LC bit to "light connection supported" in the UE network capability IE of the ATTACH REQUEST message.

If the UE supports light connection, the attach procedure is initiated in a VPLMN and the UE is configured to disable light connection in visited PLMN (see the "LightConnection" leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17]), then the UE shall set the LC bit to "light connection not supported" in the UE network capability IE of the ATTACH REQUEST message.

If EMM-REGISTERED without PDN connection is not supported by the UE or the MME, or if the UE wants to request PDN connection with the attach procedure, the UE shall send the ATTACH REQUEST message together with a PDN CONNECTIVITY REQUEST message contained in the ESM message container IE.

If EMM-REGISTERED without PDN connection is supported by the UE and the MME, and the UE does not want to request PDN connection with the attach procedure, the UE shall send the ATTACH REQUEST message together with an ESM DUMMY MESSAGE contained in the ESM message container information element.

If a valid EPS security context exists, the UE shall integrity protect the ATTACH REQUEST message combined with the message included in the ESM message container IE. When the UE does not have a valid EPS security context, the ATTACH REQUEST message combined with the message included in the ESM message container IE is not integrity protected.

# Figure 5.5.1.2.2.1: Attach procedure and combined attach procedure

* * * * Next Change * * * *

##### 5.5.3.2.2	Normal and periodic tracking area updating procedure initiation

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME, unless the UE is configured for "AttachWithIMSI" as specified in 3GPP TS 24.368 [15A] or 3GPP TS 31.102 [17] and is entering a tracking area in a new PLMN that is neither the registered PLMN nor in the list of equivalent PLMNs;

b)	when the periodic tracking area updating timer T3412 expires;

c)	when the UE enters EMM-REGISTERED.NORMAL-SERVICE and the UE's TIN indicates "P-TMSI";

d)	when the UE performs an inter-system change from S101 mode to S1 mode and has no user data pending;

e)	when the UE receives an indication from the lower layers that the RRC connection was released with cause "load balancing TAU required";

f)	when the UE deactivated EPS bearer context(s) locally while in EMM-REGISTERED, because it could not establish a NAS signalling connection, and then returns to EMM-REGISTERED.NORMAL-SERVICE and no EXTENDED SERVICE REQUEST message, CONTROL PLANE SERVICE REQUEST message or DETACH REQUEST message is pending to be sent by the UE;

g)	when the UE changes the UE network capability information or the MS network capability information or both;

h)	when the UE changes the UE specific DRX parameter;

i)	when the UE receives an indication of "RRC Connection failure" from the lower layers and has no signalling or user uplink data pending (i.e when the lower layer requests NAS signalling connection recovery);

j)	when the UE enters S1 mode after 1xCS fallback or 1xSRVCC;

k)	when due to manual CSG selection the UE has selected a CSG cell whose CSG identity and associated PLMN identity are not included in the UE's Allowed CSG list or in the UE's Operator CSG list;

l)	when the UE reselects an E-UTRAN cell while it was in GPRS READY state or PMM-CONNECTED mode;

m)	when the UE supports SRVCC to GERAN or UTRAN or supports vSRVCC to UTRAN and changes the mobile station classmark 2 or the supported codecs, or the UE supports SRVCC to GERAN and changes the mobile station classmark 3;

n)	when the UE changes the radio capability for GERAN or cdma2000® or both;

NOTE 1:	In this release of the specifications, a UE can indicate change in the UE's E UTRAN capabilities to the network by performing a detach and then a re-attach (see 3GPP TS 23.401 [10]).

o)	when the UE's usage setting or the voice domain preference for E-UTRAN change in the UE;

p)	when the UE activates mobility management for IMS voice termination as specified in 3GPP TS 24.008 [13], annex P.2, and the TIN indicates "RAT-related TMSI";

q)	when the UE performs an intersystem change from A/Gb mode to S1 mode and the TIN indicates "RAT-related TMSI", but the UE is required to perform tracking area updating for IMS voice termination as specified in 3GPP TS 24.008 [13], annex P.4;

r)	upon reception of a paging indication using S-TMSI, if the timer T3346 is running and the UE is in state EMM-REGISTERED.ATTEMPTING-TO-UPDATE;

s)	when the UE needs to update the network with EPS bearer context status due to local de-activation of EPS bearer context(s) as specified in subclause 6.5.1.4A;

t)	when the UE needs to request the use of PSM or needs to stop the use of PSM;

u)	when the UE needs to request the use of eDRX or needs to stop the use of eDRX;

v)	when a change in the eDRX usage conditions at the UE requires different extended DRX parameters;

w)	when a change in the PSM usage conditions at the UE requires a different timer T3412 value or different timer T3324 value;

NOTE 2:	A change in the PSM or eDRX usage conditions at the UE can include e.g. a change in the UE configuration, a change in requirements from upper layers or the battery running low at the UE.

x)	when the CIoT EPS optimizations the UE needs to use, change in the UE; or

y)	when the Default_DCN_ID value changes, as specified in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17].

NOTE 3:	The tracking area updating procedure is initiated after deleting the DCN-ID list as specified in annex C.

For all cases except case b, the UE shall set the EPS update type IE in the TRACKING AREA UPDATE REQUEST message to "TA updating". For case b, the UE shall set the EPS update type IE to "periodic updating".

For case n, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

For case l, if the TIN indicates "RAT-related TMSI", the UE shall set the TIN to "P-TMSI" before initiating the tracking area updating procedure.

For case r, the "active" flag in the EPS update type IE shall be set to 1.

If the UE is using only control plane CIoT EPS optimization, the case i only applies to the case that the UE has indicated to the network that subsequent to the uplink data transmission a downlink data transmission is expected during the transport of uplink user data via the control plane procedure (see subclause 6.6.4).

If the UE has to request resources for ProSe direct discovery or Prose direct communication (see 3GPP TS 36.331 [22]), then the UE shall set the "active" flag to 1 in the TRACKING AREA UPDATE REQUEST message.

If the UE does not have any established PDN connection, the "active" flag in the EPS update type IE shall be set to 0.

When the UE has user data pending and performs an inter-system change from S101 mode to S1 mode to a tracking area included in the TAI list stored in the UE, the UE shall perform a service request procedure instead of a tracking area updating procedure.

When initiating a tracking area updating procedure while in S1 mode, the UE shall use the current EPS NAS integrity key to integrity protect the TRACKING AREA UPDATE REQUEST message.

In order to indicate its UE specific DRX parameter while in E-UTRAN coverage, the UE shall send the TRACKING AREA UPDATE REQUEST message containing the UE specific DRX parameter in the DRX parameter IE to the network, with the exception of the case if the UE had indicated its DRX parameter (3GPP TS 24.008 [13]) to the network while in GERAN or UTRAN coverage. In this case, when the UE enters E-UTRAN coverage and initiates a tracking area updating procedure, the UE shall not include the UE specific DRX parameter in the DRX parameter IE in the TRACKING AREA UPDATE REQUEST message.

NOTE 4:	The UE specific DRX parameter is not used by the E-UTRAN for paging from NB-IoT cells (see 3GPP TS 23.401 [10] and 3GPP TS 36.304 [21]).

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the TRACKING AREA UPDATE REQUEST message.

If the UE supports PSM and requests the use of PSM, the UE shall include the T3324 value IE with a requested timer value in the TRACKING AREA UPDATE REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

If a UE supporting CIoT EPS optimizations in NB-S1 mode initiates the tracking area updating procedure for EPS services and "SMS only", the UE shall indicate "SMS only" in the Additional update type IE and shall set the EPS update type IE to "TA updating".

If the UE supports S1-U data transfer and multiple user plane radio bearers (see 3GPP TS 36.306 [44], 3GPP TS 36.331 [22]) in NB-S1 mode, then the UE shall set the Multiple DRB support bit to "Multiple DRB supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

If the UE has to request resources for V2X communication over PC5 (see 3GPP TS 23.285 [47]), then the UE shall set the "active" flag to 1 in the TRACKING AREA UPDATE REQUEST message.

After sending the TRACKING AREA UPDATE REQUEST message to the MME, the UE shall start timer T3430 and enter state EMM-TRACKING-AREA-UPDATING-INITIATED (see example in figure 5.5.3.2.2.1). If timer T3402 is currently running, the UE shall stop timer T3402. If timer T3411 is currently running, the UE shall stop timer T3411. If timer T3442 is currently running, the UE shall stop timer T3442.

If the UE supports neither A/Gb mode nor Iu mode, the UE shall include a valid GUTI in the Old GUTI IE in the TRACKING AREA UPDATE REQUEST message. In addition, the UE shall include Old GUTI type IE with GUTI type set to "native GUTI".

If the UE supports A/Gb mode or Iu mode or both, the UE shall handle the Old GUTI IE as follows:

-	If the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the Old GUTI IE, and include Old GUTI type IE with GUTI type set to "mapped GUTI". If a P-TMSI signature is associated with the P-TMSI, the UE shall include it in the Old P-TMSI signature IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE 5:	The mapping of the P-TMSI and RAI to the GUTI is specified in 3GPP TS 23.003 [2].

-	If the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE, and include Old GUTI type IE with GUTI type set to "native GUTI".

If a UE has established PDN connection(s) and uplink user data pending to be sent via user plane when it initiates the tracking area updating procedure, or uplink signalling not related to the tracking area updating procedure when the UE does not support control plane CIoT EPS optimization, it may also set an "active" flag in the TRACKING AREA UPDATE REQUEST message to indicate the request to establish the user plane to the network and to keep the NAS signalling connection after the completion of the tracking area updating procedure.

If a UE is using EPS services with control plane CIoT EPS optimization and has user data pending to be sent via control plane over MME but no user data pending to be sent via user plane, or uplink signalling not related to the tracking area updating procedure, the UE may set a "signalling active" flag in the TRACKING AREA UPDATE REQUEST message to indicate the request to keep the NAS signalling connection after the completion of the tracking area updating procedure.

If the UE has a current EPS security context, the UE shall include the eKSI (either KSIASME or KSISGSN) in the NAS Key Set Identifier IE in the TRACKING AREA UPDATE REQUEST message. Otherwise, the UE shall set the NAS Key Set Identifier IE to the value "no key is available". If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context. Otherwise the UE shall not integrity protect the TRACKING AREA UPDATE REQUEST message.

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode and the TIN is set to "P-TMSI", the UE shall include the GPRS ciphering key sequence number applicable for A/Gb mode or Iu mode and a nonceUE in the TRACKING AREA UPDATE REQUEST message.

When the tracking area updating procedure is initiated in EMM-CONNECTED mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode, the UE shall derive the EPS NAS keys from the mapped K'ASME using the selected NAS algorithms, nonceMME and KSISGSN (to be associated with the mapped K'ASME) provided by lower layers as indicated in 3GPP TS 33.401 [19]. The UE shall reset both the uplink and downlink NAS COUNT counters of the mapped EPS security context which shall be taken into use. If the UE has a non-current native EPS security context, the UE shall include the KSIASME in the Non-current native NAS key set identifier IE and its associated GUTI, as specified above, either in the Old GUTI IE or in the Additional GUTI IE of the TRACKING AREA UPDATE REQUEST message. The UE shall set the TSC flag in the Non-current native NAS key set identifier IE to "native security context".

When the tracking area updating procedure is initiated in EMM-IDLE mode, the UE may also include an EPS bearer context status IE in the TRACKING AREA UPDATE REQUEST message, indicating which EPS bearer contexts are active in the UE. The UE shall include the EPS bearer context status IE in TRACKING AREA UPDATE REQUEST message:

-	for the case f;

-	for the case s; and

-	if the UE has established PDN connection(s) of "non IP" PDN type.

If the UE initiates the first tracking area updating procedure following an attach in A/Gb mode or Iu mode, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports SRVCC to GERAN/UTRAN, the UE shall set the SRVCC to GERAN/UTRAN capability bit in the MS network capability IE to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported".

For all cases except case b, if the UE supports vSRVCC from S1 mode to Iu mode, then the UE shall set the H.245 after handover capability bit in the UE network capability IE to "H.245 after SRVCC handover capability supported" and additionally set the SRVCC to GERAN/UTRAN capability bit in the MS network capability IE to "SRVCC from UTRAN HSPA or E-UTRAN to GERAN/UTRAN supported" in the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports ProSe direct discovery, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct discovery bit to "ProSe direct discovery supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports ProSe direct communication, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe direct communication bit to "ProSe direct communication supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports acting as a ProSe UE-to-network relay, then the UE shall set the ProSe bit to "ProSe supported" and set the ProSe UE-to-network relay bit to "acting as a ProSe UE-to-network relay supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

If the UE supports NB-S1 mode or Non-IP PDN type, then the UE shall support the extended protocol configuration options IE.

For all cases except case b, if the UE supports the extended protocol configuration options IE, then the UE shall set the ePCO bit to "extended protocol configuration options supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports V2X communication over PC5, then the UE shall set the V2X PC5 bit to "V2X communication over PC5 supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports light connection and the tracking area updating procedure is initiated in the UE’s HPLMN or EHPLMN, then the UE shall set the LC bit to "light connection supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports light connection, the tracking area updating procedure is initiated in a VPLMN and the UE is configured to enable light connection in visited PLMN (see the "LightConnection" leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17]), then the UE shall set the LC bit to "light connection supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

For all cases except case b, if the UE supports light connection, the tracking area updating procedure is initiated in a VPLMN and the UE is configured to disable light connection in visited PLMN (see the "LightConnection" leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] or in USIM file NASCONFIG as specified in 3GPP TS 31.102 [17]), then the UE shall set the LC bit to "light connection not supported" in the UE network capability IE of the TRACKING AREA UPDATE REQUEST message.

# Figure 5.5.3.2.2.1: Tracking area updating procedure

* * * * Next Change * * * *

#### 9.9.3.34	UE network capability

The purpose of the UE network capability information element is to provide the network with information concerning aspects of the UE related to EPS or interworking with GPRS. The contents might affect the manner in which the network handles the operation of the UE. The UE network capability information indicates general UE characteristics and it shall therefore, except for fields explicitly indicated, be independent of the frequency band of the channel it is sent on.

The UE network capability information element is coded as shown in figure 9.9.3.34.1 and table 9.9.3.34.1.

The UE network capability is a type 4 information element with a minimum length of 4 octets and a maximum length of 15 octets.

NOTE:	The requirements for the support of UMTS security algorithms in the UE are specified in 3GPP TS 33.102 [18], and the requirements for the support of EPS security algorithms in 3GPP TS 33.401 [19].

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE network capability IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of UE network capability contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| EEA0 |  | 128- EEA1 |  | 128- EEA2 |  | 128- EEA3 |  | EEA4 |  | EEA5 |  | EEA6 |  | EEA7 |  | octet 3 |  |
| EIA0 |  | 128- EIA1 |  | 128- EIA2 |  | 128- EIA3 |  | EIA4 |  | EIA5 |  | EIA6 |  | EIA7 |  | octet 4 |  |
| UEA0 |  | UEA1 |  | UEA2 |  | UEA3 |  | UEA4 |  | UEA5 |  | UEA6 |  | UEA7 |  | octet 5* |  |
| UCS2 |  | UIA1 |  | UIA2 |  | UIA3 |  | UIA4 |  | UIA5 |  | UIA6 |  | UIA7 |  | octet 6* |  |
| ProSe-dd |  | ProSe |  | H.245-ASH |  | ACC-CSFB |  | LPP |  | LCS |  | 1xSR VCC |  | NF |  | octet 7* |  |
| ePCO |  | HC-CP CIoT |  | ERw/oPDN |  | S1-U data |  | UP CIoT |  | CP CIoT |  | Prose-relay |  | ProSe-dc |  | octet 8* |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | LC |  | V2X PC5 |  | multipleDRB |  | octet 9* |  |
| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | octet 10* -15* |  |
| Spare |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Figure 9.9.3.34.1: UE network capability information element

Table 9.9.3.34.1: UE network capability information element

| EPS encryption algorithms supported (octet 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| EPS encryption algorithm EEA0 supported (octet 3, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA0 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA0 supported |
| EPS encryption algorithm 128-EEA1 supported (octet 3, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA1 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA1 supported |
| EPS encryption algorithm 128-EEA2 supported (octet 3, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA2 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA2 supported |
| EPS encryption algorithm 128-EEA3 supported (octet 3, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm 128-EEA3 not supported |
| 1 |  |  |  |  | EPS encryption algorithm 128-EEA3 supported |
| EPS encryption algorithm EEA4 supported (octet 3, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA4 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA4 supported |
| EPS encryption algorithm EEA5 supported (octet 3, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA5 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA5 supported |
| EPS encryption algorithm EEA6 supported (octet 3, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA6 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA6 supported |
| EPS encryption algorithm EEA7 supported (octet 3, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | EPS encryption algorithm EEA7 not supported |
| 1 |  |  |  |  | EPS encryption algorithm EEA7 supported |
| EPS integrity algorithms supported (octet 4) |  |  |  |  |  |
| EPS integrity algorithm EIA0 supported (octet 4, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA0 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA0 supported |
| EPS integrity algorithm 128-EIA1 supported (octet 4, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA1 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA1 supported |
| EPS integrity algorithm 128-EIA2 supported (octet 4, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA2 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA2 supported |
| EPS integrity algorithm 128-EIA3 supported (octet 4, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm 128-EIA3 not supported |
| 1 |  |  |  |  | EPS integrity algorithm 128-EIA3 supported |
| EPS integrity algorithm EIA4 supported (octet 4, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA4 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA4 supported |
| EPS integrity algorithm EIA5 supported (octet 4, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA5 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA5 supported |
| EPS integrity algorithm EIA6 supported (octet 4, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA6 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA6 supported |
| EPS integrity algorithm EIA7 supported (octet 4, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | EPS integrity algorithm EIA7 not supported |
| 1 |  |  |  |  | EPS integrity algorithm EIA7 supported |
| UMTS encryption algorithms supported (octet 5) |  |  |  |  |  |
| UMTS encryption algorithm UEA0 supported (octet 5, bit 8) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA0 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA0 supported |
| UMTS encryption algorithm UEA1 supported (octet 5, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA1 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA1 supported |
| UMTS encryption algorithm UEA2 supported (octet 5, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA2 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA2 supported |
| UMTS encryption algorithm UEA3 supported (octet 5, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA3 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA3 supported |
| UMTS encryption algorithm UEA4 supported (octet 5, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA4 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA4 supported |
| UMTS encryption algorithm UEA5 supported (octet 5, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA5 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA5 supported |
| UMTS encryption algorithm UEA6 supported (octet 5, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA6 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA6 supported |
| UMTS encryption algorithm UEA7 supported (octet 5, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS encryption algorithm UEA7 not supported |
| 1 |  |  |  |  | UMTS encryption algorithm UEA7 supported |
| UCS2 support (UCS2) (octet 6, bit 8) |  |  |  |  |  |
| This information field indicates the likely treatment of UCS2 encoded character strings by the UE. |  |  |  |  |  |
| 0 |  |  |  |  | The UE has a preference for the default alphabet (defined in |
|  |  |  |  |  | 3GPP TS 23.038 [3]) over UCS2 (see ISO/IEC 10646 [29]). |
| 1 |  |  |  |  | The UE has no preference between the use of the default alphabet and |
|  |  |  |  |  | the use of UCS2. |
| UMTS integrity algorithms supported (octet 6) |  |  |  |  |  |
| UMTS integrity algorithm UIA1 supported (octet 6, bit 7) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA1 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA1 supported |
| UMTS integrity algorithm UIA2 supported (octet 6, bit 6) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA2 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA2 supported |
| UMTS integrity algorithm UIA3 supported (octet 6, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA3 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA3 supported |
| UMTS integrity algorithm UIA4 supported (octet 6, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA4 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA4 supported |
| UMTS integrity algorithm UIA5 supported (octet 6, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA5 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA5 supported |
| UMTS integrity algorithm UIA6 supported (octet 6, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA6 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA6 supported |
| UMTS integrity algorithm UIA7 supported (octet 6, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | UMTS integrity algorithm UIA7 not supported |
| 1 |  |  |  |  | UMTS integrity algorithm UIA7 supported |
| NF capability (octet 7, bit 1) |  |  |  |  |  |
| 0 |  |  |  |  | notification procedure not supported |
| 1 |  |  |  |  | notification procedure supported |
| 1xSRVCC capability (octet 7, bit 2) |  |  |  |  |  |
| 0 |  |  |  |  | SRVCC from E-UTRAN to cdma2000® 1x CS not supported |
| 1 |  |  |  |  | SRVCC from E-UTRAN to cdma2000® 1x CS supported |
|  |  |  |  |  | (see 3GPP TS 23.216 [8]) |
| Location services (LCS) notification mechanisms capability (octet 7, bit 3) |  |  |  |  |  |
| 0 |  |  |  |  | LCS notification mechanisms not supported |
| 1 |  |  |  |  | LCS notification mechanisms supported (see 3GPP TS 24.171 [13C]) |
| LTE Positioning Protocol (LPP) capability (octet 7, bit 4) |  |  |  |  |  |
| 0 |  |  |  |  | LPP not supported |
| 1 |  |  |  |  | LPP supported (see 3GPP TS 36.355 [22A]) |
| Access class control for CSFB (ACC-CSFB) capability (octet 7, bit 5) |  |  |  |  |  |
| 0 |  |  |  |  | eNodeB-based access class control for CSFB not supported |
| 1 |  |  |  |  | eNodeB-based access class control for CSFB supported (see 3GPP TS 22.011 [1A]) |
| H.245 After SRVCC Handover capability (H.245-ASH) (octet 7, bit 6) This bit indicates the capability for H.245 with support and use of pre-defined codecs, and if needed, H.245 codec negotiation after SRVCC handover. |  |  |  |  |  |
| 0 |  |  |  |  | H.245 after SRVCC handover capability not supported |
| 1 |  |  |  |  | H.245 after SRVCC handover capability supported (see 3GPP TS 23.216 [8]) |
| ProSe (octet 7, bit 7) This bit indicates the capability for ProSe. |  |  |  |  |  |
| 0 |  |  |  |  | ProSe not supported |
| 1 |  |  |  |  | ProSe supported |
| ProSe direct discovery (ProSe-dd) (octet 7, bit 8) This bit indicates the capability for ProSe direct discovery. |  |  |  |  |  |
| 0 |  |  |  |  | ProSe direct discovery not supported |
| 1 |  |  |  |  | ProSe direct discovery supported |
| ProSe direct communication (ProSe-dc) (octet 8, bit 1) This bit indicates the capability for ProSe direct communication. |  |  |  |  |  |
| 0 |  |  |  |  | ProSe direct communication not supported |
| 1 |  |  |  |  | ProSe direct communication supported |
| ProSe UE-to-network-relay (ProSe-relay) (octet 8, bit 2) This bit indicates the capability to act as a ProSe UE-to-network relay |  |  |  |  |  |
| 0 |  |  |  |  | Acting as a ProSe UE-to-network relay not supported |
| 1 |  |  |  |  | Acting as a ProSe UE-to-network relay supported |
| Control plane CIoT EPS optimization (CP CIoT) (octet 8, bit 3) This bit indicates the capability for control plane CIoT EPS optimization. |  |  |  |  |  |
| 0 |  |  |  |  | Control plane CIoT EPS optimization not supported |
| 1 |  |  |  |  | Control plane CIoT EPS optimization supported |
| User plane CIoT EPS optimization (UP CIoT) (octet 8, bit 4) This bit indicates the capability for user plane CIoT EPS optimization. |  |  |  |  |  |
| 0 |  |  |  |  | User plane CIoT EPS optimization not supported |
| 1 |  |  |  |  | User plane CIoT EPS optimization supported |
| S1-u data transfer (S1-U data) (octet 8, bit 5) This bit indicates the capability for S1-u data transfer. |  |  |  |  |  |
| 0 |  |  |  |  | S1-U data transfer not supported |
| 1 |  |  |  |  | S1-U data transfer supported |
| EMM-REGISTERED without PDN connection (ERw/oPDN) (octet 8, bit 6) This bit indicates the capability for EMM REGISTERED without PDN connectivity. |  |  |  |  |  |
| 0 |  |  |  |  | EMM-REGISTERED without PDN connection not supported |
| 1 |  |  |  |  | EMM-REGISTERED without PDN connection supported |
| Header compression for control plane CIoT EPS optimization (HC-CP CIoT) (octet 8, bit 7) This bit indicates the capability for header compression for control plane CIoT EPS optimization. |  |  |  |  |  |
| 0 |  |  |  |  | Header compression for control plane CIoT EPS optimization not supported |
| 1 |  |  |  |  | Header compression for control plane CIoT EPS optimization supported |
| Extended protocol configuration options (ePCO) (octet 8, bit 8) This bit indicates the support of the extended protocol configuration options IE. |  |  |  |  |  |
| 0 |  |  |  |  | Extended protocol configuration options IE not supported |
| 1 |  |  |  |  | Extended protocol configuration options IE supported |
| Multiple DRB support (multipleDRB) (octet 9, bit 1) This bit indicates the capability to support multiple user plane radio bearers (see 3GPP TS 36.306 [44], 3GPP TS 36.331 [22]) in NB-S1 mode. |  |  |  |  |  |
| 0 |  |  |  |  | Multiple DRB not supported |
| 1 |  |  |  |  | Multiple DRB supported |
| V2X communication over PC5 (V2X PC5) (octet 9, bit 2) This bit indicates the capability for V2X communication over PC5. |  |  |  |  |  |
| 0 |  |  |  |  | V2X communication over PC5 not supported |
| 1 |  |  |  |  | V2X communication over PC5 supported |
| Light Connection (LC) (octet 9, bit 3) This bit indicates the capability for light connection. |  |  |  |  |  |
| 0 |  |  |  |  | light connection not supported |
| 1 |  |  |  |  | light connection supported |
| All other bits in octet 9 to 15 are spare and shall be coded as zero, if the respective octet is included in the information element. |  |  |  |  |  |

* * * * End of Change * * * *