3GPP TSG CT WG4 Meeting #85	C4-184457

Osaka, Japan, 21st – 25th May 2018	was C4-184033

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.244 | CR | 0105 | rev | 1 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Duplicating the user plane packets to multiple destinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Verizon, Cisco |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | CUPS-CT |  |  |  |  |  |  | Date: |  | 2018-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is not clear that in TS 29.244, whether it is possible to require UP function to duplicate the same user plane traffic to more than one destination. At least, to support lawful interception, it should be possible, e.g. for different target IDs, e.g. IMSI, IMEI, the same user plane packets are duplicated to different SX3LIFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the Duplicating Parameters in the Create FAR IE may have multiple instances, e.g. when the same user plane packets need to be duplicated to different SX3LIF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulator requirements may not be satisfied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.3, 7.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.5.2.3	Create FAR IE within Sx Session Establishment Request

The Create FAR grouped IE shall be encoded as shown in Figure 7.5.2.3-1.

Table 7.5.2.3-1: Create FAR IE within Sx Session Establishment Request

| Octet 1 and 2 |  | Create FAR IE Type = 3 (decimal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc |  |
| FAR ID | M | This IE shall uniquely identify the FAR among all the FARs configured for that Sx session. | X | X | X | FAR ID |
| Apply Action | M | This IE shall indicate the action to apply to the packets, See subclauses 5.2.1 and 5.2.3. | X | X | X | Apply Action |
| Forwarding Parameters | C | This IE shall be present when the Apply-Action requests the packets to be forwarded. It may be present otherwise. When present, this IE shall contain the forwarding instructions to be applied by the UP function when the Apply-Action requests the packets to be forwarded. See table 7.5.2.3-2. | X | X | X | Forwarding Parameters |
| Duplicating Parameters | C | This IE shall be present when the Apply-Action requests the packets to be duplicated. It may be present otherwise. When present, this IE shall contain the forwarding instructions to be applied by the UP function for the traffic to be duplicated, when the Apply-Action requests the packets to be duplicated. Several IEs with the same IE type may be present to request to duplicate the packets to different destinations. See NOTE 1. See table 7.5.2.3-3. | X | X | - | Duplicating Parameters |
| BAR ID | O | When present, this IE shall contain the BAR ID of the BAR defining the buffering instructions to be applied by the UP function when the Apply Action requests the packets to be buffered. See table 7.5.2.6-1. | X | - | - | BAR ID |
| NOTE 1: The same user plane packets may be required, according to operator's policy and configuration, to be duplicated to different SX3LIFs. |  |  |  |  |  |  |

Table 7.5.2.3-2: Forwarding Parameters IE in FAR

| Octet 1 and 2 |  | Forwarding Parameters IE Type = 4 (decimal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc |  |
| Destination Interface | M | This IE shall identify the destination interface of the outgoing packet. | X | X | X | Destination Interface |
| Network Instance | O | When present, this IE shall identify the Network instance towards which to send the outgoing packet. See NOTE 1. | X | X | X | Network Instance |
| Redirect Information | C | This IE shall be present if the UP function is required to enforce traffic redirection towards a redirect destination provided by the CP function. | - | X | X | Redirect Information |
| Outer Header Creation | C | This IE shall be present if the UP function is required to add one or more outer header(s) to the outgoing packet. If present, it shall contain the F-TEID of the remote GTP-U peer when adding a GTP-U/UDP/IP header, or the Destination IP address and Port Number when adding a UDP/IP header. | X | X | - | Outer Header Creation |
| Transport Level Marking | C | This IE shall be present if the UP function is required to mark the IP header with the DSCP marking as defined by IETF RFC 2474 [22]. When present, it shall contain the value of the DSCP in the TOS/Traffic Class field set based on the QCI, and optionally the ARP priority level, of the associated EPS bearer, as described in sub-clause 4.7.3 of 3GPP TS 23.214 [2]. | X | X | - | Transport Level Marking |
| Forwarding Policy | C | This IE shall be present if a specific forwarding policy is required to be applied to the packets. It shall be present if the Destination Interface IE is set to SGi-LAN. It may be present if the Destination Interface is set to Core. When present, it shall contain an Identifier of the Forwarding Policy locally configured in the UP function. | - | X | X | Forwarding Policy |
| Header Enrichment | O | This IE may be present if the UP function indicated support of Header Enrichment of UL traffic. When present, it shall contain information for header enrichment. | - | X | X | Header Enrichment |
| Linked Traffic Endpoint ID | C | This IE may be present, if it is available and the UP function indicated support of the PDI optimisation feature, (see subclause 8.2.25). When present, it shall identify the Traffic Endpoint ID allocated for this Sx session to receive the traffic in the reverse direction (see subclause 5.2.3.1). | X | X | - | Traffic Endpoint ID |
| NOTE 1: The Network Instance parameter is needed e.g. in the following cases: - PGW/TDF UP function supports multiple PDNs with overlapping IP addresses; - SGW UP function is connected to PGWs in different IP domains (S5/S8); - PGW UP function is connected to SGWs in different IP domains (S5/S8); - SGW UP function is connected to eNodeBs in different IP domains. |  |  |  |  |  |  |

Table 7.5.2.3-3: Duplicating Parameters IE in FAR

| Octet 1 and 2 |  | Duplicating Parameters IE Type = 5 (decimal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc |  |
| Destination Interface | M | This IE shall identify the destination interface of the outgoing packet. | X | X | - | Destination Interface |
| Outer Header Creation | C | This IE shall be present if the UP function is required to add one or more outer header(s) to the outgoing packet. If present, it shall contain the F-TEID of the remote GTP-U peer. | X | X | - | Outer Header Creation |
| Transport Level marking | C | This IE shall be present if the UP function is required to mark the IP header with the DSCP marking as defined by IETF RFC 2474 [22]. When present, it shall contain the value of the DSCP in the TOS/Traffic Class field. | X | X | - | Transport Level Marking |
| Forwarding Policy | C | This IE shall be present if a specific forwarding policy is required to be applied to the packets. When present, it shall contain an Identifier of the Forwarding Policy locally configured in the UP function. | X | X | - | Forwarding Policy |

* * * Next Change * * * *

#### 7.5.4.3	Update FAR IE within Sx Session Modification Request

The Update FAR grouped IE shall be encoded as shown in Figure 7.5.4.3-1.

Table 7.5.4.3-1: Update FAR IE within Sx Session Modification Request

| Octet 1 and 2 |  | Update FAR IE Type = 10 (decimal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc |  |
| FAR ID | M | This IE shall identify the FAR to be updated. | X | X | X | FAR ID |
| Apply Action | C | This IE shall be present if it is changed. | X | X | X | Apply Action |
| Update Forwarding parameters | C | This IE shall be present if it is changed.See table 7.5.4.3-2. | X | X | X | Update Forwarding Parameters |
| Update Duplicating Parameters | C | This IE shall be present if it is changed. See table 7.5.4.3-3. Several IEs with the same IE type may be present to request to duplicate the packets to different destinations. | X | X | - | Update Duplicating Parameters |
| BAR ID | C | This IE shall be present if the BAR ID associated to the FAR needs to be modified. See Table 7.5.4.11-1. | X | - | - | BAR ID |
| NOTE: The IEs which do not need to be modified shall not be included in the Update FAR IE. The UP function shall continue to behave according to the values previously received for IEs not present in the Update FAR IE. |  |  |  |  |  |  |

Table 7.5.4.3-2: Update Forwarding Parameters IE in FAR

| Octet 1 and 2 |  | Update Forwarding Parameters IE Type = 11 (decimal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc |  |
| Destination Interface | C | This IE shall only be provided if it is changed. When present, it shall indicate the destination interface of the outgoing packet. | X | X | X | Destination Interface |
| Network instance | C | This IE shall only be provided if it is changed. | X | X | X | Network Instance |
| Redirect Information | C | This IE shall be present if the instructions regarding the redirection of traffic by the UP function need to be modified. | - | X | X | Redirect Information |
| Outer Header Creation | C | This IE shall only be provided if it is changed | X | X | - | Outer Header Creation |
| Transport Level Marking | C | This IE shall only be provided if it is changed | X | X | - | Transport Level Marking |
| Forwarding Policy | C | This IE shall only be provided if it is changed | - | X | X | Forwarding Policy |
| Header Enrichment | C | This IE shall only be provided if it is changed | - | X | X | Header Enrichment |
| SxSMReq-Flags | C | This IE shall be included if at least one of the flags is set to 1. - SNDEM (Send End Marker Packets): this IE shall be present if the CP function modifies the F-TEID of the downstream node in the Outer Header Creation IE and the CP function requests the UP function to construct and send GTP-U End Marker messages towards the old F-TEID of the downstream node. | X | X | - | SxSMReq-Flags |
| Linked Traffic Endpoint ID | C | This IE may be present, if it is changed and the UP function indicated support of the PDI optimization feature, (see subclause 8.2.25). When present, it shall identify the Traffic Endpoint ID allocated for this Sx session to receive the traffic in the reverse direction (see subclause 5.2.3.1). | X | X | - | Traffic Endpoint ID |

Table 7.5.4.3-3: Update Duplicating Parameters IE in FAR

| Octet 1 and 2 |  | Update Duplicating Parameters IE Type = 105 (decimal) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets 3 and 4 |  | Length = n |  |  |  |  |
| Information elements | P | Condition / Comment | Appl. |  |  | IE Type |
|  |  |  | Sxa | Sxb | Sxc |  |
| Destination Interface | C | This IE shall only be provided if it is changed. When present, it shall indicate the destination interface of the outgoing packet. | X | X | - | Destination Interface |
| Outer Header Creation | C | This IE shall only be provided if it is changed. | X | X | - | Outer Header Creation |
| Transport Level Marking | C | This IE shall only be provided if it is changed. | X | X | - | Transport Level Marking |
| Forwarding Policy | C | This IE shall only be provided if it is changed. | - | X | - | Forwarding Policy |

* * * End of Changes * * * *