3GPP TSG-CT Meeting #85	CP-192075

# Newport Beach, US; 16th – 17th September 2019             (Revision of C4-193833, C4-193670, C4-193392)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.518 | CR | 0223 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Small Data Rate Control Status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2019-09-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Alignment with stage 2, For small data rate control: The Small Data Rate Control Statuses are transferred as part of UE context between AMFs. Acoordingly, in stage 2 specificaton 23.502 clause 4.3.4.2 step 3, we can read: If the UPF included Small Data Rate Control Status in step 2 then the SMF includes Small Data Rate Control Status in the request or response to the AMF in steps 3a-3d. And 23.502 clause 5.2.2.2.2 Namf_Communication_UEContextTransfer service operation, Small Data Rate Control Status is added in the UE context, see Table 5.2.2.2.2-1: UE Context in AMF is this clause. Based on the information above, changes as below are needed: small data rate status is added in response message body of Namf_Communication_UEContextTransfer service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add smallDataRateStatusInfos in UeContext which is used by Individual ueContext resource transfer operation as part of response message boby. - Revsion of Yaml file accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Small data rate control won’t be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.25, 6.1.6.2.x (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.571 CR 0121 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features in the OpenAPI specification file of Namf_Communication API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Revoke the change on N1N2MessageTransfer; SmallDataRateStatusInfo is based on DNN and S-NSSAI. Rev.2: Add minItems: 1 for the array in OpenAPI. Rev.3: (for CT#85): Fixes to OpenAPI: SmallDataRateStatusInfo shall be SmallDataRateStatusInfo: |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Namf_Communication service based interface protocol.

Table 6.1.6.1-1: Namf_Communication specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SubscriptionData | 6.1.6.2.2 | Information within AMFStatusChangeSubscribe |  |
| AmfStatusChangeNotification | 6.1.6.2.3 | Information within AMFStatusChangeNotify |  |
| AmfStatusInfo | 6.1.6.2.4 | Information within AMFStatusChangeNotify |  |
| AssignEbiData | 6.1.6.2.5 | Represents information needed for AMF to assign EBIs. |  |
| AssignedEbiData | 6.1.6.2.6 | Represents successful assignment of EBI(s). |  |
| AssignEbiFailed | 6.1.6.2.7 | Represents failed assignment of EBI(s) |  |
| UEContextRelease | 6.1.6.2.8 | Information within ReleaseUeContext |  |
| N2InformationTransferReqData | 6.1.6.2.9 | N2 information requested to be transferred to 5G AN. |  |
| NonUeN2InfoSubscriptionCreateData | 6.1.6.2.10 | Subscription information for non UE specific N2 information notification. |  |
| NonUeN2InfoSubscriptionCreatedData | 6.1.6.2.11 | The created subscription for non UE specific N2 information notification. |  |
| UeN1N2InfoSubscriptionCreateData | 6.1.6.2.12 | Subscription information for UE specific N1 and/or N2 information notification. |  |
| UeN1N2InfoSubscriptionCreatedData | 6.1.6.2.13 | The created subscription for UE specific N1 and/or N2 information notification. |  |
| N2InformationNotification | 6.1.6.2.14 | N2 information for notification. |  |
| N2InfoContainer | 6.1.6.2.15 | N2 information container. |  |
| N1MessageNotification | 6.1.6.2.16 | N1 message notification data structure. |  |
| N1MessageContainer | 6.1.6.2.17 | N1 Message Container |  |
| N1N2MessageTransferReqData | 6.1.6.2.18 | N1/N2 message container |  |
| N1N2MessageTransferRspData | 6.1.6.2.19 | N1/N2 message transfer response |  |
| RegistrationContextContainer | 6.1.6.2.20 | Registration Context Container used to send the UE context information, N1 message from UE, AN address etc during Registration with AMF re-allocation procedure. |  |
| AreaOfValidity | 6.1.6.2.21 | Area of validity information for N2 information transfer |  |
| UeContextTransferReqData | 6.1.6.2.23 | Represents to start transferring of an individual ueContext resource from old AMF to new AMF. |  |
| UeContextTransferRspData | 6.1.6.2.24 | Indicates the transferring of the individual ueContext resource is started successfully. |  |
| UeContext | 6.1.6.2.25 | Represents an individual ueContext resource |  |
| N2SmInformation | 6.1.6.2.26 | Represents the session management SMF related N2 information data part. |  |
| N2InfoContent | 6.1.6.2.27 | Represents a transparent N2 information content to be relayed by AMF. |  |
| NrppaInformation | 6.1.6.2.28 | Represents a NRPPa related N2 information data part. |  |
| PwsInformation | 6.1.6.2.29 | Represents a PWS related information data part. |  |
| N1N2MsgTxfrFailureNotification | 6.1.6.2.30 | N1/N2 Message Transfer Failure Notification |  |
| N1N2MessageTransferError | 6.1.6.2.31 | N1/N2 Message Transfer Error |  |
| N2N2MsgTxfrErrDetail | 6.1.6.2.32 | N1/N2 Message Transfer Error Details |  |
| N2InformationTransferRspData | 6.1.6.2.33 | Indicates a successful delivery of N2 Information to the AN. |  |
| MmContext | 6.1.6.2.34 | Represents a Mobility Management Context in UE Context |  |
| SeafData | 6.1.6.2.35 | Represents SEAF data derived from data received from AUSF |  |
| NasSecurityMode | 6.1.6.2.36 | Indicates the NAS Security Mode |  |
| PduSessionContext | 6.1.6.2.37 | Represents a PDU Session Context in UE Context |  |
| NssaiMapping | 6.1.6.2.38 | Represents a map of a S-NSSAI in serving PLMN to a S-NSSAI in home PLMN. |  |
| UeRegStatusUpdateReqData | 6.1.6.2.39 | Provides information on the UE registration completion at a target AMF. |  |
| AssignEbiError | 6.1.6.3.40 | Represents the details regarding EBI assignment failure. |  |
| UeContextCreateData | 6.1.6.2.41 | Indicates a request to create an individual ueContext resource |  |
| UeContextCreatedData | 6.1.6.2.42 | Indicates a successful creation of an individual ueContext resource |  |
| UeContextCreateError | 6.1.6.2.43 | Represents an error when creating a UE context |  |
| NgRanTargetId | 6.1.6.2.44 | Indicates a NG RAN as target of the handover |  |
| N2InformationTransferError | 6.1.6.2.45 | Error within NonUeN2MessageTransfer response |  |
| PWSResponseData | 6.1.6.2.46 | Represents the type of PWS |  |
| PWSErrorData | 6.1.6.2.47 | Represents the type of PWS error |  |
| NgKsi | 6.1.6.2.49 | Represents the ngKSI (see 3GPP TS 33.501 [27]) |  |
| KeyAmf | 6.1.6.2.50 | Represents the Kamf or K'amf. (see 3GPP TS 33.501 [27]). |  |
| ExpectedUeBehavior | 6.1.6.2.51 | Represents the expected UE behavior (e.g. UE moving trajectory) and its validity period. |  |
| UeRegStatusUpdateRspData | 6.1.6.2.52 | Provides the status of UE context transfer status update at a source AMF. |  |
| N2RanInformation | 6.1.6.2.53 | Represents the RAN related N2 information data part. |  |
| N2InfoNotificationRspData | 6.1.6.2.54 | N2 information notification response data |  |
| SmallDataRateStatusInfo | 6.1.6.2.x | Represents the small data rate status |  |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Identifier |  |
| Ppi | 6.1.6.3.2 | Paging Policy Indicator |  |
| NasCount | 6.1.6.3.2 | Represents a NAS COUNT |  |
| 5GMmCapability | 6.1.6.3.2 | Represents a 5GMM capability |  |
| UeSecurityCapability | 6.1.6.3.2 | Represents a UE Security Capability |  |
| S1UeNetworkCapability | 6.1.6.3.2 | Represents a S1 UE Network Capability |  |
| DrxParameter | 6.1.6.3.2 | Indicates the UE DRX Parameters |  |
| OmcIdentifier | 6.1.6.3.2 | Represents the OMC Identifier |  |
| StatusChange | 6.1.6.3.3 |  |  |
| N2InformationClass | 6.1.6.3.4 |  |  |
| N1MessageClass | 6.1.6.3.5 |  |  |
| N1N2MessageTransferCause | 6.1.6.3.6 |  |  |
| UeContextTransferStatus | 6.1.6.3.7 | Describes the status of an individual ueContext resource in UE Context Transfer procedures |  |
| N2InformationTransferResult | 6.1.6.3.8 | Describes the result of N2 information transfer by AMF to the AN. |  |
| CipheringAlgorithm | 6.1.6.3.9 | Indicates the supported Ciphering Algorithm |  |
| IntegrityAlgorithm | 6.1.6.3.10 | Indicates the supported Integrity Algorithm |  |
| SmsSupport | 6.1.6.3.11 | Indicates the supported SMS delivery of a UE. |  |
| ScType | 6.1.6.3.12 | Indicates the security context type. |  |
| KeyAmfType | 6.1.6.3.13 | Indicates the Kamf type. |  |
| TransferReason | 6.1.6.3.14 | Indicates UE Context Transfer Reason |  |
| AMPolicyReqTrigger | 6.1.6.3.15 | AM Policy Request Triggers |  |
| RatSelector | 6.1.6.3.16 | Indicates the RAT type for the transfer of N2 information |  |
| NgapIeType | 6.1.6.3.17 | Indicates the supported NGAP IE types |  |
| N2InfoNotifyReason | 6.1.6.3.18 | N2 Information Notify Reason |  |
| SmfChangeIndication | 6.1.6.3.x | Indicates the I-SMF or V-SMF insertion, change or removal | DTSSA |

Table 6.1.6.1-2 specifies data types re-used by the Namf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Namf service based interface.

Table 6.1.6.1-2: Namf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Snssai | 3GPP TS 29.571 [6] |  |  |
| Arp | 3GPP TS 29.571 [6] |  |  |
| PduSesisonId | 3GPP TS 29.571 [6] |  |  |
| Guami | 3GPP TS 29.571 [6] | Globally Unique AMF Identifier |  |
| AmfName | 3GPP TS 29.571 [6] | The name of the AMF |  |
| Supi | 3GPP TS 29.571 [6] | Subscription Permanent Identifier |  |
| IndicationFlags | 3GPP TS 29.502 [16] | Indication Flags |  |
| Cause | 3GPP TS 29.571 [6] | 5G-AN Cause |  |
| ProblemDetails | 3GPP TS 29.571 [6] | Detailed problems in failure case |  |
| supportedFeatures | 3GPP TS 29.571 [6] | Supported Features |  |
| TimeZone | 3GPP TS 29.571 [6] |  |  |
| UserLocation | 3GPP TS 29.571 [6] |  |  |
| AccessType | 3GPP TS 29.571 [6] |  |  |
| AllowedNssai | 3GPP TS 29.531 [18] |  |  |
| NfInstanceId | 3GPP TS 29.571 [6] |  |  |
| Uri | 3GPP TS 29.571 [6] |  |  |
| Ecgi | 3GPP TS 29.571 [6] | EUTRA Cell Identifier |  |
| Ncgi | 3GPP TS 29.571 [6] | NR Cell Identifier |  |
| Uint16 | 3GPP TS 29.571 [6] |  |  |
| 5Qi | 3GPP TS 29.571 [6] | 5G QoS Identifier |  |
| CorrelationID | 3GPP TS 29.572 [25] | LCS Correlation ID |  |
| Pei | 3GPP TS 29.571 [6] |  |  |
| Dnn | 3GPP TS 29.571 [6] |  |  |
| Gpsi | 3GPP TS 29.571 [6] |  |  |
| GroupId | 3GPP TS 29.571 [6] |  |  |
| PlmnId | 3GPP TS 29.571 [6] |  |  |
| RfspIndex | 3GPP TS 29.571 [6] |  |  |
| EbiArpMapping | 3GPP TS 29.502 [16] | EBI - ARP mapping |  |
| NsiId | 3GPP TS 29.531 [18] |  |  |
| TraceData | 3GPP TS 29.571 [6] | Trace control and configuration parameters |  |
| ConfiguredSnssai | 3GPP TS 29.531 [18] |  |  |
| NgApCause | 3GPP TS 29.571 [6] | Represents the NG AP cause IE |  |
| Area | 3GPP TS 29.571 [6] |  |  |
| ServiceAreaRestriction | 3GPP TS 29.571 [6] |  |  |
| CoreNetworkType | 3GPP TS 29.571 [6] |  |  |
| Ambr | 3GPP TS 29.571 [6] |  |  |
| GlobalRanNodeId | 3GPP TS 29.571 [6] |  |  |
| NfGroupId | 3GPP TS 29.571 [6] | Network Function Group Id |  |
| DurationSec | 3GPP TS 29.571 [6] |  |  |
| SmallDataRateStatus | 3GPP TS 29.571 [6] |  |  |

* * * Next Change * * * *

6.1.6.2.25	Type: UeContext

Table 6.1.6.2.25-1: Definition of type UeContext

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supi | Supi | C | 0..1 | This IE shall be present if available. When present, this IE contains SUPI of the UE. |
| supiUnauthInd | boolean | C | 0..1 | This IE shall be present if SUPI is present. When present, it shall indicate whether the SUPI is unauthenticated. |
| gpsiList | array(Gpsi) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain the GPSI(s) of the UE. |
| pei | Pei | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain Mobile Equipment Identity of the UE. |
| udmGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the UDM Group serving the UE. |
| ausfGroupId | NfGroupId | O | 0..1 | When present, it shall indicate the identity of the AUSF Group serving the UE. |
| routingIndicator | string | O | 0..1 | When present, it shall indicate the Routing Indicator of the UE. |
| groupList | array(GroupId) | C | 1..N | This IE shall be present if the UE belongs to any subscribed internal group(s) and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall list the subscribed internal group(s) to which the UE belongs to. |
| drxParameter | DrxParameter | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain the DRX parameter of the UE. |
| subRfsp | RfspIndex | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it shall indicate the subscribed RFSP Index of the UE. |
| usedRfsp | RfspIndex | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it shall indicate the used RFSP Index of the UE. |
| subUeAmbr | Ambr | C | 0..1 | This IE shall be present if subscribed UE-AMBR has been retrieved from UDM and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall indicate the value of subscribed UE AMBR of the UE. |
| smsSupport | SmsSupport | C | 0..1 | and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. Indicates whether the UE supports SMS delivery over NAS via 3GPP access, or via non-3GPP access, or via both the 3GPP and non-3GPP access. |
| smsfId | NfInstanceId | C | 0..1 | This IE shall be present if the SMS service for UE is activated and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it indicates the identifier of the SMSF network function instance serving the UE. The NF service consumer (e.g. target AMF) may use this information to identify the SMSF NF service profile from among the SMSF NF service profiles it received from the NRF. |
| seafData | SeafData | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE contains the security data derived from data received from AUSF of the UE. |
| 5gMmCapability | 5GMmCapability | C | 0..1 | This IE shall be present if the UE had provided this IE during Registration Procedure and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE shall contain 5G MM capability of the UE. |
| pcfId | NfInstanceId | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE indicates the identity of the PCF for AM Policy and/or UE Policy. |
| pcfAmPolicyUri | Uri | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall contain the URI of the individual AM policy resource (see 3GPP TS 29.507 [32] clause 5.3.3.2) used by the AMF. |
| amPolicyReqTriggerList | array(PolicyReqTrigger) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall indicate the AM policy request triggers towards the PCF. The NF Service Consumer (e.g. target AMF) shall use these triggers to request AM policy from the PCF whenever these triggers are met. The possible AM policy control request triggers are specified in clause 6.1.2.5 of 3GPP TS 23.503 [7]. |
| pcfUePolicyUri | Uri | C | 0..1 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall contain the URI of the individual UE policy resource (see 3GPP TS 29.507 [32] clause 5.3.3.2) used by the AMF. |
| uePolicyReqTriggerList | array(PolicyReqTrigger) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present this IE shall indicate the UE policy request triggers towards the PCF. The NF Service Consumer (e.g. target AMF) shall use these triggers to request UE policy from the PCF whenever these triggers are met. The possible UE policy control request triggers are specified in clause 6.1.2.5 of 3GPP TS 23.503 [31]. |
| hpcfId | NfInstanceId | O | 0..1 | This IE indicates the identity of PCF for AM Policy and/or UE Policy in home PLMN, when the UE is roaming. |
| restrictedRatList | array(RatType) | O | 1..N | When present, this IE shall indicate the list of RAT types that are restricted for the UE; see 3GPP TS 29.571 [6] |
| forbiddenAreaList | array(Area) | O | 1..N | When present, this IE shall indicate the list of forbidden areas of the UE. |
| serviceAreaRestriction | ServiceAreaRestriction | O | 0..1 | When present, this IE shall indicate Service Area Restriction for the UE. |
| restrictedCnList | array(CoreNetworkType) | O | 1..N | When present, this IE shall indicate the list of Core Network Types that are restricted for the UE. |
| eventSubscriptionList | array(AmfEventSubscription) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, it shall indicate the event subscription(s) targeting the UE or the group the UE is part of. |
| mmContextList | array(MmContext) | C | 1..2 | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE contains the MM Contexts of the UE. |
| sessionContextList | array(PduSessionContext) | C | 1..N | This IE shall be present if available and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. When present, this IE contains the PDU Session Contexts of the UE. |
| traceData | TraceData | C | 0..1 | This IE shall be present if signalling based trace has been activated (see 3GPP TS 32.422 [30]) and if it is not case b) specified in clause 5.2.2.2.1.1 step 2a. |
| remainingServiceGapTimer | DurationSec | C | 0..1 | This IE shall be present if Service Gap Control is enabled and if the AMF has started a Service Gap Timer which has not expired yet (see clause 5.31.16 of 3GPP TS 23.501 [2]). The value of the IE indicates the remaining time before the Service Gap Timer expires in number of seconds. |
| smallDataRateStatusInfos | array(SmallDataRateStatusInfo) | O | 1..N | List of Small Data Rate Control Statuses for released PDU Sessions, see clause 5.31.14.3 of TS 23.501 [2]. |

* * * Next Change * * * *

6.1.6.2.x	Type: SmallDataRateStatusInfo

Table 6.1.6.2.x-1: Definition of type SmallDataRateStatusInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| singleNssai | Snssai | M | 1 | S-NSSAI |
| dnn | Dnn | M | 1 | DNN |
| smallDataRateStatus | SmallDataRateStatus | M | 1 | Small data rate status related to the S-NSSAI and Dnn. |

* * * Next Change * * * *

A.2	Namf_Communication API

openapi: 3.0.0

**********skipped for clarity**********

UeContext:

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

supiUnauthInd:

type: boolean

gpsiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

ausfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

groupList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

drxParameter:

$ref: '#/components/schemas/DrxParameter'

subRfsp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RfspIndex'

usedRfsp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RfspIndex'

subUeAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

smsSupport:

$ref: '#/components/schemas/SmsSupport'

smsfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

seafData:

$ref: '#/components/schemas/SeafData'

5gMmCapability:

$ref: '#/components/schemas/5GMmCapability'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfAmPolicyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

amPolicyReqTriggerList:

type: array

items:

$ref: '#/components/schemas/PolicyReqTrigger'

minItems: 1

pcfUePolicyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

uePolicyReqTriggerList:

type: array

items:

$ref: '#/components/schemas/PolicyReqTrigger'

minItems: 1

hpcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

restrictedRatList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 1

forbiddenAreaList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Area'

minItems: 1

serviceAreaRestriction:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

restrictedCoreNwTypeList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CoreNetworkType'

minItems: 1

eventSubscriptionList:

type: array

items:

$ref: 'TS29518_Namf_EventExposure.yaml#/components/schemas/AmfEventSubscription'

minItems: 1

mmContextList:

type: array

items:

$ref: '#/components/schemas/MmContext'

minItems: 1

maxItems: 2

sessionContextList:

type: array

items:

$ref: '#/components/schemas/PduSessionContext'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

remainingServiceGapTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

smallDataRateStatusInfos:

type: array

items:

$ref: '#/components/schemas/SmallDataRateStatusInfo'

minItems: 1

**********skipped for clarity**********

N2RanInformation:

type: object

properties:

n2InfoContent:

$ref: '#/components/schemas/N2InfoContent'

required:

- n2InfoContent

N2InfoNotificationRspData:

type: object

properties:

n2InfoContent:

$ref: '#/components/schemas/N2InfoContent'

SmallDataRateStatusInfo:

type: object

properties:

Snssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

Dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

SmallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

required:

- Snssai

- Dnn

- SmallDataRateStatus

**********skipped for clarity**********

* * * End of Changes * * * *