3GPP TSG-CT WG4 Meeting #93	C4-193690

# Wroclaw, Poland; 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.283 | CR | 0018 | rev | 2 | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft-ietf-dime-load published as RFC 8583 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DLoCMe |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The IETF draft on Diameter Load Information Conveyance has been published as RFC8583. Differences between the previously referenced draft and the newly referenced RFC consist of some editorial corrections and AVP values assignment. There are no technical impacts that would affect the content of the TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the reference to the I-D "draft-ietf-dime-load" by a reference to the RFC 8583 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Essential Correction. Missing normative reference, which might lead to misimplementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1.10, 7.3.1, 7.3.23, D.1, D.2, D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | subclauses 7.3.x1 and 7.3.x2 have to be numbered 7.3.24 and 7.3.25 as in the Rel-15 version. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void.

[3]	Void.

[4]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[5]	IETF RFC 4960: "Stream Control Transmission Protocol".

[6]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details".

[7]	IETF RFC 5234:"Augmented BNF for Syntax Specifications: ABNF".

[8]	IETF RFC 7944: "Diameter Routing Message Priority".

[9]	3GPP TS 29.329: "Sh interface based on the Diameter protocol; Protocol details".

[10]	3GPP TS 29.336: "Home Subscriber Server (HSS) diameter interfaces for interworking with packet data networks and applications".

[11]	IETF RFC 7683: "Diameter Overload Indication Conveyance".

[12]	3GPP TS 23.003: "Numbering, addressing and identification".

[13]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[14]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[15]	IETF RFC 3986:"Uniform Resource Identifier (URI): Generic Syntax".

[16]	Void.

[17]	IETF RFC 8583: "Diameter Load Information Conveyance".

[18]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[19]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[20]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[21]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[22]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management; Protocol specification".

[23]	IETF RFC 6733: "Diameter Base Protocol".

* * * Next Change * * * *

### 7.1.10	MC Service ID to MC Service User Database resolution

The MC Service ID to MC Service User Database resolution mechanism enables an MC Service Server or the Configuration Management Server to find the identity of the MC Service User Database that holds the MC Service related data for a given MC Service ID when multiple and separately addressable MC Service User Databases have been deployed by the MC service provider/network operator. The resolution mechanism is not required in networks that utilise a single MC Service User Database per MC Service or when an MC Service Server is configured to use pre-defined MC Service User Database.

The resolution mechanism shall use a Diameter Proxy Agent or a Diameter Redirect Agent.

The Diameter Proxy Agent or Diameter Redirect Agent shall be used to determine the MC Service User Database identity.

In networks where the use of the MC Service ID to MCPTT User Database resolution mechanism is required and the MC Server Server is not configured to use a predefined MC Service User Database, each MC Service Server shall be configured with the pre-configured address/name of a Diameter Proxy Agent or Diameter Redirect Agent to enable use of these resolution mechanisms.

To get the MC Service User Database identity, the MC Service Server or the Configuration Management Server shall send the Data Management Application request normally destined to the MC Service User Database to the pre-configured Diameter Proxy Agent or Diameter Redirect Agent.

The Diameter Proxy Agent shall determine the MC Service User Database identity based on the provided user identity and - if the Diameter load control mechanism is supported (see IETF RFC 8583 [17]) - optionally also based on previously received load values from Load AVPs of type HOST. The Diameter Proxy Agent shall then forward the Data Management Application request directly to the determined MC Service User Database. The MC Service Server shall determine the MC Service User Database identity from the response to the Data Management Application request received from the MC Service User Database. The MC Service Server and the Configuration Management Server should store the MC Service User Database identity/name/Realm and shall use it in further Data Management Application requests associated to the same MC Service ID.

The Diameter Redirect Agent shall determine the MC Service User Database address and shall send to the MC Service Server or the Configuration Management Server a notification of redirection towards the MC Service User Database, in response to the request. Multiple MC Service User Database identities may be included in the response, as specified in IETF RFC 6733 [23]. In such a case, the MC Service Server or the Configuration Management Server shall send the Request to the first MC Service User Database identity in the ordered list received in the Response from the Diameter Redirect Agent. If the MC Service Server or the Configuration Management Server does not receive a successful response to the Request, the MC Service Server or the Configuration Management Server shall send a Request to the next MC Service User Database identity in the ordered list. This procedure shall be repeated until a successful response from an MC Service User Database is received.

* * * Next Change * * * *

### 7.3.1	General

The following table (table 7.3.1-1) specifies the Diameter AVPs defined for the MCPTT-2, MCVideo-2, MCData-2 and CSC-13 interfaces, their AVP Code values, types, possible flag values and whether or not the AVP may be encrypted. The Vendor-ID header of all AVPs defined in this specification shall be set to 3GPP (10415).

For all AVPs which contain bit masks and are of the type Unsigned32 e.g., DPR-Flags, bit 0 shall be the least significant bit. For example, to get the value of bit 0, a bit mask of 0x0001 shall be used.

Table 7.3.1-1: MCPTT-2, MCVideo-2, MCData-2 and CSC-13 specific Diameter AVPs

|  |  |  |  | AVP Flag rules |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attribute Name | AVP Code | Clause defined | Value Type | Must | May | Should not | Must not | May Encr. |
| MCPTT-ID | 4500 | 7.3.2 | UTF8String | M, V |  |  |  | No |
| Data-Identification | 4501 | 7.3.3 | Grouped | M, V |  |  |  | No |
| Data-Identification-Prefix | 4502 | 7.3.11 | Unsigned32 | M,V |  |  |  | No |
| Data-Identification-Flags | 4503 | 7.3.12 | Unsigned64 | M,V |  |  |  | No |
| DPR-Flags | 4504 | 7.3.13 | Unsigned32 | M,V |  |  |  | No |
| DPA-Flags | 4505 | 7.3.14 | Unsigned32 | M,V |  |  |  | No |
| DUR-Flags | 4506 | 7.3.15 | Unsigned32 | M,V |  |  |  | No |
| DUA-Flags | 4507 | 7.3.16 | Unsigned32 | M,V |  |  |  | No |
| NDR-Flags | 4508 | 7.3.17 | Unsigned32 | M,V |  |  |  | No |
| NDA-Flags | 4509 | 7.3.18 | Unsigned32 | M,V |  |  |  | No |
| User-Data-Id | 4510 | 7.3.19 | Unsigned32 | M,V |  |  |  | No |
| MC-Service-User-Profile-Data | 4511 | 7.3.20 | Grouped | M,V |  |  |  | No |
| Sequence-Number | 4512 | 7.3.21 | Unsigned32 | M,V |  |  |  | No |
| Data | 4513 | 7.3.22 | Grouped | M,V |  |  |  | No |
| MCVideo-ID | 4514 | 7.3.24 | UTF8String | V |  |  | M | No |
| MCData-ID | 4515 | 7.3.25 | UTF8String | V |  |  | M | No |
| NOTE 1: The AVP header bit denoted as "M", indicates whether support of the AVP is required. The AVP header bit denoted as "V" indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [23]. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |  |  |  |  |  |

The following table (table 7.3.1-2) specifies the Diameter AVPs re-used by the MCPTT-2, MCVideo-2, MCData-2 and CSC-13 interfaces from existing Diameter Applications, including a reference to their respective specifications and when needed, a short description of their use within MCPTT-2, MCVideo-2, MCData-2 and CSC-13 interfaces.

Any other AVPs from existing Diameter Applications, except for the AVPs from Diameter base protocol defined in IETF RFC 6733 [23], do not need to be supported. The AVPs from Diameter base protocol defined in IETF RFC 6733 [23] are not included in table 7.3.1-2.

Table 7.3.1-2: MCPTT-2 and CSC-13 re-used Diameter AVPs

| Attribute Name | Reference | Comments | M-bit |
| --- | --- | --- | --- |
| Supported-Features | 3GPP TS 29.229 [6] |  |  |
| DRMP | IETF RFC 7844[8] |  | Must set |
| Feature-List-ID | 3GPP TS 29.229 [6] |  |  |
| Feature-List | 3GPP TS 29.229 [6] | See subclause 7.3.10 |  |
| Feature-Id | 3GPP TS 29.229 [6] |  |  |
| User-Data | 3GPP TS 29.329 [9] |  |  |
| User-Identifier | 3GPP TS 29.336 [10] | See subclause 7.3.8 |  |
| OC-Supported-Features | IETF RFC 7683 [11] | See subclause 7.3.6 | Must set |
| OC-OLR | IETF RFC 7683 [11] | See subclause 7.3.5 | Must set |
| Load | IETF RFC 8583 [17] | See subclause 7.3.23 | Must not set |
| NOTE 1: The M-bit settings for re-used AVPs override those of the defining specifications that are referenced. Values include: "Must set", "Must not set". If the M-bit setting is blank, then the defining specification applies. NOTE 2: If the M-bit is set for an AVP and the receiver does not understand the AVP, it shall return a rejection. If the M-bit is not set for an AVP, the receiver shall not return a rejection, whether or not it understands the AVP. If the receiver understands the AVP but the M-bit value does not match with the definition in this table, the receiver shall ignore the M-bit. |  |  |  |

* * * Next Change * * * *

### 7.3.23	Load

The Load AVP is of type Grouped and it is defined in IETF RFC 8583 [17]. This AVP is used to support the Diameter load control mechanism.

* * * Next Change * * * *

### 7.3.24	MCVideo-ID

The MCVideo-ID AVP is of type UTF8String, and it shall contain an URI as defined in IETF RFC 3986 [15]. For more information see 3GPP TS 23.281 [19].

* * * Next Change * * * *

### 7.3.25	MCData-ID

The MCData-ID AVP is of type UTF8String, and it shall contain an URI as defined in IETF RFC 3986 [15]. For more information see 3GPP TS 23.282 [20].

* * * Next Change * * * *

## D.1	General

Diameter load control mechanism is an optional feature.

It is recommended to make use of IETF RFC 8583 [17] on the MCPTT-2, MCVideo-2, MCData-2 and CSC-13 interfaces where, when applied, the MC Service Server and the Configuration Management Server shall behave as reacting nodes and the MC Service User Database as a reporting node.

* * * Next Change * * * *

## D.2	MC Service User Database behaviour

The MC Service User Database may report its current load by including a Load AVP of type HOST in answer commands as described in IETF RFC 8583 [17].

The MC Service User Database calculates its current load by implementation specific means. For example, the MC Service User Database may take into account the traffic over the MCPTT-2, MCVideo-2, MCData-2 and CSC-13 interfaces or other interfaces, the level of usage of internal resources (e.g. CPU, memory), the access to external resources, etc.

The MC Service User Database determines when to send Load AVPs of type HOST by implementation specific means.

* * * Next Change * * * *

## D.3	MC Service Server and Configuration Management Server behaviour

When performing next hop Diameter Agent selection for requests that are routed based on realm, the MC Service Server/Configuration Management Server may take into account load values from Load AVPs of type PEER received from candidate next hop Diameter nodes, as per IETF RFC 8583 [17].

* * * End of Changes * * * *