3GPP TSG-CT Meeting #86	CP-193121

# Sitges, SPAIN; 09th – 10th December 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.511 | CR | 0024 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ExternalDocs field and API version change in the OpenAPI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The agreed CR C4-195545 (CR#23) introduces changes to Rel-15 only. The agreed C4-195463 (CR# 0022) introduces changes to Rel-16. So a Rel-16 version of TS 29.511will be produced. Consequently the externalDocs part of the newly produced Rel-16 version needs to be move in the top as well since it needs be updated frequently, along with the version update of the API. This CR is to align the agreement for Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the externalDocs section from the bottom to the top of the specification and change the version of the externalDocs specification from V5.4.0 to V6.0.0 N5g-eir_EquipmentIdentityCheck Service API version number is incremented from 1.0.2 to 1.0.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Updating externalDocs is easily forgotten. Wrong version info in OpenAPI YAML file |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible changes to N5g-eir_EquipmentIdentityCheck API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## A.2	N5g-eir_EquipmentIdentityCheck Service API

openapi: 3.0.0

info:

version: '1.0.3'

title: '5G-EIR Equipment Identity Check'

description: |

5G-EIR Equipment Identity Check Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.511 V16.0.0; 5G System; Equipment Identity Register Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.511/'

servers:

- url: '{apiRoot}/n5g-eir-eic/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- n5g-eir-eic

paths:

/equipment-status:

get:

summary: Retrieves the status of the UE

operationId: GetEquipmentStatus

tags:

- Equipment Status (Document)

parameters:

- name: pei

in: query

description: PEI of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

- name: supi

in: query

description: SUPI of the UE

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: gpsi

in: query

description: GPSI of the UE

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/EirResponseData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'404':

description: PEI Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'414':

$ref: 'TS29571_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

n5g-eir-eic: Access to the N5g-eir_EquipmentIdentityCheck API

schemas:

EirResponseData:

type: object

required:

- status

properties:

status:

$ref: '#/components/schemas/EquipmentStatus'

EquipmentStatus:

type: string

enum:

- WHITELISTED

- BLACKLISTED

- GREYLISTED

externalDocs:

description: 3GPP TS 29.511 V15.4.0; 5G System; Equipment Identity Register Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.511/'

* * * End of Change * * * *