3GPP TSG-CT WG Meeting #86	CP-193138

Itges, SPAIN; 09th – 10th December 2019 	was C4-195611, C4-195595, C4-195504, C4-195215

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0297 | rev | 4 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Frame Routes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-10-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For supporting Framed Routing, the actual ranges of IPv4 address / IPv6 Prefixes corresponding to a Framed Route may be defined in the subscription data associated with a (DNN, S-NSSAI) received by the SMF from the UDM, see clause 5.6.14 Support of Framed Routing of TS23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Frame routes information is kind of SM subscription data, and it is per S-NSSAI, DNN level , therefore to extend SM subscription data to include frame routes information, the revisions include: - Define new data type FrameRouteInfo. - Extend DnnConfiguration to include routes information. - Revise yaml file Nudm_SDM API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Framed Routing new feature will not be fully supported by 5G system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.9, 6.1.6.2.xx(new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR will introduce backward compatible new features in the OpenAPI specification file of Nudm_SDM API, and the open API Nudr_DataRepository API will be introduced backward compatible impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1： - Change IP address range form from start/end form to networkaddredd+mask in data type FrameRouteInfo. Rev2: - Change the definition of ipv4 range in Table 6.1.6.2.xx-1: FrameRouteInfo. - Revise the yalm file of open API Nudm_SDM accordingly. Rev3: - Add the reference IPv4AddrMask and remove the reference IPv4Addr. Rev4: Correct “$ref: 'TS29571_CommonData.yaml#/components/schemas/IPv4AddrMask'” to $ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4AddrMask' in yaml file of Nudm_SDM API. |  |  |  |  |  |  |  |  |

********************The start of changes********************

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the structured data types defined for the Nudm_SDM service API. For simple data types defined for the Nudm_SDM service API see table 6.1.6.3.2-1.

Table 6.1.6.1-1: Nudm_SDM specific Data Types

| Data type |  | Clause defined |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Nssai |  | 6.1.6.2.2 |  | Network Slice Selection Assistance Information |  |
| SdmSubscription |  | 6.1.6.2.3 |  | A subscription to notifications |  |
| AccessAndMobilitySubscriptionData |  | 6.1.6.2.4 |  | Access and Mobility Subscription Data |  |
| SmfSelectionSubscriptionData |  | 6.1.6.2.5 |  | SMF Selection Subscription Data |  |
| UeContextInSmfData |  | 6.1.6.2.16 |  | UE Context In SMF Data |  |
| PduSession |  | 6.1.6.2.17 |  |  |  |
| DnnInfo |  | 6.1.6.2.6 |  | Data Network Name and associated information (LBO roaming allowed flag) |  |
| SnssaiInfo |  | 6.1.6.2.7 |  | S-NSSAI and associated information (DNN Info) |  |
| SessionManagementSubscriptionData |  | 6.1.6.2.8 |  | User subscribed session management data |  |
| DnnConfiguration |  | 6.1.6.2.9 |  | User subscribed data network configuration |  |
| PduSessionTypes |  | 6.1.6.2.11 |  | Default/allowed session types for a data network |  |
| SscModes |  | 6.1.6.2.12 |  | Default/allowed SSC modes for a data network |  |
| SmsManagementSubscriptionData |  | 6.1.6.2.14 |  | SMS Management Subscription Data |  |
| IdTranslationResult |  | 6.1.6.2.18 |  | SUPI that corresponds to a given GPSI |  |
| IpAddress |  | 6.1.6.2.22 |  | IP address (IPv4, or IPv6, or IPv6 prefix) |  |
| 3GppChargingCharacteristics |  | 6.1.6.3.2 |  | 3GPP Charging Characteristics |  |
| IwkEpsInd |  | 6.1.6.3.2 |  | Interworking with EPS Indication |  |
| ModificationNotification |  | 6.1.6.2.21 |  |  |  |
| UeContextInSmsfData |  | 6.1.6.2.23 |  |  |  |
| SmsfInfo |  | 6.1.6.2.24 |  |  |  |
| AcknowledgeInfo |  | 6.1.6.2.25 |  |  |  |
| SorInfo |  | 6.1.6.2.26 |  | Steering Of Roaming Information |  |
| UpuInfo |  | 6.1.6.2.33 |  | UE Parameters Update Information |  |
| SharedData |  | 6.1.6.2.27 |  | Subscription Data shared by multiple UEs |  |
| PgwInfo |  | 6.1.6.2.28 |  | Information about the DNNs/APNs and PGW-C+SMF FQDNs used in interworking with EPS |  |
| TraceDataResponse |  | 6.1.6.2.29 |  | Contains Trace Data or a shared data Id identifying shared Trace Data |  |
| SdmSubsModification |  | 6.1.6.2.31 |  | Modification instruction for a subscription to notifications |  |
| EmergencyInfo |  | 6.1.6.2.32 |  | Information about emergency session |  |
| EpsIwkPgw |  | 6.1.6.2.11 |  | Information of the PGW-C+SMF selected by the AMF for EPS interworking with N26 interface. |  |
| GroupIdentifiers |  | 6.1.6.2.34 |  |  |  |
| ExtGroupId |  | 6.1.6.3.2 |  |  |  |
| NiddInformation |  | 6.1.6.2.35 |  | Non-IP Data Delivery information |  |
| CagData |  | 6.1.6.2.36 |  |  |  |
| CagInfo |  | 6.1.6.2.37 |  |  |  |
| DataSetName |  | 6.1.6.3.3 |  |  |  |
| PduSessionContinuityInd |  | 6.1.6.3.7 |  |  |  |
| AdditionalSnssaiData |  | 6.1.6.2.38 |  | Additional information specific to a slice |  |
| VnGroupData |  | 6.1.6.2.39 |  |  |  |
| AppDescriptor |  | 6.1.6.2.40 |  |  |  |
| AppPortId |  | 6.1.6.2.41 |  | Application Port Id |  |
| FrameRouteInfo |  | 6.1.6.2.xx |  | Frame Route Information |  |

Table 6.1.6.1-2 specifies data types re-used by the Nudm_SDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudm_SDM service API.

Table 6.1.6.1-2: Nudm_SDM re-used Data Types

| Data type |  | Reference |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Dnn |  | 3GPP TS 29.571 [7] |  | Data Network Name; this type is used as key in a map of: - DnnConfigurations; see clause 6.1.6.2.8; - EpsIwkPgws; see clause 6.2.6.2.2; |  |
| DurationSec |  | 3GPP TS 29.571 [7] |  | Time value in seconds |  |
| ProblemDetails |  | 3GPP TS 29.571 [7] |  | Common data type used in response bodies |  |
| Snssai |  | 3GPP TS 29.571 [7] |  | Single NSSAI |  |
| Uri |  | 3GPP TS 29.571 [7] |  | Uniform Resource Identifier |  |
| Gpsi |  | 3GPP TS 29.571 [7] |  | Generic Public Subscription Identifier |  |
| RatType |  | 3GPP TS 29.571 [7] |  | Radio Access Technology Type |  |
| Area |  | 3GPP TS 29.571 [7] |  |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [7] |  |  |  |
| CoreNetworkType |  | 3GPP TS 29.571 [7] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [7] |  | see 3GPP TS 29.500 [4] clause 6.6 |  |
| PlmnId |  | 3GPP TS 29.571 [7] |  | PLMN Identity |  |
| PduSessionType |  | 3GPP TS 29.571 [7] |  |  |  |
| SubscribedDefaultQos |  | 3GPP TS 29.571 [7] |  | Subscribed Default QoS |  |
| Ambr |  | 3GPP TS 29.571 [7] |  |  |  |
| PduSessionId |  | 3GPP TS 29.571 [7] |  | PduSessionId is used as key in a map of PduSessions; see clause 6.1.6.2.16. |  |
| NfInstanceId |  | 3GPP TS 29.571 [7] |  |  |  |
| Supi |  | 3GPP TS 29.571 [7] |  |  |  |
| RfspIndex |  | 3GPP TS 29.571 [7] |  |  |  |
| SscMode |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv4Address |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv6Address |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [7] |  |  |  |
| SorMac |  | 3GPP TS 29.509 [24] |  |  |  |
| SteeringInfo |  | 3GPP TS 29.509 [24] |  |  |  |
| AckInd |  | 3GPP TS 29.509 [24] |  |  |  |
| CounterSor |  | 3GPP TS 29.509 [24] |  |  |  |
| UpuMac |  | 3GPP TS 29.509 [24] |  |  |  |
| UpuData |  | 3GPP TS 29.509 [24] |  |  |  |
| UpuAckInd |  | 3GPP TS 29.509 [24] |  |  |  |
| CounterUpu |  | 3GPP TS 29.509 [24] |  |  |  |
| TraceData |  | 3GPP TS 29.571 [7] |  | Trace control and configuration parameters |  |
| NotifyItem |  | 3GPP TS 29.571 [7] |  |  |  |
| UpSecurity |  | 3GPP TS 29.571 [7] |  |  |  |
| ServiceName |  | 3GPP TS 29.510 [19] |  |  |  |
| OdbPacketServices |  | 3GPP TS 29.571 [7] |  |  |  |
| GroupId |  | 3GPP TS 29.571 [7] |  |  |  |
| DateTime |  | 3GPP TS 29.571 [7] |  |  |  |
| CagId |  | 3GPP TS 29.571 [7] |  |  |  |
| StnSr |  | 3GPP TS 29.571 [7] |  | Session Transfer Number for SRVCC |  |
| CMsisdn |  | 3GPP TS 29.571 [7] |  | Correlation MSISDN |  |
| OsId |  | 3GPP TS 29.519 [33] |  |  |  |
| Uint16 |  | 3GPP TS 29.571 [7] |  |  |  |
| IPv4AddrMask |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [7] |  |  |  |

********************Next change********************

6.1.6.2.9	Type: DnnConfiguration

Table 6.1.6.2.9-1: DnnConfiguration

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| pduSessionTypes | PduSessionTypes | M | 1 | Default/Allowed session types |
| sscModes | SscModes | M | 1 | Default/Allowed SSC modes |
| iwkEpsInd | IwkEpsInd | O | 0..1 | Indicates whether interworking with EPS is subscribed: true: Subscribed; false: Not subscribed; If this attribute is absent it means not subscribed. |
| 5gQosProfile | SubscribedDefaultQos | O | 0..1 | 5G QoS parameters associated to the session for a data network |
| sessionAmbr | Ambr | O | 0..1 | The maximum aggregated uplink and downlink bit rates to be shared across all Non-GBR QoS Flows in each PDU Session |
| 3gppChargingCharacteristics | 3GppChargingCharacteristics | O | 0..1 | Subscribed charging characteristics data associated to the session for a data network |
| staticIpAddress | array(IpAddress) | O | 1..2 | Subscribed static IP address(es) of the IPv4 and/or IPv6 type |
| upSecurity | UpSecurity | O | 0..1 | When present, this IE shall indicate the security policy for integrity protection and encryption for the user plane. |
| pduSessionContinuityInd | PduSessionContinuityInd | O | 0..1 | When present, this IE shall indicate how to handle a PDU Session when UE the moves to or from NB-IoT. If this attribute is absent it means that Local policy shall be used. |
| invokeNefSelection | boolean | O | 0..1 | When present, this IE shall indicate whether to invoke NEF selection for NIDD session: true: to invoke the NEF selection; false or absent: not to invoke the NEF selection; If this attribute is absent it means not to invoke the NEF selection. |
| niddNefId | NfInstanceId | C | 0..1 | Indicates the identity of the NEF which is to be selected for this DNN. It is required if "Invoke NEF Selection" indicator is set. |
| niddInfo | NiddInformation | O | 0..1 | When present, this IE shall indicate information used for SMF-NEF Connection. This attribute may be present if "Invoke NEF Selection" indicator is set. |
| redundantSessionAllowed | boolean | O | 0..1 | Indicates whether redundant PDU Sessions are allowed: true: Allowed; false: Not allowed; If this attribute is absent it means not allowed. |
| ipv4FrameRouteList | array(FrameRouteInfo) | O | 1..N | List of Frame Route information of IPv4, see clause 5.6.14 of 3GPP TS 23.501 [2]. |
| ipv6FrameRouteList | array(FrameRouteInfo) | O | 1..N | List of Frame Route information of IPv6, see clause 5.6.14 of 3GPP TS 23.501 [2]. |

********************Next change********************

6.1.6.2.xx	Type: FrameRouteInfo

Table 6.1.6.2.xx-1: FrameRouteInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ipv4Mask | IPv4AddrMask | C | 0..1 | Indicates IPv4 address mask. |
| ipv6Prefix | Ipv6Prefix | C | 0..1 | Indicates IPv6 prefix. |
| NOTE: Either ipv4Mask or ipv6Prefix shall be present. |  |  |  |  |

********************Next change********************

A.2	Nudm_SDM API

openapi: 3.0.0

info:

version: '2.1.0.alpha-2'

title: 'Nudm_SDM'

description: |

Nudm Subscriber Data Management Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.503 Unified Data Management Services, version 16.1.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.503/'

servers:

- url: '{apiRoot}/nudm-sdm/v2'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause clause 4.4 of 3GPP TS 29.501.

security:

- oAuth2ClientCredentials:

- nudm-sdm

- {}

paths:

/{supi}:

get:

summary: retrieve multiple data sets

operationId: GetDataSets

tags:

- Retrieval of multiple data sets

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: dataset-names

in: query

style: form

explode: false

description: List of dataset names

required: true

schema:

$ref: '#/components/schemas/DatasetNames'

- name: plmn-id

in: query

description: serving PLMN ID

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SubscriptionDataSets'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/nssai:

get:

summary: retrieve a UE's subscribed NSSAI

operationId: GetNSSAI

tags:

- Slice Selection Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

description: serving PLMN ID

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Nssai'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/am-data:

get:

summary: retrieve a UE's Access and Mobility Subscription Data

operationId: GetAmData

tags:

- Access and Mobility Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

description: serving PLMN ID

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/smf-select-data:

get:

summary: retrieve a UE's SMF Selection Subscription Data

operationId: GetSmfSelData

tags:

- SMF Selection Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

description: serving PLMN ID

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmfSelectionSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/ue-context-in-smf-data:

get:

summary: retrieve a UE's UE Context In SMF Data

operationId: GetUeCtxInSmfData

tags:

- UE Context In SMF Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/UeContextInSmfData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/ue-context-in-smsf-data:

get:

summary: retrieve a UE's UE Context In SMSF Data

operationId: GetUeCtxInSmsfData

tags:

- UE Context In SMSF Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/UeContextInSmsfData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/trace-data:

get:

summary: retrieve a UE's Trace Configuration Data

operationId: GetTraceConfigData

tags:

- Trace Configuration Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

description: serving PLMN ID

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/TraceDataResponse'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/sm-data:

get:

summary: retrieve a UE's Session Management Subscription Data

operationId: GetSmData

tags:

- Session Management Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: single-nssai

in: query

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

- name: dnn

in: query

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

- name: plmn-id

in: query

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

minItems: 1

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/sms-data:

get:

summary: retrieve a UE's SMS Subscription Data

operationId: GetSmsData

tags:

- SMS Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/sms-mng-data:

get:

summary: retrieve a UE's SMS Management Subscription Data

operationId: GetSmsMngtData

tags:

- SMS Management Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SmsManagementSubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/sdm-subscriptions:

post:

summary: subscribe to notifications

operationId: Subscribe

tags:

- Subscription Creation

parameters:

- name: supi

in: path

description: SUPI of the user

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SdmSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudm-sdm/<apiVersion>/{supi}/sdm-subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

callbacks:

datachangeNotification:

'{request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'307':

description: Temporary Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'Contains the new Callback URI of the target NF Service Consumer (e.g. AMF) to which the request is redirected'

required: true

schema:

type: string

'308':

description: Permanent Redirect

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Location:

description: 'Contains the new Callback URI of the target NF Service Consumer (e.g. AMF) to which the request is redirected'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/sdm-subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications

operationId: Unsubscribe

tags:

- Subscription Deletion

parameters:

- name: supi

in: path

description: SUPI of the user

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: subscriptionId

in: path

description: Id of the SDM Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: modify the subscription

operationId: Modify

tags:

- Subscription Modification

parameters:

- name: supi

in: path

description: SUPI of the user

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: subscriptionId

in: path

description: Id of the SDM Subscription

required: true

schema:

type: string

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SdmSubsModification'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SdmSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{ueId}/id-translation-result:

get:

summary: retrieve a UE's SUPI or GPSI

operationId: GetSupiOrGpsi

tags:

- GPSI to SUPI Translation

parameters:

- name: ueId

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VarUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: app-port-id

in: query

description: Application port identifier

content:

application/json:

schema:

$ref: '#/components/schemas/AppPortId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/IdTranslationResult'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/am-data/sor-ack:

put:

summary: Nudm_Sdm Info service operation

operationId: SorAckInfo

tags:

- Providing acknowledgement of Steering of Roaming

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AcknowledgeInfo'

responses:

'204':

description: Successful acknowledgement

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/am-data/upu-ack:

put:

summary: Nudm_Sdm Info for UPU service operation

operationId: UpuAck

tags:

- Providing acknowledgement of UE Parameters Update

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AcknowledgeInfo'

responses:

'204':

description: Successful acknowledgement

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/am-data/subscribed-snssais-ack:

put:

summary: Nudm_Sdm Info operation for S-NSSAIs acknowledgement

operationId: S-NSSAIs Ack

tags:

- Providing acknowledgement of S-NSSAIs Update

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AcknowledgeInfo'

responses:

'204':

description: Successful acknowledgement

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/am-data/cag-ack:

put:

summary: Nudm_Sdm Info operation for CAG acknowledgement

operationId: CAG Ack

tags:

- Providing acknowledgement of CAG Update

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AcknowledgeInfo'

responses:

'204':

description: Successful acknowledgement

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/shared-data:

get:

summary: retrieve shared data

operationId: GetSharedData

tags:

- Retrieval of shared data

parameters:

- name: shared-data-ids

in: query

description: List of shared data ids

required: true

style: form

explode: false

schema:

$ref: '#/components/schemas/SharedDataIds'

- name: supportedFeatures

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SharedData'

minItems: 1

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/shared-data-subscriptions:

post:

summary: subscribe to notifications for shared data

operationId: SubscribeToSharedData

tags:

- Subscription Creation for shared data

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SdmSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nudm-sdm/<apiVersion>/shared-data-subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

description: Unexpected error

callbacks:

datachangeNotification:

'{request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/shared-data-subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications for shared data

operationId: UnsubscribeForSharedData

tags:

- Subscription Deletion for shared data

parameters:

- name: subscriptionId

in: path

description: Id of the Shared data Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: modify the subscription

operationId: ModifySharedDataSubs

tags:

- Subscription Modification

parameters:

- name: subscriptionId

in: path

description: Id of the SDM Subscription

required: true

schema:

type: string

requestBody:

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SdmSubsModification'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SdmSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/group-data/group-identifiers:

get:

summary: Mapping of Group Identifiers

operationId: GetGroupIdentifiers

tags:

- Group Identifiers

parameters:

- name: ext-group-id

in: query

description: External Group Identifier

required: false

schema:

$ref: '#/components/schemas/ExtGroupId'

- name: int-group-id

in: query

description: Internal Group Identifier

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/GroupIdentifiers'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nudm-sdm: Access to the nudm-sdm API

schemas:

# COMPLEX TYPES:

DatasetNames:

type: array

items:

$ref: '#/components/schemas/DataSetName'

minItems: 2

uniqueItems: true

SubscriptionDataSets:

type: object

properties:

amData:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

smfSelData:

$ref: '#/components/schemas/SmfSelectionSubscriptionData'

uecSmfData:

$ref: '#/components/schemas/UeContextInSmfData'

uecSmsfData:

$ref: '#/components/schemas/UeContextInSmsfData'

smsSubsData:

$ref: '#/components/schemas/SmsSubscriptionData'

smData:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

smsMngData:

$ref: '#/components/schemas/SmsManagementSubscriptionData'

UeContextInSmsfData:

type: object

properties:

smsfInfo3GppAccess:

$ref: '#/components/schemas/SmsfInfo'

smsfInfoNon3GppAccess:

$ref: '#/components/schemas/SmsfInfo'

SmsfInfo:

type: object

required:

- smsfInstanceId

- plmnId

properties:

smsfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

AccessAndMobilitySubscriptionData:

type: object

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

gpsis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

internalGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

vnGroupInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/VnGroupData'

minProperties: 1

sharedVnGroupDataIds:

type: object

additionalProperties:

$ref: '#/components/schemas/SharedDataId'

minProperties: 1

subscribedUeAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmbrRm'

nssai:

$ref: '#/components/schemas/Nssai'

ratRestrictions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

forbiddenAreas:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Area'

serviceAreaRestriction:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

coreNetworkTypeRestrictions:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CoreNetworkType'

rfspIndex:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RfspIndexRm'

subsRegTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSecRm'

ueUsageType:

$ref: '#/components/schemas/UeUsageType'

mpsPriority:

$ref: '#/components/schemas/MpsPriorityIndicator'

mcsPriority:

$ref: '#/components/schemas/McsPriorityIndicator'

activeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSecRm'

dlPacketCount:

$ref: '#/components/schemas/DlPacketCount'

sorInfo:

$ref: '#/components/schemas/SorInfo'

upuInfo:

$ref: '#/components/schemas/UpuInfo'

micoAllowed:

$ref: '#/components/schemas/MicoAllowed'

sharedAmDataIds:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

minItems: 1

odbPacketServices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OdbPacketServices'

subscribedDnnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

serviceGapTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

cagData:

$ref: '#/components/schemas/CagData'

stnSr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/StnSr'

cMsisdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CMsisdn'

nbIoTUePriority:

$ref: '#/components/schemas/NbIoTUePriority'

CagData:

type: object

required:

- cagInfos

properties:

cagInfos:

description: A map (list of key-value pairs where PlmnId serves as key) of CagInfo

type: object

additionalProperties:

$ref: '#/components/schemas/CagInfo'

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

CagInfo:

type: object

required:

- allowedCagList

properties:

allowedCagList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CagId'

cagOnlyIndicator:

type: boolean

SmfSelectionSubscriptionData:

type: object

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

subscribedSnssaiInfos:

type: object

additionalProperties:

$ref: '#/components/schemas/SnssaiInfo'

sharedSnssaiInfosId:

$ref: '#/components/schemas/SharedDataId'

SnssaiInfo:

type: object

required:

- dnnInfos

properties:

dnnInfos:

type: array

items:

$ref: '#/components/schemas/DnnInfo'

minItems: 1

DnnInfo:

type: object

required:

- dnn

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

defaultDnnIndicator:

$ref: '#/components/schemas/DnnIndicator'

lboRoamingAllowed:

$ref: '#/components/schemas/LboRoamingAllowed'

iwkEpsInd:

$ref: '#/components/schemas/IwkEpsInd'

dnnBarred:

type: boolean

Nssai:

type: object

required:

- defaultSingleNssais

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

defaultSingleNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

singleNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

additionalSnssaiData:

type: object

additionalProperties:

$ref: '#/components/schemas/AdditionalSnssaiData'

minProperties: 1

nullable: true

UeContextInSmfData:

type: object

properties:

pduSessions:

description: A map (list of key-value pairs where PduSessionId serves as key) of PduSessions

type: object

additionalProperties:

$ref: '#/components/schemas/PduSession'

pgwInfo:

type: array

items:

$ref: '#/components/schemas/PgwInfo'

minItems: 1

emergencyInfo:

$ref: '#/components/schemas/EmergencyInfo'

EmergencyInfo:

type: object

oneOf:

- required:

- pgwFqdn

- required:

- pgwIpAddress

properties:

pgwFqdn:

type: string

pgwIpAddress:

$ref: '#/components/schemas/IpAddress'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

PduSession:

type: object

required:

- dnn

- smfInstanceId

- plmnId

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

PgwInfo:

type: object

required:

- dnn

- pgwFqdn

properties:

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

pgwFqdn:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

SessionManagementSubscriptionData:

type: object

required:

- singleNssai

properties:

singleNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnnConfigurations:

description: A map (list of key-value pairs where Dnn serves as key) of DnnConfigurations

type: object

additionalProperties:

$ref: '#/components/schemas/DnnConfiguration'

internalGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

vnGroupInfo:

type: object

additionalProperties:

$ref: '#/components/schemas/VnGroupData'

minProperties: 1

sharedVnGroupDataIds:

type: object

additionalProperties:

$ref: '#/components/schemas/SharedDataId'

minProperties: 1

sharedDnnConfigurationsId:

$ref: '#/components/schemas/SharedDataId'

odbPacketServices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OdbPacketServices'

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

sharedTraceDataId:

$ref: '#/components/schemas/SharedDataId'

DnnConfiguration:

type: object

required:

- pduSessionTypes

- sscModes

properties:

pduSessionTypes:

$ref: '#/components/schemas/PduSessionTypes'

sscModes:

$ref: '#/components/schemas/SscModes'

iwkEpsInd:

$ref: '#/components/schemas/IwkEpsInd'

5gQosProfile:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SubscribedDefaultQos'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

3gppChargingCharacteristics:

$ref: '#/components/schemas/3GppChargingCharacteristics'

staticIpAddress:

type: array

items:

$ref: '#/components/schemas/IpAddress'

minItems: 1

maxItems: 2

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

pduSessionContinuityInd:

$ref: '#/components/schemas/PduSessionContinuityInd'

invokeNefSelection:

type: boolean

niddNefId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

niddInfo:

$ref: '#/components/schemas/NiddInformation'

redundantSessionAllowed:

type: boolean

ipv4FrameRouteList:

type: array

items:

$ref: '#/components/schemas/FrameRouteInfo'

minItems: 1

ipv6FrameRouteList:

type: array

items:

$ref: '#/components/schemas/FrameRouteInfo'

minItems: 1

NiddInformation:

type: object

required:

- afInstanceId

properties:

afInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

IpAddress:

type: object

oneOf:

- required:

- ipv4Addr

- required:

- ipv6Addr

- required:

- ipv6Prefix

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

PduSessionTypes:

type: object

required:

- defaultSessionType

properties:

defaultSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

allowedSessionTypes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

minItems: 1

SscModes:

type: object

required:

- defaultSscMode

properties:

defaultSscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

allowedSscModes:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

minItems: 1

maxItems: 2

SmsSubscriptionData:

type: object

properties:

smsSubscribed:

$ref: '#/components/schemas/SmsSubscribed'

sharedSmsSubsDataId:

$ref: '#/components/schemas/SharedDataId'

SmsManagementSubscriptionData:

type: object

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

mtSmsSubscribed:

type: boolean

mtSmsBarringAll:

type: boolean

mtSmsBarringRoaming:

type: boolean

moSmsSubscribed:

type: boolean

moSmsBarringAll:

type: boolean

moSmsBarringRoaming:

type: boolean

sharedSmsMngDataIds:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

SdmSubscription:

type: object

required:

- nfInstanceId

- callbackReference

- monitoredResourceUris

properties:

nfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

implicitUnsubscribe:

type: boolean

expires:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

callbackReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

amfServiceName:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/ServiceName'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

singleNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

subscriptionId:

type: string

plmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

SdmSubsModification:

type: object

properties:

expires:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

monitoredResourceUris:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

ModificationNotification:

type: object

required:

- notifyItems

properties:

notifyItems:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotifyItem'

minItems: 1

IdTranslationResult:

type: object

required:

- supi

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

AcknowledgeInfo:

type: object

required:

- provisioningTime

properties:

sorMacIue:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SorMac'

upuMacIue:

$ref: 'TS29509_Nausf_UPUProtection.yaml#/components/schemas/UpuMac'

securedPacket:

$ref: '#/components/schemas/SecuredPacket'

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

SorInfo:

type: object

properties:

steeringContainer:

$ref: '#/components/schemas/SteeringContainer'

ackInd:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/AckInd'

sorMacIausf:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SorMac'

countersor:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/CounterSor'

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- ackInd

- provisioningTime

SharedDataIds:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

minItems: 1

uniqueItems: true

UpuInfo:

type: object

properties:

upuDataList:

type: array

items:

$ref: 'TS29509_Nausf_UPUProtection.yaml#/components/schemas/UpuData'

minItems: 1

upuRegInd:

$ref: '#/components/schemas/UpuRegInd'

upuAckInd:

$ref: 'TS29509_Nausf_UPUProtection.yaml#/components/schemas/UpuAckInd'

upuMacIausf:

$ref: 'TS29509_Nausf_UPUProtection.yaml#/components/schemas/UpuMac'

counterUpu:

$ref: 'TS29509_Nausf_UPUProtection.yaml#/components/schemas/CounterUpu'

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- upuDataList

- upuAckInd

- upuRegInd

- provisioningTime

SharedData:

type: object

required:

- sharedDataId

properties:

sharedDataId:

$ref: '#/components/schemas/SharedDataId'

sharedAmData:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

sharedSmsSubsData:

$ref: '#/components/schemas/SmsSubscriptionData'

sharedSmsMngSubsData:

$ref: '#/components/schemas/SmsManagementSubscriptionData'

sharedDnnConfigurations:

type: object

additionalProperties:

$ref: '#/components/schemas/DnnConfiguration'

sharedTraceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

sharedSnssaiInfos:

type: object

additionalProperties:

$ref: '#/components/schemas/SnssaiInfo'

TraceDataResponse:

type: object

properties:

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

sharedTraceDataId:

$ref: '#/components/schemas/SharedDataId'

SteeringContainer:

oneOf:

- type: array

items:

$ref: 'TS29509_Nausf_SoRProtection.yaml#/components/schemas/SteeringInfo'

minItems: 1

- $ref: '#/components/schemas/SecuredPacket'

GroupIdentifiers:

type: object

properties:

extGroupId:

$ref: '#/components/schemas/ExtGroupId'

intGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

VnGroupData:

type: object

properties:

pduSessionTypes:

$ref: '#/components/schemas/PduSessionTypes'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

singleNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

appDescriptors:

type: array

items:

$ref: '#/components/schemas/AppDescriptor'

minItems: 1

AppDescriptor:

type: object

properties:

osId:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/OsId'

appId:

type: string

AdditionalSnssaiData:

type: object

properties:

requiredAuthnAuthz:

type: boolean

support3Gpp:

type: boolean

supportNon3Gpp:

type: boolean

AppPortId:

type: object

properties:

destinationPort:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint16'

originatorPort:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint16'

FrameRouteInfo:

type: object

properties:

ipv4Mask:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4AddrMask'

ipv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

# SIMPLE TYPES:

UeUsageType:

type: integer

MpsPriorityIndicator:

type: boolean

McsPriorityIndicator:

type: boolean

DnnIndicator:

type: boolean

LboRoamingAllowed:

type: boolean

SmsSubscribed:

type: boolean

3GppChargingCharacteristics:

type: string

DlPacketCount:

type: integer

minimum: -1

MicoAllowed:

type: boolean

SharedDataId:

type: string

pattern: '^[0-9]{5,6}-.+$'

IwkEpsInd:

type: boolean

SecuredPacket:

type: string

format: base64

UpuRegInd:

type: boolean

ExtGroupId:

type: string

pattern: '^extgroupid-[^@]+@[^@]+$'

NbIoTUePriority:

type: integer

# ENUMS:

DataSetName:

anyOf:

- type: string

enum:

- AM

- SMF_SEL

- UEC_SMF

- UEC_SMSF

- SMS_SUB

- SM

- TRACE

- SMS_MNG

- type: string

PduSessionContinuityInd:

anyOf:

- type: string

enum:

- MAINTAIN_PDUSESSION

- RECONNECT_PDUSESSION

- RELEASE_PDUSESSION

- type: string

********************The end of changes********************