3GPP TSG-CT Meeting #86	CP-193225

# Sitges, SPAIN; 09th – 10th December 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.571 | CR | 0159 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Wireline Service Area Restrictions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CableLabs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson, CableLabs |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-11-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Service Area Restrictions for wireline access are defined in TS 23.316, 4.3.3.3 The subscription data in the UDM for a 5G-BRG includes a Service Area Restriction which may contain either Allowed or Non-Allowed Areas specified by using explicit Line IDs (and corresponding operator identifiers) and/or other geographical information (e.g., longitude/latitude, zip code, etc.). The subscription data in the UDM for a 5G-CRG and FN-CRG includes a Service Area Restriction which may contain either Allowed or Non-Allowed Areas specified by using explicit HFC Node IDs and/or other geographical information (e.g., longitude/latitude, zip code, etc.). Since the representation of Line IDs, and HFC Node Ids, in the Area data type cannot be done in a backwards compatible way without increasing its complexity by defining a Supported Feature, it is proposed to define a new data type WirelineArea. In the same way, to avoid increasing the complexity of the ServiceAreaDescription data type, it is proposed to define a new data type for wireline access service area restrictions, named WirelineServiceAreaRestriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of WirelineArea and WirelineServiceAreaRestriction data types |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wireline Service Area Restrictions cannot be defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4.x1 (new), 5.4.4.x2 (new), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible new features to the OpenAPI file for TS29571_CommonData API |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changes in relation to 5236 are: - Adding CableLabs as co-author of the document. Changes in relation to 5364 are: - Correction of table 5.4.4.x1-1: hfNId attribute cardinality and hfNId definition in the NOTE. - Correction of table 5.4.4.x2-1: NOTE 1 text is updated to indicate that the "restrictionType" attribute and the "areas" attribute shall be either both present or absent. C4-195453 agreed in CT4#95 is revised to: - Replace in the OpenAPI file the attribute gobalLineId in WirelineArea type to globLineId to align it with the globLineId name used in table 5.4.4.x1. - Indicate the GlobalLineId data type is FFS - Define GlobalLineId type as string in the OpenAPI file as interim solution. |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 5.4.4.x1	Type: WirelineArea

Table 5.4.4.x1-1: Definition of type WirelineArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| globLineIds | array(GlobalLineId) | C | 1..N | List of Global Line Identifiers over NGAP. It shall be present if the 5G-BRG is accessing via wireline access network. |  |
| hfcNIds | array(HfcNId) | C | 1..N | List of HFC Node Ids. It shall be present if the 5G-CRG/FN-CRG is accessing via wireline access network. |  |
| areaCodeB | AreaCode | C | 0..1 | Area Code for for 5G-BRG accessing via wireline access network |  |
| areaCodeC | AreaCode | C | 0..1 | Area Code for 5G-CRG/FN-CRG is accessing via wireline access network |  |
| NOTE: One and only one of the "globLineIds", "hfcNIds", "areaCodeB" and "areaCodeC” attributes shall be included in a WirelineArea data structure. |  |  |  |  |  |

Editor's note:	The GlobalLineId data type is FFS.

# *** 2nd Change ***

#### 5.4.4.x2	Type: WirelineServiceAreaRestriction

Table 5.4.4.x2-1: Definition of type WirelineServiceAreaRestriction

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| restrictionType | RestrictionType | C | 0..1 | string "ALLOWED_AREAS" or "NOT_ALLOWED_AREAS" (NOTE 1) |
| areas | array(WirelineArea) | C | 0..N | A list of Areas. These areas are: - allowed areas if RestrictionType is "ALLOWED_AREAS" - not allowed areas if RestrictionType is "NOT_ALLOWED_AREAS" (NOTE 1) (NOTE 2) |
| NOTE 1: The "restrictionType" attribute and the "areas" attribute shall be either both present or absent. NOTE 2: The empty array is used when service is allowed/restricted nowhere. |  |  |  |  |

# *** 3rd Change ***

## A.2	Data related to Common Data Types

openapi: 3.0.0

info:

version: '1.2.0.alpha-2'

title: 'Common Data Types'

description: |

Common Data Types for Service Based Interfaces.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.571 Common Data Types for Service Based Interfaces, version 16.1.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.571/'

[…]

Area:

type: object

oneOf:

- required:

- tacs

- required:

- areaCode

properties:

tacs:

type: array

items:

$ref: '#/components/schemas/Tac'

minItems: 1

areaCode:

$ref: '#/components/schemas/AreaCode'

ServiceAreaRestriction:

type: object

properties:

restrictionType:

$ref: '#/components/schemas/RestrictionType'

areas:

type: array

items:

$ref: '#/components/schemas/Area'

maxNumOfTAs:

$ref: '#/components/schemas/Uinteger'

maxNumOfTAsForNotAllowedAreas:

$ref: '#/components/schemas/Uinteger'

allOf:

#

# 1st condition: restrictionType and areas attributes shall be either both absent

#                or both present

#

- oneOf:

- not:

required: [ restrictionType ]

- required: [ areas ]

#

# 2nd condition: if restrictionType takes value NOT_ALLOWED_AREAS,

#                then maxNumOfTAs shall be absent

#

- anyOf:

- not:

required: [ restrictionType ]

properties:

restrictionType:

type: string

enum: [ NOT_ALLOWED_AREAS ]

- not:

required: [ maxNumOfTAs ]

#

# 3rd condition: if restrictionType takes value ALLOWED_AREAS,

#                then maxNumOfTAsForNotAllowedAreas shall be absent

#

- anyOf:

- not:

required: [ restrictionType ]

properties:

restrictionType:

type: string

enum: [ ALLOWED_AREAS ]

- not:

required: [ maxNumOfTAsForNotAllowedAreas ]

WirelineArea:

type: object

properties:

globLineIds:

type: array

items:

$ref: '#/components/schemas/GlobalLineId'

minItems: 1

hfcNIds:

type: array

items:

$ref: '#/components/schemas/HfcNId'

minItems: 1

areaCodeB:

$ref: '#/components/schemas/AreaCode'

areaCodeC:

$ref: '#/components/schemas/AreaCode'

WirelineServiceAreaRestriction:

type: object

properties:

restrictionType:

$ref: '#/components/schemas/RestrictionType'

areas:

type: array

items:

$ref: '#/components/schemas/WirelineArea'

# EDITOR’S NOTE: The GlobalLineId definition is FFS.

#                The string type defined below is only an interim solution.

GlobalLineId:

type: string

PresenceInfo:

type: object

properties:

praId:

type: string

presenceState:

$ref: '#/components/schemas/PresenceState'

trackingAreaList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 1

ecgiList:

type: array

items:

$ref: '#/components/schemas/Ecgi'

minItems: 1

ncgiList:

type: array

items:

$ref: '#/components/schemas/Ncgi'

minItems: 1

globalRanNodeIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

minItems: 1

PresenceInfoRm:

type: object

properties:

praId:

type: string

presenceState:

$ref: '#/components/schemas/PresenceState'

trackingAreaList:

type: array

items:

$ref: '#/components/schemas/Tai'

minItems: 0

ecgiList:

type: array

items:

$ref: '#/components/schemas/Ecgi'

minItems: 0

ncgiList:

type: array

items:

$ref: '#/components/schemas/Ncgi'

minItems: 0

globalRanNodeIdList:

type: array

items:

$ref: '#/components/schemas/GlobalRanNodeId'

nullable: true

[…]

*** End of Changes ***