3GPP TSG-CT WG3 Meeting #108-e	C3-201085

# E-Meeting, 19th -28th  February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.523 | CR | 0018 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DNN Clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | en5GPccSer |  |  |  |  |  | Date: |  |  | 2020-01-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DNN is defined as common data type, will be used by several APIs, and it is clarified whether the Dnn data type contains just the DNN Network Identifier, or the Network Identifier plus the Operator Identifier, shall be documented in each API where this data type is used. We propose that DNN over N5 can be a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. It is consistent with the N5 and N7 interfaces. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Both NI and OI are included, or only NI is included in the DNN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear encoding definition of DNN may lead to inter-operation issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2.2, 5.6.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not change the open API. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Begin of Change * * * *

#### 5.6.2.2	Type PcEventExposureSubsc

Table 5.6.2.2-1: Definition of type PcEventExposureSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventSubs | array(PcEvent) | M | 1..N | Subscribed Policy Control events. |  |
| eventsRepInfo | ReportingInformation | O | 1 | Represents the reporting requirements of the subscription. |  |
| groupId | GroupId | C | 0..1 | Represents an internal group identifier and identifies a group of UEs. It shall be present when the subscription is targeting a Group of UE(s). |  |
| filterDnns | array(Dnn) | O | 1..N | Represents the DNNs for which the policy event report shall apply. Each DNN is a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only. If omitted it represents any DNN. |  |
| filterSnssais | array(Snssai) | O | 1..N | Represents the S-NSSAIs for which the policy event report shall apply. If omitted it represents any S-NSSAI. |  |
| filterServices | array(ServiceIdentification) | O | 1..N | Represents the services for which the policy event report shall apply. If omitted, the policy event report shall apply for all the active services. | ExtendedSessionInformation |
| notifUri | Uri | M | 1 | Notification URI for Policy Control event reporting. |  |
| notifId | string | M | 1 | Notification Correlation ID assigned by the NF service consumer. |  |
| suppFeat | SupportedFeatures | M | 1 | This IE represents a list of Supported features used as described in subclause 5.8. In the HTTP POST request it represents the set of NF service consumer supported features. In the HTTP POST and GET responses it represents the set of PCF Event Exposure agreeable supported features. |  |

* * * * Next Change * * * *

#### 5.6.2.9	Type PduSessionInformation

Table 5.6.2.9-1: Definition of type PduSessionInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| snssai | Snssai | M | 1 | S-NSSAI of the PDU session. | ExtendedSessionInformation |
| dnn | Dnn | M | 1..N | Dnn of the PDU session, a full DNN with both the Network Identifier and Operator Identifier, or a DNN with the Network Identifier only.. | ExtendedSessionInformation |
| ueIpv4 | Ipv4Addr | C | 0..1 | The IPv4 address of the served UE. (NOTE 1) | ExtendedSessionInformation |
| ueIpv6 | Ipv6Prefix | C | 0..1 | The IPv6 prefix of the served UE. (NOTE 1) | ExtendedSessionInformation |
| ipDomain | string | O | 0..1 | Identifies the IP domain. (NOTE 2) | ExtendedSessionInformation |
| ueMac | MacAddr48 | C | 0..1 | UE MAC address. (NOTE 1) | ExtendedSessionInformation |
| NOTE 1: Either the served UE IP address (an Ipv4Addr or Ipv6Prefix or both if available) or UE MAC address shall be present. NOTE 2: An "ipDomain" attribute, may be provided in combination with a "ueIpv4" attribute. |  |  |  |  |  |

* * * * End of Change * * * *