3GPP TSG-CT WG4 Meeting #97e	C4-202118

# E-Meeting, 15th – 24th April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.562 | CR | 0007 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource-Level Authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMS5G_SBA |  |  |  |  |  | Date: |  |  | 2020-03-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA3 has introduced in TS 33.501 the functionality that allows an NF Service Producer to enforce Oauth2-based authorization per resource and operation level. This allows the service producer to specify a number of additional scopes (on top of the existing scope containing the service name); these additional scopes are standardized on the OpenAPI specification of each API, and the service producer indicates in its NF Profile which of these scopes can be granted to different NF Service Consumers (based on their NF Type or their NF Instance ID). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add additional scopes for each resource and operation in the OpenAPI definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The capability of Oauth2-based resource-level authorization in not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.x (new), 6.2.x (new), 6.3.x (new), A.2, A.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards-compatible new features, with impacts on the following APIs: - TS29562_Nhss_imsUECM.yaml - TS29562_Nhss_imsSDM.yaml - TS29562_Nhss_imsUEAU.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 29.335: "User Data Repository Access Protocol over the Ud interface; Stage 3".

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	OpenAPI Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md

[10]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	IETF RFC 7807: "Problem Details for HTTP APIs".

[12]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[13]	3GPP TS 23.003: "Numbering, addressing and identification".

[14]	3GPP TS 33.203: "Access security for IP-based services".

[15]	3GPP TS 29.503: "Unified Data Management Services; Stage 3".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[17] 	IETF RFC 4740: "Diameter Session Initiation Protocol (SIP) Application".

[18]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[19]	IETF RFC 4412: "Communications Resource Priority for the Session Initiation Protocol".

[20]	3GPP TS 29.228: " IP Multimedia (IM) Subsystem Cx and Dx interfaces;Signalling flows and message contents".

[21]	3GPP TS 29.218: " IP Multimedia (IM) session handling; IM call model; Stage 2".

[22]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[23]	IETF RFC 8497: "Marking SIP messages to be logged".

[24]	3GPP TS 24.323: "3GPP IP Multimedia Subsystem (IMS) service level tracing Management Object (MO)".

[25]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[26]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[27]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[28]	IETF RFC 2045: "Multipurpose Internet Mail Extensions(MIME) Part One: Format of Internet Message Bodies".

[29]	ETSI 083 034: "Network Attachment Sub-System (NASS); e4 interface based on the DIAMETER protocol".

[30]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[bb]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[cc]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

* * * Next Change * * * *

### 6.1.x	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_imsUECM API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [bb]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [cc]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_imsUECM API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [cc], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_imsUECM service.

The Nhss_imsUECM API defines the following scopes for OAuth2 authorization:

Table 6.1.x-1: Oauth2 scopes defined in Nhss_imsUECM API

| Scope | Description |
| --- | --- |
| "nhss-ims-uecm" | Access to the Nhss IMS UE Context Management API |
| "nhss-ims-uecm:authorize:invoke" | Access to invoke the Authorize custom operation |
| "nhss-ims-uecm:registration:create" | Access to create S-CSCF Registration resources |
| "nhss-ims-uecm:restoration:read" | Access to read the S-CSCF Restoration resource |
| "nhss-ims-uecm:restoration:modify" | Access to create/update/delete the S-CSCF Restoration resource |

* * * Next Change * * * *

### 6.2.x	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_imsSDM API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [bb]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [cc]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_imsSDM API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [cc], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_imsSDM service.

The Nhss_imsSDM API defines the following scopes for OAuth2 authorization:

Table 6.2.x-1: Oauth2 scopes defined in Nhss_imsSDM API

| Scope | Description |
| --- | --- |
| "nhss-ims-sdm" | Access to the Nhss IMS Subscription Data Management API |
| "nhss-ims-sdm:registration-status:read" | Access to read the Registration Status resource |
| "nhss-ims-sdm:profile-data:read" | Access to read the Profile Data resource |
| "nhss-ims-sdm:priority-levels:read" | Access to read the Priority Levels resource |
| "nhss-ims-sdm:ifcs:read" | Access to read the Initial Filter Criteria resource |
| "nhss-ims-sdm:service-level-trace-information:read" | Access to read the Service Level Trace Information resource |
| "nhss-ims-sdm:server-name:read" | Access to read the Server Name resource |
| "nhss-ims-sdm:scscf-capabilities:read" | Access to read the S-CSCF Capabilities resource |
| "nhss-ims-sdm:ps-domain:location-data:read" | Access to read the PS-Domain Location Data resource |
| "nhss-ims-sdm:cs-domain:location-data:read" | Access to read the CS-Domain Location Data resource |
| "nhss-ims-sdm:repository-data:modify" | Access to create/update/delete the Repository Data resource |
| "nhss-ims-sdm:repository-data:read" | Access to read the Repository Data resource |
| "nhss-ims-sdm:identities:read" | Access to read the Identities resource |
| "nhss-ims-sdm:srvcc:read" | Access to read the SRVCC resource |
| "nhss-ims-sdm:srvcc:modify" | Acess to update the SRVCC resource |
| "nhss-ims-sdm:psi-status:read" | Access to read the PSI Status resource |
| "nhss-ims-sdm:psi-status:modify" | Acess to update the PSI Status resource |
| "nhss-ims-sdm:subscriptions:create" | Access to create Subscriptions resources |
| "nhss-ims-sdm:subscriptions:modify" | Access to update/delete a Subscription resource |
| "nhss-ims-sdm:shared-subscriptions:create" | Access to create a Shared-Data Subscriptions resource |
| "nhss-ims-sdm:shared-subscriptions:modify" | Access to update/delete a Shared-Data Subscription resource |
| "nhss-ims-sdm:shared-subscriptions:read" | Access to read the Shared-Data resource |

* * * Next Change * * * *

### 6.3.x	Security

As indicated in 3GPP TS 33.501 [5], the access to the Nhss_imsUEAU API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [bb]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [cc]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nhss_imsUEAU API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [cc], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nhss_imsUEAU service.

The Nhss_imsUEAU API defines the following scopes for OAuth2 authorization:

Table 6.3.x-1: Oauth2 scopes defined in Nhss_imsUEAU API

| Scope | Description |
| --- | --- |
| "nhss-ims-ueau" | Access to the Nhss IMS UE Context Management API |
| "nhss-ims-ueau:generate-sip-auth-data:invoke" | Access to invoke the Generate SIP Auth Data custom method |

* * * Next Change * * * *

## A.2	Nhss_imsUECM API

openapi: 3.0.0

info:

version: '1.0.0.alpha-2'

title: 'Nhss_imsUECM'

description: |

Nhss UE Context Management Service for IMS.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Unified Data Management Services, version 16.0.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-uecm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

paths:

/{impu}/authorize:

post:

summary: Authorize IMS Identities to register in the network or establish multimedia sessions and return CSCF location if it is stored

operationId: Authorize

tags:

- Authorize Ims identities

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:authorize:invoke

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizationRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AuthorizationResponse'

'400':

description: Bad Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'500':

description: Internal Server Error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

description: Unexpected error

callbacks:

deregistrationeNotification:

'{request.body#/deregCallbackUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DeregistrationData'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/scscf-registration:

put:

summary: SCSCF registration information

operationId: SCSCF registration

tags:

- SCSCF registration and deregistration

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:registration:create

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRegistration'

'204':

description: No content

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: '#/components/schemas/ExtendedProblemDetails'

'404':

description: Not found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

description: Unexpected error

/{impu}/scscf-registration/scscf-restoration-info:

put:

summary: Update the S-CSCF restoration information of the UE

operationId: UpdateScscfRestorationInfo

tags:

- Update the S-CSCF restoration information of the UE

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:modify

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoRequest'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss_imsUECM/v1/{impu}/scscf-registration/scscf-restoration-info'

required: true

schema:

type: string

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

'204':

description: No content

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

get:

summary: Retrieve the S-CSCF restoration

operationId: GetScscfRestorationInfo

tags:

- S-CSCF Restoration Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:read

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfRestorationInfoResponse'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

delete:

summary: Delete the S-CSCF restoration

operationId: DeleteScscfRestorationInfo

tags:

- S-CSCF Restoration Info Delete

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-uecm

- oAuth2ClientCredentials:

- nhss-ims-uecm

- nhss-ims-uecm:restoration:modify

parameters:

- name: impu

in: path

description: Public identity of the user.

required: true

schema:

$ref: '#/components/schemas/Impu'

responses:

'204':

description: No content

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-uecm: Access to the Nhss IMS UE Context Management API

nhss-ims-uecm:authorize:invoke: Access to invoke the Authorize custom operation

nhss-ims-uecm:registration:create: Access to create S-CSCF Registration resources

nhss-ims-uecm:restoration:read: Access to read the S-CSCF Restoration resource

nhss-ims-uecm:restoration:modify: Access to create/update/delete the S-CSCF Restoration resource

schemas:

(… text not shown for clarity …)

* * * Next Change * * * *

## A.3	Nhss_imsSDM API

openapi: 3.0.0

info:

version: '1.0.0.alpha-2'

title: 'Nhss_imsSDM'

description: |

Nhss Subscriber Data Management Service for IMS.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 Unified Data Management Services, version 16.0.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-sdm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

paths:

/{imsUeId}/ims-data/registration-status:

get:

summary: Retrieve the registration status of a user

operationId: GetRegistrationStatus

tags:

- Registration Status retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:registration-status:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsRegistrationStatus'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/profile-data:

get:

summary: Retrieve the complete IMS profile for a given IMS public identity (and public identities in the same IRS)

operationId: GetProfileData

tags:

- IMS Profile Data Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:profile-data:read

parameters:

- name: imsUeId

in: path

description: IMS Identity. In this case it shall be an IMS public identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsProfileData'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/priority-levels:

get:

summary: Retrieve the service priority levels associated to the user

operationId: GetPriorityInfo

tags:

- Priority Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:priority-levels:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PriorityLevels'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/ifcs:

get:

summary: Retrieve the Initial Filter Criteria for the associated IMS subscription

operationId: GetIfcs

tags:

- IFCs Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ifcs:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Ifcs'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/profile-data/service-level-trace-information:

get:

summary: Retrieve the IMS service level trace information for the associated user

operationId: GetServiceTraceInfo

tags:

- Service Trace Info Retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:service-level-trace-information:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceLevelTraceInformation'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/location-data/server-name:

get:

summary: Retrieve the server name for the associated user

operationId: GetSserverName

tags:

- Server Name retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:server-name:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsLocationData'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/ims-data/location-data/scscf-capabilities:

get:

summary: Retrieve the S-CSCF capabilities for the associated IMS subscription

operationId: GetScscfCapabilities

tags:

- Retrieval of the S-CSCF capabilities for the IMS subscription

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:scscf-capabilities:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ScscfCapabilityList'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/ps-domain/location-data:

get:

summary: Retrieve the location data in PS domain

operationId: GetLocPsDomain

tags:

- PS location retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:ps-domain:location-data:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: requestedNodes

in: query

description: Indicates the serving node(s) for which the request is applicable.

schema:

$ref: '#/components/schemas/RequestedNodes'

- name: servingNode

in: query

description: Indicates that only the stored NF id/address of the serving node(s) is required

schema:

$ref: '#/components/schemas/ServingNode'

- name: localTime

in: query

description: Indicates that only the Local Time Zone information of the location in the visited network where the UE is attached is requested

schema:

$ref: '#/components/schemas/LocalTime'

- name: currentLocation

in: query

description: Indicates whether an active location retrieval has to be initiated by the requested node

schema:

$ref: '#/components/schemas/CurrentLocation'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PsLocation'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/access-data/cs-domain/location-data:

get:

summary: Retrieve the location data in CS domain

operationId: GetLocCsDomain

tags:

- CS location retrieval

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:cs-domain:location-data:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: servingNode

in: query

description: Indicates that only the stored NF id/address of the serving node(s) is required

schema:

$ref: '#/components/schemas/ServingNode'

- name: localTime

in: query

description: Indicates that only the Local Time Zone information of the location in the visited network where the UE is attached is requested

schema:

$ref: '#/components/schemas/LocalTime'

- name: currentLocation

in: query

description: Indicates whether an active location retrieval has to be initiated by the requested node

schema:

$ref: '#/components/schemas/CurrentLocation'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CsLocation'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/repository-data/{serviceIndication}:

put:

summary: Update the repository data associated to an IMPU and service indication

operationId: UpdateRepositoryDataServInd

tags:

- Update repository data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:modify

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serviceIndication

in: path

description: Identifier of a service related data

required: true

schema:

$ref: '#/components/schemas/ServiceIndication'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

required: true

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-sdm/v1/{imsUeId}/repository-data/{serviceIndication}'

required: true

schema:

type: string

'200':

description: OK

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

'204':

description: No content

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'409':

$ref: 'TS29571_CommonData.yaml#/components/responses/409'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

delete:

summary: delete the Repository Data for a Service Indication

operationId: DeleteRepositoryDataServInd

tags:

- Delete repository data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:modify

parameters:

- name: imsUeId

in: path

description: Identifier of the UE

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serviceIndication

in: path

description: Identifier of a service related data

required: true

schema:

$ref: '#/components/schemas/ServiceIndication'

responses:

'204':

description: Expected response to a valid request

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

get:

summary: Retrieve the repository data associated to an IMPU and service indication

operationId: GetRepositoryDataServInd

tags:

- Repository data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:repository-data:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: serviceIndication

in: path

description: Identifier of a service related data

required: true

schema:

$ref: '#/components/schemas/ServiceIndication'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/RepositoryData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/identities/msisdns:

get:

summary: retrieve the Msisdns associated to requested identity

operationId: GetMsisdns

tags:

- Retrieval of the associated Msisdns

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:identities:read

parameters:

- name: imsUeId

in: path

description: IMS Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: privateId

in: query

description: Private identity

schema:

$ref: '#/components/schemas/PrivateId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/MsisdnList'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/identities/ims-associated-identities:

get:

summary: Retrieve the associated identities to the IMS public identity included in the service request

operationId: GetImsAssocIds

tags:

- Retrieval of associated IMS public identities

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:identities:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PublicIdentities'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/{imsUeId}/srvcc-data:

get:

summary: Retrieve the srvcc data

operationId: GetSrvccData

tags:

- Retrieval of UE SRVCC capability and STN-SR

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:srvcc:read

parameters:

- name: imsUeId

in: path

description: IMS Public Identity or IMS Private Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SrvccData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Patch

operationId: UpdateSrvccData

tags:

- Update SRVCC data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:srvcc:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity or IMS Private Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful response

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

description: Unexpected error

/{imsUeId}/service-data/psi-status:

get:

summary: Retrieve the PSI activation state data

operationId: GetPsiState

tags:

- Retrieval of PSI activation state

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:psi-status:read

parameters:

- name: imsUeId

in: path

description: IMS Private Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/PsiActivationState'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch:

summary: Patch

operationId: UpdatePsiState

tags:

- Update PSI state data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:psi-status:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Service Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

'204':

description: Successful response

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

description: Unexpected error

/{imsUeId}/subscriptions:

post:

summary: subscribe to notifications

operationId: ImdSdmSubscribe

tags:

- SDM Subscription Creation

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:subscriptions:create

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-sdm/<apiVersion>/{imsUeId}/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

callbacks:

datachangeNotification:

'{request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{imsUeId}/subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications

operationId: ImsSdmUnsubscribe

tags:

- SDM Subscription Deletion

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:subscription:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: modify the subscription

operationId: ImsSdmSubsModify

tags:

- SDM Subscription Modification

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:subscription:modify

parameters:

- name: imsUeId

in: path

description: IMS Public Identity

required: true

schema:

$ref: '#/components/schemas/ImsUeId'

- name: subscriptionId

in: path

description: Id of the Subscription

required: true

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

'204':

description: Successful modification

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/shared-data-subscriptions:

post:

summary: subscribe to notifications for shared data

operationId: SubscribeToSharedData

tags:

- Subscription Creation for shared data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-subscriptions:create

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

required: true

responses:

'201':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nhss-ims-dm/<apiVersion>/shared-data-subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

default:

description: Unexpected error

callbacks:

datachangeNotification:

'{request.body#/callbackReference}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ModificationNotification'

responses:

'204':

description: Successful Notification response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/shared-data-subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications for shared data

operationId: UnsubscribeForSharedData

tags:

- Subscription Deletion for shared data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-subscription:modify

parameters:

- name: subscriptionId

in: path

description: Id of the Shared data Subscription

required: true

schema:

type: string

responses:

'204':

description: Successful response

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

patch:

summary: modify the subscription

operationId: ModifySharedDataSubs

tags:

- Subscription Modification

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-subscription:modify

parameters:

- name: subscriptionId

in: path

description: Id of the SDM Subscription

required: true

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/ImsSdmSubscription'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/shared-data:

get:

summary: retrieve shared data

operationId: GetSharedData

tags:

- Retrieval of shared data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-sdm

- oAuth2ClientCredentials:

- nhss-ims-sdm

- nhss-ims-sdm:shared-data:read

parameters:

- name: shared-data-ids

in: query

description: List of shared data ids

required: true

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/SharedDataId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/SharedData'

minItems: 1

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-sdm: Access to the Nhss IMS Subscription Data Management API

nhss-ims-sdm:registration-status:read: Access to read the Registration Status resource

nhss-ims-sdm:profile-data:read: Access to read the Profile Data resource

nhss-ims-sdm:priority-levels:read: Access to read the Priority Levels resource

nhss-ims-sdm:ifcs:read: Access to read the Initial Filter Criteria resource

nhss-ims-sdm:service-level-trace-information:read: Access to read the Service Level Trace Information resource

nhss-ims-sdm:server-name:read: Access to read the Server Name resource

nhss-ims-sdm:scscf-capabilities:read: Access to read the S-CSCF Capabilities resource

nhss-ims-sdm:ps-domain:location-data:read: Access to read the PS-Domain Location Data resource

nhss-ims-sdm:cs-domain:location-data:read: Access to read the CS-Domain Location Data resource

nhss-ims-sdm:repository-data:modify: Access to create/update/delete the Repository Data resource

nhss-ims-sdm:repository-data:read: Access to read the Repository Data resource

nhss-ims-sdm:identities:read: Access to read the Identities resource

nhss-ims-sdm:srvcc:read: Access to read the SRVCC resource

nhss-ims-sdm:srvcc:modify: Acess to update the SRVCC resource

nhss-ims-sdm:psi-status:read: Access to read the PSI Status resource

nhss-ims-sdm:psi-status:modify: Acess to update the PSI Status resource

nhss-ims-sdm:subscriptions:create: Access to create Subscriptions resources

nhss-ims-sdm:subscription:modify: Access to update/delete a Subscription resource

nhss-ims-sdm:shared-subscriptions:create: Access to create a Shared-Data Subscriptions resource

nhss-ims-sdm:shared-subscription:modify: Access to update/delete a Shared-Data Subscription resource

nhss-ims-sdm:shared-data:read: Access to read the Shared-Data resource

schemas:

(… text not shown for clarity …)

* * * Next Change * * * *

## A.4	Nhss_imsUEAU API

openapi: 3.0.0

info:

version: '1.0.0.alpha-2'

title: 'Nhss_imsUEAU'

description: |

Nhss UE Authentication Service for IMS.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.562 HSS Services for Interworking with IMS, version 16.0.0

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.562/'

servers:

- url: '{apiRoot}/nhss-ims-ueau/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ueau

paths:

/{impi}/security-information/generate-sip-auth-data:

post:

summary: Generate authentication data for the UE based on the Auth-Scheme provided

operationId: GenerateSipAuthData

tags:

- Generate Auth Data

security:

- {}

- oAuth2ClientCredentials:

- nhss-ims-ueau

- oAuth2ClientCredentials:

- nhss-ims-ueau

- nhss-ims-ueau:generate-sip-auth-data:invoke

parameters:

- name: impi

in: path

description: IMS Private Identity for the UE (IMPI)

required: true

schema:

$ref: '#/components/schemas/Impi'

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/SipAuthenticationInfoRequest'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/SipAuthenticationInfoResult'

'400':

description: Bad Request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'405':

$ref: 'TS29571_CommonData.yaml#/components/responses/405'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

description: Not implemented

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

description: Gateway timeout

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nhss-ims-ueau: Access to the Nhss IMS UE Authentication API

nhss-ims-ueau:generate-sip-auth-data:invoke: Access to invoke the Generate SIP Auth Data custom operation

schemas:

(… text not shown for clarity …)

* * * End of Changes * * * *